S I M P L Y    L A B E L S

Version 2.2

Copyright 1989

By

Robert E. Pitcher

P.O. Box 877

Windsor, Nfld.

Canada, AOH 2HO

Table of Contents

1.0 - Introduction . . . . . . . . . . . . . . . . . . . . .   1

2.0 - Who May Use This Software  . . . . . . . . . . . . . .   2

Warranty . . . . . . . . . . . . . . . . . . . . . . .   3

3.0 - Getting Started  . . . . . . . . . . . . . . . . . . .   3

4.0 - Conventions  . . . . . . . . . . . . . . . . . . . . .   5

5.0 - Using Simply Labels  . . . . . . . . . . . . . . . . .   6

5.1 - Defining Label Formats . . . . . . . . . . . . . . . .   6

5.1.1 - Creating New Formats  . . . . . . . . . . . . . .   7

5.1.2 - Editing Current Formats . . . . . . . . . . . . .   7

Size  . . . . . . . . . . . . . . . . . . . . . . . .   7

Description . . . . . . . . . . . . . . . . . . . . .   8

Background Text . . . . . . . . . . . . . . . . . . .   8

Print . . . . . . . . . . . . . . . . . . . . . . . .   8

Typeface  . . . . . . . . . . . . . . . . . . . . . .   9

5.1.3 - Deleting a Format . . . . . . . . . . . . . . . .   9

5.1.4 - Saving Your Formats . . . . . . . . . . . . . . .   9

5.2 - Creating New Label Files . . . . . . . . . . . . . . .   9

5.3 - Using Your Label Files . . . . . . . . . . . . . . . .   10

5.3.1 - Loading a Label File  . . . . . . . . . . . . . .   10

5.3.2 - Adding Labels . . . . . . . . . . . . . . . . . .   11

5.3.3 - Browsing Around . . . . . . . . . . . . . . . . .   12

Searching for Labels  . . . . . . . . . . . . . . . .   12

Moving Around Your File . . . . . . . . . . . . . . .   14

Editing a Label . . . . . . . . . . . . . . . . . . .   15

Deleting a Label  . . . . . . . . . . . . . . . . . .   15

5.3.4 - Deleting Labels . . . . . . . . . . . . . . . . .   15

5.3.5 - Sorting your Files  . . . . . . . . . . . . . . .   15

5.3.6 - Importing Labels From Other Files . . . . . . . .   16

Label Files . . . . . . . . . . . . . . . . . . . . .   17

Column Files  . . . . . . . . . . . . . . . . . . . .   20

5.3.7 - Printing your Labels  . . . . . . . . . . . . . .   21

Selecting an Output Device  . . . . . . . . . . . . .   21

Printing a Group of Labels  . . . . . . . . . . . . .   22

Searching for Labels to Print . . . . . . . . . . . .   23

Print Functions . . . . . . . . . . . . . . . . . . .   23

Print Options . . . . . . . . . . . . . . . . . . . .   24

Label Format . . . . . . . . . . . . . . . . . . .   24

Envelope Format  . . . . . . . . . . . . . . . . .   26

Report Format  . . . . . . . . . . . . . . . . . .   28

Number of Copies . . . . . . . . . . . . . . . . .   29

Typeface   . . . . . . . . . . . . . . . . . . . .   30

5.4 - Saving your Labels . . . . . . . . . . . . . . . . . .   30

5.5 - Deleting Old Label Files . . . . . . . . . . . . . . .   30

6.0 - Setting Program Options  . . . . . . . . . . . . . . .   30

6.1 - General Settings . . . . . . . . . . . . . . . . . . .   31

6.2 - Printer Settings . . . . . . . . . . . . . . . . . . .   32

6.2.1 - Default Printer Settings  . . . . . . . . . . . .   34

7.0 - Potential Problems . . . . . . . . . . . . . . . . . .   35

8.0 - Technical Support  . . . . . . . . . . . . . . . . . .   36

9.0 - Registration Form  . . . . . . . . . . . . . . . . . .   37

ii

1.0 Introduction

Welcome to  Simply Labels.  While it is not necessary to read all

of this manual, it is recommended  that you  at least  read up to

the section  that explains  the conventions used in this program.

The documentation for Simply Labels can be read  like a  book but

is best  used as  a reference.   If you have problems while using

the program, just find the  topic  causing  the  problems  in the

table of contents and then read the section on that topic.

Simply Labels  started out  as a simple basic program written for

the Sanyo MBC 55x series computers.    It  was  written  in Sanyo

Basic and  had very  few features.   Despite  its problems people

still used  it  and  because  of  their  interest  version  2 was

written.   Sanyo users have not been forgotten with this version.

Simply Labels is now a compiled program and can be  configured to

run on  most MS-DOS  machines.   With version 2.2 the program has

been made easier to use and many new features have been added.

Many people have a very narrow view of  what makes  a label.   Of

course the  most common  type of  label is  the mailing label and

this program handles  them  with  ease.    But  Simply  Labels is

capable  of  so  much  more.    In  this  program  a  label  is a

rectangular area which can  contain text,  (background or normal)

and instructions on how to print any text contained in the label.

This information  is combined  to create  what is  called a label

format.

Simply Labels  allows you to define label formats and then create

files containing labels that  use  your  formats.    It  has many

advanced  features  such  as  searching and sorting your files in

many different  ways,  importing  labels  from  text  files, full

screen  label  editing,  printing  to  a  printer  or  disk file,

printing  multiple  copies  with   multiple   labels   per  line,

background label  text, graphic characters in your labels and the

ability to use all  of  your  printer's  capabilities.    In this

version  your  label  files  can  be  printed in two new formats.

Instead of the standard label  format  you  can  also  print your

label files in Envelope format or in a Report format.

Simply Labels  is powerful enough to meet the needs of most small

business and flexible enough  for the  many uses  at home.   When

used for  mailing labels  it can  handle over 400 labels per file

with up to 75 files per  disk or  sub directory.   It  can easily

transfer information  between label  files even  if the files use

different formats.  With  the ability  to use  graphic characters

and background  text you can design custom labels to meet most of

your needs.

1

Some types of labels that can be created with Simply Labels:

Mailing Labels           Labels for Book Reports

Recipe Cards             Labels for Preserves

Disk Labels              Labels for Part Bins

Name Tags                Inventory Cards

Cassette Jackets         VCR Labels

These are only sample ideas and you may find you have other types

of  labels  in  mine.    If you currently use several programs to

handle different types of  labels you  may be  surprised that one

program is capable of meeting all your label needs.

2.0 Who May Use This Software

This software  is not  free software  and it is not in the public

domain.  It is distributed  by  the  "ShareWare"  method  and all

rights  are  reserved  by  the  author.   Non-registered users of

Simply Labels are permitted to use the program  on a  trial basis

for the sole purpose of determining whether or not it meets their

needs.   Any and  all other  uses after  a trial  period of three

weeks  requires  registration.    To register your copy of Simply

Labels you may use the  registration  form  at  the  end  of this

document or use the file REGISTRA.LAB to print a form.

You are  encouraged to  copy Simply Labels and pass it on to your

friends or anyone else who would  like  a  copy  as  long  as the

following conditions are met.

Simply  Labels  must  be  distributed in a complete and

unmodified form including the following files.

SL.COM  -  Main program

SL.OVL  -  Program file

SL.DOC  -  Documentation for the program

SL.CNF  -  Program configuration

SL.DEF  -  Label Definitions

??.LAB  -  Sample Label Files

CODES.DOC  -  Keyboard codes for Graphic mode

No charge, other than  a  maximum  disk  coping  fee of

$5.00, may be charged for Simply Labels.

Simply  Labels  may  not  be distributed as part of any

other  product  without  written  permission  from  the

author.

This software was created using the Zbasic compiler.

Portions of this program are (C) Copyrighted, 1985 by Zedcor Inc.

2

Warranty

The author  makes no  warranty of any kind, expressed or implied,

as to the suitability of this  program to  perform any particular

purpose or  function.   The author  shall in no way be liable for

any damages, of any kind, including  lost savings,  lost profits,

or other  incidental or  consequential damages resulting from the

use of, or inability to use Simply Labels.  The  author shall not

be liable  for any  damage to data or property caused directly or

indirectly by the use of this program.

Every effort has been  made  to  insure  that  this  program will

perform the  task it  was designed  to do  and that it is free of

defects but you USE IT AT YOUR OWN RISK!

3.0 Getting Started

To use this program you must have the following equipment:

Personal Computer capable of running MS-DOS

MS-DOS or PC-DOS 2.11 or higher

Minimum of 256K of memory

A Printer

Setting up Simply Labels is very easy.  The first  thing you must

do is make a backup of your program disk.  If you do not know how

to do this  check  your  DOS  manual  and  look  up  the DISKCOPY

command.   If you  use a  hard disk you should create a directory

called LABELS and copy  all  files  to  this  directory  with the

following DOS commands.

MD\LABELS

COPY A:*.* C:\LABELS

If  you  have  problems  with  this  you should refer to your DOS

manuals.  If you use a  floppy system  you should  use the backup

disk and store the original in a safe place.

To run  Simply Labels  from a  hard disk  you must  first set the

default directory to the directory containing  the program files.

This is  necessary so  that the  program will be able to find the

configuration files it needs to run properly.   Use the following

command to do this:

CD\LABELS

To start the program type the following:

3

SL [B:FileName.LAB]

The  filename  is  optional  and  will tell the program to load a

label file you wish to work on.  The drive and extension are both

optional.   If the  drive is omitted the default drive set in the

program options will be used.  See the section on Setting Program

Options (6.0) for more details.

4

4.0 Conventions

The ESCape  key is  very popular in this program and will prevent

you from making many mistakes.  If you find yourself somewhere in

the  program  where  you  don't  want  to  be  you can back up by

pressing the ESC key.    This  will  work  anywhere  including at

Menus, Input  prompts and while printing.  If you press it enough

times you will usually end up back at the DOS prompt.

The Menus used in this program are very  simple and  easy to use.

The menus  are the wrap around type with all choices displayed on

a single line. There is also a line  just below  the menu  with a

description of  the highlighted  choice.  There are three ways to

make a menu choice.    First  you  may  use  the  cursor  keys to

highlight  a  choice  and  then  press  RETURN.  This will be the

preferred method until you are familiar with the  program because

it will  allow you  to read the descriptions for each choice. The

second method is to press  the  key  corresponding  to  the first

letter of the choice you wish to make.  The third and fastest way

is to use the function keys.

At the right of the screen two lines above the menu line you will

see a block which will help you see where you are in the program.

This block will show  the name  of the  parent menu.   The parent

menu is the menu one level above or where you just came from.

All  input  to  the  program  may  be edited by using the Insert,

Delete, Backspace, Home, End,  Left and  Right cursor  keys.  The

only exception to this is when the program is asking for a single

character response.  Upper or Lower  case  may  be  used  for all

input.   Depending on  the type of input asked for, some keys may

not be active.  For example  if  the  program  is  looking  for a

numeric entry  you will  not be able to type letters.  ESC may be

used at  any input  prompt to  exit and  usually return  you to a

menu.

When entering  file names  the program will convert your entry to

upper case characters.  Also do not add an extension  because the

program supplies  the proper  one.  One exception to this is when

importing text  files.    In  this  case  the  extension  must be

included and you will be asked for the full file name.

The bottom line of the screen is reserved for messages.  Whenever

you have problems look  at this  line for  help.   All errors are

also reported on this line.

5

5.0 Using Simply Labels

When  you  first  start  the  program a title screen will appear.

Press any key to continue after reading it.  What you see next is

the Main menu and it can be recognized by the words SIMPLY LABELS

printed in big letters in the  label window.  When you  start the

program with  a filename  on the command line, the file is loaded

and you go straight to the Use  menu  so  the  Main  menu  is not

displayed.

Main Menu

F1 Create  F2 Use  F3 Delete  F4 Options  F5 Formats  F6 Quit

There are  two main  functions in this program.  The first allows

you to define label formats and  the second  lets you  create and

work with your label files.

To get  familiar with  the program you may want to use one of the

sample label files that came with the program.  A label file is a

file  containing  a  group  of  labels that have the same format.

When a label file  is first  created there  are no  labels in the

file.   You have  to Use the file and Add the labels.  The number

of labels allowed in a file will depend on the  label format used

for  the  file  and  is  displayed  after the file is loaded into

memory.

5.1 Defining Label Formats

Selecting Formats  from the  Main menu  will bring  up the Format

menu.   From this  menu you can create custom label formats which

you will use in your label files.  A label  file must  use one of

the defined  formats.  A maximum of 25 formats are allowed and 12

standard label sizes are provided to get you started.   These may

be deleted or modified in any way you like.

A label  format consists  of the  size of  the label, the spacing

between labels, a description of  the  label  and  the background

text for  the label  as well  as instructions on how to print the

label.  The sizes allowed for labels are from 30 to 74 characters

wide by  2 to  15 lines high.  The description is just a name you

give to the format for your  own reference.   The  spacing is the

number of  lines skipped between each label and is normally 1 but

may be as many as 66.

6

The idea of background text may not be familiar to  you but  is a

very simple  idea.   Background text is text which will be common

to all labels in the file and cannot be written over.   It may or

may  not  be  used  depending  on  the application.  For standard

mailing labels  you would  not want  any background  text but for

cassette tape labels you would want to design boxes and areas for

side A and B.  Background text allows  you to  create custom fill

in the blank type labels.

Format Menu

F1 New  F2 Edit  F3 Delete  F4 Quit

5.1.1 Creating New Formats

To create a new label format you select New from the Format menu.

You will be asked for the  size, spacing  and description  of the

label.   After these  are entered  you will  go to  the Edit menu

where you can work on all parts of  your format  and print sample

labels on your printer to check your design.  See Editing Current

Formats below for information on editing formats.  When you leave

the Formats menu your formats are automatically saved.

5.1.2 Editing Current Formats

When you  select Edit  from the  Format menu  you must select the

format you wish to edit.  This is done by pressing the  SPACE BAR

to view  the different  formats on  the screen.  You may also use

the UP and DOWN cursor keys to cycle  through the  formats.  When

the one  you wish to edit is displayed press RETURN to select it.

The Edit menu is then displayed  which allows  you to  change any

part of that label format.

Edit Format Menu

F1 Size  F2 Description  F3 Background  F4 Print  F5 Typeface

Size

This will  allow you  to change  the size of the label as well as

the spacing.  The current values for the label are  displayed and

may  be  edited  or  you  may  press  RETURN to keep the original

7

values.  The label will then  be redrawn  at the  new size.   One

note of caution when making a label smaller.  Any background text

that no longer fits  in the  label will  be lost.   The  size you

enter will  determine the amount of text that can be entered into

the label.   The  actual  printed  label  may  look  quite  a bit

different than  the label  displayed on the screen because of the

different typefaces  that  can  be  used  when  printing  to your

printer.   A good  idea is  to make  the label just big enough to

hold the text you need and then you can fit it to  your labels by

using  the  spacing  and  different typefaces.  Remember to check

your format design for proper appearance  on your  printer before

using it in a file.

Description

Allows you  to edit  the description  of the  label format.  This

description is only for  your  reference  and  can  be  up  to 40

characters  long.    Full  editing  functions  are available when

entering the description.

Background Text

This allows you to edit the  background text  of the  label.  See

the section on Adding Labels (5.3.2) to a file for information on

how to edit label text.  Press ESC when you have finished editing

and you  will be returned to the Edit Formats menu.  This feature

allows your blank labels to contain text and  graphic characters.

The background  text will be part of all labels using this format

and cannot be edited.  This is handy  for a  lot of applications.

You can  design boxes around your labels or you can design labels

with blanks that will be filled in by hand.

If you know how to program your printer  you can  even download a

custom  character  set  to  create  company logos on your mailing

labels.

Print

This will print a sample of your label format to your printer. It

will let  you check  all parts of your label format.  A series of

"+" signs are printed for each line in your format.   This sample

label will  use any  typeface selections you may have assigned to

your format.  This is handy  for checking  sizes and  making sure

your format fits your labels!!

8

Typeface

With this  option you  can assign a typeface to each line of your

label.  See the section  on  Setting  Program  Options  (6.0) for

information on  setting up your printer definitions or typefaces.

This is a powerful  feature which  lets you  create all  kinds of

special  labels.    You  can  use big bold headings or very small

print or any combination you like.  When  you select  this option

you will  see a list of all printer definitions and a list of the

label lines.  All you have to do to use this feature is place the

number of  the printer definition next to the line where you want

to use that setup.   You  can use  the up  and down  keys to move

between lines.  Once a setup is selected it will stay on until it

is turned off so it is not necessary to fill  in all  lines.  For

example if  you wanted  all lines  of your  label printed in Near

Letter Quality you would just place the number  of that  setup on

line 1.   The  typeface selections are saved as part of the label

format.  Any label file you  create using  this format  will also

use the typeface selections.

5.1.3 Deleting a Format

You will  select this  option to get rid of formats you no longer

need.  First select the format you want  to delete.   See Editing

Current Formats  above on  how to  select a  format.  The program

will ask if you are sure  you want  to delete  this format. Press

"Y" to  delete it.   Deleting  a format will in no way affect any

label files that already use that format.

5.1.4 Saving Your Formats

Simply Labels will know if any changes have been made  by editing

a format,  deleting a format or by creating a new format and will

automatically save your formats if necessary.   Your  formats are

stored in  the default  directory in  a file called  SL.DEF .  If

you delete this  file  the  program  will  only  remember  the 12

standard formats that come as part of the program.

5.2 Creating New Label Files

A new label file may be created by selecting Create File from the

Main menu.  Creating a label file is a simple  matter of entering

a file  name and  selecting a  label format  to use.  The program

will add a ".LAB" extension to  your  name  so  do  not  enter an

extension.   After this  is done  the Use  menu will appear. From

9

here you can add labels and do any work you wish with  your file.

See section  5.3 for  information on  using your  label files.  A

newly created file is not saved until you are finished  using it.

See the section on Saving your labels (5.4) for more information.

This was done  to  allow  you  to  create  quick  one-time labels

without having to save the file and clutter up your disks.

5.3 Using Your Label Files

From the  Main menu  you can select Use to work with your labels.

Once a label file has been selected and loaded the  Use menu will

be displayed.   This  is the  only part  of the program that uses

your label files and all files must be saved before  leaving this

menu or any changes you made will be lost.

Use Menu

F1 Browse  F2 Print  F3 Add  F4 Delete  F5 Sort  F6 Import

From this menu you can do all of your work with your label files.

You can search, edit, delete, sort, import and print your labels.

When you first come to the Use menu the first  label in  the file

is displayed.   If  there are no labels in the file a blank label

will be on the screen.  At almost any time while in  the Use menu

you can  use the  PAGE-UP and PAGE-DOWN keys to move through your

file.  Just above the menu line you will see two blocks that give

you information  about your  file.  The one on the left will show

the number of the displayed label and how many labels  are in the

file.   The block on the right will show how many more labels may

be added to this file.

5.3.1 Loading a Label File

Before you can use  a label  file you  must load  it into memory.

When Use is selected from the Main menu the program will read the

directory of the data drive and  display the  names of  the label

files in  alphabetical order  for you to select one.  This can be

done by using the cursor keys to highlight the name  you want and

pressing RETURN.  Also you may press the key corresponding to the

first  letter  in  the  name  you  want  and  the  name  will  be

highlighted.  If there is more than one file that starts with the

same letter just keep  pressing the  key until  it highlights the

file you want and then press RETURN.  After a file is selected it

will be loaded into memory and  the Use  menu will  be displayed.

If there  are no  label files  on the  disk it  will tell you and

10

return you to the  Main menu.   Pressing  ESC before  selecting a

file will  also return  you to the Main menu.  If an error occurs

for any reason while  loading a  file the  program will  beep and

display an  error message.  You will then be returned to the Main

menu.

5.3.2 Adding Labels

Selecting Add from the Use menu will allow  you to  add labels to

your label  files.  If you are using a new file with no labels in

it a blank label will appear on the screen, allowing you to enter

your label  text.   When you  finish entering text into the label

press ESC and if the label is not blank it  will be  added to the

file.   There will be no blank labels in your files.  If the file

you are using  already  contains  labels  the  Add  menu  will be

displayed.

Add Menu

F1 Blank  F2 Copy  F3 Quit

If Blank is selected from the Add menu a blank label is displayed

for you to fill  in.   If you  select Copy  instead, you  will be

asked which  label you  wish to  copy.   After entering the label

number, that label is displayed and may be  edited to  create the

new label.   The  Copy feature is very handy if you have a lot of

labels with similar text.  A  good use  for this  feature is when

entering  a  lot  of  mailing  labels  that  have  the same City,

Province, and Code.  You could enter the first label and then use

the Copy feature to Copy the rest and edit the name and address.

When editing text inside a label you have a full label editor and

what you see  on  the  screen  is  exactly  what  the  label will

contain.  A help screen showing the editing commands is available

by pressing the F10 function key.  You can use the cursor keys to

move  around  the  label.    The  HOME key will move to the first

position in  the label  and the  END key  will move  to the last.

Pressing the  RETURN key  will move to the start of the next line

or field if your label has background text.  The  INSERT key will

toggle between  Insert and Typeover modes and the current mode is

displayed above the menu line in the block on the left.  This was

done because  the Sanyo MBC 5XX series computers does not support

different cursor sizes.  The DELETE key will delete the character

under the  cursor and move all remaining text on that line to the

left.  The BACKSPACE key will delete the character to the left of

the cursor  and move  all remaining  text and the cursor back one

position.  When in insert mode all text pushed outside  the label

is lost.   Pressing CNTRL-LEFT will move you to the left one word

at a time and CNTRL-RIGHT will move one word to  the right.   You

11

can center a line of text within your label by placing the cursor

on the line and pressing the ALT-C keys.  An erase to  the end of

the line function is provided which will delete all text from the

cursor to the end of the current line.  This is done  by pressing

the ALT-E keys.

When  editing   labels  with   background  text  there  are  some

differences.  First of all the  cursor  will  never  rest  on any

background  text,  preventing  you  from changing it.  Background

text breaks  your label  up into  fields.   A field  is any blank

spaces between  Background text.   The  word left  and word right

functions also function as field left  and field  right functions

when background  text is  used in  your label.   Also the INSERT,

DELETE and BACKSPACE keys  will only  affect text  in the current

field.   Any remaining  text on  that line  will not be affected.

Also the center a line and  erase to  end of  line functions only

affect text in the current field.

Simply Labels  allows you to use the graphic characters contained

in the extended character set as  text  in  your  labels.   These

characters may  be used for either background text or normal text

and are very useful for making fancy labels  with boxes  or logos

in them.   To use these characters in Simply Labels you just have

to press F1 at any time while editing label text and you  will be

switched to graphic mode.  A small happy face is displayed on the

menu line  to  show  you  that  you  are  in  the  graphics mode.

Pressing F1  a second  time will  return you to normal text mode.

All of the graphic characters are  represented by  standard ASCII

characters and  can be  entered from  the keyboard.  To see which

keys represent which graphic  characters copy  the file CODES.DOC

to your  printer.   If this  file does not print properly on your

printer you should not use graphic characters in your labels.

5.3.3 Browsing Around

Selecting Browse from the Use menu will bring up the Browse menu.

This menu  is useful  for looking  at your labels and general the

updating of your label files.

Browse Menu

F1 Search  F2 Top  F3 Bottom  F4 Move  F5 Edit  F6 Delete

Searching For Labels

Simply labels includes two  different types  of searches  to help

12

you  find  labels  in  your  files.  The search routines are used

throughout the program to find,  delete  and  print  your labels.

The two types of searches are called complete and conditional and

are explained below.

When searching for labels from  the  Browse  menu  you  enter the

text, up  to thirty  characters, to  search for and press return.

If you are using a conditional search you  also must  specify the

line to search.  The program will then search for matching labels

and if none are found you  will be  returned to  the Browse menu.

When using  search from the Browse menu, the first label matching

the search text is displayed on the screen and the Search menu is

displayed.  This will give you the option of editing the label or

continuing with the search.  If you choose to edit  the displayed

label you can continue the search after the editing is completed.

This function is useful  if you  have to  edit a  group of labels

that have something in common.

Search Menu

F1 Continue Search  F2 Edit  F3 Quit

Complete Searches

A complete  search allows  you to  enter text, up to a maximum of

thirty characters, to be  searched for.   A  complete search will

find any  occurrence of  your text  any place in your label.  The

text can be in any position on any line and the search  will find

it.

Conditional Searches

Conditional  searches  will  find  all  labels that match certain

conditions that you specify.  A  big difference  between complete

searches and  conditional searches  is that  the latter will only

search one line of your label and you must tell the program which

line to search.  If any of the condition symbols ( <, >, <>, .. )

are entered in your search text  the program  assumes you  want a

conditional search and will then ask you which line in your label

you want to search.   The  conditions that  may be  used are Less

Than (<),  Greater Than (>), Not Equal To (<>) and a Range Search

(..). When you enter the text you wish to search for, you specify

the condition  by including the condition symbols at the start of

the line.  For example:

<Smith

13

This will find all labels that are alphabetically before Smith on

the line you choose to search.

>Smith

This will find all labels that are alphabetically after Smith.

<>Smith

This search will find all labels that are not equal to Smith.

The  Range  Search  is  a  little different in that the condition

characters (..) are not placed at the beginning of the line. They

are placed between two words.  For Example:

Barry..Smith

This can  be read  as Greater  Than Barry and Less Than Smith and

the search will find  all labels  matching this  condition.  This

search will  work just  as well with numbers, making it great for

working with zip codes.

In all searches the case of the text does not matter.

SMITH

This search will find Smith,  SMith  ,smith  and  smithers.   The

search  only  checks  the  number  of  characters you enter for a

match.  This  is  handy  if  you  are  not  sure  of  the correct

spelling.   For example  searching for "Smi" will find all labels

containing  those  three  letters.    Remember  that  conditional

searches only  check one line for a match and do not care what is

in the rest of the label.

Moving Around Your File

The PAGE-UP and PAGE-DOWN keys may be used  to move  through your

file one label at a time but from the Browse menu there are three

choices that make moving around your label file quick and easy.

Two of the choices are called  TOP and  BOTTOM and  these options

just move  you to  the first  and last  labels in  you file.  The

third option is called MOVE  and  lets  you  move  any  number of

labels in either direction.  When you select this option you have

to enter the number of labels you want to skip.  To  move towards

the first label in your file just enter a negative number.

14

Editing A Label

Selecting Edit  from the  Browse menu  will allow you to edit the

text of  the  label  currently  displayed  on  the  screen.   For

information  on  editing  label  text  see  the section on Adding

Labels (5.3.2).

Deleting A Label

This  option  will  allow  you  to  delete  the  label  currently

displayed on  the screen.   You will be asked to confirm that you

want to delete this label.   Pressing "Y"  will delete  the label

and move all labels above it to fill in the blank.  There will be

no blank labels in your file.

5.3.4 Deleting Labels

You can delete a group of labels by selecting Delete from the Use

menu.   The labels  are selected  by using  the search functions.

See the section on Browsing  Around  (5.3.3)  for  information on

searches.   After you  enter the  text to  search for the program

will tell you how many matches it found and present the labels on

the  screen  one  at  a  time  and ask if you want to delete this

label.  If you press "Y" the displayed  label is  deleted and the

next label  that was  found in the search is displayed.  Pressing

ESC will return you to the Use menu.

5.3.5 Sorting Your Files

Most standard mailing label programs are  set up  as a  data base

type  program  with  fields  for  Name,  Address, City and so on.

While this is fine for mailing labels it severely limits the type

of  labels  you  can  generate.   Simply Labels takes a free form

approach allowing what you see is  what you  get type  of labels.

This allows  much more flexibility in designing label formats but

presents a problem for sorting labels.  When using mailing labels

the most basic sort is to sort the file by Last Name.  To do this

in Simply Labels the sort routines were written  to allow  you to

sort on  only a  part of  a line.   It is possible to sort on the

whole line or you can select  a sort  on only  the first  or last

word in  the line.   There  is a primary and a secondary sort and

both can specify which part of the line to use.  For  example you

could sort  a mailing  label file  on the  line containing Zip or

15

Postal codes and then as a secondary sort, sort on  the last word

in the first line.  This would sort your file by Zip codes and by

Last Name.  The sort routines  in Simply  Labels are  quite fast,

sorting 400 labels in about one minute if complete line sorts are

used.  It may take much longer if  first or  last word  sorts are

used.

Sort Menu

F1 All  F2 First  F3 Last  F4 Quit

Selecting  Sort  from  the  Use  menu will allow you to sort your

label file in alphabetical order.  You will be asked to enter the

line for  the primary  sort.   Next you  have to select, from the

sort menu, which part of the  line to  use for  the primary sort.

After this is done you have to repeat the above for the secondary

sort. Next you have a choice of an ascending or  descending sort.

Select the  type of  sort and the program will begin sorting your

file.  When the sort is  finished the  program will  beep to tell

you that it has finished.  If you press any key while the program

is sorting you have the option of continuing with the  sort or to

abort and  return to the Use menu.  When you save your files they

will be saved in the sorted  order.   This way  you only  have to

sort your file when labels are added or changed.

5.3.6 Importing Labels From Other Files

The import  option used  in Simply  Labels is  quite flexible and

allows you to import labels from most types of text  files.  This

has many  uses such  as splitting  your label  files into smaller

files, transferring information between  label  files,   creating

sub-files containing  only parts  of the  original file,   and to

transfer information from other  application  programs  into your

label files.

This program can read ASCII text files created by any application

such as your spreadsheet or data base programs  and pick  out the

information you  want to  include in your labels.  The only thing

that is assumed about the text files is that they fit into one of

two broadly defined types.  They are called Label type and Column

type text files and are  explained  below.    To  use  the Import

option you  will have  to tell  the program  the name of the text

file to import from and exactly what text you want  to include in

your labels.  After you enter this information Simply Labels will

read the file, build  a label  according to  your input  and then

display it  on the  screen.  At this point you can add it to your

file or read another label.  Labels may be added until your label

16

file is  full or  the end  of the text file is reached.  Esc will

abort the import operation and return you to the Import menu.

Import Menu

F1 Label  F2 Column  F3 Quit

Label Type Files

Label type files are text files in which the information required

to  make  your  label  is  contained  in  24 lines or less.  Some

examples of this type of  file  are  the  text  files  created by

Simply Labels  when printing  labels to  a disk  file.  Most data

base programs will also print this type of  file.   It is assumed

that if more than one label is to be read that the information in

the text file be repeated every so many  lines.   If a  data base

program prints  mailing labels  to a  disk file,  the first label

will take from lines 1 to 6 and the second  will be  from lines 7

to 12.   This is an example of a repeating label type file.  When

you create text files from your applications you may want to turn

off  options  such  as  page  headings  which  will  destroy  the

repeating nature of the text file.

To import labels from this type  of  text  file  you  will select

Label from  the Import Menu.  Next you will be asked to enter the

full name of the text file.  This name may contain a drive letter

and you must include the extension.

[d:]SAMPLE.EXT

The program will then check to see that the file can be found and

if it  can't find  the file  you will  be returned  to the Import

menu.  If you enter a valid file name the label make-up screen is

then displayed and it is from here that you will tell the program

how to read the text file and build your labels.

The first  information that the program requires is the number of

lines from the text file required to build  one label.   This can

be from  1 to  24 lines  and represents how many lines before the

text file repeats.  If you enter a  value such  as 6  the program

will read  6 lines  from the text file, build your label and then

read the next 6 lines to build the second label.

Once you tell the program how  many lines  to read  from the text

file you  have to  tell it what text to include from those lines.

For each line in your label you must enter the line from the text

file to  use, and  what part  of that line to use.  On the screen

17

you will see the line from  your  label  that  you  are currently

defining.   You will  be asked  to enter which line from the text

file will be used to build the current  label line,  the starting

position and  the length of text to use.  A sample make-up screen

is shown below.

-----------------------------------------------------------------

Label Make-Up From Label Report

Enter number of lines from text file needed to make label. 24

Label Line # 2

Enter line number from text file to use.  7

Start: 6

Length: 35

-----------------------------------------------------------------

The values entered on the screen above would place the  text from

positions 6 to 35 on line 7 in the text file into the second line

of your label.  All lines in your label will  be defined  in this

way.    To  help  make  this easier all lines have default values

which may be selected  by  just  pressing  RETURN.    The default

values for  each line in the label will be the corresponding line

in the  text file,  with a  starting position  of 1  and a length

equal to  the width  of your  label.  If these default values are

what you want or you have defined as many lines  as you  need you

may press ESC to start importing labels.

This  method  of  specifying  how  to build your label allows the

program to read any text file and rearrange  it any  way you like

to fit  it into your label.  You can even take text from one line

in your text file and use it on several lines in your label.

Some notes on importing text.  When a line is read from  the text

file it is filtered to remove any non-printable characters before

it is placed into the label.  If you try to place  more text than

is allowed  on one  line in  your label the program will clip the

line.  Any part of the label that is not read from  the text file

be taken from your label format, including background text.

For a practical example of how to use this type of importing lets

make a label file called COMPANY.LAB with a size of 60 x  11 with

one line  for spacing.   This file is used to record all contacts

you deal with in your business.   This  file will  use background

text to  separate your label into fields and is shown below.  See

the  section  on  Creating  New  Label   Files  (5.2)   for  more

18

information.   This example  will show  how to make a sub file of

mailing labels for those contacts.

COMPANY.LAB

-------------------------------------------------------------

|NAME-OF-BUSINESS:                       BUSS-#:              |

|ADDRESS:                                                     |

|CITY:                                                        |

|PROVINCE:                                                    |

|POSTAL CODE:                                                 |

|-------------------------------------------------------------|

|NAME-OF-CONTACT:                                             |

|PHONE-#:(123) 456-7890                                       |

|DATE-LAST-CONTACTED:                                         |

|COMMENTS:                                                    |

|                                                             |

-------------------------------------------------------------

The first step in making mailing  labels for  your contacts would

be to  print your label file (COMPANY.LAB) to a disk file.  To do

this you would select Print from the Use menu, select  Range from

the Print  menu and  select Disk from the Device menu.  After all

the labels have been printed to the disk file return  to the Main

menu.   Remember you  can also  use the search functions to print

only contacts that meet certain conditions, such as  those living

in Ontario.

After you  have done  this you  need a  file to  hold the mailing

labels you are going to import.  Lets create another  file called

MAILING.LAB with  a size  of 30  x 5  with a spacing of one line.

After you create this  file, select  Use from  the Main  menu and

load MAILING.LAB  into memory.  The Use menu is displayed and the

Import option is selected from here.  Next select Label  from the

Import  menu  and  when  asked  how many lines needed to make one

label, answer 12.  Remember the size of  our labels  are 60  x 11

with one  line spacing.  Next enter  the following values for the

label make-up:

Text Line# to Use        Start     Length

Line#1:          6                  17         30

Line#2:          2                   9         30

Line#3:          3                   6         30

Line#4:          4                  10         30

Line#5:          5                  13         30

After these values are entered the text file will be read and the

19

mailing  label  for  the  first  contact will be displayed on the

screen.  If you want to add this to your  file you  may press any

key or "N" to skip and display the next label.

Column Files

Column type  files are  text files  in which  all the information

needed to make a label is contained on a single line.   This type

of  text  file  is  very  common  with  data base and spreadsheet

programs.  When Simply Labels reads  this  type  of  text  file a

label  is  built  from  every  line  in the text file and you may

include only the labels that you want.

Sample Column Type Text File:

EMP#      Name     Age     Address           City

--------------------------------------------------------------

1999   John Doe     29   P.O. Box 23      St. John's, Nfld.

1234   Janet Smith  34   P.O. Box 1234    Corner Brook, Nfld.

9876   Dick Jones   23   P.O. Box 543     St. John's, Nfld.

Importing labels from this type of text file is done by selecting

Column from  the Import menu.  Next you have to enter the name of

the text file  to  import  from.    This  name  must  contain the

extension and  may contain  an optional  drive specification.  If

the file cannot be found you will be returned to the Import menu.

Next you  have to  tell the  program how  you want  to build your

labels.  This is very similar to importing  from label  type text

files as described above.

For each  line in your label you must tell the program which part

of the line from the text file to use.   To do  this you  have to

enter a  starting position  and the  length of  text to use.  For

example to make mailing  labels from  the sample  text file above

you would enter the following values at the Label Make-Up screen:

Start          Length            Result

Line#1:        8              13             (Name)

Line#2:        26             17             (Address)

Line#3:        43             20             (City)

Line#4:

Line#5:

20

After  the  values  needed  to  build your labels are entered the

program will read each line of  the  text  file  and  display the

resulting  label  on  the  screen.    You then have the choice of

pressing any key to add it to your file or  pressing "N"  to skip

that label  and display  the next one.  Labels may be added until

your file is full  or  the  end  of  the  text  file  is reached.

Pressing ESC will abort the import operation.

5.3.7 Printing Your Labels

To print your labels select Print from the Use menu and the Print

menu will appear.  This menu allows you to print your labels in a

variety of  ways.   You can  print a  single label,  any Group of

labels or you can Search for the labels you want to print.

Print Menu

F1 Group  F2 Search  F3 Test  F4 Repeat  F5 Edit  F6 Options

Selecting an Output Device

Simply Labels allows you to  print  your  labels  to  the screen,

printer or  to a  disk file.  When you choose to print labels you

will be asked to select an output device from the Device Menu.

Device Menu

F1 Printer  F2 Disk  F3 Screen

The screen is handy for checking to make  sure the  proper labels

will be printed.  When you are sure you have selected the correct

labels you can print the same labels to the printer to get a hard

copy.   When labels  are printed  to the  screen they are printed

inside the label window.  This window will scroll when full.  See

the section  on Setting  Program Options (6.0) for information on

setting the screen printing speed.  The printing  will pause when

any  key  is  pressed  and  you  have the option of continuing or

aborting the printing.  You cannot print multiple labels per line

to the  screen.   This option  is only available when printing to

21

the printer or a disk file.

Printing your labels  to  a  disk  file  is  very  important when

combined with  the import  option because  it allows you to split

your files into  smaller  ones  and  to  transfer  labels between

files.   When you  select Disk  from the  Device menu you will be

asked for a file name for the  text  file.    If  a  file already

exists with  the name  you entered  you will  be asked  for a new

name.  The program adds a ".TXT" extension  to the  file name for

you so  there is  no need  to add  an extension.   The files that

Simply Labels prints to disk are standard ASCII files and  may be

read  by  any  program.    For  example  you  can  use the search

functions in Simply  Labels  to  select  certain  mailing labels,

print them  to a  disk file  and your  word processor can use the

file to create form letters.  If any errors occur during printing

to the disk file the printing will stop and an error message will

be displayed.

When using  your printer  as the  output device  the program will

check to  make sure your printer is ready to print before sending

any labels to be printed.   If  your  printer  is  not  ready the

program will beep and ask you to check it.  At this point you can

fix your printer and continue or press ESC to abort and return to

the Print menu.  Also at any time while printing pressing any key

will  pause  the  printing  and  then  you  have  the  option  of

continuing or quitting.

When printing  labels to  the printer,  any typeface selected for

that label will be turned on before the  label is  printed.  Note

that these  typefaces are not used when printing to the screen or

to a disk file.  Therefore  if you  print your  labels to  a disk

file and  then later  print them  on your  printer, they will not

contain the typefaces selected  in  your  label  file.    See the

section on Setting Program Options (6.0) for more information.

Printing a Group of Labels

A  group  of  labels  can  be  a  single  label  or any number of

consecutive labels from your file.   To print  a group  of labels

you just  select Group  from the  Print menu and then you will be

asked to select the output device you want.   Next  you are asked

to enter  the group  of labels to print by entering the first and

last label to be printed.  The defaults  for these  are the first

and  last  labels  in  your  file and may be selected by pressing

RETURN. If you enter  a starting  label other  than label  #1 the

program assumes you want to print a single label and will set the

default for the last  label to  the value  you entered.   Finally

your labels will be printed to the output device you selected.

Note that  if your  file contains  only one label you will not be

22

asked to enter the first and last label to print.   When printing

to the  printer or to a disk file the program will use the number

of copies and print format selected in the  print options.   When

printing to the screen one label per line is always used.

Searching for Labels to Print

There are  times you may want to print a single label and are not

sure where it is in your file or you may want to print all labels

that have  something in  common, like having the word "Canada" in

them.  This can be done very easily because Simply  Labels allows

all of its search functions to be used while printing.  To search

for the labels you wish to  print  just  select  Search  from the

Print menu and you will be asked to enter the text to search for.

See the section on Browsing Around  (5.3.3) for  more information

on using  the search  functions.   Simply Labels will then search

for labels matching your text and tell you how many it found.  If

the search  found no  matches you  will be  returned to the Print

menu.  Next you will have to select an output device.

Print Search Menu

F1 All  F2 Selective  F3 Quit

With Simply Labels you  will  have  the  option  of  printing all

labels  that  are  found  with the search or selectively printing

some of them.   To do  this the  Print Search  menu is displayed.

Choosing All  from this menu will print all labels that are found

in the search.  Choosing Selective will instruct Simply Labels to

display labels  found in  the search  on the screen one at a time

and ask you if you want  to print  this label.  Pressing "Y" will

print the label and then display the next label that was found in

the search.  When using the screen as the output device the Print

Search menu  is not  used and  all labels found in the search are

printed.

Print Functions

From the Print menu you can also select Test which  will help you

set up  your printer  before printing your labels.  This function

will print sample labels  using whatever  options are  set in the

Print Options menu.

You can force your printer to top of form by pressing ALT-P while

23

at the Print menu.  This  will  flush  the  print  buffer  of any

remaining labels  and cause  your printer  to go  to top of form.

Using this option  is  better  than  pushing  form  feed  on your

printer  because  it  flushes  the  print  buffer  first. In this

program Top  of  Form  means  one  label  for  Label  format, one

envelope for Envelope format, and the normal form feed when using

Report  format.    See  the  section  on  Label  Format  for more

information on the print buffer.

By pressing  ALT-L you  can advance  the paper in your printer by

one line.  This also flushes the print buffer before your printer

advances one line.

Print Options

The last  choice in  the Print  menu is  Options and  it lets you

change all  options used  when printing  your labels.   When this

choice  is  selected  a  screen  is displayed showing all current

print options along with the Options Menu.  From the Options Menu

you  can  select  one  of  three  different  formats  to use when

printing your  labels,  change  the  number  of  copies  that are

printed and  the typeface  selections for  your label  file.  All

options set from this menu are saved as part of  your label file.

If you  select a  different format,  all options  for that format

will be in effect just by selecting that format.  This allows you

to quickly change between formats while maintaining all settings.

Print Options Menu

F1 Labels  F2 Envelopes  F3 Report  F4 Copies  F5 Typeface

Label Format

Selecting this choice will tell the program to print your file in

standard label format  and  let  you  set  the  options  for this

format.   To change any label options you must select Labels from

the Options Menu.  When this is done  a screen  will be displayed

showing all  options that are available with the Label format and

allow you to change them.  The options  available are  the number

of labels per line, the spacing between labels, the lines between

labels and the left margin.  When the options are set the way you

like just press ESC to return to the Options Menu.

24

Options Screen for Label Format

-----------------------------------------------------------------

****  Label Set-Up ****

Number of labels across. (1 - 5) [ ]

Spacing between labels.  [  ]

Lines between labels. [  ]

Left Margin. [  ]

Print Sample label. [ ]

-----------------------------------------------------------------

Simply labels  will allow  you to  print up  to 5 labels across a

page.  It does this  by  including  a  buffer  which  stores your

labels until the line is full.  There may be times when you print

a label and nothing seems to  happen.   Do not  worry, your label

was  stored  in  the  buffer.    When the number of labels in the

buffer reaches the number of labels across that you selected, the

labels will  be printed.  If you are using one label per line the

label is printed right away.  The buffer was included  to prevent

the wasting  of expensive  labels.   When you are finished all of

your printing you will want to print the labels remaining  in the

buffer.   This can  be done  in several ways.  Any time you leave

the Print menu  or  change  the  output  device  the  buffer will

automatically be flushed of all remaining labels.  Selecting Line

or Page while at the Print menu will also flush the buffer before

moving your  paper.   You may  also flush  the buffer manually by

pressing the F10 function key or the ALT-F keys.

If you select more than 1 label across you will be able  to enter

the spacing between your labels.  You may have to experiment with

this value to find the  proper  spacing.    The  values  for this

option are  from 0  to 99 characters.  If only one label per line

is selected you will not be able to set this option.

The next option is the number of lines between your labels.  This

is  first  set  when  you  create  the label format but with this

version you can now change it at any time.

The left  margin is  normally set  from the  main program options

(See  section  6.0)  but  may  be changed from this screen.  This

allows you to set  different margins  for different  label files.

25

If this  option is  left blank the left margin is set to whatever

is entered from the main program options.

The next  option  allows  you  to  print  sample  labels  on your

printer.   This is the same as selecting test from the Print menu

but it lets you  see  what  your  options  are  set  like without

leaving the  options screen.   To  print sample labels just press

the space bar when the cursor is at this option.

Envelope Format

By choosing  Envelope from  the Print  Options menu  you tell the

program to print your labels as envelopes.  This format is really

only useful if your file contains mailing labels but there  is no

reason it  can't be  used with any type of label file.  To change

or set any envelope  options you  must select  Envelopes from the

Option Menu.   When this format is selected a screen is displayed

showing all options available.  When  you have  finished entering

the  options  just  press  ESC  to  return  to  the Options menu.

Whenever the envelope format  is  selected  all  labels  that are

printed will be printed as envelopes.

Options Screen for Envelope Format

-----------------------------------------------------------------

****  Envelope Set-Up ****

Print Return Address (Y/N) [Yes]             Printer Definitions

--------- Return Address ------------         1 - N L Q

Label # for return address [   ]              2 - N L Q   U L

Address            Typeface      3 - Condensed

1 - [Your Name                ] - [1]         4 - Condensed U L

2 - [Your Address             ] - [8]         5 - Bold Italic

3 - [Your City                ] - [ ]         6 - Wide

4 - [Postal Code or Zip       ] - [ ]         7 - Big and Bold

5 - [                         ] - [9]         8 - Baby Print

9 - Normal Print

----------- Envelope Size -----------

Width (Char)[90 ]  Height(Lines) [25]   Print Sample Envelope [ ]

Starting Line for Address [10]

-----------------------------------------------------------------

The first option on this screen gives you the choice  of printing

a return address or not.  If "N" is entered for this choice, only

the main address from your label file will be printed.   You will

26

not be  able to  enter any  return address options and the cursor

will move down to the envelope Width.  Entering "Y"  will display

the  current  return  address  and  allow  you to change it.  The

return address shown on the envelope options screen  is used with

all envelopes printed from the current label file.

The next  option asks  for a label number for the return address.

You can enter any  label number  from your  file and  the program

will use  that label as the return address.  If you don't want to

use a label from your file you can just press return to  enter it

manually.  In either case you may edit the return address and add

any typeface you wish  to  use  with  the  return  address.   The

typefaces entered  here are  only used for the return address and

are different from the selections entered from the  Options Menu.

The typeface  selections made  from the  Option menu are used for

the main address of the envelope.

Simply Labels will print any size  envelope you  can fit  in your

printer.   Instead of just allowing one or two size envelopes you

can define any size you like by entering the width  and number of

lines in the envelope.

When  entering  the  size  options  just remember that the return

address is fixed at the top  left and  you can  position the main

address any  where you  like.  The width option is used to center

the main address.  If you want to  move the  main address  to the

right just  increase the  width number.  The height option is the

total lines in the envelope and should be adjusted to move you to

the top  of the next envelope. (Tractor feed envelopes)  The last

option for envelope size is the Starting Line for Address  and it

controls which  line in  the envelope the main address starts on.

To move the main address up, just decrease this number or to move

it down just increase it.

The  last  option  on  this  screen  allows  you  to print sample

envelopes.  When the cursor is  on  this  option  just  press the

space bar  and a sample envelope will be printed on your printer.

This is the same as selecting  Test from  the Print  Menu but you

don't have to leave the envelope option screen.

When entering  options you  can move  freely around the screen by

using the UP and DOWN cursor  keys.   The method  of entering the

envelope size  was chosen  because it allows any size envelope to

be used and it will allow  you to  adjust for  special typefaces.

For example  if the size of the envelope was fixed and you wanted

the main address printed in  wide  characters,  the  main address

would be  way over  on the  right side of the envelope.  This way

you can center the  main address  by adjusting  the width option.

Don't be  afraid to  print out lots of sample envelopes until you

get the set up just right.   It  might  be  a  good  idea  to use

standard paper  while you do this.  You won't have to enter these

values each time you use your file because all options  are saved

27

along with your labels.

Report Format

This  choice  will  tell  the  program  to print your labels in a

report format and allow you to set all  available report options.

The Report  format consists  of a  page heading  or title and the

body of the report  which will  be your  labels.   Each label you

print from  the Print  Menu will  make up one line in the report.

This is  a good  way to  get summaries  or lists  from your label

files.   The report can contain up to five columns, each of which

can be any line in your label.  You can have two  title lines and

the report  can be printed on any size page.  This format is very

useful for all types of label  files.   You could  use the report

format to  print out  a list  of all parts from a parts file or a

list of all cassettes from a tape file.  To  use this  format all

you have  to do is enter or select the options you want and press

ESC when  you  are  finished.    Whenever  the  report  format is

selected  all  labels  that  are  printed  will be printed in the

report format.

Options Screen for Report Format

--------------------------------------------------------------------

****  Report Set-Up  ****

Title 1 - [                                                ]

Title 2 - [                                                ]

Repeating Titles (Y/N) [Yes]    Print Date (Y/N) [Yes]

Typeface for Titles [ ]         Report Width [   ]

---------------------------------------------------------------

Printer Definitions

Report Options          Report Contents       1 - N L Q

2 - N L Q  U L

Print Label Numbers [Yes]  Column  Label Line#    3 - Condensed

Typeface for Report [ ]       1        [1 ]       4 - Condensed U L

Page Length [66]              2        [  ]       5 - Bold Italic

Top Margin [3]                3        [  ]       6 - Wide

Bottom Margin [3]             4        [  ]       7 - Big and Bold

Left Margin [ ]               5        [  ]       8 - Baby Print

9 - Normal Print

--------------------------------------------------------------------

The top part of this screen allows you  to enter  the options for

the report  titles.   Two title lines are printed and can be left

blank if desired.  The repeating  titles option  will if selected

cause the  title lines to be printed at the top of each new page.

28

If you enter "N" for this option the title lines are only printed

on  the  first  page  and  two  extra  labels  are printed on all

remaining pages.

If you want to print the date as part of your report, just select

that option  and the  date will  be printed as part of the second

title line.  The report format does not use any typeface settings

from  the  main  label  file  but  instead allows you to select a

typeface for the title  lines and  a different  typeface for your

labels or  the body of the report.  The report width is just used

to center the title lines and should be set accordingly.

The bottom part of the screen  allows  you  to  set  options that

affect the  body of  the report.   The first option allows you to

print the label numbers as  part  of  the  report.    If  this is

selected the label numbers are printed as the first column in the

report.  The typeface for the body of the report will normally be

condensed to allow as much as possible to fit on one line but can

be any typeface you have defined.

The page  length, top  and bottom  margins combine  to define the

size of  the printed  page.   The defaults are set for a standard

page but can be  set for  almost any  size page.   If  you want a

continuous  printout  without  any  page breaks just set the page

length less than the top margin plus the bottom margin.   Setting

it to one works just fine.

The next  set of options will control what is actually printed in

your report.  To print a line from your label file in your report

all you  have to do is put the line number next to the column you

want the  line to  be printed  in.   When printing  in the report

format blank or empty columns are not printed.

Number of Copies

When Copies  is selected  from the Print Options menu you will be

asked how many copies of each label you would like  printed.  You

can print  from 1 to 99 copies of each label.  When you enter the

number of copies the number becomes the default  until you change

it.   All copies  of a  label will be printed together before the

next label is printed.   This  option will  work with  either the

label or envelope formats but is not available when Report format

is used.

29

Typeface

Selecting Typeface from the Print Options menu will  allow you to

select any  of the defined typefaces (See Setting Program Options

(6.0) for more information.) for any line in your label.  See the

section on  Editing Label  Formats (5.1.2) for information on how

to use this option.  This option is  not available  when printing

labels in  Report format.   The  typefaces are  set in the report

options when using Report formats.

5.4 Saving Your Labels

Simply Labels is smart enough to know when any changes  have been

made to  your label file.  If you add, delete, or edit any labels

or if you change any typeface  selections you  will be  told that

your file  has been  changed and given a chance to save your file

when you leave the  Use menu.   If  you return  to the  Main menu

without saving  your file the changes you made will be lost.  Any

time you are at the Use menu you may manually  save your  file by

pressing the  F9 function  key or  the ALT-S keys.  This is handy

when you are adding a lot of labels and want  to save  your work.

After you manually save your file you will be returned to the Use

menu.

When you choose to save your label file you will be asked  if you

want it  saved under the default or previous name.  You can press

RETURN to keep the same name or edit it to save the  file under a

new name  which is  useful for  making backups of your files.  If

the file already exists  you will  be asked  to confirm  that you

want to replace the file and then your labels will be saved.

5.5 Deleting Old Label Files

You can delete label files you no longer need by selecting Delete

from the Main menu.  You will have to select  the file  to delete

from a list of label files on your data disk. (See 5.3.1 for more

information on selecting files)  To delete the file you will have

to confirm  that you  do indeed  want to delete it.  Pressing "Y"

will delete the file.

6.0 Setting Program Options

Selecting Options from the  Main menu  will bring  up the options

screen and  display the default settings.  You can use the UP and

DOWN cursor keys or the RETURN key to  move between  the options.

Options can  be edited using the INSERT, DELETE, BACKSPACE, HOME,

END and LEFT and RIGHT  cursor  keys.    When  you  have finished

30

setting your  options press  ESC and you will be given the chance

to save them.  If you choose not to save  your options  they will

remain in  effect until  you change  them or  until you leave the

program.  If you  save your  options they  are written  to a file

called SL.CNF  in the  default directory.  If this file cannot be

found when you start Simply Labels the program will reset  to its

default settings.

6.1 General Settings

Main Program Options screen

-----------------------------------------------------------------

Data Drive  [ ]

Type of Display  [ ]                 Screen Write Method [ ]

1 - B/W                                 1 - Bios

2 - Color   3 - Two Color               2 - Direct

Screen Printing Speed (1-9)  [5]     Left Margin [  ]

*** Printer Definition Strings ***

Name                  Definition

[N L Q          ]-[27/@/27/x/1               ]

[N L Q   U L    ]-[27/@/27/x/1/27/-/1        ]

[Condensed      ]-[27/@/27/15                ]

[Condensed U L  ]-[27/@/27/15/27/-/1         ]

[Bold Italic    ]-[27/@/27/!/24/27/4         ]

[Wide           ]-[27/@/27/!/49              ]

[Big and Bold   ]-[27/@/27/G/27/E/27/h/01    ]

[Baby Print     ]-[27/@/27/S/0/27/3/18/27/M  ]

[Normal Print   ]-[27/@                      ]

-----------------------------------------------------------------

The first  option will set the default data drive.  This is where

the program will store your label files.  If this  option is left

blank the  system default  drive will be used.  Valid entries for

your data drive are letters  from  A-I.    You  cannot  specify a

directory for  Simply Labels to use.  If you want to do your work

in a sub-directory  you  must  change  to  that  directory before

starting the program.

The type  of display  may be selected from three choices.  If you

have a black and white monitor with a color card  select option 1

31

(B/W).   Option 2  is called  Color and is suitable for all color

displays including CGA, EGA and VGA.  The third choice of display

is  called  "Two  Color"  and  is suitable for liquid crystal and

other mono displays.

The method the  program  uses  to  write  to  the  screen  may be

selected.   Printing with  the BIOS  will be much slower than the

direct option but it has its advantages.  If you have a  CGA card

that generates a lot of snow it can be eliminated by choosing the

BIOS method.    On  faster  computers  you  may  not  notice much

difference between the two.

The  next  option  controls  how  fast  labels are printed to the

screen.  Valid selections are  from  1  to  9  with  9  being the

fastest speed.   This  is only  used when  printing labels to the

screen from  within  the  program.    This  was  included because

personal computers have such a wide range of speeds.

You can  set a left margin with the next option which may help in

lining up your labels.  This setting is used with all label files

and with  all options.   However  you can,  from within the Print

menu, override this normal setting for special type labels.

A special note to  owners  of  Sanyo  MBC  55x  or  other  not so

compatible  computers.    This  program  will  run  fine  on your

computer if you select a Color  screen with  Bios screen writing.

If you have the standard green monitor you may find the Two Color

option easier to read.  If you can  not see  the screen  when you

start Simply  Labels just press the ESC key three times to return

to DOS and delete  the file  called SL.CNF.   When  you start the

program again  you should  be able to see the screen.  You should

then select Options from the Main menu to set up  the program for

your computer.   If  your Sanyo  has a  video board installed the

program will also work with direct screen printing.

6.2 Printer Settings (@#/!*#$)

This is a major problem for individuals who write software.   Big

software companies  have the  resources to write drivers for most

brands of printers but an individual  is lucky  to have  one good

printer.   There are  advantages to the printer driver method but

what happens if your printer is not supported.  Tough Luck!!

A different approach was taken with  Simply Labels  that lets the

program work  with any  printer connected as the default printer.

This approach also allows you to use options  that may  be unique

to your printer.  In effect what the program does is allow you to

write a limited driver to match  your printer  which assures that

the program will work with all different brands of printers.  One

disadvantage of this method is that  it requires  you to  dig out

32

your printer  manual and  look up the codes needed to set up your

printer the way you want.

On the option screen in the program you will see an area for nine

printer definition strings.  A printer definition string consists

of a name, which describes the setup, and the codes needed to set

your printer  to that  mode.  In most cases the codes can be read

directly from a table in your  printer manual.   The  program was

written to make setting up these codes as easy as possible.  Full

editing features can be used when entering the printer definition

strings.

If you  happen to  own a  Star NX10 printer, then you are in luck

because that is the default printer for the program.  Even if you

don't it  is a  good idea  to look at the defaults to see how the

settings are entered.  In any case here is how to design your own

settings.   First you  have to  decide which features you want to

define and then look up the codes needed  to put  your printer in

that mode.   The  rules for  entering the  codes are very simple:

The backslash is used as a separator between the codes to be sent

to your  printer.  Everything between the backslashes will result

in one character being sent to  your  printer.    If  there  is a

single  character  between  the  backslashes  it  is  sent to the

printer exactly as you typed it,  but if  there is  more than one

character between  the backslashes the ASCII code that the number

represents will be sent.   The  first and  last backslash  do not

need to be entered.  For Example:

27/M      Will send ESCAPE  "M" to your printer

27/77     Will send  exactly the  same codes to your printer

because 77 is the ASCII code  for "M".   This will

set most dot matrix printers to Elite pitch.

27/x/1    Will send ESCAPE  "x1" to your printer.  This will

set most printers to Near Letter Quality

If  you  cannot  type  the  character  needed  directly  from the

keyboard  use  the  ASCII  code  for that character.  To send the

ASCII codes from 0 to  9  just  place  a  zero  in  front  of the

number(00 to 09).

One  caution   about  using   your  printer   definitions.    The

definitions you enter are common to all of  your label  files and

if you  change a  definition after using it in a label file, that

file will use the new definition and not  the original  one.  The

label files  only contain  a reference to the definitions and not

the definitions themselves.   You should  decide what definitions

you want and not change them.

33

6.2.1 Default Printer Settings

Star NX10

1 -  [N L Q          ]-[27/@/27/x/1               ]

This is standard Near Letter Quality.

2 -  [N L Q   U L    ]-[27/@/27/x/1/27/-/1        ]

This is Near Letter Quality With underline turned on.

3 -  [Condensed      ]-[27/@/27/15                ]

This is standard condensed pitch.

4 -  [Condensed U L  ]-[27/@/27/15/27/-/1         ]

This is condensed with underlining.

5 -  [Bold Italic    ]-[27/@/27/!/24/27/4         ]

This is the Italic font with double strike and Emphasized.

6 -  [Wide           ]-[27/@/27/!/49              ]

This is double width Elite with double strike.

7 -  [Big and Bold   ]-[27/@/27/G/27/E/27/h/01    ]

This is double size printing with emphasis and double strike.

8 -  [Baby Print     ]-[27/@/27/S/0/27/3/18/27/M  ]

This is  Elite Superscript with line spacing set to 18/216"  Line

spacing is set to  fit two  lines of  baby print  in one  line of

normal text.

9 -  [Normal Print   ]-[27/@                      ]

This is the reset command.

Note that  all the  default settings  start with the code "27/@".

This code is the reset code for the printer and  is used  to make

sure  that  all  previous  options  are  cancelled.  Most printer

options will stay on until they  are turned  off, so  the easiest

way to  make sure the printer is set the way you want is to reset

it to the default mode first.   You  do not  have to  follow this

example but it makes it easier to get exactly what you want.

Star SG-10/15

Because  these  printers  are  so  popular  their codes have been

included in this manual.   If  you own  this printer  just select

34

Options from the Main menu, enter and save these codes.

1 -  [N L Q          ]-[27/@/27/4                 ]

2 -  [N L Q   U L    ]-[27/@/27/4/27/-/1          ]

3 -  [Condensed      ]-[27/@/27/15                ]

4 -  [Condensed U L  ]-[27/@/27/15/27/-/1         ]

5 -  [Bold Italic    ]-[27/@/27/!/24/27/I/01      ]

6 -  [Wide           ]-[27/@/27/W/01/27/G         ]

7 -  [Big and Bold   ]-[Not supported in the SG-XX]

8 -  [Baby Print     ]-[27/@/27/S/O/27/3/18/27/M  ]

9 -  [Normal Print   ]-[27/@                      ]

7.0 Potential Problems

Simply Labels  was written  with the  computer novice in mind and

provides traps for most errors.  For the  most part  the problems

you may  encounter won't  be errors but rather unexpected results

which occur mostly when printing labels to your printer.

Areas which  cause  the  most  problems  are  printing  labels or

envelopes  that  use  more  than  one  typeface.    If you select

typefaces that change the  pitch or  characters per  inch of your

printer  you  will  not  get  the  desired  results when printing

multiple labels per line.  Remember the pitch is also changed for

the spacing  between the  labels which causes the different lines

in your labels not  to  line  up.    This  will  also  apply when

printing  envelopes.    If  your labels use graphic characters to

draw boxes in your  labels you  may also  run into  this problem.

The best  way around  this problem is to stick to typefaces which

don't change the  pitch  of  the  printer.    You  can  still use

settings such as bold, near letter quality and italics.  Remember

this is only a problem when  using  more  than  one  typeface per

label.    You  can  use  any  typeface  with  no problems if that

typeface is used for the whole label.  Underlining can also cause

some problems because some printers will underline the whole line

including all the spaces between the labels.

Another area which may cause  unexpected  results  is  when using

typefaces that change the line spacing of your printer.  This may

cause your labels to drift and  not to  line up  on your printer.

This problem  can be  avoided by  careful design of your formats.

When designing label formats check them  from the  Format menu by

using the Print option to print out sample labels.  When entering

printer definitions  that change  the line  spacing it  is a good

idea to use multiples of the normal line spacing.  For example if

your normal line spacing is 1/6" and you  wanted small  print you

could set  your printer  to superscript with 1/12" spacing.  This

will allow two lines of small print to fit in one line  of normal

print.   With this set up you could have 10 lines instead of five

in a standard mailing label.  Remember when  designing your label

35

formats that the size of your label only determines the number of

characters that will fit in your label and not the actual printed

size.   This will depend on the number of characters per inch and

the line spacing set on your  printer and  you have  full control

over this.

When Importing  labels and  it appears not to be working check to

make sure you are using a text file.  If you are  returned to the

Import  menu  without  pressing  ESC, an error has occurred while

reading the text file.   Remember  a line  must be  less than 256

characters long  and it  must end with a CR.  Reaching the end of

the text file will also return you to the Import menu.   No error

messages are displayed when importing labels.

8.0 Technical Support

If you  have registered  the program every effort will be made to

support you in your use of Simply  Labels.    If  you  are  not a

registered  user  you  may  contact  the  author but there are no

promises.  Whenever looking    for  support  please  include your

name, telephone  number, version  number and an exact description

of your problem.

Support will be provided in one of the two following ways.  First

you may  write the  author explaining  in detail exactly what the

problem is.  The second method of getting support is  to call The

FoxNet BBS  and leave  a message  to the author or to the sys-op.

By using the BBS you will usually get a reply within 24 hours.

Registered users will receive a phone number they can use  to get

support but  the preferred  method is  one of the two ways listed

above.

The latest version of Simply Labels will be  posted and available

for downloading on the FoxNet BBS.

F o x N e t    B B S

PC Board  24 hrs a day / 7 days a week

300/1200/2400 baud

(709) 489-4758    24 hrs a day

Please send  any suggestions  for improvements to the author.  If

enough interest is  shown  the  suggestions  may  be  included in

future versions.

36

9.0 Registration Form

----------------------------------------------------------------

Registration Form  for Simply Labels Ver 2.2

Name: __________________________  To Register your copy of

Address: __________________________  Simply Labels please send

City: __________________________  $25.00 to the author at

Prov/State: __________________________  the following address:

Code/Zip: __________________________

Robert E. Pitcher

Phone Number: ___________________________     P.O. Box 877

Windsor, Nfld.

Computer Type:  ___________________________     Canada AOH 2HO

Printer Type: ___________________________

Where did you

get your copy:____________________________

Comments: _________________________________________________

Thank You for your Support!!

----------------------------------------------------------------

37