DRLOCK

version 0.1 (c) copy right 1988 All Rights Reserved

This is a simple security program to turn on/off diskette access to

drive A: or B:, as well as to the hard disk of AT, XT 286 and PS/2.  You can

independently allow read, write or format to each and change it as needed

from a pop up menu.  The pop up menu itself can be protected by a password

security code and the number of tries you are allowed to try the pass can be

set at various values as well.  This program can be wiped from memory, along

with any other program ran after this to allow big programs as much space as

possible.  OR, this program can be locked to prevent you from removing this

program.  Whether you wipe this out or if it's locked, the program will tell

you what time this program was originally ran.  When you first run this

program, this program will also tell you the time this program first became

memory resident, so by comparing the two, you can tell if someone has

tampered with your computer by turning the computer off, also.

This program intercepts the disk BIOS interrupt 13h.  Any disk system

which does not use this will not be affected.  The PS/2 read and write uses

the interrupt 13h for disk read and write, but when formatting the hard disk,

the format.com uses interrupt 21h function 440dh major code 8, minor code 42h

for format and verify track on a logical device on the hard disk.  This is

intercepted and checked if the default drive or drive C:.  If either the

default or C: and the HD format is turned off, then format.com will abort

with the message

Attempted write-protected violation

Format failure

Format another (Y/N)?

The presence of a PS/2 is checked with the system services interrupt 15h

(formerly the cassette interrupt) function 0c0h for 'return system

configuration parameters'.

This program was tested on a PS/2 with a hard disk and an XT without a

hard disk running on the latest DOS 3.3 and was found to work without any

trouble.  If you are not sure that this program will work for your particular

machine, assume that only the read/write features can be turned on or off and

assume that the format cannot be turned off and try this out as an experiment.

This program is designed to work on CGA, EGA, monochome or VGA in either

the 80 column wide mode or the 40 column wide mode.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

If you simply type

DRLOCK

then you get the help screen only.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

To run this, simply type

DRLOCK RUN

and end it with an <ENTER> or <ntr> or <cr> or anything else you want to call

the carriage return.  Just the letter 'r' in 'run' will work also.  The <alt>

and 'M' will pop up the menu.

You can load any other program AFTER the DRLOCK to be removed along with

DRLOCK, provided that the other programs have my name mentioned somewhere or

it mentions that this program will wipe out that particular program also.  Do

not run any other memory resident program after DRLOCK if not made by this

group or specifically mentions that DRLOCK will wipe out that software also.

With the default above, there is no need to type a password to pop up

the pop up menu.  All drives will allow read/write/format.  The program can

be wiped from memory, and a chirping sound will be generated when you try to

access a drive whose feature (read/write/format) is turned off.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

To lock this program so that it will not come out of memory, type

DRLOCK L

If you have other commands entered at this command line, it is a good

practice to separate the commands with at least one space.  There is no need

to enter the Run command in this case.  If you enter the Run command,

everything afterwards is ignored.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

To prevent the chirping sound (No-sound) when your program tries to use

a feature which is turned off, type

DRLOCK N

In this case, you do not have to type the 'R' parameter to run this.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

To wipe this program and any other program ran after this from memory,

DRLOCK W

And DRLOCK will tell you what time it was ran.  If the time does not

match the time you noted as the first time you ran DRLOCK, then someone

turned off the computer and played around with it.

NOTE:  A version of MSPOOL which was originally uploaded to CompuServe cannot

remove this program.  A corrected version should be present at this time.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

To disallow write/format to drive B: and disallow write to the hard

disk, type

DRLOCK BWF HW

The letters BWF stands for 'drive B:  do not allow Write/Format.

The letters HW stands for 'Hard disk:  do not allow Write.

You do not have to specify the 'R'un parameter since the presence of any

parameter other than 'W'ipe indicates that you want to run this program.

Similarly, to disallow for drive A: or C:, follow the drive letter with

the letters RWF for Read, Write or Format.  The letters may be in capital or

small letters in any order but must NOT be separated from the drive letter

with any space or tab.  In other words, DRLOCK B R will not disallow Read

for drive B: when you first run DRLOCK because of the space separating 'B'

for drive B:.  And 'R' will be interpreted as the end of this command line

and taken as meaning 'R'un.

The accesses allowed may be changed later using the pop up menu.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

To prevent the pop up menu from popping up unless you know the password,

type

DRLOCK Ppassword

All letters after the letter 'p' until a space, tab or the end of a line

will be taken as the password.  Although the letter 'P' to indicate the next

line to follow can be in capitals or small letters, the password itself must

match each capital or small letter you type at the pop up prompt.  So if your

password is 'Cathy', then using 'cathy' or 'cAthy' at the pop up prompt for a

pass will not work.  The password may be up to 78 characters long, including

any character from chr$(33) through chr$(255).

If you literally type

drlock pcAThy

to run DRLOCK, then when the prompt to enter the password appears, you must

literally type the 'cAThy' as is to access the pop up menu to change the

accesses.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

When a feature which is not allowed is detected, DRLOCK sets a carry

flag and returns an error condition in the AH register back to the operating

system or the program which tried to access a feature such as read/write/

format which was not allowed at the time.  For read request, the default is

to set the time-out error to say that the disk drive failed to respond only.

For write and format, the default is the set the write protected error.  To

change the read error status returned in the AH register to 'bad command

passed to diskette i/o', type

DRLOCK !1

To change the write error status to 'time-out', type

DRLOCK @128

To change the format error status to 'time-out' and 'bad-command', type

DRLOCK #129

The values which can be combined with each are the following:

time-out          128          disk drive did not respond at all

bad-seek           64          disk head did not seek to requested area

bad-nec            32          the NEC disk controller failed

bad-crc            16          cyclic redundancy check error

dma-boundary        9          tried to access over 64kb of space

dma-bad             8          direct-memory-access overrun

no-such-record      4          sector requested did not exist

write-protected     3          disk is write protected

bad-adr-marker      2          no address marker found

bad-command         1          incorrect command was sent to disk drive

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

If a password is required, then you are given a default of three times

to type the proper password.  To change the number of tries allowed to 2,

type

DRLOCK T2

The default is to allow three tries.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

PROMPTS AND MESSAGES

=-=-=

DRLOCK:  Enter password or <esc>

=-=-=

When the pop up menu is protected, this message will appear to ask you

to enter the password.  End the password with a carriage return or type the

escape key to get out any time.

|--------------------------------------|

|A: read write format   cursor         |

|B: read write format   and <ENTER> to |

|C: read write format   change setting |

|HD read write format   <ESC> exits    |

|v-0.1-DrMS&ASSOC----------------------|

This is roughly what the pop up will look like when you turn it on.

Move the cursor up/down/left/right to the desired feature and then press on

the carriage return.  If the cursor is on the first 'read' when you do, then

the word 'read' will disappear from the drive A: row and you will be left

with

|--------------------------------------|

|A:      write format   cursor         |

|B: read write format   and <ENTER> to |

|C: read write format   change setting |

|HD read write format   <ESC> exits    |

|v-0.1-DrMS&ASSOC----------------------|

Now, any attempt to read the diskette in drive A: will be intercepted

and a chirping sound will be made to tell you that nothing actually happened.

This program will not intercept a RAM drive configured as drive C: if it does

not use the standard BIOS to access the diskette drive.

Similarly, moving the cursor to the HD (hard disk) line and pressing

down on the carriage return once at the 'read' feature will disallow reading

the hard disk.  Press down on the carriage return once more to allow reading

the hard disk.

=-=-=

Sorry, I cannot remove DRLOCK because it is not in memory

=-=-=

If you tried to wipe DRLOCK from memory when you did not have one in

memory, then you get this message.

=-=-=

You don't have to do it, DRLOCK is already in memory

=-=-=

If you tried to run DRLOCK when DRLOCK was already in memory, then you

get this message.

=-=-=

Internal memory organization is destroyed !!!

I recommend that you warm reBOOT WITH THE CTL-ALT-DEL key combination

as soon as possible since the behavior of the machine is no longer

predictable.  Do not ignore this error message!

=-=-=

You will get this long message if you tried to wipe DRLOCK from memory

and something unexpected happened.  This can happen often if you were running

BASIC to poke values into memory without checking to see if you are allowed

to do so by consulting with DOS.  DRLOCK itself consults with DOS and should

not by itself cause this error.

=-=-=

DRLOCK was run at ...........

If the "run time" displayed at wipe out time does not match this,

then someone has turned off the machine and tampered with it.

Note the time on a piece of paper and carry it with you.

=-=-=

This message appears when you first make DRLOCK memory resident.  Note

the time somewhere and check the time against the time displayed when you

wipe or at least try to wipe DRLOCK from memory.  If the two times do not

match exactly, then it implies that someone turned off your machine and did

something with it.

The time is given in the 24 hour format such that 23:12:45.00 is 45

seconds past 11:12 PM with the '.00' representing the 1/100 th of a second.

Some clones may consistently give '.00' for the 1/100 th of a second.

=-=-=

This was turned on at ...........

This is locked and cannot be wiped from memory

=-=-=

If you try to wipe DRLOCK from memory but DRLOCK was locked with the 'L'

parameter, you get this message.  This can be defeated by turning off the

machine and running the program over again, but anyone who does will have to

change the time, match it exactly and go through a few other time consuming

steps to try to fool you.  If you had a password to pop up the menu, and it

no longer pops up, then you know that someone was playing with your machine.

=-=-=

Run time was ...........

Wipe time was ...........

=-=-=

If DRLOCK was not locked and can come out of memory, then you get this

message.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

All products and names mentioned are Trademarks or Registered Trademarks of

their respective corporations or companies.

All enclosed programs, documents and other files are provided AS IS, without

any warranty, expressed or implied, including but not limited to fitness for

a particular purpose.

A contribution of $10 US/Canadian is appreciated if you find this useful, or

$20 for an improved one which wipes out any and all programs (not just mine

or associate(s)) loaded after this, among other features.  (This later uses

more memory, of course)

Your negative criticisms are also welcome.

P.S.  The program keeps track of when the first 100 attempts to access the

password happened at, but this version does not display the times.

Dr. Masaaki Sawada & Assoc.

University of Waterloo, Faculty of Science

Waterloo, Ontario  Canada  N2L 3G1

No school time nor equipments were used in producing this program and

documentation.