Introduction

CompuServe has implemented a version of the XMODEM protocol that

allows most XMODEM users to successfully transfer files securely between

their micros and CompuServe's hosts.  This implementation was not and

cannot be 100% faithful to the original XMODEM specification (see

reference [1]), and the differences between micro to micro XMODEM and

micro to host XMODEM has caused some people problems.  The purpose of

this short article is to describe, in detail, the most frequent problems

people have with micro to host XMODEM, and to offer some suggestions on

how to overcome these problems.

XMODEM's origins

The XMODEM (aka MODEM7) protocol was originally devised by Ward

Christensen as a protocol for communicatons between microcomputers.  As

it was originally devised, the user runs a program called "MODEM", and

dials up another computer. The user then instructs the remote computer

to run a program called "XMODEM".  XMODEM and MODEM then use the XMODEM

protocol to transfer files between the two computers.

The XMODEM protocol has several assumptions implicit in its design,

and these assumptions are the source of the problems people have in

using XMODEM protocol on CompuServe.  Some of these assumptions are:

o File lengths are exact multiples of 128 bytes.

o Word size is 8 bits on both computers.

o Data is transmitted 8 bits, no parity, one stop bit.

o Both computers are dedicated (single user) microcomputers.

o Both computers are talking to each other directly over a phone line.

The XMODEM protocol

The XMODEM protocol is a half duplex protocol -- infomation travels in

only one direction at once.  The basic protocol works something like

this:

1.  When the receiver is

ready to start

receiving data, it

transmits a <nak>

(negative

acknowledgment)

character to the

sender, and continues

to do so every 10

seconds until:

2.  The sender sends a

block.  Each block

contains the block

number (modulo 256),

128 bytes of data, and

a checksum.  Once the

block is sent, the

sender waits while:

3.  The receiver verifies

that he received the

block correctly;  if

the received checksum

matches, then the

receiver sends an <ack>

character, and the

sender sends the next

128 bytes of the file

(step 2).  If the block

was not received

correctly -- or if 10

seconds expires before

a complete block is

received, the receiver

sends a <nak> character

to the sender.  When

the sender receives a

<nak>, it re-transmits

the last block.

4.  Steps 2 3 are repeated

until the end of the

file.  At the end of

the file, the sender

sends an <eot>

character instead of a

block of data.

5.  When the receiver sees

the <eot>, it sends an

<ack> to the sender,

and then the file

transmission process is

complete.

There is a variant of the XMODEM protocol that CompuServe also

supports, called the CRC option.  In the original XMODEM protocol, the

checksum of each block was only one byte long. This small of a checksum

(along with the algorithm used to generate it) allows errors to easily

sneak into a supposedly error free protocol.

The CRC option works roughly like this:  Instead of the receiver

sending out <nak>s at the very beginning, it sends out the letter "C".

If the sender has the CRC option, it treats this just like a <nak>, but

knows to compute the two-byte CCITT CRC-16 and add it to the end of all

blocks instead of the one byte checksum.  If the sender does not have

the CRC option, then the "C" character has no meaning to it and is

ignored.  The receiver may try about three "C"s before deciding that the

sender will not handle CRCs, and then the receiver transmits a <nak> to

start a checksum-XMODEM file transfer.

1.  XMODEM Problems on CompuServe

Because of the assumptions made in the XMODEM protocol, various

difficulties arise in using XMODEM over CompuServe. Here are some major

classes of problems:

XMODEM Timing

Since XMODEM was originally devised for micro-to-micro communications,

it has some problems fitting into an environment where one of the

computers happens to be a mainframe and the two computers are not

connected over a simple phone line but via a packet switching network.

CompuServe's network can often introduce long delays in what should be

continuous communications, as can a very busy host.

When these delays arise, the microcomputer running XMODEM may mistake

them for a breakdown in communications.  If a long delay occurs in the

middle of a packet -- in the middle of the data -- many XMODEM

implementations see that as meaning there has been data lost.  These

delays result in the following symptoms:

o The receiver cannot successfully receive even the first block of

data.

o The receiver consistently <nak>s blocks or <nak>s a majority of

blocks, although eventually receives the file.

o The receiver just suddenly gives up in the middle of a seemingly

successful file transfer.

o The receiver has great difficultly in receiving the file, and gives

up anywhere from 5 to 20 blocks through the transfer.

o The file transfer proceeds very smoothly with several successfully

transfered blocks when all of a sudden every block transmitted is

rejected (<nak>ed) by the host.

Note that all of these problems are indistinguishable from other

communications problems (e.g., noisy phone lines, bad parity settings).

A key question to ask people who are having XMODEM problems is "Have you

ever successfully done a XMODEM transfer to CompuServe?"  If the answer

is yes, then the following problems may be at fault.

Delays within blocks

The XMODEM documentation recomends that a gap of more than 1 second be

treated as a breakdown of communications.  On a microcomputer, this

makes sense -- there is no reason for a dedicated micro computer to

suddenly stop transmitting and then resume.  But on CompuServe's systems

and network, a 1 second delay is not only possible, but happens

frequently!

If the microcomputer decides that this delay is a breakdown of some

sort, it will send a <nak> to CompuServe, requesting that the block be

retransmitted. Unfortunately, the rest of the first block is still "in

the pipe" -- and when the receiver gets this block the receiver may do

anything from abort the transfer to ignore it.  Assuming the receiver

handles the rest of the block gracefully (and the rest of the block

doesn't look like the start of another block), the host will see the

<nak> the receiver sent and retransmit the block.  If there's a delay in

the middle of that transmission, the receiver will <nak> it too, and so

on forever.

Thus, when the receiver is too sensitive to delays in the middle of

transmissions, either the receiver gives up very quickly or the receiver

<nak>s every block transmitted.

Delays between blocks

Fortunately, most implementations of the XMODEM protocol aren't that

sensitive to delays in the middle of blocks.  But other delays can cause

problems too.  When a block is received by the microcomputer, for

example, the micro will almost immediately <ack> or <nak> the block and

wait for the next block to be sent.  The XMODEM specification [1] states

that if the next block does not arrive within 10 seconds, the receiver

should send its <ack> or <nak> again.

Ten seconds is not a very long time, however, when the network and/or

mainframe is heavily loaded.  Often, what will happen is that the <ack>

or <nak> will arrive at the host, the host will begin transmission of

the next block, but the block won't make it to the micro in time. Then,

the micro transmits a <nak>, even though the host has correctly received

the first <ack>/<nak>.

What happens, then, is that a <nak> enters the network, even though

the host has seen and responded to the first <ack> or <nak>.  After

that, things can easily degrade.  The host may find itself one block

ahead of the receiver, causing the receiver to give up on the spot. The

host may find itself stuck one <ack> or <nak> behind the receiver, which

could result in a successful, although very difficult, file transfer.

And the network problems could be severe enough -- or the micro

sensitive enough -- that the micro goes through about 6 or so blocks

(mostly <nak>s with a few <ack>s) and gives up.

Network Flow Control

When the user is sending data to CompuServe, the node will send an

XOFF (S) character to the micro if the micro is transmitting faster than

the node can send the data to the host.  This happens even during an

XMODEM transfer (although very infrequently).  If the micro

misinterprets the S as a <nak>, it will immediately retransmit the block

-- causing the network to send even more Ses and the micro to see even

more naks.  Usually what happens is that the micro thinks that every

block it transmits is being <nak>ed, and the micro gives up almost

immediately. This problem can strike anytime during an upload to the

host, and things can be proceeding very smoothly upto that point.

Timing suggestions and hints

To overcome and avoid these problems, an XMODEM protocol

implementation should wait 20 seconds before sending a <nak> on a lost

block, wait at least 10 seconds during delays in the middle of a block,

and honor (or at least ignore) flow control from the node during

uploading of files.

Many programs which have timing problems with CompuServe can be

patched to relax their timing restrictions.  A typical place to look is

for calls to a "timed input" routine (where the routine will only wait

so long for a character to arrive). Either patching that routine (make

it wait twice as long, perhaps) or patching the code that calls the

routine (to use a longer time out value) would help to make the XMODEM

work on CompuServe.

Host file Format Concerns

While almost all microcomputers have an 8 bit word (or multiple of 8

bits), CompuServe has a 36 bit word. Text files are stored with 7 bit

bytes (5 of them per word). While this poses no problems for pure text

files (as the ASCII character set is defined for only 7 bits of data),

"binary" files cannot be stored as is on our host systems (an example of

binary data would be an executable program, or a WordStar file).  Files

in which 8 bits of data per byte must be preserved -- binary files --

are stored in a special file format (currently Intel hex format).

Thus, when people upload or download to CompuServe using XMODEM, they

are asked:

Transfer types available,

1 - ASCII  (7 - bit)

2 - Binary (8 - bit)

Please make a selection:

People often pick the wrong answer, the results are either an

unsuccessful or useless upload or download.

The correct approach

For uploading, the choice of a transfer type can be as simple as:  "If

it's a text file, use ASCII.  If it's not, use binary."

For downloading, the correct transfer type is the same one that was

used to upload the file, i.e., if the file was uploaded with an ASCII

transfer it must be downloaded with an ASCII transfer;  if the file was

uploaded with a binary transfer it must be downloaded with a binary

transfer.

At the moment, it is a problem for users to determine which type the

file was uploaded in. Many SYSOPs have the file's description specify

the correct approach for downloading.  Other SYSOPs enforce the

convention that files with a ".BIN" extension should be downloaded with

the binary transfer while other files should use the ASCII transfer.  It

is best to ask what policy is in use if there is some doubt.

In the future, the SIG program will be improved so that the choice of

which transfer type won't be asked during a download. bl

What happens if...

The user downloads a binary file with ASCII mode?

The user gets the Intel hex format of the file.  The downloaded file

can be salvaged by use of a program to convert Intel hex format back to

binary. Currently, the IBM Novice forum (PCS-129) has such a program

available.

The user downloads an ASCII file in binary mode?

The user usually receives one or two blocks of garbage data, at best.

The user uploads a binary file in ASCII mode?

The file transfer will not succeed, and any files left over after the

upload will be useless.

The user uploads an ASCII file in binary mode?

The file transfer will succeed, but the uploaded file will be encoded

into Intel hex

format.  Thus, any people who downloaded the file in ASCII mode will

have the hex representation of the file.  People who download the file

in Binary mode, however will be able to receive the file correctly.

PC-TALK III

PC-Talk will work fine with CompuServe, so long as the user is running

PC-Talk in 8 bits / no parity mode.  The most recent version(s) of

PC-Talk are set up to switch to 8 bits / no parity automatically on an

XMODEM transfer, but these versions are still rare in the user

community.

By default, CompuServe communicates in 7 bits, even parity.  Thus,

before PC-Talk can be thrown into 8 bits no partiy, the user must tell

CompuServe to talk 8 bits no parity.

The steps involved in making this change are as follows:

1.  The user must log into

CompuServe as usual

(i.e., 7 bits even

parity).

2.  The user must then run

the Default program.

This program can be

reached either by

typing "R DEFALT" from

command mode in the

personal file area (the

"OK" prompt), or by

typing "GO DEFALT" from

any "!"  prompt.

3.  In Default, the user

should select option

5, "View or Change

current terminal

parameters."  Then, he

should select option

8, "Parity is", change

parity to ZERO, and

then exit the default

program, making all

changes permanent.

4.  At this point, PC-Talk

can be switched into 8

bits no parity.  On

some modems (but not

most of the Hayes line

of modems), doing this

switch will cause the

modem to hang up.

Thus, it may be

advisible to log off

CompuServe before

switching to 8 bits no

parity.

5.  Once PC-Talk is running

in 8 bit mode, it is

possible to easily

upload and download

using XMODEM.

The next time the user dials into CompuServe (with PC Talk set to 8

bits no parity) and types control-c to catch the system's attention, the

"User ID:"  prompt will be garbled. Once the system recognizes the User

ID, however, the password prompt will be presented in clear text.  Thus,

users should just muddle through the garbled User ID prompt and

everything will work well after that.

If the user's modem does not hang up when the parity setting is

changed, it is also possible to change to 8 bits no parity just before

performing an XMODEM transfer, and change back to 7 bits even parity

when it completes.  This is a rather involved, manual process, though,

and is not recomended.

Note:  Putting PC-Talk into 8 bits no parity is necessary even to

upload or download ASCII files.