====================================

The Amstrad CPC emulator (v0.3beta).

====================================

The emulator demands the following to run:

- i386sx or compatible

- 101-keys 'US', or 102-keys 'GR', 'NL', 'SP' or 'UK' keyboard

- VGA or (register) compatible

- DOS 2.00 or higher, or Windows'95 in 'MS-DOS' mode.

- 160Kb free conventional memory

-------------

The emulator:

-------------

The emulator doesn't need parameters, so start it up with:

CPC [Enter]

!! If anything could go wrong, you ALWAYS can terminate the program !!

!!    by holding down the [F12] key for at least 1/4 of a second     !!

Of course there are many differences between the CPC and the PC:

- Video

Cause the display of the PC gives a much sharper picture than the

CPC's, all CPC's display lines are showed twice on the PC's display.

This results in a 'squared' picture. A higher resolution (I prefer

800x600) undoes this a little.

- Keyboard

The CPC's keyboard layout is (much) different from that of the PC.

The emulator translates the symbols(!) on the PC keyboard to

appropriate scancodes on the CPC. If the PC keyboard doesn't have a

Pound-sterling symbol (), another symbol is used which doesn't resides

on the CPC's keyboard. For the US keyboard, I've chosen the tilde (~).

The numeric part of the CPC is often called 'Function keys'. That's

why the emulator sees no difference between the numeric keys and

the [F1] - [F10] keys on the PC keyboard.

The PC keyboard doesn't have a [Copy] or [Clr] key. I've chosen the

'grey' [Insert] and [Delete] key to represent them (sounded most

logical to me).

The CPC doesn't have a LED which shows the status of the [Caps Lock].

That's why the emulator doesn't control the PC's keyboard LEDs.

The [F11] key switches the emulator between 100% and full speed

operation. With the [F12] key (when hold down for at least 1/4 of

a second) the emulator terminates.

----------------------

Possible errormessages

----------------------

...........

At start-up:

...........

- '80x86 : Requires at least an i386sx'

Sorry, the emulator needs at least an AT-386(sx), and so doesn't work

on (simulated) XT's and AT-286s.

- 'V86 : Cannot run on a virtual machine'

The emulator has its own operating system, and therefor cannot be run

from another OS except DOS and Windows'95 in 'MS-DOS' mode.

PC/XT simulators like Emm386 and Qemm also prevent the emulator to

start.

The emulator doesn't have any problems with memory managers like HIMEM,

UMB drivers (not simulators) and background programs (a.k.a TSRs).

- 'File cpc.rom not found'

- 'File cpc.cfg not found'

The files HAVE TO BE in the CURRENT directory. The emulator NEEDS

these files.

- 'Error reading file cpc.rom'

- 'Error reading file cpc.cfg'

Uh oh, something went wrong reading these files. The file CPC.ROM

has to be 32Kb exactly!!

- 'Expansion ROM file not found'

In the configuration file (CPC.CFG) an extra ROM is mentioned, but

could be found (in the current directory).

- 'Out of base memory'

After initialisation and loading the extra ROMs, there isn't enough

RAM anymore to emulate the CPC's RAM.

.................

During emulation:

.................

- '80x86 : Exception x occurred!!'  (x = a number between 0 and 18)

The is the largest possible error. Normally, this error SHOULD not

occur at any time. This error is only given when there's something

good wrong with the system or emulator.

- 'V86 : Invalid OP-Code'

For compatibility reasons, this version of the emulator tries to

run background programs (like the DOS clock, SMARTDRV, MOUSE, etc.)

in a 'virtual' DOS environment. Apparently, one of those programs

messed up.

!!! ATTENTION !!! This COULD be a VIRUS !!!

--------------------------------

The configuration file (cpc.cfg):

--------------------------------

For your (and my) convenience, I've already put all options in

the configuration file. Normally you don't have to edit it (much).

The emulator ignores lines beginning with a '#', and lines it cannot

recognise.

If a line starts with a number, it expects the filename of an extra

ROM hereafter. This ROM will be placed at slot <number>.

E.g.

7  vdos10.rom

will place the contents of the file VDOS10.ROM at slot 7.

Beware:

- A ROM-file may ONLY contain the contents of a ROM. Headers and/or

header-info are not allowed.

- A ROM-file may NOT be larger than 16Kb (16384 bytes).

Other options:

- JOY {0|1}

With this option you can enable the use of a PC joystick. Beware

that reading the positions of a joystick takes (much) time.

If you don't need the joystick, then you're better off keeping it

disabled.

If you have a 'speed-compensating' joystick port, then try to set

it to a as slow as possible setting. This will (oddly) speed up

the reading.

- KDB nn

If you have a keyboard layout other than 'US', than here is the place

to set yours. The emulator expects a keyboard code for 'nn'. These

codes are the same as for DOS's KEYB command (see DOS manual).

At the moment this is written, I have made keyboard definitions for:

GR, NL, SP and UK.

You can order other definitions.

- LPT {0|1|2}

In most cases, you have attached a printer to LPT1. If you have

a printer attached to the printer port of an IBM MDA or Hercules HGC,

you'll have to use:

LPT 0

- VDM x

The video mode!!! Normally the emulator will use mode 14 (640x200,

16 colours), which is enough to emulate all possible documented modes

on one screen.

To have better aspects, you can use mode 18 (640x480, 16 colours).

At a higher resolution (800x600, 16 colours), the emulator will use

the 'extra' space for a nice border as you expect from a real CPC.

Although controlling a 800x600 mode works the same at all SVGAs,

setting it up works very different to each one. At most times you

can use mode 106 or 'VESA' mode 258.

For your convenience, I've made a list of some common used SVGAs.

Most SVGA's                     :       106

VESA compatible SVGA's          :       258

ATI (Graphics Ultra/VGA Wonder) :        84, 106

Cirrus Logic                    :       100, 106

Genoa (6x00)                    :       106, 121(?)

NCR (77C2?)                     :        88

OAK (OTI 067/077/087)           :        82, 106

Paradise/WD90C??                :        88, 106

S3 (86C???)                     :       106, 258

Trident (8800/8900/9000)        :        91

Tseng (ET3000/ET4000)           :        41, 106(ET4000)

Weitek (W5?86/P9000/P9100)      :        88, 106

If your SVGA isn't mentioned here, you can look up the mode number

in your SVGA's manual.

Beware, these modes are mostly given with hexadecimal numbers, the

emulator expects (normal) decimal numbers.

-------------------

The disc utilities:

-------------------

The disc utilities are mode to transfer CPC files to and from the PC.

Beware that the utilities are very basic at this moment.

- CAT [A|B]

CAT works as expected, but doesn't give free disc space.

- ERA [A|B]:<filename>

ERA can ONLY erase a file from a VDOS disc. Wildcards

are not allowed yet, sorry.

- GET [A|B]:<filename>

GET copies a file (AMSDOS AND VDOS) to the CURRENT directory.

As with ERA, no wildcards are allowed.

- PUT [A|B]:<filename>

PUT copies a file from the CURRENT directory to a VDOS disc.

Beware that the parameter is the DESTINATION file.

Wildcards are not allowed. However, you can do a lot with

DOS's FOR command. E.g.:

C:\CPC\TMP> for %a in (*.bin) do put a:%a

Copies all .BIN files from the current directory to drive A:

- VORMAT [A|B]

VORMAT formats a disc using the VDOS format (720Kb).

Only double density discs can be used, and the drive

has to have 80 tracks.

You actually can use 5.25" DD discs (normally 360Kb),

and format them to 720Kb using a HD (normally 1.2Mb) drive.

The emulator will read those discs correctly.

- XDSK <filespec>

This program is meant to extract CPC files from a .DSK file,

so you can use PUT and VORMAT to copy them to a disc.

This program is made within hours after I got the first .DSK

file, and therefor can only use 180Kb files (yet)!!