%OP%JUN%OP%TM1%OP%HM2%OP%FM2%OP%BM1%OP%LM5%OP%HE/%H2%Z88 Users' Club Software Library%H2%/%OP%FO/%H2%Z109%H2%//@P@/%CO:A,12,72%                         %H1%%H2%MANUAL Z88 FORTH v 2.0                   %H2%Copyright (c) 1990 Salvador Merino%H2%%H1%IntroductionZ88 Forth is based in the Book Threaded Interpretive Languages by R.G. Loeliger (1981 BYTE Publications Inc).I have developed my Z88 FORTH  in one of my three SINCLAIR QL with the help of a Editor and a Assembler Z80 for QL (Quantum Z80 by Marcos Cruz, Acacias 44, 28023 MADRID (SPAIN)). The following files are enclosed for the Z88 Software Library:- FORTH.BAS.- Basic Loader.- FORTH.CDE.- FORTH Code.- Forth.doc & Update.doc - my poor Z88 Manual.- ASM.FTH.- Z80 FORTH ASSEMBLER.The following descriptions have the format:(A) Function(B) Input/Output(C) Usage%H1%%H2%Arithmetic Keywords                                   %H2%*%JL%A) Does a signed multiply of the second stack word by the low-order %JR%byte of the top stack entry and replaces both entries by the 16-bit (word) product. B) Two Stack entries/One stack entry C) Signed integer arithmetic.                                  %H2%*/ %JR%A) Does a signed multiply of the third stack word by the low-order byte %JR%of the second stack word and signed divide of the 24-bit product by the %JR%low-order byte of the top stack entry. Replaces the three entries with the 16-bit quotient.  B) Three stack entries/One stack entry. Signed C) integer arithmetic.                                %H2%*/MOD%JR%A) Does a signed multiply of the third stack word by the low-order byte %JR%of the second stack entry and a signed divide 24-bit product by the %JL%low-order byte of the top stack entry. Replaces the three entries with %JR%the 16-bit quotient as the second and the 8-bit residual as the top stack entry. B)Three stack entries/Two stack entries.%P0%                                  %H2%+           %JL%A) Adds the second stack entry and the top stack entry and replaces both with the single two's complement sum as the top stack entry. B) Two stack entries/One stack entry.C) Signed arithmetic.                                   %H2%-%JL%A) Pops the top two stack entries and the two's complement subtracts the top entry from the second entry and pushes the result.B) Two stack entries/One stack entry.                                   %H2%/%JR%A) Does a signed divide of the second stack word by the low-order byte %JR%of the top stack entry. Replaces both entries with an 8-bit quotient expanded to 16 bits.B) Two stack entries/One stack entry.C) Signed integer arithmetic.                                %H2%/MOD%JR%A) Does a signed divide of the second stack entry by the low-order byte %JR%of the top stack entry. Replaces these entries with the 8-bit quotient expanded to 16 bits as the top entry.B) Two stack entries/Two stack entries.C) Signed integer arithmetic.                                   %H2%1+A) Increment th top stack entry by 1.B) One stack entry/One stack entry.C) Signed arithmetic, byte addressing and index                                   %H2%1-   A) Decrements the top stack entry by 1.B) One stack entry/One stack entry.C) Signed arithmetic, byte addressing and index decrementing.                                  %H2%2* A) Multiplies the top stack entry by 2.B) One stack entry/One stack entry.C) Signed integer arithmetic.                                  %H2%2+A) Increments the word at the top of the stack by 2.B) One stack entry/One stack  entry.C) Word addressing and incrementing.                                  %H2%2-A) Decrements the word at the top of the stack by 2.B) One stack entry/One stack entry.C) Word addressing and decrementing.                                 %H2%2/A) Divide (signed) the top stack word by 2.B) One stack entry/One stack entry.C) Signed integer arithmetic.                                  %H2%ABS%JL%A) if the top stack entry is two's complement negative, its two's %JR%complement (a positive integer) is returned to the stack. otherwise the original positive integer is returned to the stack.B) one stack entry/One stack entry.                                  %H2%D*%JL%A) Does a signed multiply of the second stack word by the low-order %JR%byte of the top entry and replace both entries by the 24-bit product %JL%with the 8 most significant  bits sign extended as the second stack entry and the 16 least significant bits as the top stack entry.B) Two stack entries/Two stack entries.C) Signed integer arithmetic.                                 %H2%D/MOD%JR%A) Does a signed divide of the 24-bit number in the third (16 least %JL%significant bits) and second (8 most significant bits) stack entries by %JR%the low-order byte of the top stack entry. Replace these entries with %JL%the 16 bit quotient as the second stack entry and the positive 8-bit remainder expanded to 16 bits as the top entry.B) Three stack entries/ Two stack entries.C) Signed Integer arithmetic.                                 %H2%MAX%JR%A) Replaces the top two stack entries by the entry with the higher value (signed).B) Two stack entries/One stack entry.C) Signed integer arithmetic tests.                                 %H2%MIN%JL%A) Replaces the top stack entries with the entry with the smaller value (signed).B) Two stack entries/One stack entry. C) Signed integer arithmetic tests.                                 %H2%MINUSA) replaces the top stack entry with its two's complement.B) One stack entry/One stack entry.C) Signed integer arithmetic.%P0%                                 %H2%MOD%JR%A) Does a signed divide of the second stack word by the low-order byte %JL%of the top stack entry. Replace both entries with the 8-bit remainder expanded to 16 bits.B) Two stack entries/One stack entry.C) Signed integer arithmetic.                                %H2%MODU/%JR%A) Does a signed divide of the second stack entry by the low-order byte %JL%of the top stack entry. Replaces both entries with the positive 8-bit remainder expanded to 16 bits as the second stack entry. B) Two stack entries/two stack entries.                                  %H2%S*%JR%A) Does a signed multiply of the low-order byte of the second stack %JL%entry by the low-order byte of the top stack entry and replaces both entries by the 16-bit product.B)Two stack entries/one stack entry.C) Signed integer arithmetic.%H1%%H2%Compiler Directives                               %H2%+LOOP   %JR%A) Adds the word address of the program control directive *+LOOP to the %JL%dictionary, then computes the difference between the current free %JR%dictionary address and the address at the top of the stack and encloses the low-order byte in the dictionary as the relative jump byte.B) One entry/none.C) Used to terminate DO...+LOOP construct in the compile mode.                                %H2%BEGIN%JL%A) Pushes to the stack the address of the next available free dictionary location.B) None/One stack entry.C) initiates a BEGIN...END loop in the compile mode.                                %H2%C+LOOP%JR%A) Adds the word address of the program control directive *C+LOOP to %JL%the dictionary, then computes the difference between the current free %JR%dictionary address and the address at the top of the stack and encloses the low-order byte in the dictionary as the relative jump byte.B) One stack entry/None.C) Used to terminate a CDO....C+LOOP construct in the compile mode.                                 %H2%CDO%JL%A) Encloses the word address of the program control directive *CDO in %JR%the dictionary and then pushes the address of the next free dictionary location to the stack.%JL%B) Used to initiate a CDO...CLOOP or CDO...C+LOOP construct in the compile mode.                                %H2%CLEAVE%JR%A) Enclose the word address of the program control directive *CLEAVE in the dictionary.B) None/None.%JL%C) Compiles a command to cause an immediate exit from a byte loop construct at execution time. Used within a conditional branch structure.                               %H2%CLOOP%JR%A) Adds  the word address of the program control directive *CLOOP to %JL%the dictionary, then computes the difference between the current free %JR%dictionary address and the address at the top of the stack and encloses the low-order byte in the dictionary.B) One stack entry/None.C) Used to terminate a CDO...CLOOP construct in the compile mode.                                  %H2%DO %JL%A) Enclose the word address of the program control directive *DO in the %JR%dictionary and then pushes to the stack the address of the next free dictionary location.B) None/One stack entry. %JL%C) Used to initiate a DO...LOOP or DO....+LOOP construct in the compile mode.                                 %H2%ELSE%JR%A) Encloses the word address of the program control directive *ELSE in %JL%the dictionary, saves the address of the next free dictionary location %JR%on the stack, reserves 1 byte in the dictionary, swaps the top  two %JL%stack entry, computes the difference between the top stack entry and %JR%the current free dictionary location and encloses the low-order byte in the dictionary as a relative jump byte.B) One stack entry/One stack entry%JL%C) Used to terminate the true code portion in an IF....ELSE...THEN construct in the compile mode.                                %H2%END%JR%A) Encloses the address of the program control directive at the top of %JL%the stack in the dictionary, computes the relative jump byte using the %JR%top entry and the current free dictionary location and encloses the low-order byte in the dictionary.B) Two stack entries/None.C) Used in

defining compiler immediate keywords.                                  %H2%IF  %JL%A) Encloses the word address of the program control directive *IF in %JR%the dictionary, pushes the address of the next free dictionary %JL%location to the stack and reserves 1 byte in the dictionary for relative jump byte.B) None/One stack entry.C) Used to initiate a conditional branch construct in the compile mode.%P0%                                 %H2%LEAVE %JR%A) Encloses the word address of the program control  directive *LEAVE in the dictionary.B) None/None.%JL%C) Compile a command to cause an immediate exit from a word-length loop construct execution time. Used within a conditional branch construct.                                 %H2%LOOP%JR%A) Encloses the word address of the program control directive *LOOP in %JL%the dictionary, then pops the stacks and computes the difference between %JR%this address and the next free dictionary address and encloses the low-order byte in the dictionary as the relative jump byte.B) One stack entry/None.C) Used to terminate a DO....LOOP construct in the compile mode.                                 %H2%THEN%JL%A) Pops the address from the stack, computes the difference between %JR%this address and the current free dictionary location as the relative %JL%jump byte, and stores the byte at the address popped from the stack initially.C) Used to terminate a branch construct in the compile mode.                                 %H2%WHILE%JR%A) Encloses the word address of the program control directive *WHILE in %JL%the dictionary. The top two stack entries are swapped, the top stack %JR%entry is popped. the offset from this address to the current free %JL%dictionary location is enclosed in the dictionary as a relative jump %JR%byte. The function also pops the top stack entry, computes the %JL%difference between this address and the current free dictionary location %JR%and stores the low-order byte to the address popped from the stack as a relative jump byte.B) Two stack entries/None.C) Used to terminate a loop construct containing a *WHILE.                                  %H2%."  %JL%A) Encloses the literal handler *[ in the dictionary, changes the token %JR%separator to " and scans the next token from the input buffer and then encloses the token in extended header format in the dictionary.B) None/none. %JL%C) Compiling literal strings into secondary keywords or display formatting.%H1%%H2%Compile Mode Terminators                                   %H2%;%JR%A) Encloses the word address of the inner interpreter SEMI routine in the dictionary and sets the system mode to the execute mode.B) None/None.%JL%C) Terminates the definition of a secondary and re-establishes the execute mode.                                %H2%;CODE %JR%A) Encloses the word address of the keyword SCODE in the dictionary and sets the system mode to execute.B) None/None.%JL%C) Terminates a defining keyword definition. Always followed by generic %JR%machine code that defines the execution time action of the defined class.%H1%%H2%Defining Words                                   %H2%:%JL%A) Sets the CONTEXT vocabulary equal to the CURRENT vocabulary, creates %JR%a secondary header for the token following ":" in the input buffer and %JL%links it to the CURRENT vocabulary and sets the system mode to the compile mode.B) None/None.C) Initiate compilation of secondary keywords.                                 %H2%<BUILDS %JR%A) Creates a CONSTANT keyword definition with an initial value of 0.The %JL%keyword name is the next available token  in the input buffer when  <BUILDS is executed.B) None/None.%JR%C) Used to initiate a high-level defining word which must later be terminated with a DOES>.                                          %H2%CCONSTANT  %JL%A) Creates a byte constant keyword dictionary entry whose name is the %JR%token following CCONSTANT and whose value equals the low-order byte of the top stack entry.B) One stack entry/None.C) Defining byte-length named constants.                                %H2%CONSTANT%JL%A) Creates a word-length constant keyword dictionary entry whose name %JR%is the token following CONSTANT and  whose value equals the top stack entry.B) One stack entry/None.C) Defining word-length named constants.                                %H2%CREATE%JL%A) Creates a dictionary header for a primitive keyword whose name is the token following CREATE and links it to the CURRENT vocabulary.B) None/None.C) Used to create all dictionary headers.%P0%                               %H2%CVARIABLE     %JR%A) Creates a byte variable keyword dictionary entry whose name is the %JL%token following CVARIABLE and whose initial value is the low-order byte of the entry popped from the stack.B) One stack entry/None.C) Defining byte-length of the entry popped from the stack.                               %H2%VARIABLE%JR%A) Creates a word-length variable dictionary keyword entry whose name %JL%is the token following VARIABLE and whose initial value is the value popped from the stack.B) One stack entry/None.C) Defining and initializing word-length named variables.                             %H2%VOCABULARY  %JR%A) Creates a vocabulary keyword dictionary entry whose name is the %JL%token following VOCABULARY, with an initial link to the last entry in %JR%the CURRENT vocabulary. and which, when the vocabulary name is executed, sets the system variable CONTEXT to the link address.B) None/None.C) Defining vocabularies.%H1%%H2%I/O                                   %H2%#%JL%A) Pops the top stack entry, computes the quotient and remainder %JR%relative to the system number base, converts the remainder to an ASCII %JL%character (0 Thru 9, A thru Z), pushes the character, then pushes the quotient.B) One stack entry/Two stack entries.%JR%C) Does one conversion in the process of generating formatted display outputs.                                  %H2%#>%JL%A) Pops the sigh byte left on the return stack by <#, discards it and %JR%then displays the string on the stack using the DISPLAY format convention. B) One return stack entry and a variable length stack string/None.C) Display formatted strings built onto the stack.                                  %H2%#S %JL%A) Converts the top stack entry to a sequence of ASCII characters %JR%equivalent to the entry given the current system number base. %JL%Sequentially pushes the number characters with the most significant character to the top stack entry. B) One stack entry/One to sixteen stack entries.C) Converting numbers to a display string.%P0%                                   %H2%.%JR%A) Displays the top entry number to the operator (given the current %JL%number base) and follows by space. destroys the top stack entry in the process.B) One stack entry/None.C) Displaying signed numbers to the operator.                                 %H2%<#%JR%A) Pops the top stack entry, pushes an ASCII space code with its %JL%high-order  bit set to the stack, restores the top stack entry and copies the high-order byte to the return stack.B) One stack entry/Two stack entries and one return stack byte entry.%JR%C) Prepares for number conversion and display by  saving the number %JL%sign on the return stack, pushing the string termination character to the stack and leaving the original top stack entry on the top.                                  %H2%?%JR%A) Displays to the operator (using the current system number base) the %JL%word whose address is the top stack entry. Number display is always followed by a space.B) One stack entry/None.%JR%C) Displaying signed numbers to the operator, generally the contents of variables.                               %H2%APART%JL%A) Displays eight characters of ASCII using the top entry as a pointer. %JR%The pointer is incremented with each character access. ASCII control code is converted to a displayable form before being echoed.B) One stack entry/One stack entry.C) Displaying  memory.                                 %H2%ASCII%JL%A) Converts the low-order byte of the top stack entry from a binary number to an ASCII code in the set 0 thru 9. A thru Z.B) One stack entry/One stack entry.%JR%C) Converts binary numbers to their equivalent ASCII code for conversion to displayable formats.                                  %H2%C?%JL%A) Displays to the operator (using the current system number base) the %JR%byte whose address is popped from the stack. The number is always followed by a space.B) One stack entry/None.%JL%C) Displaying signed numbers to the operator; generally the contents of byte variables or constants.                                %H2%CLEAR        A) Clears the CRT display and home the cursor.B) None/None.C) Control of display formatting.                                 %H2%CRA) Issues a carriage-return line-feed sequence to the display.B) None/None.C) Display formatting.                               %H2%DISPLAY%JR%A) Outputs to the display the low-order byte of successive top stack %JL%entries until a non-ASCII code is output (a character with the high-order bit 1 set).B) One to N stack entries/None.                                 %H2%EMIT     %JR%A) Pops the top stack entry and outputs the low-order byte to the display.B) One stack entry/None.C) Direct Control of the display for formatting.                                 %H2%KEY%JL%A) Pushes to the stack in the low-order byte position the next ASCII code entered via the keyboard.B) None/One stack entry.C) Interfaces the keyboard to the system.                                %H2%SIGN%JR%A) Pushes the ASCII code for a minus sign to the stack (in the low-order

%JL%byte position) if the top byte on the return stack is two's complement negative. %JR%B) One return stack byte entry/One return stack byte entry and zero or one stack entries.%JL%C) Adds a leading negative sign to the stack string number if the %JR%original binary number was negative. Used in designing formatted displays.                                %H2%SPACE      A) Echo displays a space to the display.B) None/None.C) Display formatting.                                 %H2%TYPE%JL%A) Pops the top stack entry which points to a string consisting of a %JR%length argument followed by that many ASCII characters. Outputs the string characters to the display.B) One stack entry/None.C) Display of system messages to the operator.%P0%%H1%%H2%Interstack                                  %H2%R>A) Pops the top stack word and pushes it to the return stack. B) One stack entry/One return stack word entry.%JL%C) Temporary storage of data within a definition or direct return stack control.                                 %H2%C<R%JR%A) Pops the top stack entry and pushes the low-order byte to the return stack.B) One stack entry/One return stack byte entry.%JL%C) Temporary storage of byte data within a definition or direct return stack control.                                 %H2%CI>%JR%A) Pushes to the stack the loop index for the innermost byte-length loop which is the top return stack byte.%JL%B) One return stack byte/One stack entry and one return stack byte entry.C) Retrieval of current byte loop index.                                 %H2%CJ>%JR%A) Pushes to the stack the loop index for the second innermost byte-length loop.%JL%B) Three return stack byte entries/Three return stack entries and one stack entry.C) Retrieval of the next level byte index.                                 %H2%CK>%JR%A) Pushes to the stack the loop index for the third innermost byte-length loop.%JL%B) Five return stack entries/Five return stack entries and one stack entry.C) Retrieval of second next level byte loop index.                                 %H2%CR>%JR%A) Pops the byte at the top of the return stack and pushes it to the stack in sign-extended format.B) One return stack byte entry/One stack entry.%JL%C) Retrieval of temporary data stored on the return stack to the stack in a format compatible with signed 16-bit arithmetic.                                   %H2%I>%JR%A) Pushes to the stack the loop index for the innermost word-length loop which is the top stack word.B) One return stack word/One return stack word and one stack word.C) Retrieval of the current word loop index.%P0%                                  %H2%J>     %JL%A) Pushes to the stack the loop index for the second innermost word-length loop.%JR%B) Three return stack word entries/Three return stack word entries and one stack entry.C) Retrieval of the second level word-length loop index.                             %H2%K>%JL%A) Pushes to the stack the loop index for the third innermost word-length loop.%JR%B) Five return stack word entries/Five return stack word entries and one stack entry.C) Retrieval of the third level word-length loop index.                                 %H2%R>%JL%A) Pops The word at the top of the return stack and pushes it to the stack.B) One return stack word entry/One stack entry.%JR%C) Retrieval of temporary data stored on the return stack or direct control of return stack addresses.%H1%%H2%Logical                                  %H2%AND%JL%A) Pops the top two stack words, does a logical AND of all bits on bit-by-bit basis and pushes the result to the stack.B) Two stack entries/One stack entry. C) Logical operations.                                  %H2%IOR %JR%A) Replaces the top two stack entries  by the logical inclusive or of the entries on a bit-by-bit basis.B) Two stack entries/One stack entry.C) Logical operations.                                 %H2%NOT  A) Inverts the logic state of the flag at the top of the stack.B) One stack entry/One stack entry.C) Inverting the results of the relational test or other flags.                                  %H2%XOR%JL%A) Replaces the top two stack entries  by the logical exclusive or of the entries on a bit-by-bit basis.B) Two stack entries/One stack entry.C) Logical operations.%P0%%H1%%H2%Memory Reference                                   %H2%!%JR%A) Stores second stack entry at the address at the top stack entry, removing both entries.B) Two stack entries/None.C) Storage of word length data in programmable memory.                                  %H2%+!  %JL%A) Pops two stack entries and adds the word at the second entry to the word whose address is the top entry.B) Two stack entries/None.%JR%C) Incrementing and decrementing word length data stored in programmable memory.                                 %H2%0SET%JL%A) Pops the top entry and sets the word whose address was the top entry to 0.B) One stack entry/None.%JR%C) initializing word length data in programmable memory to 0 or setting word length flags in programmable memory to false.                                 %H2%1SET %JL%A) Pops the top stack entry and sets the word whose address was the top entry to one.B) One stack entry/None.%JR%C) Initialising word length data in programmable memory to one or setting word-length flags in programmable memory to true.                                   %H2%@%JL%A) Replaces the address at the top of the stack with the word at that address.B) One stack entry/One stack entry.C) Returns word length data stored in memory.                                  %H2%C!%JR%A) Stores the low-order byte of the second stack entry at the address at the top stack entry, removing both entries.B) Two stack entries/None.C) Storage of byte length data in programmable memory.                                  %H2%C+!%JL%A) Pops two stack entries and adds the byte in the low-order byte of %JR%the second stack entry to the byte whose address is the top stack entry.B) Two stack entries/None.%JL%C) Incrementing/decrementing byte-length data stored in programmable memory.%P0%                                  %H2%C0SET%JR%A) Pops the top stack entry and sets the byte whose address was the top entry to 0.B) One stack entry/None.%JL%C) Initializing byte-length data programmable memory to 0 or setting byte-length flags in programmable memory to false.                                   %H2%C@%JR%A) Replaces the address at the top of the stack with the byte at that address (in sign extended format).B) One stack entry/One stack entry. %JL%C) Returns byte-length data stored in memory in format compatible with 16-bit signed arithmetic.%H1%%H2%Relational                                   %H2%0<%JR%A) If the top stack entry is two's complement negative, it is replaced by a true flag. Otherwise it is replaced by a false flag.B) One stack entry/One stack entry.C) Test conditioning prior to branching.                                  %H2%0=%JL%A) If the top value is 0, it is replaced by a true flag. Otherwise it is replaced by a false flag.B) One stack entry/one stack entry.C) Test conditioning prior to branching.                                   %H2%<%JR%A) If the second stack entry is less than the top entry, both entries %JL%are replaced by a true flag. Otherwise both are replaced by a false flag.B) Two stack entries/One stack entry.C) Test conditioning prior to branching.                                   %H2%=%JR%A) If the top entries are equal, both are replaced by a true flag. Otherwise both are replaced by a false flag.B) Two stack entries/One stack entry.C) Test conditioning prior to branching.                                   %H2%>%JL%A) If the second stack entry in greater than the top entry, both %JR%entries are replaced by a true flag. Otherwise both are replaced by a false flag.B) Two stack entries/One stack entry.C) Test conditioning prior to branching.%P0%%H1%%H2%Stack                                 %H2%2DUPA) Duplicates the top stack entry twice.B) One stack entry/Three  stack entries.C) Duplication of data on the stack.                                 %H2%2OVER%JL%A) Duplicates the third stack entry over the top two and pushes the word to the stack.B) Three stack entries/Four stack entries.C) Stack data management.                                %H2%2SWAPA) Interchanges the top and third stack words.B) Three stack entries/Three stack entries.C) Stack management.                                 %H2%CJOIN                                          %JR%A) Pops the top two stack entries a nd combines them to a single word %JL%by moving the low-order byte of the top entry into the high-order byte of the second entry and pushes the resulting 16-bit word to the stack.B) Two stack entries/One stack entry.C) Stack manipulation for multi-byte signed integers.                                 %H2%CSPLIT%JR%A) Pops the top stack entry and creates two 16-bit numbers. The %JL%high-order byte is moved to low-order byte of the second entry in %JR%sign-extended format. The low-order byte is returned as the top stack entry as a positive 16-bit number.B) One stack entry/Two stack entries.C) Stack manipulation of multi-byte integers.                                  %H2%DROPA) Pops the top stack entry and discards it.B) One stack entry/None.C) Stack  clean up.                                 %H2%DUP A) Duplicates the top stack entry and pushes it to the stack.B) One stack entry/two stack entries.C) Stack management.                                  %H2%LROT%JL%A) Rotates the top three stack entries left in an infix cyclic sense (input A  B C  into B C A

with A the final top stack entry).B) Three stack entries/Three stack entries.C) Control of stack order.%P0%                                 %H2%OVER%JR%A) Duplicates the second stack entry and pushes it the top of the stack.B) Two stack entries/Three stack entries.C) Control of stack order.                                 %H2%RROT%JL%A) Rotates the top three stack entries right in an infix cyclic sense (input A B C  into C A B with B the top stack entry).B) Three stack entries/Three stack entries.C) Control of the stack order.                                 %H2%SWAPA) Interchanges the order of the top two stack entries.B) Two stack entries/Two stack entries.C) Stack data management.%H1%%H2%System                                   %H2%'%JR%A) Scans the token following the '  (tick) in the input buffer and %JL%searches the CURRENT and CONTEXT vocabularies for the keyword %JR%corresponding to the token. Returns the word address of the keyword as %JL%the top stack entry if it is located. If not found, the token is echoed to the operator and followed by a "?".B) None/One stack entry or none.C) Operator location of keywords.                                  %H2%+SP%JR%A) Adds the current stack pointer to the number at the top of the stack.B) One stack entry/One stack entry.C) Direct addressing of data on the stack.                                  %H2%,%JL%A) Pops the top stack entry word and encloses it in the free dictionary space.B) One stack entry/None.C) Used to build dictionary keywords.                                 %H2%-SP%JR%A) Subtracts the current stack pointer from the number at the top of the stack.B) One stack entry/One stack entry.C) Direct addressing of stack data.%P0%                                %H2%?RSA) Pushes to the stack the current return stack pointer.B) None/One stack entry. C) Return stack display and control.                                 %H2%ABORT%JL%A) Does an unconditional jump to START/RESTART routine to re-initialize the system and the stacks.B) None/None%JR%C) Used when the operator is totally at sea. (The system knows exactly what's going on.)                                %H2%ASPACEA) Pushes an ASCII space code to the low-order byte of the stack.B) None/One stack entry.%JL%C) The normal token separator and to insert blanks in formatted displays.                                 %H2%BINARY    A) Sets the system number base to 2 decimal or the binary radix.B) None/None.C) Sets I/O to the binary radix notation.                                   %H2%C,%JR%A) Pops the top stack word and encloses the low-order byte in the dictionary.B) One stack entry/None.C) Used to build dictionary keywords.                                  %H2%CA!%JL%A) Stores the address at the top of the stack in the word address %JR%location of the latest entry in the CURRENT vocabulary, ie: the top %JL%stack entry is the code address of the keyword currently in the process of being defined.B) ONe stack entry/None.C) Used to define defining keywords.                                %H2%DECIMALA) Sets the system variable BASE to 10 decimal to evoke decimal I/LO.B) None/None.C) Evokes radix 10 I/O.                                  %H2%DO, %JR%A) Stores the program control directive at the top of the stack to the dictionaryB) One stack entry/One stack entry.C) Used to define compiler directive immediate keywords.%P0%                                  %H2%END,%JL%A) Encloses the address of the program control directive at the top of %JR%the stack in the dictionary, computes the relative jump byte using the %JL%top stack entry and  the current free dictionary location and encloses the low-order byte in the dictionary.B) Two stack entries/None.C) Used in defining compiler directive immediate keywords.                                  %H2%ENTRY%JR%A) Pushes to the stack the address of the first header byte of the latest entry in the CURRENT vocabulary.B) None/None.%JL%C) Used to locate the address of the latest vocabulary definition which will become the link address of the next keyword.                                %H2%EXECUTE%JR%A) Pops the top stack entry to word address register and jumps to the inner interpreter RUN routine to cause the execution of a keyword.B) One stack entry/None.%JL%C) Used by the system for keyword execution and by operator for defining  conditional execution keywords.                                 %H2%HERE %JR%A) Pushes the address of the next free dictionary location to the stack (the address stored at the system variable DP).B) None/One stack entry. %JL%C) Used by the system in building dictionary entries and by the operator to determine dictionary space usage.                                  %H2%HEX    %JR%A) Sets the system variable BASE to 16 decimal to evoke hexadecimal I/O.B) None/None.C) Evokes radix 16 I/O.                                 %H2%OCTALA) Sets the system variable BASE to 8 decimal to evoke octal I/O.B) None/None.C) Evokes radix 8 I/O.                                %H2%SINGLE  %JL%A) If the top stack entry is a valid 8-bit number (the high-order byte %JR%is all zeros or all ones), a false flag is pushed to the stack. otherwise a True flag is pushed to the stack.B) One stack entry/Two stack entries.C) Used to determine storage or display requirements for stack numbers.%P0%%H1%%H2%System Directives                              %H2%DEFINITIONS%JL%A) Sets the system variable CURRENT to the value in the system variable CONTEXT.B) None/None.C) Sets the vocabulary into which new definitions will be linked.%H1%%H2%System Variables                               %H2%BASE         A) Pushes to the stack the address of the number base variable.B) None/One stack entry.%JR%C) Used to access the system variable which contains the radix for system I/O.                                %H2%COMPILER%JL%A) Pushes to the stack the address of the compiler variable which points to the last entry in the COMPILER vocabulary.B) None/One stack entry.C) Used to access the link to the last COMPILER vocabulary entry.                                 %H2%CONTEXTA) Pushes to the stack the address of the system context variable.B) None/One stack entry. %JR%C) Used to access system variable which contains the address of the vocabulary that will be searched to locate keywords.                               %H2%CURRENTA) Pushes to the stack the address of the current vocabulary variable.B) None/One stack entry.%JL%C) Used  to access the current vocabulary variable which contains the address of the vocabulary where new keywords will be added.                                  %H2%DPA) Pushes to the stack the address of the dictionary pointer variable.B) None/One stack entry.%JR%C) Used to access the system variable which contains the address of the next free dictionary location.                                %H2%LBPA) Pushes to the stack the address of the line buffer pointer variable.B) None/One stack entry.%JL%C) Used to access the line buffer pointer variable which contains the address of the start of the next token in the input line buffer.%P0%                                %H2%MODEA) Pushes to the stack the address of the system mode variable.B) None/One stack entry.%JR%C) Used to access the system variable which contains the system execution state.                                %H2%STATEA) Pushes to the stack the address of the system state variable.B) None/One stack entry.%JR%C) Used to access the system variable which contains the compiler immediate state. %H1%%H2%Utility                                 %H2%ERASE    %JL%A) Fills a region of memory with ASCII spaces. The starting memory %JR%address is the second stack entry and the ending memory address is the top entry. Removes both entries.B) Two Stack entries/None.C) Clearing string data.                                %H2%FILL%JL%A) Fill a region of memory with specified byte. The byte is the third %JR%stack entry low-order byte. The starting memory address is the second %JL%stack entry and the ending memory address in the top entry. Removes all three entries.B) Three stack entries/none.C) Loading memory to some initial value.                                 %H2%MOVE   %JR%A) Move the region of memory specified by the starting address of the %JL%third stack entry and the ending address of the second stack entry to %JR%the memory region specified by the starting address of the top stack entry. Removes all three entries.B) Three stack entries/none.C) Used to move memory data.                                %H2%WAIT%JL%A) if a keyboard entry (any) has been received, a loop is entered waiting for the next keyboard entry.B) None/None.C) Used to hold the display screen fixed to allow inspection.%P0%%H1%%H2%Vocabulary                                  %H2%CORE  %JR%A) Sets the CONTEXT system variable to the address of the code body of CORE which contains the address of the latest entry in the vocabulary.B) None/None.C) Evokes the CORE vocabulary.                                %H2%FORGET%JL%A) Searches the current vocabulary for the token following FORGET. If %JR%located, the current link address is set to the  address of the link in %JL%the keyword located and the dictionary pointer is reset to the start of %JR%the header of the located keyword. if not located, the token is echo displayed and followed by a "?".B) None/None.C) Used to delete keyword definitions in a spatial sense.                               %H2%IMMEDIATE %JL%A) Removes the latest entry from the current vocabulary and links it to %JR%the compiler vocabulary. The previous second entry in the current

vocabulary becomes the latest entry.B) None/None.C) Adding keywords to the compiler vocabulary.%H1%%H2%Z88 Keywords                                                      %H2%BLEEPA) Make a bleep. Mark count, space count and sound count in the top stack.B) Three entries/None.For example : 12 20 10 BLEEP "ENTER"                                %H2%*CLI"A) Invoke new CLI.B) None/None.C) Compiling Z88 FORTH Programs from a Pipedream Plain Text File.For example: - PLAIN TEXT FILE.J: HOLA ." HOLA, SOY Z88 FORTH V$ 1.00 " CR ;CLEAR HOLA- INLINE EDITOR*CLI" .*PROGRAM.FTH"%P0%                                %H2%NUMBERA) Inputs a character string from keyboard. Converts it to a number, using the current numeric base. Puts flag false or true in the top stack. If flag true, number is the second stack number.B) None/Two or one entries.The INLINE editor in Z88 FORTH is very primitive. Please use only "ENTER" & "DEL" Special keys.%CO:B,12,60%%CO:C,12,48%%CO:D,12,36%%CO:E,12,24%%CO:F,12,12%