MOY - Download from PC to 48K Speccy via IF1 RS232.         File: MOY.DOC

Copyright 1994 Staffan Kjerrstrom                           Date:94-12-04

***********************************

MOY - User's guide.   Revision 1.0

***********************************

1. General

***********

MOY is a set of tools for  upload and download of speccy programs via

a local RS232 link to a PC.

MOY is intended for those who prefer "the real thing" instead of emulators.

Exchange of speccy programs via Internet or BBS'es is easier today than

in the ancient times when we used VTX5000 for downloads from Prestel type

systems. The MOY tools will help some speccy owners to enjoy these

new resources.

Two different program formats are handled:

1) The .SNA format which is useful for program exchange via BBS'es

and Internet.

2) The Multiface One format (M1) which is useful if you want to store many

Spectrum programs on the PC hard disk. The M1 format is compressed and

this reduces loading time and disk space.

2. Requirements

****************

48K Spectrum and Interface 1.

IBM PC compatible computer with MSDOS.

The SPEC232 programs for sending files on

the RS232 local link and for converting

between snapshot formats.

Cable for RS232 link, as described in SPEC232.

For uploads:

Multiface One (M1) snap box.

3. Installation

****************

Copy the MOY files to a PC directory and extract the files from RS232.ZIP.

Install and check out the RS232 link according to the RS232 documentation.

(Test the programs SLOAD, WRITERS and READRS)

4. Packing list

****************

File extentions:

Ext. Use

------------------------------

ASM  Assembler source code

BAS  ZX Basic program (Loads to speccy with:  LOAD * "b")

BAT  DOS Batch program

ZIP  Set of files to be upacked with UNZIP

ZX   ZX machine code program

The following files are included in this MOY distribution pack:

File          Contents

------------------------------------------------------------------------

LDCODE.ASM   Load one block of CODE from speccy tape.

LDCODE.ZX    Machine code for  -"-

M1MSAV1.BAS  Save M1 Microdrive snap to PC as M1 files.

M1MSAV2.BAS     -"-

MM1MSAVE.BAT    -"-

M1MSNA.BAS   Save M1 Microdrive snap to PC as .SNA file.

M1MSNA.BAT     -"-

M1RUN.BAS    Load M1 files from PC and RUN it.

M1RUN.BAT      -"-

M1TSAVE.BAS  Save M1 Tape snap to PC as M1 files.

M1TSAVE.BAT    -"-

M1TSNA.BAS   Save M1 Tape snap to PC as .SNA file.

M1TSNA.BAT     -"-

MOY.DOC      This document.

MOY.TXT      Short message about upload of MOY.

SNARUN.ASM   Load .SNA file from PC and RUN it.

SNARUN.BAS     -"-

SNARUN.BAT     -"-

SNARUN.ZX      -"-

SPEC232.ZIP   Rs232 communication an snapshot conversion.

Copyright 1992 Henk de Groot.

Not yet developed:

SNASAVE.BAS  Save snap direct to PC as .SNA file.

SNASAVE.BAT    -"-

SNASAVE.ASM    -"-

SNASAVE.ZX     -"-

5. Programs for .SNA files.

**************************

M1MSNA  Save M1 Microdrive snap to PC as .SNA file.

---------------------------------------------------

Use Multiface One (M1) to make a snapshot on a Microdrive cartridge

with the program name 'run'.

Put he cartridge in drive 1.

Speccy:  RANDOMIZE USR 0        PC: m1msna <filename-first-part>

LOAD *"b"

Eg:    m1msna sabre

M1TSNA Save M1 Tape snap to PC as .SNA file.

--------------------------------------------

Use Multiface One (M1) to make a snapshot on a tape

with any program name.

(The first snap on the tape will be used.)

Speccy:  RANDOMIZE USR 0        PC: m1tsna <filename-firt-part>

LOAD *"b"

Eg:    m1tsna sabre

Press PLAY on tape unit.

SNARUN Load .SNA file from PC and RUN it.

-----------------------------------------

This part can be used without a Multiface One connected.

The only drawback is that the start screen will be

disturbed. Normally this doesn't matter because the program

will redraw the screen when running.

Speccy:  RANDOMIZE USR 0        PC: snarun <snafilename>

LOAD *"b"

Eg:    snarun sabre.sna

SNASAVE Save snap direct to PC as .SNA file.

--------------------------------------------

This part is not developed yet.

Please send me a note if you are interested.

The SPCONV program (in SPEC232.ZIP) can be used to produce

.SNA files from speccy programs. However this requires access

to the source code to enter the Basic statement:

SAVE*"b" CODE 16384,49152: ...

If you have no source code AND want to produce a .SNA snap

for use in systems without Multiface One (eg emulators)

AND ALSO want to avoid the temporary screen disturbance

THEN you need SNASAVE (and M1).

6. Programs for M1 files.

*************************

M1MSAVE  Save M1 Microdrive snap to PC as M1 files.

---------------------------------------------------

Use Multiface One (M1) to make a snapshot on a Microdrive cartridge

with the program name 'run'.

Put he cartridge in drive 1.

Speccy:  RANDOMIZE USR 0        PC: m1msave <filename-first-part>

LOAD *"b"

Eg:    m1msave sabre

The result will be three files with the extentions: M1, M2 and M3.

Eg: SABRE.M1, SABRE.M2 and SABRE.M3

These are then all downloaded when M1RUN is used.

M1RUN Load M1 files from PC and RUN it.

---------------------------------------

This part can be used without a Multiface One connected.

The only drawback is that the start screen will be

disturbed. Normally this doesn't matter because the program

will redraw the screen when running.

Speccy:  RANDOMIZE USR 0        PC: m1run <filename-first-part>>

LOAD *"b"

Eg:    m1run sabre

M1TSAVE Save M1 Tape snap to PC as M1 files.

--------------------------------------------

Use Multiface One (M1) to make a snapshot on a tape

with any program name.

(The first snap on the tape will be used.)

Speccy:  RANDOMIZE USR 0        PC: m1tsave <filename-first-part>>

LOAD *"b"

Eg:    m1tsave sabre

Press PLAY on tape unit.

The result will be three files with the extentions: M1, M2 and M3.

Eg: SABRE.M1, SABRE.M2 and SABRE.M3

These are then all downloaded when M1RUN is used.

7. Copyright

*************

SPEC232.ZIP   Rs232 communication an snapshot conversion.

Copyright 1992 Henk de Groot.

MOY

Distribute freely as complete package.

Commercial use not allowed without permission.

No liability accepted.

Source codes are included.

Staffan Kjerrstroem            Tel: +46 21 125567

Jakobsgatan 4                  Internet: skjerrst@senet.abb.se

S-72464 Vaesteraas

Sweden

8. Change history

*****************

94-12-04   First version ready. /SKj