REPLAY PROFESSIONAL DEMO

by 2-Bit Systems

SEE REVIEW IN THIS MONTH'S ST FORMAT

FOR FURTHER DETAILS CONTACT MICRODEAL ON 0726 68020

OR WRITE TO: PO BOX 68, ST AUSTELL, CORNWALL, PL25 4YB

Note: only part of the Replay Professional Editor is present. Neither

Drumbeat, MIDIplay or the sampling/playback cartridge are present with

this demo.

Note: if you have the Replay 4 cartridge, you will be able to sample

software with this demo version of the Replay Professional Editor.

REPLAY CARTRIDGE SPECIFICATIONS

Anti-aliasing filters              -    2:  1 each on I/P & O/P

Signal to noise ratio    (input)   -    48 dBs Best case

Max. sample frequency              -    100 kHz (not guaranteed)

Frequency selection                -    software controlled

Resolution     (input)             -    8  BITS

(output)                  12 BITS

Linearity      (input)             -    Within 1 LSB

Max. input / output signal         -    2 V (P-P) / 0.7 V (R.M.S)

REPLAY PROFESSIONAL EDITOR

Sample frequencies                 -    5.5,8,11,16,22,32 Khz

(Through Monitor/HI-FI)

-    Plus  44 and 48 Khz

(Through HI-FI mode only)

Max. sample buffer size            -    UP TO 4 MEGA BYTES

(On 4 'MEGA ST')

Max. sample magnification          _    to 600 sample BYTES

Sample display modes               -    Filled,Outline & Envelope

Sample editing facilities          -    Reverse, Fade in/out, Overlay,

Insert, Delete, Copy, Clear,

Fill,

Shrink, Load, Save, Filter,

Echo,

Flange and up to 10 sample

preset

function keys.

MIDI facilities                    -    Mono channel (MIDI 1 to 16)

10 note assignable samples

Loop points with full A/S/R

(Attack, Sustain, Release)

Real time pitch shift

MIDI trigger/shift

Store MIDI sets to disc

Total free memory   520  ST        -    better than 230 kBytes

(inc paste buffer)                      ( > 10 secs @ 22 kHz)

1040 ST             better than 750 kBytes

( > 34 secs @ 22 kHz)

DRUMBEAT PROFESSIONAL

Play back frequency                -    22 kHz

Max. kit size                      -    15 SAMPLES

Max. sample size                   -    10 kBytes (half second

samples)

Max. patterns                      -    50

Max. pattern steps per song        -    100

Tempo                              -    40 to 239 beats per minute

Beat registration                  -    1/8, 1/16, 1/32 and triplets

(2,4,8,3,6,9 Beats/quarter

note)

Synchronisation                    -    INTERNAL TEMPO or EXTERNAL

MIDI

MIDIPLAY

Max. number of samples             -    128 (Memory permitting)

RAM control                        -    Dynamic allocation of space

Polophony                          -    UP TO 4 VOICE / NOTE POLY

Keyboard range                     -    C2 to G7 (9 octave)

Features                           -    Pitch bend, A/S/R looping,

MONO/POLY operation.

4 separate keyboards at once

each with its own channel.

Free RAM                 520  ST   -    Better than 280 kBytes

1040 ST   -    Better than 820 kBytes

REPLAY PROFESSIONAL EDITOR.

When the EDITOR is first run, it automatically splits the computers

memory into TWO large chunks, one of these is visible as the SAMPLE

DISPLAY, the other is invisible and is called the PASTE BUFFER. This

buffer is used for the temporary storage of samples during editing

operations. Obviously, the larger the PASTE BUFFER, the smaller the

SAMPLE memory is. For this reason it is possible to make the PASTE

BUFFER smaller or even to remove it entirely. This means that larger

sample times can be achieved though it does have the nasty side effect

of restricting the size of the temporary sample which can be held or

restricting the number of edit functions which are available at any

one time.

The MENU bar

DESK

Selecting Desk will allow you to see the REPLAY program credits and

version number only. Any desk accessories loaded will be displayed in

grey lettering, indicating that it has been disabled and is not

available for use within the REPLAY software. Desk accessories are

disabled because of their ability to indiscriminately corrupt samples

held in memory.

FILE

The FILE title will reveal options for loading and saving individual

samples to and from disc, loading and saving SETs of samples complete

with PRESET and MIDI data and finally, the option to leave the program

and return to the desktop. For more information on the LOAD, SAVE and

SET facilities.

FREQUENCY

Selection of the sample and play back speed is achieved by clicking on

the desired value and is put into effect immediately. The 22 KHz value

is the default speed. Roughly speaking, the faster the sample speed,

the higher the sound quality but the shorter the sample time.

OUTPUT

The default output mode is MONITOR but selecting HI-FI will cause all

future output to be heard via an amplifier connected to the REPLAY

cartridge output socket. All output can be heard via HI-FI but certain

restrictions are placed upon the MONITOR mode. For example, an attempt

to select a frequency above 32 KHz in monitor mode will not be

allowed. Similarly, an attempt to return to MONITOR whilst FREQUENCY

is at say, 44 KHz, will be prevented.

DISPLAY

The options shown here will select the mode used to display the sample

in the edit window. The different modes are available to enhance

particular detail in a sample. The general purpose selection, FILLED,

is the default, but OUTLINE would usually be fine. The main use of

outline is for revealing the true shape of a waveform under high

MAGNIFICATION where the true wave shape is displayed. The ENVELOPE

option will display the full wave envelope reflected along the central

axis. This display mode is useful for showing bursts of speech or

drums (for example) in amongst a lot of otherwise confusing data.

The sample display

You will notice a Black line to the left of the display. This is the

LOWER edit cursor. To the right you will notice a Black and White

dashed line. This is the UPPER edit cursor but it is covered by

another cursor called the GHOST cursor.

UPPER & LOWER cursors

The UPPER and LOWER edit cursors are the key to the operation of the

whole EDITOR because it is between these two points that ALL

operations are performed. To be useful, it must be possible to move

these two pointers. This can be achieved in two ways. One is to use

the cursor movement buttons found on the cursor control panel (section

4.5), the other is to use the mouse directly within the sample

display. First, Move the mouse 'ARROW' pointer over the dashed line to

the right of the sample display, press the left mouse button and keep

it pressed. If you are over the cursor line, the ARROW should change

into a POINTING FINGER. This means that you have found the cursor and

are ready to move it. Now, keeping your finger on the left mouse

button, move the mouse to the left, a black line should follow. Move

the cursor to the centre of the screen and release the mouse button.

The POINTING FINGER should revert back to an ARROW. There should now

be three vertical lines on the screen, two black and one dashed. The

same operation can be performed on the LOWER cursor to the left of the

screen, try it now. You will find that it is not possible to move a

cursor further than the position of its opposite number.

Both of the UPPER and LOWER cursors will be visible AT ALL TIMES in

the SAMPLE display while the GHOST cursor (the dashed line) may

disappear from view from time to time.

The GHOST cursor

Quite simply, the GHOST cursor is a position marker. For example, when

a sample is played back and stopped early by pressing the escape key,

the GHOST will mark the position at which the sample play back was

stopped allowing easy location of parts of a sample of interest. The

GHOST may appear to do strange things. If, for example, a sample is

loaded from Disc, the UPPER cursor is automatically placed at the end

of the new sample on screen and the GHOST will mark the position of

the UPPER cursor BEFORE it was moved to the end of the newly loaded

sample. This is because the EDITOR will perform an operation by

default, but might do something that the user did not want or expect

it to do. Therefore the user can correct the mistake by moving the

UPPER cursor back to the position marked by the GHOST cursor. The

GHOST cursor has a special function assigned to it called the SWAP

button located on the PASTE BUFFER control panel. Pressing this button

(or the space bar) will cause the EDITOR to swap the locations of the

UPPER and GHOST cursors instantly, thus enabling fast and accurate

relocation of the UPPER cursor to its old position. A little

experimentation with the EDITOR will show the exact nature of this

very useful facility.

ZOOM (magnify)

The ZOOM button will cause the display to magnify in upon the data

currently displayed between the cursors. Magnification can be

performed as much as desired, but magnification beyond 600 sample

values between the cursors has no effect.

SAMPLE SIZE DISPLAY / SET

This box displays the size of the sample currently displayed between

the UPPER and LOWER cursors. However, this box also has the hidden

ability to set the size of the sample. This is achieved by clicking

the mouse inside the sample display, upon which the numbers in the box

will be replaced with '________'. This indicates that the computer is

waiting for the user to type in a number and press the RETURN key when

finished. If the RETURN button is pressed without entering a number,

then the old size is retained by default. Any valid number is one

between the values of 2 (minimum size) and the present size of the

SAMPLE buffer. If a value that is too great is entered, then the

previous size is returned to. Please note that as an alternative to

terminating the number with RETURN, the letters 'K' and 'S' may be

substituted. The significance of these is to set the size to kbytes or

seconds respectively. Hence entering 10K will set the size to 10,000

bytes. Entering 10S will attempt to set the size to that equivalent to

10 seconds of sampling. Only integer values are permitted, decimal

points are not allowed.

RESET magnification

The RESET key has the effect of un magnifying back up to maximum

screen size. The UPPER and LOWER cursors will remain at their previous

positions. If the cursors are too close together, then they may

disappear from the display. This is because they are virtually on top

of one another. Simply use MAGNIFY again to relocate them, or move one

or the other cursor, one position either way via its key on the cursor

control line. Pressing RESET when no magnification is in effect will

cause the cursors to return to their respective extremes on either

side of the edit window. All cursor position data will be lost. If the

positions were accurately located and were important, then they could

have been stored in one of the 10 PRESET memories in case of mishap.

SAMPLING / PLAY BACK control

SAMPLE

Clicking on this button will cause the program to start sampling the

sound currently being played on the input of the REPLAY cartridge. The

sound will be heard simultaneously via the Monitor speaker or

Amplifier, depending upon the current setting of the MENU bar OUTPUT

control. Sampling will stop automatically when the sample space is

used up, or when the the sampling has been terminated prematurely by

pressing the ESCape key. The sample display will be redrawn to reflect

the changes within the SAMPLE buffer.

PRE-SAMPLE

This very useful feature samples continuously. The only way to exit

PRE-SAMPLE is to use the ESCape key, after which the sample will be

drawn on the screen. There may be a small delay between halting the

function and control being returned to the editor, especially when

using very large sample areas. When using the straight SAMPLE button,

TRIGGER can be used to actually start the sampling but does not

guarantee to catch the end of the desired sample. PRE-SAMPLE allows

continuous monitoring of the sound and may be terminated upon

completion of the sample. Of course, we can not guarantee to have

captured the start of the sample in memory, but with use of the set

sample size facility we can set the size of the record buffer to any

size we choose. The best option however is probably to use the entire

sample memory to be quite confident of catching the sample required.

TRIGGER ON / OFF

With TRIGGER selected, selecting SAMPLE has no immediate effect.

Sampling is suspended until the incoming sample volume exceeds the

value displayed in the box to the right of the TRIGGER button. As soon

as SAMPLE is selected, the MOUSE pointer will turn into a BEE to

indicate that it is waiting to start sampling. As soon as the sample

volume exceeds the preset value the sample button will then turn

black. Sampling may be terminated BEFORE the TRIGGER occurs by

pressing the ESCape key. No sample operation will take place under

these circumstances.

TRIGGER sensitivity

The box to the right of the TRIGGER button displays the trigger

sensitivity. The smaller the value, the more sensitive, the larger the

value, the less sensitive the TRIGGER is. The two arrows on either

side of this display are used to adjust this threshold value up and

down. Sensitivity is incremented in steps of two. When related to the

SCOPE display, a value of 1 represents a small movement of the scope

display, where as a value of 127 requires the signal in the SCOPE

display to touch either the top or bottom of the display to cause a

sample TRIGGER to occur.

HEAR (audio monitor)

Upon selecting HEAR, audio input to the REPLAY cartridge may be

monitored in real time. It is possible to start sampling at any time

by pressing the 'S' key, otherwise selecting ESCape will halt the

function in the normal fashion.

LOOP ON / OFF

With LOOP selected, forward or backward play back of a sample will

continue looping from start to end and back again, until the ESCape

key is pressed.

PASTE BUFFER control

As has previously been explained, the PASTE BUFFER is an area of the

computers memory which is automatically set aside by the EDITOR for

the temporary storage of samples or parts of samples. The paste buffer

can only hold one sample at a time and may occasionally be required

for other uses by the EDITOR, the SPECIAL EFFECTS for example.

Whenever an operation to clear or overwrite the contents of the buffer

is required, the EDITOR will always request permission to destroy the

contents of this otherwise hidden sample. On occasions, the EDITOR may

throw up the strange message 'CURSOR GAP TOO SMALL' and give the

option to 'OVERFLOW', 'CLIP' or 'CANCEL'. This is trying to say that

the sample in the BUFFER at present is larger than the size of the gap

between the UPPER and LOWER cursors. You are given the opportunity to

cancel the function, to just take sufficient of the sample from the

BUFFER to fit the cursor gap or simply to take the entire sample from

the BUFFER and ignore the size of the cursor gap entirely.

STORE sample

Selecting STORE will copy the current sample between the UPPER and

LOWER cursors into the PASTE BUFFER. It will remain there until it is

erased by another sample or when cleared out for use by another

function, such as reducing the size of the BUFFER itself.

INSert sample

The LOWER cursor position is taken as the start of this operation. The

EDITOR will move the entire contents of the sample space, starting

from the lower cursor position, towards the right of the screen by

just enough space to fit the sample in the PASTE BUFFER into the new

space created. The sample in the PASTE area is then copied into the

sample edit area. Any sample that used to exist at the very end of the

sample buffer will be lost forever. To clarify exactly what the

function does, a little demonstration is required. Suppose that we had

just sampled the word 'HELLO' into the computers memory. We now move

the UPPER cursor down towards the start of the sample until we only

have the letter 'H' surrounded by the cursors. Now press the STORE

button. Next, press the INSert button twice, allowing time for the

operation to complete between presses. Now move the UPPER cursor back

to the end of the sample and press PLAY. The computer will now say 'H-

H-HELLO'.

REPeat sample

REPeat performs a similar function to insert. It will copy the entire

contents of the sample edit area from the LOWER cursor position and

place the copy at the position pointed to by the UPPER cursor. Again,

any sample at the end of the sample area will be lost. Again, a simple

explanation will clarify the exact function of this useful feature.

Suppose that we had just sampled the 'HELLO' into the computers

memory, as before. Again we surround the 'H' with both cursors. Now

press the REPEAT button twice, allowing time for the operation to

complete between presses. Now move the UPPER cursor back to the end of

the sample and press PLAY, it should now say 'H-H-HELLO'. Notice that

we have created EXACTLY the same effect as before, but we did not have

to use the PASTE BUFFER directly as before.

Create a GAP

GAP inserts a blank space between the cursors. The whole sample,

starting from the lower cursor running right up to the end of memory,

will be shifted up to the UPPER cursor leaving the space between both

cursors. This space may now be recorded into or have something PASTED

or loaded into it.

PASTE sample

The contents of the PASTE BUFFER are copied from the buffer into the

sample area overwriting what ever was in the sample space before the

operation. The copy is placed at the start of the area marked by the

LOWER cursor position.

CUT sample

The sample between the cursors is removed and the contents of the

sample edit area, starting from the UPPER cursor, are copied down to

the LOWER cursor. Any trace of the old sample is lost unless it was

first saved with a STORE operation. This function DOES NOT work in the

manner that some WORD PROCESSOR users might imagine. The choice of the

words in the EDITOR for CUT, PASTE and STORE are more logical and

describe EXACTLY what they do.

CLR

Selecting CLR will erase the sample between the UPPER and LOWER

cursors. Again, the action of this button is NOT reversible unless

preceded by a STORE operation.

SWP

SWP causes the editor to SWAP the positions of the UPPER and GHOST

cursors. Whilst not sampling, pressing the space bar performs the same

function.

BUFFER size display

The value shown in this box shows the size of the current PASTE BUFFER

and therefore the largest sample size that can be held in the buffer.

However, in the same way that the sample size display can be used to

set the cursor width, clicking in this box with the mouse will again

display '________'. The size of the PASTE BUFFER can not be larger

than the sample edit work space.

PASTE BUFFER status

Finally a little box is located to the right of the BUFFER size

display. The contents of the display are either E for Empty or F for

Full. If the user clicks on the button when 'Full' the contents of the

buffer are played out through the currently selected output channel.

EDITOR FUNCTIONS

FADE IN / OUT

Selecting either of the Fade options will cause the sample to fade in

from zero to maximum volume or fade out from maximum volume to zero.

As with ALL of the edit functions, the area between the UPPER and

LOWER cursors is the area faded in or out.

FILL

The area between the cursors is filled with as many repetitions of the

sample presently held in the PASTE BUFFER as can be fitted in. Please

note that this may include a copy of the sample at the end of the FILL

area which is INCOMPLETE ! (I.E. two and a half copies of the PASTE

sample).

VOLUME

Upon selecting VOLUME, the computer will ask you to select volume up

or down. Select the desired direction and the EDITOR will increase or

decrease the sample between the sample cursors by about 12%. Please

note that repeated use of VOLUME increase will result in a

proportional increase in any noise present in the sample !

FIND

FIND will locate the occurrences of a sample segment held in the PASTE

BUFFER. Upon selecting FIND, the EDITOR will ask were to start

searching from. The search will either look from the very START of the

sample buffer to find the FIRST occurrence of the sample in the sample

buffer, or start searching from the LOWER cursor position (the NEXT

occurrence). Please note that FIND may take some while to search a

very large sample buffer. Also, to make a match, the sample being

searched for must be an EXACT copy of that held in the PASTE BUFFER.

It is therefore a waste of time to search for a sample which has had

its volume adjusted !

REVERSE

This function reverses the sample in memory and has the same effect in

RAM as having played the sample backwards using the BACK option from

the sample play control panel.

FINE

FINE allows accurate positioning of the UPPER and LOWER cursor

positions. The EDITOR will give the user the option of which cursor to

adjust. Upon selecting a cursor, the sample display window will be

replaced with a magnified display of the sample surrounding the

selected cursor. Moving the mouse pointer into the display area and

pressing the LEFT mouse button will cause the cursor to be moved to

that position. Pressing the RIGHT mouse button in the display area

will cause the cursor to be positioned back at the exact same location

found upon entry into the display mode. To exit from this mode of

operation, move the mouse pointer outside of display area and press

the RIGHT mouse button. The display will be returned to that relevant

at the time before entry into the FINE mode with the selected cursor

at its new position. This mode of location is useful for accurately

identifying the start and end of sample segments, or for locating the

cursors in an attempt to remove MIDI looping clicks.

SHRINK

SHRINK compresses a sample to be half of its original size. As a

result, the speed of the sample will now be double its original value,

so to correct for this, it will be necessary to select a new sample

frequency from the FREQUENCY menu on the MENU bar. For example, assume

that a sample was made at 22 kHz and then shrunk, then a new value of

11 kHz will need to be selected to make the sample sound correct. It

is advisable to sample sounds at the highest possible speed and then

shrink them down to the desired speed. This will normally result in a

higher quality sound than if it had simply been sampled at a lower

speed since the sample will have been made at a speed many times

faster than the filtering threshold of the REPLAY cartridge and will,

therefore, be subject to much less ALIASING than at lower speeds.

JOIN

The controls now in the centre of the screen, effectively allow fine

positioning of both of the UPPER and LOWER cursors by selecting the

appropriate buttons. Alternatively, the user can actually draw, on the

screen, the waveform between the sample start and end points.

Provision is made for testing the sample from within this mode of

operation. On leaving the JOIN mode, the EDITOR central control panel

will be redrawn and the sample display will also be redrawn with the

cursors at their new locations.

Micro adjust.

Selecting the double arrows located either side of the legends

marked LEFT and RIGHT will increment the cursor positions in steps of

10 samples at a time where as the single arrow keys will move 1 sample

at a time. The display is redrawn after each selection to display the

latest cursor position.

Screen draw.

Selecting screen draw will enable the user to place the mouse

pointer any where within the sample display and hand draw the

waveform by pressing the LEFT mouse button and moving the mouse

pointer slowly around the screen. To exit draw mode, move the mouse

pointer to some point OUTSIDE of the sample display area and press the

RIGHT mouse button, upon which the editor will return to normal loop

edit mode. Screen draw should only be used where the cursors represent

the START AND END of the sample and not where the cursors represent

loop pointers located somewhere inside a MIDI looping sample.

OVERLAY

The sample in the PASTE BUFFER is superimposed on top of the sample

between the current cursor positions. Please note that there will be a

resulting reduction in the overall sample volumes after the operation

has been performed. Crude ECHO effects can be created by overlaying

samples on top of themselves but displaced by a small amount.

PRESET

To assign a sample or memorise the cursor positions at any time,

select the PRESET key, it should turn black. The EDITOR is now waiting

for the user to select the function key to which the assignment should

be made. Select an unused key from f1 to f10. Once this is done, the

user will be requested to enter a name for the sample. The name can

consist of up to 8 valid printable characters, and should be typed in

from the computer keyboard. Press RETURN when finished. If a key

already has a name, then just pressing the RETURN key will keep the

old name. The name of the current sample is displayed in the edit

window after selecting or naming a function key.

F1 to F10

The function keys can be used on their own or jointly with the PRESET

key. When the PRESET key is selected, pressing a function key will

cause the EDITOR to assign a sample or memorise the positions of the

cursors within the sample edit display window. Pressing a function key

while the PRESET key is NOT selected, will cause the EDITOR to

relocate the screen to the exact position that it was in when the

definition was last set. The entire contents of the screen are

remembered including the state of magnification.

SPECIAL EFFECTS (S.F.X)

***NOT IMPLEMENTED IN DEMO***

DIGITAL FILTERING

***NOT IMPLEMENTED IN DEMO***

MIDI

Within the EDITOR, it is possible to play samples under the control of

an external MIDI keyboard. While the facilities offered here are some

what more restrictive than within the MIDIPLAY software, the user has

the chance at least to try out samples under MIDI KEYBOARD control and

test the loop points accurately using the TEST function.

MIDI looping

Throughout the use of the EDITOR, it is made clear that performing an

operation upon a sample is always relevant to the sample or part

sample held between the LOWER and UPPER cursors, until now ! Because

of the fact that REPLAY PROFESSIONAL can now perform MIDI LOOPING, the

cursors now perform a slightly different function. It is always

assumed that the extreme Left of the display is the start of the

sample and the the extreme right of the display is the end of the

sample to be played REGARDLESS of the present setting of the cursors.

The cursors now form the body of the sample to be looped. The LOWER

cursor defines the start of the loop and the UPPER cursor, the end.

But what is MIDI looping ? Well, consider an instrument such as a

guitar. When the guitarist strums a note, the note plays, loud at

first and decays away until it can no longer be heard. In this case

the sample plays once and decays away naturally. Now consider an

instrument such as a violin. As the violinist draws the bow across the

violin, the instrument makes a noise. The violinist can make the

instrument play a short or a long note depending on how far the bow is

pulled across the strings, the violinist can SUSTAIN the note for a

longer period. As soon as the bow is removed, the note will decay to

nothing. By carefully defining loop points in some samples we can add

our own SUSTAIN period. We do this by surrounding the loop point by

the cursors. REPLAY PROFESSIONAL supports a form of A.S.R. looping

(Attack, Sustain, Release) which can be individually selected to be on

or off for each sample.

The attack is the first and often loudest part of a sample and is

defined as the part of the sample between the Left of the display and

the Left cursor. The Sustain period is the 'Flat' part between the

cursors. The Release phase of the sample is defined as the point

between the Right cursor and the Right hand side of the display. When

a looped sample is played, the sample is played from the far left of

the screen through the Left cursor to the Right cursor. If the MIDI

key is still pressed then the sample goes BACK to the Left cursor

again and plays to the right cursor. If the MIDI key is now released,

then the sample will continue to play until the end, upon which it

will stop.

Sample data box

In the centre of the display, the editor shows data on the PRESETS. It

shows the sample PRESET number and the name of the sample assigned to

it, if any. Next the MIDI note assignment, if valid followed by an 'L'

if the sample is defined to be a looping sample.

Sample selection

To select a sample, simply click the Mouse on the relevant function

key legend on the screen.

The SCREEN pointer

At the end of the row of Function buttons is a large button called

SCREEN. This button is a button which will allow the user to

experiment with looping pointers without having to go through the

rather tedious business of setting up a PRESET every time a change is

to be tested.

Assigning to a MIDI note or KEY

To assign a PRESET to a MIDI note for the TRIGGER mode, select the

button called NOTE. Now click on the legend of the function key that

you wish to assign the NOTE to. The Mouse pointer should now change to

the small image of a MIDI keyboard. The computer is now waiting to

receive a note on the MIDI input, simply press a key on the MIDI

keyboard or press the ESCape key. The mouse pointer should now return

to its Arrow shape.

MIDI CHANNEL selection

For the TRIGGER, KEYBOARD and TEST modes to work, the MIDI channel

must be set up to be the same as that of the MIDI keyboard in use. The

MIDI channel number can be selected by clicking on the arrows

surrounding the MIDI channel display.

TRIGGER performance mode

Upon selecting TRIGGER, the computer will wait for a key on the MIDI

keyboard to be played that matches on of those in the sample data

display. As soon as a selected key is pressed, then the sound assigned

to that key will be played. To exit trigger mode press the ESCape key.

KEYBOARD performance mode

As soon as keyboard mode is selected, the the computer is waiting for

a note on the MIDI keyboard to be played. When a key is pressed, then

the current sample will be played in that KEY. To change the sample,

simply press the associated FUNCTION key on the COMPUTERS keyboard.

The new sample will become the active one straight away. To exit the

KEYBOARD mode simply press the ESCape key.

TEST mode

TEST mode is identical to the KEYBOARD mode with the exception that on

entry, its default sample is whatever is setup on the SCREEN button.

The other point of interest is that setting the LOOP button before

selecting TEST will cause the sample to adopt A.S.R. loop mode.

SAMPLE change in KEYBOARD mode

While in KEYBOARD mode, the simple method of changing the 'ACTIVE'

sample is by pressing a Function key on the computer keyboard. An

alternative exists if the MIDI keyboard in use is equipped with voice

change buttons which are MIDI controllable. By pressing the voice

button which issues the MIDI data 'VOICE 1' will cause the computer to

select that sample number.

LOOP CLICKS

Loop clicks are probably the biggest enemy of the musician wanting to

use samplers as serious alternatives to REAL instruments. Use of the

FINE and JOIN modes in the editor are highly recommended to try and

help this often arduous and thankless task. Remember the famous saying

'Patience is a virtue' ! Perseverance really is the name of the game

here.

FILE handling

LOAD sample

Clicking on LOAD SAMPLE will cause the EDITOR to place the computers

file requester on the screen. The default file type to load will be

.SPL It may be necessary to change the extension to .AVR depending on

the type of sample to load (see APPENDIX iii for further details).

SAVE sample

***NOT IMPLEMENTED IN DEMO***

SETS

A SET is a disc file which contains a number of samples (between 1 &

10) which are used in conjunction with the EDITORS MIDI facilities

(section 4.13).

LOAD SET

Selecting LOAD set will cause the file selector to appear. Upon

selecting the desired SET, the computer will proceed to load all of

the samples into memory and assign them to the Function keys

accordingly. MIDI key assignment and looping data is also

automatically loaded in along with the cursor positions. The samples

are all loaded in as one block and it is up to the user to ensure that

sufficient space is available to load the SET into memory since, with

all other operations, the samples are loaded into the memory space

starting from the LOWER cursor position. If prior to loading a SET

into memory, some function keys are already defined, then these

definitions will be wiped. All samples are assumed to have been made

at the same sample speed. After loading, the computer will attempt to

set the system play back speed to be that relevant at the time of

saving the SET to disc.

SAVE SET

***NOT IMPLEMENTED IN DEMO***

ADD SET

After selecting ADD SET and choosing the set to load from disc, the

computer will start to load the samples into memory. The new sample(s)

will again be loaded into the sample edit area starting from the LOWER

cursor position so great care must be taken to ensure that the new

sample data does not overwrite the sample space of existing PRESETS !

When assigning the key definitions, the computer will start to assign

sample from the lowest available spare function key and will only

assign the new samples to empty keys. If in the process of allocating

the definitions, the computer runs out of spare function keys, then a

warning will appear on the screen. All sample will be loaded (space

permitting) but some samples may NOT be assigned to a key if none are

available.

WIPE SET

Upon selecting WIPE set, the central control panel will be replaced

with a new control panel. The top line of control buttons represent

the Ten function keys. It will now be possible to select a function

key or series of keys to de-allocate. Upon selecting ERASE, the

definitions highlighted in black will be made empty. Only the Function

key definition is removed, the sample in memory is left intact ! This

must be cleared out separately if required.