GoZone is a MIDI mapping utility program for the ATARI ST.

It provides the capabilities found in many hardware mappers.

It would probably be best if you would print this off and go ahead

and start the program.  That way you can see what I'm talking about.

Set up your equipment with a master controller into the ST's midi

in and your slave units from the ST's midi out.  Start out with

each slave on a different channel.

I have also included a separate file called MAP.DOC which

describes three simple example maps I have included.

GoZone works only on the monochrome, hi-res monitor.

If you find this program to be of value please send a donation

to the address listed below.  Donations encourage me to continue

working on projects like these.  Send any suggestions as well.

Have fun,

Loren Claypool

157 Timberlake Circle

Scott Depot, WV 25560

Gozone v1.0 - soft midi control

Gozone is a realtime midi control program.  It allows the user to

define "zones" based on key value and velocity value.  Four velocity

zones are allowed per key zone and a maximum of 128 key zones (all

possible midi notes) may be defined for a maximum of 512 discrete

zones.  An output midi channel is assigned to each zone.  Within a

zone the user may optionally define velocity and pitch adjustment

values for the notes.

Gozone also provides for controller mapping.  The user may map

continuous controllers to continuous controllers and switch controllers

to switch controllers.  For continuous controllers, Gozone automatically

adjusts the LSB's to correspond with the MSB's (i.e., 32-63 are adjusted

when the user adjusts 0-31).  The user may assign any controller, along

with pitch bend and channel pressure after touch to only work on a fixed

midi channel or to be dynamic, that is, to be "active" on the last

channel triggered until the controller is reset.  All of this provides

for extensive controller manipulation.

Gozone also allows the user to define a patch change table whereby

a patch change request from the master controller may change patch

values on any or all midi channels.

When the program is first started, a screen is displayed with a

menu on the left side.  The menu options are:

Exit      - end program

Edit Map  - change map values

Save Map  - save map to disk

Load Map  - load map from disk

Print Map - print map values to printer

Perform   - enter performance mode.

All screens in Gozone use the menu in the same place.  Menu options are

selected by pointing with the mouse and clicking the left button.

Values are changed by using the left button to decrement and the

right button to increment.

A set of zone definitions, controller mappings, and the patch

change table are collectively known as a map.

Selecting Edit Map will allow the user to modify the map values.

The user may save and load maps to and from disk.  Gozone forces

a file extension of MAP.

Print Map allows the user to obtain a hardcopy of the map.  The

user may print all of the map, or select to print only the controller

mappings, the patch change table, or the zones.

Perform puts Gozone into Performance mode.  The program uses the

map setting to manipulate the data coming into the midi in port and

sends it out the midi out port.

To exit Performance mode, press F10.

The Edit map menu contains the following options:

Exit      - return to the main menu

Next Zone - page to next key zone

Prev Zone - page to prev key zone

Copy Def  - copy the default velocity zone settings to the

velocity zones of the current key zones

Def Cntrl - define controller mappings

Def Patch - define the patch change table

Init Map  - intialize all map settings

A "key zone" is defined by a range of note values (i.e., a low note

and a high note).  The possible midi notes are 0 to 127.

A "velocity zone" is defined by a range of velocity values.

The possible velocity values are 0 (no velocity, used by some synths

as note off) to 127.

A velocity zone within a key zone defines a zone.

The default zones at the top allow the user to specify velocity

zone settings which may be copied to multiple key zones.  It is provided

merely as a short cut for zone definitions.  With the exception of the

pitch adjustment, the edit criteria for the default zones are the same

as the performance zones.

Gozone does not allow the user to leave gaps between zones or to

define conditions which may lead to invalid midi values.  The user will

notice changes on the screen other than the value being modified.  This

is to correct possible error conditions.

To set the boundaries for a key zone, the user adjusts the high

note of the zone.  The user may not adjust the low note.  The low note

of the first key zone will always be 0.  The low note of the second

key zone will be one note higher than the high note of the first key

zone.  The high note of the last key zone will be 127.  From an

initialized map the low note is 0 and the high note is 127.  If the

user decrements the high note, Gozone will create and maintain another

key zone.  Try lowering the high note of the initial setting (127) to,

say, 120.  Select Next Zone from the menu and notice that the low note

of this key zone is 121 while the high note is 127.  Select Prev Zone.

Adjust the high note up by one.  Select Next Zone.  Notice the low

note is now 122.  Select Prev Zone.  Adjust the high note back to 127.

The next zone is now deleted.  This approach to key zone definitions

makes Gozone self-correcting and does not allow the user the chance

to leave gaps in the map definition.

The velocity split points in both the default and performance

zones may have values of 1 to 127.  Notice the initial value in velocity

zone 1 is 127.  In other words, no velocity zones have been defined.

Reduce the split value to 126.  Velocity zone 2 is created.  When the

value of velocity zone 1 is incremented back to 127.  Velocity zone 2

is deleted.  Experiment.

The output channel is the midi channel to be triggered when a note

satisfies the requirements for the key zone and the velocity zone.

The output note may be modified by a velocity and/or pitch adjustment to

be described below.  Notice the output note need not be the note played

nor have the same velocity value.

The edit values for the velocity adjustment are as follows:

1) may be no lower previous velocity split point * -1

2) may be no higher than 127 - current velocity split point.

This criteria prevents a value being outside the midi standard range.

Example                          1           2              3

Velocity split point       64          96            127

For velocity zone 1 the velocity adjustment may be from    0   to    63

"               " 2 "                                 "  -64   to    31

"               " 3 "                                 "  -96   to     0.

The obvious use for this feature is to allow the triggering of a patch

by striking the key very hard, with a velocity of between 96 and 127,

but have the tone sound as if the key were struck with medium velocity.

In this case a velocity adjustment value between -32 and -64 would

cause the desired result.  Of course the adjusting output velocity of

a lightly struck note is just as easy.

When editing the pitch adjustment value of the default zones, the

possible values are -127 to 127 (in semitones).

In the performance zones, the pitch adjustment may be:

1)  no lower than the current low note * -1

2)  no higher than 127 - the current high note.

Whenever a pitch adjustment value becomes invalid, it is automatically

reset to 0.

Def Cntrl allows the user to create controller mappings.  The user

may specify whether a controller is to be dynamic or fixed as described

above.  Also, the user may assign the output value of a controller to be

mapped by midi channel to another controller value.  Example:

The Mod Wheel (controller 1) may be mapped to breath control

(controller 2) on midi channel 1, portamento time (controller 5)

on midi channel 6, and volume (controller 7) on midi channel 9,

while remaining defined as modulation of all other midi channels.

Paging up and down allows access to all defined controllers.  Again,

Gozone automatically adjust's controllers 32 - 63 to conform to

controllers 0 - 31.  These controllers are LSB's or least significant

bytes of the lower controllers and are used by some manufacturers to

provide a finer degree of controller resolution.  Also, remember that

only continuous controllers may be mapped to continuous controllers and

switch controllers to switch controllers.

Def Patch allows the user to build a patch change table for a map.

Patch 0 is sent as soon as the user enters Performance mode.  Entries

1 thru 15 are sent when a patch change is intiated from the master

controller.  If necessary Gozone will start wrap back to the beginning

of the table.

Possible values for each patch/midi channel are 0 - 127 and blank.

A blank specifies that no change message be sent for the current patch/

channel combination.  The change values correspond to patch numbers

in each synth so be sure the value is valid for the receiving unit.