Preliminary Documentation for DESKTOP.ACC  6/5/90

-------------------------------------------------

ICD Hard Disk Accessory

DESKTOP.ACC is named DESKTOP.ACX on our distribution diskette.  This is because

there is not sufficient memory in a 520 ST for the ICDBOOT, ICDFMT, and DESKTOP

all at once.  Users of computers with more than 512 Kbytes of RAM will not have

any problem.  To use DESKTOP, it must be renamed by doing a Show Info, change

its name to DESKTOP.ACC, and place it in the main or root directory of the

drive which you selected as your Autoboot drive in HDUTIL.  (This will usually

be drive C).  After placing the accessory in the correct directory you must

reboot the computer to have the accessory in memory.  Alternatively you may

run DESKTOP from MultiDesk of Codehead Software.  See below.

Note that you must have version 4.8.0 or greater of ICDBOOT installed in order

to install and run DESKTOP.ACC as of this date.

DESKTOP.ACC is a hard disk partition management tool which works in conjunction

with the GEM desktop.  It will read your hard drive volume names and put them

in the drive icons.  It also provides valuable partition information and will

allow you to swap, rename, enable and disable active partitions, and toggle

the write verify and write cache functions.  Any changes made will be reflected

in your GEM drive icons (and open windows, if applicable).

DESKTOP.ACC does NOT work with NEODESK, in that it will not put the partition

names in the icons, or change paths in open windows.  Maybe we can do this

later.  Otherwise, it apparently works fine with NEODESK.

DESKTOP.ACC works fine with MultiDesk, from Codehead Software, under the

following conditions: as MULTDESK.PRG, no problems known.  As MULTDESK.ACC,

loaded at bootup, no problem.  Loaded dynamically, from MULTDESK.ACC, as

long as Auto-Run is turned off, no problem, but with Auto-Run on, a problem

seems to occur which we have been unable to track down.  This problem shows

up when you go to rename a partition.  Therefore, do NOT load DESKTOP.ACC

dynamically (i.e. not at bootup) with MULTDESK.ACC, unless you make sure

Auto-Run is OFF!!!  If you wish to load DESKTOP.ACC dynamically, the best

way to do it (under TOS 1.4 at least) is to click on MULTDESK.PRG and install

it as an application for ACC files.  Then you can double-click on DESKTOP.ACC;

MultiDesk program will load and run it for you automatically.

Swapping is performed in the Logical Drives window.  Most of the other

functions are accomplished in the Available Partitions window.  To swap

click on any drive letter or partition name in this window.  Drag the name

to the place in the logical boot order where you want it, and release.  If

you get the flat hand staying up, clicking anywhere outside of the Logical

drives window will get out of swap mode.

A check mark under the Boot column in the Available partitions window indicates

that this partition's boot sector contains bootable code and that ICDBOOT.SYS

can be loaded from this partition when it is the first GEM partition enabled

in the system.  This does NOT necessarily mean that the AUTO folder and the

ACCessories will be loaded from this drive.

An asterisk ('*') next to a drive partition's name in the Logical drives

window indicates that the drive is the current Autoboot drive.  This is

the drive you selected in HDUTIL when you set up your hard disk to Autoboot.

This is the drive where the AUTO folder and ACCessories are loaded from.

To revert to "pristine" logical drive order, click on Unswap.  This makes

your first partition C, next D, etc.

To save your swap order permanently, click on Save.  This will write the

swap order into ICDBOOT.SYS.  Next time you boot, you will be using the

new swap order.  If you swap your Autoboot drive (the drive where the

system looks for the AUTO folder and ACCessories), you will be asked if you

want to save a copy of your DESKTOP.INF file on the new Autoboot drive.

This is usually a good idea.  You will also want to make sure the ACC is

residing on the new Autoboot drive, else when you reboot you will have

to find it and copy it over, then reboot, to be able to swap back.  If you

have swapped any drives corresponding to open windows in your DESKTOP.INF

file, you will be asked to update the DESKTOP.INF.  This is also a good idea;

otherwise, you will come up without your windows open.

When DESKTOP.ACC terminates, the new swap order will be reflected in the

drive icons; also any open windows affected by the new swap order will be

changed.

A check mark under the On column in the Available partitions window means that

that partition is currently enabled (or 'visible' to ICDBOOT).

To enable or disable active partitions, click on the appropriate line under the

ON column.  Up to 14 GEM partitions may be enabled at any given time.  You

can have a total of 64 partitions for a physical hard drive (LUN) with a

maximum of 128 partitions in the system.  Actually more than 14 partitions

can be enabled, but ICDBOOT and TOS will only see the first 14 GEM/BGM partitions.

To name or rename a partition, click on any GEM or BGM partition name in

the Available Partitions window.  To get out of rename mode, click anywhere

else.  You can only edit (name/rename) GEM and BGM volume names.  The ACC

has no way of knowing how non-GEM partitions keep their volume names, hence

you can't edit these.

Bear in mind, your volume names may be upper or lower case, with spaces.

If you do a Show Info on the partition, the volume name is truncated at

the first space.  In the ACC dialog, volume names are shown with periods at

the ninth character.  This is strictly in keeping with the GEM convention

of showing a volume name as a file name.  The period will NOT show up in

the icon.  Be advised that it is NOT a good idea to use the pound sign ('#')

character as part of a volume name.  This confuses TOS.

To turn on the write verify TEMPORARILY click on the On or Off button as

desired.  This is the same function provided by our old VERIFY.ACC, and does

not affect the Write Verify configuration in ICDBOOT.  It DOES affect the

verify on your floppy disk drives, so you should probably leave it On most

of the time.  The verify does slow down file transfers, however it is usually

worth the extra time to insure good data.

To toggle the write cache function, click on the appropriate On or Off as

desired.  When turning write cache off, all buffers will be flushed if needed,

and write cacheing is then disabled until rebooting or until you explicitly

turn it back on.  This is done for those using the Supercharger PC emulator.

The Supercharger MUST have write cacheing disabled, otherwise it will NOT

write to the hard disk.  Once again, this is TEMPORARY.  It does not make any

permanent changes to your ICDBOOT file.  It only changes the booter in memory.

While in the ACC, you can force a rescan of all hard drives by clicking on

the Rescan button.  This is equivalent to running the ACC from the beginning.

You MUST finalize any actions undertaken here by clicking on OK or Reboot.

If you click on Cancel, we will undo what we can.

ICD, Inc.

We have tested this accessory thoroughly and believe it to be free from problems.

However under no circumstances will ICD, Inc. be held responsible for any damage

to information on your hard drives.