In brief, here are a few things you SHOULD know about this

software:

o    This current release includes full software support for the

Supercharger PC emulator.  You MUST determine the SCSI ID of your

Supercharger.  They are shipped as ID 3.  After setting up your

hard drive to Auto boot with HDUTIL, go to the Config option of

that program.  Near the bottom of the screen you will see a line

that says 'Skip SCSI ID's:'.  Normally only ID 6 is skipped; this

is the ICD real time clock.  You MUST also set the ID of your

Supercharger here, then save.  This means that all ICD programs

will not do any SCSI commands on this ID.  (This means that you

need no longer hold down the reset button when booting the computer

with the Supercharger attached).

You must also disable write cacheing when running the Supercharger,

otherwise you will never write to your hard disk.  This can be

accomplished in several ways.  First, you can set up your booter to

disable write cacheing.  Second, you can permanently enable write

cacheing and turn it off temporarily with the DESKTOP.ACC or with

the CACHEOFF.PRG.  There is also a CACHEON.PRG to turn the cacheing

back on.  See below for a more complete explanation.

Please note that the Supercharger is fully compatible with OLD

ICD Host Adapters.  It is NOT compatible with early release versions

of the ICD Advantage and Advantage Plus host adapters.  In addition

to the software you will need a hardware upgrade kit which replaces

one of the GAL chips on the Advantage and Advantage Plus boards. The

cost for this upgrade kit is $15.00.

Address:  ICD Advantage ST Upgrade Offer

1220 Rock St.

Rockford, IL 61101

Note that Advantage and Advantage Plus host adapters shipped from ICD

after the date of this file will have hardware upgrades in place to be

compatible with the Supercharger.