****************************************************************************

Welcome to RAP v1.0, the third smallest Point mailer presently available

for the AMIGA.

RAP is based upon POP, the world's smallest mailer, by Denis Turcotte

and JAZ, , the world's cutiest mailer, by Robert Williamson

****************************************************************************

If you like this release, let me and everyone else know.

If you don't, delete it from your system and keep quiet :)

To properly unarchive files, directories and preserve file attributes you

should use the -xra option of LHA, or manually set the RAP script to

'swerd'.

Thanks to:

Denis Turcotte       pop.wpl, control panel basics and the

original design concept of a minimal

point mailer

Russell McOrmond     wpl.library

David Jones          xferq.library

Yves Konigshofer     xprzedzap.library

Alan Kelm            LogProc

Christian Warren     CliVa

Installation  is pretty easy, just CD to where you extracted RAP and type

RAP.   The best place to extract RAP is in the volume Mail:  if you have it

assigned.   RAP will assign RAP:  to this directory and your S:User-Startup

will  be  modified to add the RAP:  assign.  If RAP:RAP.x is not found, the

RAP Control Panel will come up, allowing you to configure your mailer.

------------------------------------------------------------------

|                             WARNINGS                             |

|                                                                  |

|   If you are presently using a WPL mailer,  please be aware that |

| the  RAP  Mailer  assigns XFERQ:  to T:.  If you are just trying |

| RAP  out,  be  sure  to  REassign  XFERQ:   back  to your normal |

| location.                                                        |

|                                                                  |

| Please  also  be  aware  that  RAP  will  not work properly with |

| earlier versions of xferq.library.  The version included is 1.9, |

| if  you  are  using  1.8  or  earlier, and some files are set to |

| delete  when  sent,  they  WILL  BE DELETED EVEN IF THEY ARE NOT |

| SENT.  This bug is fixed in the included 1.9 xferq.library.      |

|                                                                  |

------------------------------------------------------------------

RAP scripts usage:

RAP <no arguments>

If  the  mailer  has been configured, this will load the mailer and

call   the  default  BOSS  node  that  was  defined  and  saved  as

ENV:RAPBOSS,  otherwise  the Control Panel will be loaded.  The RAP

script exits when the mailer is launched.

RAP PANEL

will bring up the RAP Control Panel

RAP domain#zone:net/node.point

will  load  mailer  and  prompt  you  for phone number and optional

password  for  this  site.   It  will  then call the site.  The RAP

script exits when the mailer is launched.

POLL <no arguments>

If  the  mailer  has been configured, this will load the mailer and

call   the  default  BOSS  node  that  was  defined  and  saved  as

ENV:RAPBOSS,  otherwise  the  Control  Panel  will be loaded.  This

script does not exit until the mailer exits.

CONFIG <no arguments>

executes RAP PANEL

When the RAP Mailer starts up, it will open up a non-draggable backdrop

window on the WorkBench backdrop.  This window has three regions, the upper

portion  having  fields  in  which  progress  info is displayed, the middle

portion  is  a  scrolling  status display and the lower portion display the

logfile.   If the middle portion is clicked-on, a bar will appear indicated

it is now interactive.  Phone number and password prompts appear here.  One

may also use ctl-C to kill the mailer in this area.

RAP also opens up a logfile named RAP:RAP.LOG.  Writes to this file are

buffered by LogProc and displayed in the lower portion of the window.

During a file transfer, the XPR status window will open on top, leaving

one line of the log visible.

RAP  uses  FTS6  (WaZoo)  handshaking with ZedZap transfer protocol only.

Points  don't really need anything else, since modern tossers can route all

domains..

A quick setup takes 5 minutes, well...ok...even less.

Configuration notes:

You  should configure the three panels under Configuration.  Once you are

satisfied,  click  the 'Write RAP Config' button.  RAP's mailer config will

be saved and your settings will also be saved to ENV:RAP/ for later recall.

These ENV:  settings are NOT used during mailer operation.

The  RAP  Control  panel  allows  you  to reload the default hard-coded

settings,  or  your own previous settings that were saved in ENV:RAP/.  You

are  also  able  to  reload  your  previous settings for each configuration

panel.

Please  note that Reloading Settings does NOT CHANGE the mailer config,

only a WRITE or SELECT will do that.

Please  also note that selecting a Saved Mailer Config, does NOT CHANGE

either the Control Panel settings or the default BOSS address.

Display:

The  window  and regions are sized and positioned based on FONTSIZE and

the X and Y co-ordinates for the top left-hand corner.  The sizing has only

been  tested  on  a Non-Interlace Hires screen with topaz fontsize 8 and 9.

Any improvements to the algorythms used in rap.w are welcomed.

The  mailer  window  is  opened  on  the  workbench  backdrop.  If, for

unforeseen  reasons,  the  windows do not open in the proper sequence, some

regions may not be visible.

You may specify the console device to use for the scrolling portion and

log  display  portions.   You  can use Wshell or KCON:  for example, if you

wish to use a scroll bar.

Addresses:

All addresses are specified as 5D in the format:

domain#zone:net/node.point

Your  system  name  should  not exceed about 30 characters, this is not

checked.

You  can  enter  up  to three phone numbers for your BOSS, each will be

dialed  in turn if the previous is busy.  You must enter at least one phone

number.   Phone  numbers  should not contain spaces.  Special characters as

described below can be used.

Session password must be no longer than 8 characters.  This password is

assigned to you by your BOSS. It should not contain spaces.

Modem Config Notes:

DTR control:

0 - No support for DTR

1 - Internal port 'twiddling the bits' - *ONLY USE THIS ON the internal

serial hardware!!!*

2 - SET_CTRL_LINES method (ASDG DSB, PDIO, CMI Multiport, CSA Magnum)

SerReadBuf  - Serial Receive buffer in K. This is a value given to

the particular serial driver, and the driver itself

allocates this buffer.

SerWriteBuf - Serial Write buffer in K.  This is the switch to turn

on the double buffered writes to the serial port.  The

value has to be greater than the size of the largest

single write that would be made. For example, a 9600bps+

modem uses an 8K block to write, so some value LARGER than

this (Say 10) should be used.  Two buffers of this size are

allocated.

*NOTE* : This feature separates the good serial drivers

from the bad.  If your system does not work with this

turned on, a value of 0 can be used to turn this feature

off.

Modem Response Strings:

OK          (Config Panel)

The string the modem returns when it has executed a command

sucessfully.

BUSY        (Config Panel)

The strings that will cause a redial attempt.

You  should  enter  all  modem  responses  you  wish  to be

interepreted as a BUSY.

EG: BUSY NO CARRIER

CONNECT     (Internal)

It is expected that the modem wil return a string containing

the word CONNECT and at least the link bps.

All other modem responses will cause the mailer to exit.

Modem Command strings:

Commands  sent to the modem are interpreted using the following special

characters:   These  are  in  addition  to any special characters the modem

understands (comma for example).

~      add a delay 240ms

`      add a delay 60ms

|      carriage return

^      raise DTR       Use ONLY if DTRCTL set to 1

v      drop dtr        Use ONLY if DTRCTL set to 1

\      escape special character

(in case you USE some of the above as text :)

One would precede these characters with the '\' escape character if the

special character needed to be sent out the modem).  In order to be able to

send an "AT\" type command to the modem, you must use 4 '\' in the string.

eg:

AT\\\\X3

SLOWMODEM  TRUE  will  cause  a  60ms  delay  to be placed between each

character sent during initialisations.

CALLDELAY is the number of seconds to wait between dial attempts.

MODEMWAIT is the number of seconds to wait for a modem response

(connect, busy, etc...)

INBOUND and OUTBOUND directories should be set to same as your point

tosser.

The various EXTERNAL COMMAND gadgets are provided for your convience.

For a more permanent installation, be sure your startup-sequence copies

ENVARC: to ENV:.

You may also move the libraries to you LIBS: directory and edit the RAP

script to change all references from 'RAP:libs/' to 'LIBS:'.

If  you  are  running  WB3.1,  you  should  be able to use LoadResource

instead of LoadLib.

Those other utils:

LaunchWPL:

A  WPL program load, launch and flush utility  by Robert Williamson

for JamMail, Pop, RAP and other single module mailers

FREQ: LaunchWPL12.lha

CliVa:

A powerful gadget interface by Christian Warren (The DekSid guy)

Freq: CLIVA23.LHA

LogProc:

A powerful window and file log manager with arexx interface

by Alan Kelm

Freq: LogProc1_3a.lha

All are available on AmiNet as well.

Technical Notes:

RAP  uses  xferq.library for queue management; but unlike other WPL

mailers, does so in a way that is transparent to your tosser.  The flatfile

format (?LO and ?UT files) is converted to xferq format only when connected

to  the  site.  This removes the necessity of other FLO file convertors.  A

second advantage is that memory and disk requirements are reduced.

RAP  will  use  OwnDevUnit.library,  if  available,  to request the

serial port from port master.  It will not respond to ODU requests.  ODU is

NOT included however.

RAP Control Panel:

The  Control  Panel  is  created  using CliVa and is written in the

CliVa  gadget  language.  It is provided as an interface to the RAP Mailer.

Configurable  gadgets are provided for commonly used commands and well as a

few extras. For those who wish to learn to use Cliva, freq CLIVA23.LHA.

Robert Williamson 1:167/104@fidonet

robert@ecs.mtlnet.org

bobw@amigaecs.1choice.login.qc.ca