1230ScsiFix V1.6 by Chris Hodges

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Introduction

~~~~~~~~~~~~

This is a small patch  which  will  fix  some  problems  occuring  on  some

machines  with  Blizzard  1230/III  and  SCSI-Kit.  This  'bug'  causes the

controller to perform very slowly. The patch  just  turns  off  the  caches

during scsi chip accesses. However, this made a change of the transfer rate

of under 80 KB/s to 2,6 MB/s of a friend's harddisk.

The patch can be made resident. If you don't use  BlizKick  you  *have*  to

REMOVE  the MAPROM Jumper on your board to avoid losing the RomTag at every

bootup!

NOTE: The blue stripes showing up in previous versions during booting  have

been removed.

Removal of the patch is possible both during reboot (by pressing  the  left

mouse button and not the right one), or using "1230ScsiFix OFF".

A BlizKick Module is also supplied.

I would really appreciate, if you could tell me, if the patch does its  job

on your machine.

Requirements

~~~~~~~~~~~~

1230ScsiStub needs Kick 2.04 or higher. Obviously also requires a  Blizzard

SCSI Controller.

Usage

~~~~~

1230ScsiFix [OFF] [RESIDENT] [ICACHE] [DCACHE]

This line should be placed  just  behind  your  SetPatch  command  in  your

startup-sequence (e.g "1230ScsiFix >NIL: RESIDENT").

Specifying the RESIDENT switch will make the  patch  reset-resident  (using

RomTags).  The  RESIDENT option only will take effect when the patch is not

already in memory (i.e calling 1230ScsiFix and  then  1230ScsiFix  RESIDENT

will not add the RomTag).

By using the OFF switch you can turn off  the  patch  again,  but  normally

there  is  no  need  to  do this. If you have installed the patch using the

BlizKick Module, the memory of the patch is not freed for obvious reasons.

The ICACHE and DCACHE  switches  can  be  used  to  optimize  your  system.

Normally,  both  caches  are turned off during scsi chip access. Specifying

ICACHE or DCACHE will turn off only the instruction or the data caches. Try

out  these  to  find  out  the  best  performance of your controller. These

switches can be used at any time (e.g after installing the patch) and  even

in conjunction with the previously installed BlizKick Module.

History

~~~~~~~

V1.6 (24-Apr-97):

- Added ICACHE and DCACHE switches.

- CPU Caches are turned on again on removal.

- Internal changes and little fixes.

V1.5 (20-Apr-97):

- Probably fixed RomTag with help from Laire.

- Removed blue stripes on bootup.

V1.4 (17-Apr-97):

- Probably is now a BlizKick module - not tested yet.

V1.3 (13-Apr-97):

- First release.

Contact address

~~~~~~~~~~~~~~~

Any mail, comments or donations welcome:

Chris Hodges                    Account: 359 68 63

Kennedystr. 8                   BLZ    : 700 530 70

D-82178 Puchheim                Bank   : Sparkasse F�rstenfeldbruck

Germany

Tel.: +49-89/8005856

Email: platon@cum.de            WWW: http://www.cum.de/user/platon/