___   ___   ___  _____

/ _ \ / _ \ / _ \| ____|

| (_) | | | | (_) | |__

> _ <| | | |> _ <|___ \

| (_) | |_| | (_) |___) |

\___/ \___/ \___/|____/

*************************

Symbols and abbreviations

accumulator     register A

addr            16-bit address

data            8-bit data

data16          16-bit data

port            8-bit address for I/O

r,r1,r2         one of the registers A,B,C,D,E,H,L

DDD,SSS         pattern of a register (DDD is destination, SSS is source)

DDD or SSS | register

-----------+---------

111    |    A

000    |    B

001    |    C

010    |    D

011    |    E

100    |    H

101    |    L

rp              one of the register pairs BC,DE,HL,SP

RP              pattern of a register pair

RP  | register pair

------+--------------

00  |      BC

01  |      DE

10  |      HL

11  |      SP

SP              stack pointer

PC              program counter

h bevor or

after symbol    the high byte of 16-bit data or address

l bevor or

after symbol    the low byte of 16-bit data or address

Z               Zero flag

S               Sign flag

P               Parity flag

CY              Carry flag

AC              Auxiliary carry flag

flags           xxxxx represent CY P AC Z S

where x can be

- = is not changed

X = is changed

0 = becomes cleared

1 = becomes set

()              content of register or address

a <-- b         a gets overwritten by b

a <--> b        a and b are exchanged

AND             logical AND

EOR             logical exclusive OR

OR              logical OR

n               number from 0..7

NNN             binary representation of n

x[n]            bit n from x

cc and CCC        cc  | condition        |  CCC

------+------------------+-------

NZ  | not zerro (Z=0)  |  000

Z   | zero (Z=1)       |  001

NC  | not carry (CY=0) |  010

C   | carry (CY=1)     |  011

PO  | parity odd (P=0) |  100

PE  | parity even (P=1)|  101

P   | plus (S=0)       |  110

M   | minus (S=1)      |  111

8085 Commands

Name        : Move register

Syntax      : MOV r1,r2

Semantic    : (r1) <-- (r2)

Code        : 01DDDSSS

Length      : 1

Cycles      : 4

Flags       : -----

Description : Content of register r2 is written into r1

Name        : Move from memory

Syntax      : MOV r,M

Semantic    : (r) <-- ((H) (L))

Code        : 01DDD110

Length      : 1

Cycles      : 7

Flags       : -----

Description : Content of cell where (H,L) points to is written into r

Name        : Move to memory

Syntax      : MOV M,r

Semantic    : ((H) (L)) <-- (r)

Code        : 01110SSS

Length      : 1

Cycles      : 7

Flags       : -----

Description : Content of register r is written into cell where (H,L) points to

Name        : Move immediate

Syntax      : MVI r,data

Semantic    : (r) <-- data

Code        : 00DDD110 data

Length      : 2

Cycles      : 7

Flags       : -----

Description : Value data is written into register r

Name        : Move to memory immediate

Syntax      : MVI M,data

Semantic    : ((H) (L)) <-- data

Code        : 00110110 data

Length      : 2

Cycles      : 10

Flags       : -----

Description : Value data is written into cell where (H,L) points to

Name        : Load register pair immediate

Syntax      : LXI rp,data16

Semantic    : (rh) <-- hdata16

(rl) <-- ldata16

Code        : 00RP0001 ldata16 hdata16

Length      : 3

Cycles      : 10

Flags       : -----

Description : Value data16 is written into register pair HL

Name        : Load H and L direct

Syntax      : LHLD addr

Semantic    : (L) <-- (addr)

(H) <-- (addr+1)

Code        : 00101010 laddr haddr

Length      : 3

Cycles      : 16

Flags       : -----

Description : Content of cell where addr points to is written into

register L, content of cell where addr+1 points to is written

into register H

Name        : Store H and L direct

Syntax      : SHLD addr

Semantic    : (addr)   <-- (L)

(addr+1) <-- (H)

Code        : 00100010 laddr haddr

Length      : 3

Cycles      : 16

Flags       : -----

Description : Content of register L is written into cell where addr points

to, content of register H is written into cell where addr+1

points to

Name        : Load accumulator indirect

Syntax      : LDAX rp

Semantic    : (A) <-- ((RP))

Code        : 00RP1010

Length      : 1

Cycles      : 7

Flags       : -----

Description : Content of cell where RP (only BC or DE allowed) points to

is written into accumulator

Name        : Store accumulator indirect

Syntax      : STAX rp

Semantic    : ((RP)) <-- (A)

Code        : 00RP0010

Length      : 1

Cycles      : 7

Flags       : -----

Description : Content of accumulator is written into cell where RP (only

BC or DE allowed) points to

Name        : Load accumulator direct

Syntax      : LDA addr

Semantic    : (A) <-- (addr)

Code        : 00111010 laddr haddr

Length      : 3

Cycles      : 13

Flags       : -----

Description : Content of cell where addr points to is written into accumulator

Name        : Store accumulator direct

Syntax      : STA addr

Semantic    : (addr) <-- (A)

Code        : 00110010 laddr haddr

Length      : 3

Cycles      : 13

Flags       : -----

Description : Content of accumulator is written into cell where addr points to

Name        : Exchange H and L with D and E

Syntax      : XCHG

Semantic    : (H) <--> (D)

(L) <--> (E)

Code        : 11101011

Length      : 1

Cycles      : 4

Flags       : -----

Description : Exchange the register pairs HL and DE

Name        : Add register

Syntax      : ADD r

Semantic    : (A) <-- (A) + (r)

Code        : 10000SSS

Length      : 1

Cycles      : 4

Flags       : XXXXX

Description : Add the register r to accumulator

Name        : Add memory

Syntax      : ADD M

Semantic    : (A) <-- (A) + ((H) (L))

Code        : 10000110

Length      : 1

Cycles      : 7

Flags       : XXXXX

Description : Add the content of cell where (H,L) points to the accumulator

Name        : Add immediate

Syntax      : ADI data

Semantic    : (A) <-- (A) + data

Code        : 11000110 data

Length      : 2

Cycles      : 7

Flags       : XXXXX

Description : Add data to accumulator

Name        : Add register with carry

Syntax      : ADC r

Semantic    : (A) <-- (A) + (r) + (CY)

Code        : 10001SSS

Length      : 1

Cycles      : 4

Flags       : XXXXX

Description : Add the register r and the carryflag to accumulator

Name        : Add memory with carry

Syntax      : ADC M

Semantic    : (A) <-- (A) + ((H) (L)) + (CY)

Code        : 10001110

Length      : 1

Cycles      : 7

Flags       : XXXXX

Description : Add the content of cell where (H,L) points and the carryflag

to the accumulator

Name        : Add immediate with carry

Syntax      : ACI data

Semantic    : (A) <-- (A) + data + (CY)

Code        : 11001110 data

Length      : 2

Cycles      : 7

Flags       : XXXXX

Description : Add data and the carryflag to accumulator

Name        : Subtract register

Syntax      : SUB r

Semantic    : (A) <-- (A) - (r)

Code        : 10010SSS

Length      : 1

Cycles      : 4

Flags       : XXXXX

Description : Subtract register r from accumulator

Name        : Subtract memory

Syntax      : SUB M

Semantic    : (A) <-- (A) - ((H) (L))

Code        : 10010110

Length      : 1

Cycles      : 7

Flags       : XXXXX

Description : Subtract content of cell where (H,L) points from accumulator

Name        : Subtract immediate

Syntax      : SUI data

Semantic    : (A) <-- (A) - data

Code        : 11010110 data

Length      : 2

Cycles      : 7

Flags       : XXXXX

Description : Subtract data from accumulator

Name        : Subtract register with borrow

Syntax      : SBB r

Semantic    : (A) <-- (A) - (r) - (CY)

Code        : 10011SSS

Length      : 1

Cycles      : 4

Flags       : XXXXX

Description : Subtract register r and carryflag from accumulator

Name        : Subtract memory with borrow

Syntax      : SBB M

Semantic    : (A) <-- (A) - ((H) (L)) - (CY)

Code        : 10011110

Length      : 1

Cycles      : 7

Flags       : XXXXX

Description : Subtract content of cell where (H,L) points and carryflag

from accumulator

Name        : Subtract immediate with borrow

Syntax      : SBI data

Semantic    : (A) <-- (A) + data - (CY)

Code        : 11011110 data

Length      : 2

Cycles      : 7

Flags       : XXXXX

Description : Subtract data and the carryflag from accumulator

Name        : Increment register

Syntax      : INR r

Semantic    : (r) <-- (r) + 1

Code        : 00DDD100

Length      : 1

Cycles      : 4

Flags       : -XXXX

Description : Increment register r

Name        : Increment memory

Syntax      : INR M

Semantic    : ((H) (L)) <-- ((H) (L)) + 1

Code        : 00110100

Length      : 1

Cycles      : 10

Flags       : -XXXX

Description : Increment content of cell where (H,L) points to

Name        : Decrement register

Syntax      : DCR r

Semantic    : (r) <-- (r) - 1

Code        : 00DDD101

Length      : 1

Cycles      : 4

Flags       : -XXXX

Description : Decrement register r

Name        : Decrement memory

Syntax      : DCR M

Semantic    : ((H) (L)) <-- ((H) (L)) - 1

Code        : 00110101

Length      : 1

Cycles      : 10

Flags       : -XXXX

Description : Decrement content of cell where (H,L) points to

Name        : Increment register pair

Syntax      : INX rp

Semantic    : (rh)(rl) <-- (rh)(rl) + 1

Code        : 00RP0011

Length      : 1

Cycles      : 6

Flags       : -----

Description : Increment register pair rp

Name        : Decrement register pair

Syntax      : DCX rp

Semantic    : (rh)(rl) <-- (rh)(rl) - 1

Code        : 00RP1011

Length      : 1

Cycles      : 6

Flags       : ------

Description : Decrement register pair rp

Name        : Add register pair to H and L

Syntax      : DAD rp

Semantic    : (H)(L) <-- (H)(L) + (rh)(rl)

Code        : 00RP1001

Length      : 1

Cycles      : 10

Flags       : X-----

Description : Add register pair rp to HL

Name        : Decimal adjust accumulator

Syntax      : DAA

Semantic    : (A) <-- adjusted (A)

Code        : 00100111

Length      : 1

Cycles      : 4

Flags       : XXXXX

Description : Correct the BCD number in the accumulator that has been

corrupted by an addition or subtraction. In particular

following two steps are performed:

1.) If the four lower bits of the accumulator are greater

nine or the AC flag is set then six is added to the

accumulator.

2.) If the four higher bits of the accumulator are now greater

nine or if the CY flag is set the six is added to the

higher four bits.

Name        : AND register

Syntax      : ANA r

Semantic    : (A) <-- (A) AND (r)

Code        : 10100SSS

Length      : 1

Cycles      : 4

Flags       : 0X1XX

Description : The accumulator is combined with the register r by a logical

AND

Name        : AND memory

Syntax      : ANA M

Semantic    : (A) <-- (A) AND ((H) (L))

Code        : 10100110

Length      : 1

Cycles      : 7

Flags       : 0X1XX

Description : The accumulator is combined with the content of cell where (H,L)

points to by a logical AND

Name        : AND immediate

Syntax      : ANI data

Semantic    : (A) <-- (A) AND data

Code        : 11100110 data

Length      : 2

Cycles      : 7

Flags       : 0X1XX

Description : The accumulator is combined with data by a logical AND

Name        : Exclusive OR register

Syntax      : XRA r

Semantic    : (A) <-- (A) EOR (r)

Code        : 10101SSS

Length      : 1

Cycles      : 4

Flags       : 0X0XX

Description : The accumulator is combined with the register r by a logical

exclusive OR

Name        : Exclusive OR memory

Syntax      : XRA M

Semantic    : (A) <-- (A) EOR ((H) (L))

Code        : 10101110

Length      : 1

Cycles      : 7

Flags       : 0X0XX

Description : The accumulator is combined with the content of cell where (H,L)

points to by a logical exclusive OR

Name        : Exclusive OR immediate

Syntax      : XRI data

Semantic    : (A) <-- (A) EOR data

Code        : 11101110 data

Length      : 2

Cycles      : 7

Flags       : 0X0XX

Description : The accumulator is combined with data by a logical exclusive OR

Name        : OR register

Syntax      : ORA r

Semantic    : (A) <-- (A) OR (r)

Code        : 10110SSS

Length      : 1

Cycles      : 4

Flags       : 0X0XX

Description : The accumulator is combined with the register r by a logical OR

Name        : OR memory

Syntax      : ORA M

Semantic    : (A) <-- (A) OR ((H) (L))

Code        : 10110110

Length      : 1

Cycles      : 7

Flags       : 0X0XX

Description : The accumulator is combined with the content of cell where (H,L)

points to by a logical OR

Name        : OR immediate

Syntax      : ORI data

Semantic    : (A) <-- (A) OR data

Code        : 11110110 data

Length      : 2

Cycles      : 7

Flags       : 0X0XX

Description : The accumulator is combined with data by a logical OR

Name        : Compare register

Syntax      : CMP r

Semantic    : (A) - (r)

Code        : 10111SSS

Length      : 1

Cycles      : 4

Flags       : XXXXX

Description : Flags are set as if the register r was subtracted from

accumulator.

Name        : Compare memory

Syntax      : CMP M

Semantic    : (A) - ((H) (L))

Code        : 10111110

Length      : 1

Cycles      : 7

Flags       : XXXXX

Description : Flags are set as if the content of the cell where (H,L) points

to was subtracted from the accumulator

Name        : Compare immediate

Syntax      : CPI data

Semantic    : (A) - data

Code        : 11111110 data

Length      : 2

Cycles      : 7

Flags       : XXXXX

Description : Flags are set as if the data was subtracted from the accumulator

Name        : Rotate left

Syntax      : RLC

Semantic    : (A[n+1]) <-- (A[n]) ; (A[0]) <-- (A[7]) ; (CY) <-- (A[7])

Code        : 00000111

Length      : 1

Cycles      : 4

Flags       : X----

Description : The accumulator is shifted cyclical to the left. The highest

bit is also copied in the carry flag

Name        : Rotate right

Syntax      : RRC

Semantic    : (A[n]) <-- (A[n+1]) ; (A[7]) <-- (A[0]) ; (CY) <-- (A[0])

Code        : 00001111

Length      : 1

Cycles      : 4

Flags       : X----

Description : The accumulator is shifted cyclical to the right. The lowest

bit is also copied in the carry flag

Name        : Rotate left through carry

Syntax      : RAL

Semantic    : (A[n+1]) <-- (A[n]) ; (CY) <-- (A[7]) ; (A[0]) <-- (CY)

Code        : 00010111

Length      : 1

Cycles      : 4

Flags       : X----

Description : The accumulator is shifted cyclical to the left through the

the carry flag

Name        : Rotate right through carry

Syntax      : RAR

Semantic    : (A[n]) <-- (A[n+1]) ; (CY) <-- (A[0]) ; (A[7]) <-- (CY)

Code        : 00011111

Length      : 1

Cycles      : 4

Flags       : X----

Description : The accumulator is shifted cyclical to the right through the

the carry flag

Name        : Complement accumulator

Syntax      : CMA      _

Semantic    : (A) <-- (A)

Code        : 00101111

Length      : 1

Cycles      : 4

Flags       : -----

Description : All 0 bits in the accumulator are replaced by 1 and all

1 bits by 0.

Name        : Complement carry

Syntax      : CMC       __

Semantic    : (CY) <-- (CY)

Code        : 00111111

Length      : 1

Cycles      : 4

Flags       : X----

Description : The carry flag is complemented

Name        : Set carry

Syntax      : STC

Semantic    : (CY) <-- 1

Code        : 00110111

Length      : 1

Cycles      : 4

Flags       : 1----

Description : The carry flag is set

Name        : Jump

Syntax      : JMP addr

Semantic    : (PC) <-- addr

Code        : 11000011 laddr haddr

Length      : 3

Cycles      : 10

Flags       : -----

Description : The program counter is set to addr

Name        : Conditional jump

Syntax      : Jcc addr

Semantic    : If (cc)

(PC) <-- addr

Code        : 11CCC010 laddr haddr

Length      : 3

Cycles      : 7/10

Flags       : -----

Description : The program counter is set to addr if the conition is true

Name        : Call

Syntax      : CALL addr

Semantic    : ((SP)-1) <-- (hPC)

((SP)-2) <-- (lPC)

(SP) <-- (SP) - 2

(PC) <-- addr

Code        : 11001101 laddr haddr

Length      : 3

Cycles      : 18

Flags       : -----

Description : The current program counter is saved on the stack and the

program counter is set to addr, i.e. a subroutine is called

Name        : Condition call

Syntax      : Ccc addr

Semantic    : If (cc)

((SP)-1) <-- (hPC)

((SP)-2) <-- (lPC)

(SP) <-- (SP) - 2

(PC) <-- addr

Code        : 11CCC100 laddr haddr

Length      : 3

Cycles      : 9/18

Flags       : -----

Description : If the conition is true the current program counter is saved on

the stack and the program counter is set to addr, i.e. a

subroutine is called

Name        : Return

Syntax      : RET

Semantic    : (lPC) <-- ((SP))

(hPC) <-- ((SP)+1)

(SP) <-- (SP) + 2

Code        : 11001001

Length      : 1

Cycles      : 10

Flags       : -----

Description : Return from a previous subroutine call

Name        : Conditional return

Syntax      : Rcc

Semantic    : If (cc)

(lPC) <-- ((SP))

(hPC) <-- ((SP)+1)

(SP) <-- (SP) + 2

Code        : 11CCC000

Length      : 1

Cycles      : 6/12

Flags       : -----

Description : Return from a previous subroutine call if the conition is true

Name        : Restart

Syntax      : RST n

Semantic    : ((SP)-1) <-- (hPC)

((SP)-2) <-- (lPC)

(SP) <-- (SP) - 2

(PC) <-- 8 * n

Code        : 11SSS111

Length      : 1

Cycles      : 12

Flags       : -----

Description : Same as CALL 8*n

Name        : JumpHandLindirect

Syntax      : PCHL

Semantic    : (lPC) <-- (L)

(hPC) <-- (H)

Code        : 11101001

Length      : 1

Cycles      : 6

Flags       : -----

Description : HL is written into the program counter

Name        : Push

Syntax      : PUSH rp

Semantic    : ((SP)-1) <-- (rh)

((SP)-2) <-- (rl)

Code        : 11RP0101

Length      : 1

Cycles      : 12

Flags       : -----

Description : Register pair rp is saved on the stack

Name        : Push processor status word

Syntax      : PUSH PSW

Semantic    : ((SP)-1) <-- (A)

((SP)-2[0]) <-- (CY)

((SP)-2[2]) <-- (P)

((SP)-2[4]) <-- (AC)

((SP)-2[6]) <-- (Z)

((SP)-2[7]) <-- (S)

(SP) <-- (SP) - 2

Code        : 11110101

Length      : 1

Cycles      : 12

Flags       : -----

Description : The accumulator and the processor status word are saved on the

stack

Name        : Pop

Syntax      : POP rp

Semantic    : (rl) <-- ((SP))

(rh) <-- ((SP)+1)

(SP) <-- (SP) + 2

Code        : 11RP0001

Length      : 1

Cycles      : 10

Flags       : -----

Description : Register pair rp is loaded from stack

Name        : Pop processor status word

Syntax      : POP PSW

Semantic    : (CY) <-- ((SP)[0])

(P)  <-- ((SP)[2])

(AC) <-- ((SP)[4])

(Z)  <-- ((SP)[6])

(S)  <-- ((SP)[7])

(A)  <-- ((SP)+1)

(SP) <-- (SP) + 2

Code        : 11110001

Length      : 1

Cycles      : 10

Flags       : XXXXX

Description : The processor status word and the accumulator are loaded from

the stack

Name        : Exchange stack top with H and L

Syntax      : XTHL

Semantic    : (L) <--> ((SP))

(H) <--> ((SP)+1)

Code        : 11100011

Length      : 1

Cycles      : 16

Flags       : -----

Description : The register pair HL is exchanged with the stack top

Name        : Move HL to SP

Syntax      : SPHL

Semantic    : (SP) <-- (H) (L)

Code        : 11111001

Length      : 1

Cycles      : 6

Flags       : -----

Description : Register pair HL is written into SP

Name        : Input

Syntax      : IN (port)

Semantic    : (A) <-- data

Code        : 11011011 port

Length      : 2

Cycles      : 10

Flags       : -----

Description : The data of the specified port is read into the accumulator

Name        : Output

Syntax      : OUT (port)

Semantic    : data <-- (A)

Code        : 11010011 port

Length      : 2

Cycles      : 10

Flags       : -----

Description : The content of the accumulator is sent to the specified port

Name        : Enable interrupts

Syntax      : EI

Semantic    : (enable interrupts) <-- 1

Code        : 11111011

Length      : 1

Cycles      : 4

Flags       : -----

Description : Interrupts are enabled

Name        : Disable interrupts

Syntax      : DI

Semantic    : (enable interrupts) <-- 0

Code        : 11110011

Length      : 1

Cycles      : 4

Flags       : -----

Description : Interrupts are disable

Name        : Read interrupt masks

Syntax      : RIM

Semantic    : (A) <-- (interrupt mask)

Code        : 00100000

Length      : 1

Cycles      : 4

Flags       : -----

Description : Read interrupt masks and seriell data into accumulator.

A[7] A[6] A[5] A[4] A[3] A[2] A[1] A[0]

SID  I7.5 I6.5 I5.5 I7.5 M7.5 M6.5 M5.5

Name        : Set interrupt masks

Syntax      : SIM

Semantic    : (interrupt mask) <-- (A)

Code        : 00110000

Length      : 1

Cycles      : 4

Flags       : -----

Description : Set interrupt masks and seriell data from accumulator.

A[7] A[6] A[5] A[4] A[3] A[2] A[1] A[0]

SOD  SOE  X    R7.5 MSE  M7.5 M6.5 M5.5

Name        : Halt

Syntax      : HLT

Semantic    : (proccessor) <-- halt

Code        : 01110110

Length      : 1

Cycles      : 5

Flags       : -----

Description : The proccessor is stopped

Name        : No operation

Syntax      : NOP

Semantic    :

Code        : 00000000

Length      : 1

Cycles      : 4

Flags       : -----

Description : Do nothing