Cross64 v0.03   6510 Cross assembler

------------------------------------

INTRODUCTION

Cross64 is a cross assembler for the CMOS 6510 processor used in

Commodore 64. It is written in portable ANSI-C, so it should be

compilable on just about any computer system there is. Cross64 has

been developed on an Amiga 1200 running AmigaOS with SAS/C compiler

version 6.58.

Features:

- assembles all standard 6510 machine code instructions and addressing

modes

- case insensitivity

- user definable labels (either constant or address labels)

- optionally prints out the source, compiled code and defined labels

for debugging purposes

USAGE

cross64 [-<options>] <source> [<executable>]

The <options> are defined as a string of characters preceded with a

'-' character. <source> is the name of the source file. <executable>

is the name of the executable file. If no <executable> is given, the

name will be constructed from the <source> by replacing the suffix

with '.prg'.

Available options:

s	Prints the source code with line numbers.

a	Prints the compiled code with line numbers, hex values and

source code.

l	Prints the labels defined in the soource.

SPECIAL COMMANDS

ORG <address>

Specifies the address in C64 memory where all following code will be

placed. Must be specified before the first machine code instruction.

Can be used multiple times. However, no "backwards jumping" allowed,

ie. the address must be greater than the current Program Counter.

.BYTE or DC.B <byte>[,<byte>,...]

Defines byte data.

.WORD or DC.W <word>[,<word>,...]

Defines word data.

INCBIN <filename>	(not implemented)

Inserts the contents of a binary file in the program.

DS.B <amount>

Defines an empty area of <amount> bytes.

DS.W <amount>

Defines an empty area of <amount> words.

HISTORY

v0.03 (03.09.97):

- first working version

- compiling simple programs is possible