O M T I - S O F T W A R E

#=#=#=#=#=#=#=#=#=#=#=#=#

1.0. Hardware

-------------

This  archive contains the sources and executeables to a harddisk-device, and

some  support  programs, who work together with the so called 'omti-harddisk'

interface,  which  interfaces an PC Omti 5520 / 5527 controller to the Amiga.

This  hardware  was published by the german computer magazine c't.  I haven't

designed a PCB by myself.  The c't board must be modified with one more chip:

Omti-Controller         Amiga Bus

DREQ   ----|)*---+

INTREQ ----|)*---+----- INT2

2/6 74LS05

No,  sorry,  I  don't  know  the  exact  pin  numbers  anymore.   I made this

modification  to  my  computer  about  three  years  ago and haven't kept the

documentation.   Best thing is to solder the 74LS05 on the back of one of the

other chips of the c't harddisk interface.

The  c't  harddisk  interface  is *A HACK*!  Its controller has no autoconfig

capability and is fixed at 0xee0000.

1.1. The software

This  software was written with an Omti 5527 controller.  It was never tested

with an Omti 5520 and MFM drives.  I'm in doubt, whether it will work without

modifications.

I'm  using  this  software  together  with two Seagate Drives, a ST238R and a

ST296R  drive  on  an A500.  The board of the A500 is built together with the

two  disks,  a 2 Meg Memory expansion, a serial board and two 3"5 Drives in a

big case.  If you're on the UseNet, you may have heard of 'castle.franken.de'

This is this machine...  :)

This archive contains sources and executeables to the following programs:

- Boot-Startup-Code for AmigaOS 1.3 and 2.04

- Custom-Bootblocks for AmigaOS 1.3 and 2.04

- example Bootcode

- A Drive-LED Simulator in two versions (only one working :)

- A Low-Level Formatter

- An information-program

- An Disk-installer

- A program to read and to write the boot tracks

- sources for the device

You'll need a directory 'DEVS:OMTI' where various files are stored.

2.0. Outline

------------

It  always annoyed me, that, while using harddisks, I had to boot from floppy

disk  in  order  to get the omti.device etc.  loaded into memory.  I tried to

build  a  piece  of  hardware, which contained EPROMs with the boot code, but

this failed miserably. So I started moving another way:

The  AmigaOS  *MUST*  access  the  floppy  drive in any case.  It must check,

whether  there  is  a  disk  in  the  drive(s) and boot from them, if they're

bootable.   Why  not  write  a custom bootblock and use a custom disk layout,

which can boot from the harddisk with just reading the disk-bootblocks.

So the following concept was created:

- Loading the bootblock from floppy disk. It contains a bootstrap-loader, which

is made resident under AmigaOS 1.3 (see below). This one loads the boot-tracks

(normally tracks 1 - 3 on Cyl 0) from the harddisk). This code is checked for

a 'BOOT' label, and if this exists, it is executed.

- Under AmigaOS 1.3, this code mounts the following devices:

- CON

- RAW

- NEWCON

- PAR

- PRT

- RAM

- DFx

- one harddisk

assigns the following Volumes:

- SYS

- LIBS

- DEVS

- FONTS

- C

- L

- S

loads the Devs:System-Configuration

installs the FastFileSystem

loads and installs the omti.device

opens the first CLI Window with NEWCON:

- Under AmigaOS 2.0, this code is more sparse (it needn't to do so much patches

as under AmigaOS 1.3, because many of these things are already done by the OS.

- it just loads and installs the omti.device

- mounts up to four partitions on each of the two possible drives

I  also  wanted  to  set  up  some  kind  of  parameter  block, who keeps all

information  about the drive.  This became Block 0 on Track 0 on Cyl 0 of the

harddisk.

In  general, you can say, that the Cyl 0 of a harddisk is reserved for system

useage.

3.0 Bootblocks

--------------

The code for the bootblocks is found in src/bootblock/1.3 and src/bootblock/2.0

3.1. Code for AmigaOS 1.3

-------------------------

This  code  makes itself resident and reset-able, if you use the 'setpatch R'

option,  which  is  also used for the RAD:  ramdisk.  It prevents itself from

being overwritten by the flaky reset routine in AmigaOS 1.3

If  you  use  this  code,  you'll need exactly *ONE* disk access *EVER*.  All

further  attempts to boot from harddisk are made from this resident bootblock

code.

You  can  override  this  by either inserting a bootable disk in drive DF0 or

pressing the left mousebutton while the reset sequence.

3.2 Code for AmigaOS 2.0

------------------------

This  code  does  *NOT*  make  itself  resident.  Reason for this was, that I

wanted  to  insert  the  reboot  code  properly  into  the  AmigaOS 2.0 reset

sequence,  i.a.   after  the initalisation but before the boot menu (the boot

menu should show all drives already mounted) but I never managed to get it to

work.   Then  again  I  lost  interest  in  writing the resident routines and

dropped  them alltogether.  So this bootblock must be read for every reset or

you'll see the disk animation.

Nevertheless,   this   bootblock   sets   the   'Silent   Startup'   Bit   in

expansion.library,  so  you'll  boot without a window up to the first console

write.

4.0. StartUp-Code

-----------------

*WARNING* YOU MUST CUSTOMIZE THIS CODE FOR YOUR OWN GOOD & HARDDISK!

4.1. StartUp-Code for AmigaOS 1.3

---------------------------------

This  code  is rather crude and not documented.  It relies heavily on the not

documeted  features  of  AmigaOS  1.3  and  dos.library.  However, it was not

written by me and I'm not willing to support it.  Use AmigaOS 2.0.

If  you really want to bother someone with this code, ask the author, Michael

Schr�der  (mlschroe@immd4.informatik.uni-erlangen.de).  If he's in good mood,

he may help.

4.2. StartUp-Code for AmigaOS 2.0

---------------------------------

This code first sets up the device (it contains some kind of small relocator)

and binds it into the device-list.

Then it mounts all of your harddisk-partitions. So you don't need any mountlist.

Every harddisk has an entry like this in the boot-code:

part0           dc.b    'DH1',0,0,0,0,0

dc.l    20           ; Boot-Priority

dc.l    0            ; Name of the Drive

dc.l    0            ; Name of the Device

dc.l    0            ; Unit for omti.device (Unit 0)

dc.l    0            ; Flags f�r OpenDevice

;STRUCTURE DosEnvec

dc.l    16           ;     ULONG de_TableSize         ; Size of Environment vector

dc.l    128          ;     ULONG de_SizeBlock         ; in longwords: standard value is 128

dc.l    0            ;     ULONG de_SecOrg            ; not used; must be 0

dc.l    4            ;     ULONG de_Surfaces          ; # of heads (surfaces). drive specific

dc.l    1            ;     ULONG de_SectorPerBlock    ; not used; must be 1

dc.l    26           ;     ULONG de_BlocksPerTrack    ; blocks per track. drive specific

dc.l    2            ;     ULONG de_Reserved          ; DOS reserved blocks at start of partition.

dc.l    0            ;     ULONG de_PreAlloc          ; DOS reserved blocks at end of partition

dc.l    0            ;     ULONG de_Interleave        ; usually 0

dc.l    1            ;     ULONG de_LowCyl            ; starting cylinder. typically 0

dc.l    300          ;     ULONG de_HighCyl           ; max cylinder. drive specific

dc.l    30           ;     ULONG de_NumBuffers        ; Initial # DOS of buffers.

dc.l    1            ;     ULONG de_BufMemType        ; type of mem to allocate for buffers

dc.l    256          ;     ULONG de_MaxTransfer       ; Max number of bytes to transfer at a time

dc.l    $fffffffe    ;     ULONG de_Mask              ; Address Mask to block out certain memory

dc.l    0            ;     LONG  de_BootPri           ; Boot priority for autoboot

dc.l    $444f5301    ;     ULONG de_DosType           ; ASCII (HEX) string showing filesystem type;

This is mainly the information, that you'll find in a mountlist entry.

You'll find a little more information in the dos/ includes.

Please don't forget, that Cyl 0 is reserved for the system!

Bootpriority is not yet used. (I doubt if it ever will be)

4.3. How to put startup code on the disk?

-----------------------------------------

This is the most interesting issue for you.

You'll  need  a  good monitor for this.  I recommend FMon, written by Michael

Schr�der, which you can find in the UseNet Archives (comp.binaries.amiga).

4.3.1. For AmigaOS 1.3

----------------------

Don't  do  it  yourself.   You'll  need  more  help that I can give you.  Ask

Michael.   Use the supplied boottracks for AmigaOS 1.3 as you starting point.

These contain all four boottracks from 0 - 3.

Track layout for AmigaOS 1.3:

0000 - 00FF			Parameter-Block  / Track 0

3400 - 34FF			Basic boot code  / Tracks 1 - 3

3500 - 3CFF			omti.device

3D00 - 6CFF			Fast File System

6D00 - 8AFF			NewCon-Handler

8B00 - A6FF			Shell-Seg

A700 - AFFF			StartUp-Code

B000 - CFFF			free

FFS,  NewCon  and  ShellSeg  have  been  *HEAVILY* modified to work with this

layout.

Don't  worry  if you can't get your system alife with AmigaOS 1.3.  Don't use

it.  Use AmigaOS 2.0.

4.3.2. For AmigaOS 2.0

----------------------

At first, allocate a memory block of $6800 bytes. This will be your bootracks.

Then,  load the omti-device absolute to the offset $2004.  *DO NOT RELOCATE!*

The device must be for address $0.  The bootcode will relocate it.

The hexdump should look like this:

[...]

1FF0: 00000000 00000000 00000000 00000000    ................

2000: 00000000 70014E75 4AFC0000 00040000    ....p.NuJ�......

2010: 077A8004 03000000 006E0000 007A0000    .z.......n...z..

[...]

Load  the  bootcode to offset 0.  It may be relocated, because it contains no

absolute addresses.

The hexdump should look like this:

0000: 424F4F54 43E81FFC 22095881 D3FC0000    BOOTC�.�".X.��..

0010: 00082C79 00000004 20290002 D0812340    ..,y.... )..�.#@

0020: 00022029 0006D081 23400006 2029000E    .. )..�.#@.. )..

[...]

Important is the 'BOOT' in the first four bytes.

Then safe this memory block to a file. It must be exactly 26624 Bytes long.

Then  use the supplied 'OmtiWrite' to put the boottrack on the disk.  Do this

*AFTER* lowlevel format.  Format will supply the Parameter block.

If  you set the disk mount entries (see above) correct, you should be able to

boot from harddisk and get your partitions mounted correctly.

5.0. The device

---------------

Look at the code. I can't say much more. It will not work with disks without

PARA block. So format new disks immediately.

The  device  supports  two  drives,  in fact I'm using it with two drives for

about three years under both, AmigaOS 1.3 and 2.0.

6.0. Support programs

---------------------

6.1. OmtiFormat

---------------

This is the lowlevel formatter for the harddisks. It's quite self explanatory.

You  can read Err-Lists from DEVS:omti/ and store them there.  So it's a good

idea to have a disk handy, from which you boot before formatting.

An Error-List looks like this:

ERRLIST   <- Yes, it's really an ERRLIST

37        <- Number of bad tracks

4,75      <- head, track

4,76

[..]

5,19

5,171

5,172

[..]

5,418

5,441

You  can  make this list with a text editor and store them into DEVS:omti/ as

'Errorlist.0'  and 'Errorlist.1' for unit 0 and 1.  You'll find the errorlist

for your drive on a label glued to the drive.  (At least with Seagate drives)

You  must click on the necessary gadgets to activate certain functions.  E.g.

at  least  unit and steprate must be clicked to be able to use 'Get Parameter

from Harddisk'

The  user  interface  is  crude  and  not  written  to the C= style guide.  I

apologize for it.  But then again, it's about three and a half years old.

You  can't  click  Nof WrCyl.  These are calculated by subtracting the tracks

needed for error correction tracks.

The formatter can also read and write the PARA blocks on the harddisk. So you

needn't reformat to change e.g. the Autopark Flag.

Parameter blocks are stored as 'ParaBlock.0' and 'ParaBlock.1' in DEVS:Omti/

*WARNING*  If you low-level format your drive, you SHOULD DEFINITELY be aware

what  you're  doing.  Drives which you low-level formatted are *EMPTY* and no

force  in  the  world  (except a backup) can give you your data back.*

You've been warned!

All other tools are less harmful to your data :)

6.2. OmtiInstall

----------------

This rather simple tool installs the custom bootblocks to boot from unit 0 or

unit 1 on a floppy disk.

It  tries  to read DEVS:Omti/OmtiBoot.0 or DEVS:Omti/OmtiBoot.1, which should

contain the bootblocks together with checksum and all.

You  must create these bootblock files from the executeables with the help of

a monitor, which can also correct the bootblock checksum.  Did I mention FMon

yet?  Yes?  ;)

6.3. OmtiInfo

-------------

This tool reads the PARA block of a harddisk and displays some statistics.

This is how my drives show up:

Parameter fuer Drive 0:

----------------------

Device-Parameter:

Hoechster Zylinder:                   614

Hoechster schreib-/lesbarer Zylinder: 614

Write-Precompensation Zylinder:       256

Park-Zylinder:                        670

Koepfe:                               4

Steprate im uSec:                     10

Init-Parameter:

AutoPark aktiv:                       NEIN

Write-Protect:                        NEIN

Boot-Parameter:

Boot-LUN:                             0

Startkopf fuer Bootspur:              1

Startsektor fuer Bootspur:            0

Startspur fuer Bootspur:              0

Anzahl der geladenen Bloecke:         78

Format-Parameter:

Verwendeter Interleave:               2

Sektoren pro Spur:                    26

Sonstiges:

Bruttokapazitaet des Drive: 31.23 MByte = 31980 KByte = 32747520 Byte

Nettokapazitaet des Drive:  31.18 MByte = 31928 KByte = 32694272 Byte

Parameter fuer Drive 1:

----------------------

Device-Parameter:

Hoechster Zylinder:                   819

Hoechster schreib-/lesbarer Zylinder: 812

Park-Zylinder:                        0

Koepfe:                               6

Steprate im uSec:                     10

Init-Parameter:

AutoPark aktiv:                       NEIN

Write-Protect:                        NEIN

Boot-Parameter:

Boot-LUN:                             1

Startkopf fuer Bootspur:              1

Startsektor fuer Bootspur:            0

Startspur fuer Bootspur:              0

Anzahl der geladenen Bloecke:         26

Format-Parameter:

Verwendeter Interleave:               2

Sektoren pro Spur:                    26

Sonstiges:

Bruttokapazitaet des Drive: 62.46 MByte = 63960 KByte = 65495040 Byte

Nettokapazitaet des Drive:  61.85 MByte = 63336 KByte = 64856064 Byte

(Yeah,  they're  old, they're slow, they're hot and they were EXPENSIVE.  But

they're still running fine.  Why should I change?  ;)

6.4. OmtiRead and OmtiWrite

---------------------------

These  tools  are used to read and write 2.04 compatible boot tracks from the

harddisk  and  to  the  harddisk.   They  both  work  on track 1 and 2 of the

harddisk (1.3 boottracks are on 1-3).

OmtiWrite may damage your boottrack, but can do no harm to your data.

6.5. OmtiDisplay_Timer and OmtiDisplay_Vector

---------------------------------------------

These tools have the same purpose.  Their only difference:  OmtiDisplay_Timer

works and OmtiDisplay_Vector crashes.  :)

They  both  simulate the LEDs of the drives in a small window up in the right

corner of the screen.  This is useful if you don't have the drive led on your

front panel of if your computer is faaaaar away from the monitor.

If I think about it, there may be only a 'geta4()' missing in .._Vector', but

I'm much too lazy to look at the code again.  As I told you, this whole thing

is  a  hack  from  the  beginning  and I do not really want to support it any

longer.  But you and you and you asked me about a release.  So there.

.._Timer  uses  the timer-device to poll all 1/10 of a sec.  the state of the

drive motors and draw the led symbol.  .._Vector should use the callback hook

in the device.

If you get .._Vector to work, tell me. I'll be happy, too.

7.0. And that's a wrap!

-----------------------

At  least  I think.  If you still have followed me up to here (and understood

everything :), I wish you good luck with installing this software.  You'll be

rewarded  with  blinding  performance (This thingie is *MUCH* faster than the

original c't device and about 10 % faster that the ALF device).

If you got it to work on your system, tell me. I'll be happy to hear.

You  still  have  questions?   Well,  ask me.  And may the source be with you

(that's why I included it.  :)

Erlangen, in the Fall of 1992

Henning