Preface

=======

PortDiag  and  its  documentation is Copyright (c) Short Software, Prague,

Czech Republic.

This  archive  can be freely redistributed as long as all of its files are

included   in  their  original  form  without  any  additions,  deletions  or

modifications  (excluding addition of other README-style files and icons) and

no  more  than  a nominal fee is charged for its distribution.  All copyright

notices  in  the  programs  and  accompanying documentation files must remain

intact.   It's especially forbidden to add various '.displayme' files and BBS

advertisements.  This style of distribution is generally known as FREEWARE.

Special permission is given to Fred Fish to distribute this program on his

"Fish Disks".

This  software  is  provided  "AS IS" without warranty of any kind, either

expressed or implied.  The author is not responsible for any damage caused by

it.

Description

===========

PortDiag  is generally used for low-level hardware watching and modifying.

It  has  support  for  all the bits of the four CIA ports, digital and analog

parts of the game ports, serial port TXD and RXD signals and selective system

allocation of this hardware.

PortDiag executable is not PURE and thus _cannot_ be made resident.

Requirements

============

PortDiag executable requires Kickstart version 37 (2.04) or higher and the

ss.library  V5.0  or  higher  (from package SSLib*.lha).  The Special Support

Library is Copyright (c) Martin Mares, MJSoft System Software.

Usage

=====

PortDiag  can be run from the CLI or the Workbench.  The only one existing

argument can be specified as parameter from the command line or as a tooltype

from the Workbench.

Argument template:  "OMIT_ANY_OF_ALL_THOSE_ANNOYING_WARNING_MESSAGES

=FORCE/S"

You will know what this option does after you run this sw ;-).

Graphical user interface description

====================================

The  main  (and the only) window is divided to nine parts:  four CIA ports,

two game ports, "Start potgo" gadget, serial bits and allocation.

Four CIA ports

--------------

The  first  digit says the bit position in the CIA hardware register.  It's

followed  by  the  descriptive  signal name (suffixed by '/' for the negative

signals).   The  third column (physical port pin description) has a character

code  saying  which  connector  it is related to and the number of the pin on

that connector.  Character codes:

D  = Disk drive

GA = Game port 1

GB = Game port 2

P  = Parallel port

S  = Serial Port

The  first  of  the two control buttons controls DATA (up{0} - 0, down{1} -

1),  the second one controls bit direction (up{I} - input, down{O} - output).

Changing of the DATA signal in the input mode is not possible.

The  "Default"  button  in  the bottom sets the values of all the direction

bits of this port to their startup values.  It doesn't set their data values,

only the directions!

Note:  If you have some of the output signals connected to ground / +5, the

DATA bit displays the state of the external signal not depending on what does

the processor want to be there.  So to minimize the possible damage, PortDiag

checks the port after each write and if it reports still the same value, it's

set back to the external one.

Two game ports

--------------

The  first  two  counters ("X" and "Y") display the mouse position counters

(JOYxDAT  registers).   Right  to them are lights labeled as (R)ight, (D)own,

(L)eft,  and  (U)p  which have meaning only if a joystick is connected to the

game  port.   These buttons are decoded from the counters and they are only a

different representation of the counter values.

The  third  and  fourth  counter  ("A"  and  "B") displays the pot counters

(JOYxPOT  registers).   They  are  used  for the right ("B") and middle ("A")

mouse  buttons  and  for  the coordinates on analog joysticks.  You can start

these counters by the "Start potgo" button (see below).

The  six buttons in the bottom right corner are grouped horizontally to two

triples.   In  each triple, the first (leftmost) button controls the data bit

of  output ({0}/{1}), the second one displays the data bit of input ({0}/{1})

and the third (rightmost) is the direction ({I}/{O}) select.

Don't   forget   that   the   "gameport.device"   (at   least  under  usual

circumstances) periodically resets the first Game Port so you can't change it

manually.

"Start potgo" gadget

--------------------

This  button  dumps  capacitors and starts the "A" and "B" counters on both

game ports (see above).

Serial bits

-----------

This  sub-window displays the state of the input-only RXD signal and allows

setting  of  TXD  signal.  TXD is controlled by manipulating the serial break

(ADKCON->UARTBRK)  so  you  can't  see level transitions during normal serial

transmission.

Allocation

----------

These  four buttons ("Serial", "Parallel", "Disk" and "Game Ports") prevent

other  tasks  from  using  the  corresponding  ports.   It's always better to

allocate the part of the hardware which you are going to play with, otherwise

some other task using this port may be confused by you.

Supplied icons

==============

There are several alternative icons from Pavouk named "PortDiagX.info" in this

package.  You may cHoose one of them and replace the "PortDiag.info".

Final words

===========

Before  sending  any  questions,  please read the hardware manual (at least

twice).  If it doesn't solve your problem, read it once again.

BUGS: Terribly written, sorry.

Thanks to MJSoft for support and the ss.library.

Thanks  to  Pavouk for the idea, icons and images (flames about similiarity

of '1' - 'I' and '0' - 'O' send to pavouk@k332.feld.cvut.cz).

Send all comments and bug reports to short@k332.feld.cvut.cz.