A500 RAM/ROM overlay Modification

by Leslie Ayling

p.o box 334 Glebe 2037

Yet another in the series of El-Cheapo modifications for the A500/2000

Those of you who arrived at the amiga via the APPLE ][ route will

no doubt be aware of the APPLE language card (otherwise known as a 16k ram

card) and how it could be loaded with all manner of different operating

systems, such as INTEGER BASIC, PASCAL, ASSEMBLER DEV PAC etc.

Once loaded, there was either a HARD or SOFT toggle switch which disabled

the ACTIVE ROMS (namely APPLESOFT BASIC) then enabled the RAM CARD.

The system then behaved as though you had the appropriate roms for whatever

package you wanted to run.

A similar function can be enabled within the Amiga 500/2000 with an

A501 card to allow the KICKSTART roms to be overlayed with a ram image.

This will allow you to not only load whatever version of kickstart

you desire (1.1, 1.2, 1.3, 1.4????) (a'la Amiga 1000) but also to customize

it for auto-config of NON-auto-config memory boards or various other

devices. Or even further still modify some of the rom-resident libraries

for more efficient action (That infernal drive clicking for one!). Lets face

it, look what difference the ARP verions of most commands make!! Any takers

on this PROJECT????

HOW IT WORKS:

-------------

To overlay the Roms with RAM we first have to choose a 256k memory

area that we are happy to do without. I picked $C40000-$C7FFFF becase it

required the least circuitry (less power drain on that commodore power

supply!) which is after all what this series of articles is all about!

The easiest way to do any re-addressing on the Amiga 500 I have

found is to confuse GARY about what he is really looking at. This time we

have to make the ram image of $C00000-$C7FFFF also appear at

$F80000-$FFFFFF. Refer to the following table :-

A23	A22	A21	A20	A19	A18	A17	A16

$F8..    1       1       1       1       1       0       0       0

$C0..    1       1       0       0       0       0       0       0

When we wan't to read $F80000-$FF0000 Gary must think we are

looking at $C00000-$C7FFFF.From the table this means that when A23,22,21,20

and 19 are HIGH (or LOGIC "1") we must force the inputs for GARY for

A21,20,19 LOW (or LOGIC "0").

Refer to the accompanying circuit digram. The 74S30 chip decodes

the top 5 address lines and the enable switch and outputs a low when they

are all HIGH. the 74F08 gates this signal with the true address lines for

A21,20,19 then applies the signals to GARY.

When the Amiga is addressing anywhere but $f80000-$FFFFFF the

output of the 74S30 is high. This is passed through logic AND gates with

the true address lines.In this state the true addresses are applied to GARY

In the address range $F80000-$FFFFFF (when the switch is enabled)

the 74S30 outputs a LOW signal.This is passed to the AND gates.In this

state since one of the inputs of each AND gate is LOW the outputs are also

forced LOW.Hence A21,20,19 are forced low in this address range, causing

GARY to think it is looking at $C00000-$C7FFFF and enable the RAM it finds

there!

If you want to write protect the RAM you can cut the R/W* trace

that leads to the A501 , join either side of the cut to a single pole

single throw switch and wire a 4.7k resistor between 5v and the A501 side

of the switch.When the switch is open writes to the A501 RAM are inhibited.

Keep the leads to this switch SHORT.

HOW TO USE IT

-------------

First of all the A501 memory must become invisible , so the system

dosen't go writing over our ROM image.Boot with a disk which has the CCS

boot or similar boot block program for turning off extra memory. **NOTE**

It must actually REMOVE the memory from the system not just ALLOCATE it all

like some programs otherwise system stacks and execbase structures will

still be sitting in fast memory!!!!

Alternatively if you have the switch on the A501 as previously described in

Megadisk, flick the switch to off and reset the system. This will also make

the A501 invisible to the system. When the Hand appears on the screen just

flick the switch back to ON.

Secondly load a Monitor program such as CMON and load the desired

image into $C40000-$C7FFFF.

This image can be created in one of two ways.

1. Find a friend with an Amiga who has the desired kickstart version you

want and get him to dump it to disk for you ($FC0000-$FFFFFF).

2. Get a kickstart disk to suit the 1000. The rom image starts on track 0

sector 1 and continues sequentially up the disk to about track 22 or

thereabouts. Load it in track by track then dump the resultant image to a

disk file. This takes more time but if you don't know anyone with

appropriate roms then it is your last resort!

Once the image is loaded modify the bytes at $C401DF and $C401B7

to #$C4. This prevents the system allocating and over writing our ROM image

when we reset .Hold down the <CTRL>-amiga-amiga keys, flick the switch and

then release the keys. The machine should reset as normal , however at this

point it is running from the RAM. If the machine crashes RESET. If it still

dosen't work recheck your work as you have made a mistake  (99.9% chance the

mistake is in the software because if you have got this far the hardware more

than likely works.)

If you are using the same version in RAM of kickstart that you have in ROM

you will have to corrupt the EXECBASE vector as described below otherwise

the system will not re-evaluate the memory attached to the system.

WHAT HAPPENS DURING RESET?

--------------------------

The RESET* signale is applied to CIA-A which forces its PA0 output

high. This is applied to GARY as OVL (overlay). This signal causes the roms

to appear at $000000-$07FFFF. The 680000 then reads the long word at

$000004 and transfers control to the address it finds there ($FC0062 for

all versions so far) it then turns off the PA0 line so that the normal chip

ram can now be accessed.With the mod enabled, the jmp $FC0062 should

jump into our Ram image,if enabled.

MODIFYING KICKSTART

-------------------

So far I have only made changes to auto-configure my 2meg memory

board as described in Megadisk 15.

Change the following bytes:-

$FC01D9 : #$20

$FC01DF : #$40

$FC01F1 : #$20

$FC01FD : #$20

$FC01B7 : #$40

$FC01BF : #$24

This will auto-configure anything up the 2megs between $200000 and

$400000. Change the #$40 bytes to more if you have more than two megs.

To enable this to work, once you have loaded the image and made

the changes (and saved it for future loading) with your monitor put the

value $FFFF into memory location $000006. This will corrupt the execbase

vectors and make the machine instantaneously GURU. Hit the RESET keys

BEFORE the led flashes 11 times. This will cause our kickstart image to

re-evaluate what memory is present and auto-configure the extra memory!

*NOTE* If you add these changes do not include the previously mentioned

patch as the two are mutually exclusive.

SUMMING UP

----------

All of the software steps could be written into one program, but at

present I don't have a sufficient grasp of C or making use of devices etc.

Maybe one of you software boys could do it and send it in to TIM at

Megadisk!

The mod is very simple and allows compatibility betwwen different

versions of kickstart without the need to buy seperate versions in ROM.

For example you now have access to kickstart 1.3 and the benefits it gives

even if you only have 1.2 in your machine.

Availability has also proved to be a problem with the 1.3 roms as an

upgrade for 1.2 owners. When I finally got mine they were $41.86 from the

chatswood service centre. You can bet your bottom dollar that 1.4 when it

comes out will be a damn sight harder to get hold of!. With this mod you

are ready as soon as the Kickstart disk is made available or when you can

dump a copy from a friend with the latest.!

If you have any problems or questions contact me either direct or via megadisk.

If you can't find the CCS boot program, it is available DIRECT from me on a

disk with ARP1.3 and many other useful utilities including the latest

versiom of VIRUSX (4.0) for the measly sum of $5.00 plus $2.00 P/P.

Coming up...More usefull hardware mods!

Hope this is of some help.....................regards  Leslie Ayling