1541 Disk Reader (and related utilities)

Copyright  1994 by Dan Babcock. These programs may be freely

copied and used for non-commercial purposes, including distribution

via FTP (Aminet) and CDROM (Fish, Aminet).

General

The programs are CLI-based and use standard command-line

parsing (e.g. typing  "program ?" will display the argument template).

Kickstart 2.04 or above is required (developed under 3.1). You can

generally abort execution by pressing Control-C.

This document is rather brief and assumes you have a solid

understanding of both the Amiga and the 1541. If you need 1541 in

formation I strongly recommend _Inside Commodore DOS_ by Rich

ard Immers and Gerald Neufeld. That book was the sole reference

used to develop this software.

If you want to contact me for some reason (such as reporting

a bug), my email address is d6b@ecl.psu.edu.

I apologize for the terrible formatting of this text. The ASCII

export of this version of FinalWriterII (release 1...I need to upgrade)

really sucks!

1541

Command line: 1541 DriveNumber FileName [Retry=RetryNumber]

[TOP]

DriveNumber: drive number (0-3)

FileName: file name for disk image

[RetryNumber]: number of times to retry a track before giving up

(default 10)

[TOP]: Read from "other" side of disk. THIS DOES NOT WORK. You

must physically flip the disk. But since I don't exactly know _why_

this option doesn't work, I've left it in for experimentation purposes.

Example: 1541 2 ram:dump retry=20

The "1541" program reads a 1541 disk and produces a disk

image. A "disk image" is simply a block-by-block copy of the disk,

stores as a file. 1541 disk images are frequently given a ".d64" ex

tension, and referred to as "d64" files. (The term "disk image" is pre

ferred, however).

"1541", as may already be obvious, reads from an Amiga

5.25 inch drive; I use the Commodore A1020, and recommend it to

you as well. There is a catch, however. A 5.25 inch drive running at

the standard 300RPM speed can only read 1541 tracks 18-35, that

is, all but speed zone 1. (I'm not going to explain the 1541 speed

zones, so if you don't understand, don't worry about it). Luckily,

though, there is an easy fix: simply slow down the drive. At a some

what slower speed the Amiga is able to read all 4 1541 speed

zones.

If you don't know how to use a screwdriver, you may want to

have an expert technician perform the speed adjustment; otherwise,

keep reading. Disassemble the drive casing. Examine (carefully!) the

drive, looking for any turn-able adjustment (potentiometer). In the

case of my Alps drive, there's a rather large adjustment nicely slot

ted for a screwdriver on the circuit board surrounding the drive mo

tor.

After you've located the (presumed) speed control, get the

drive in a position in which you can adjust the speed while the drive

is connected and functional. Run the "speed" program (see below),

which continuously prints drive speed information (in RPM) to the

console. I've found that 280RPM works satisfactorily, so that's what I

recommend you use, at least as a start. If you have problems read

ing tracks 1-17 at 280RPM you can try reducing the speed slightly

(but I wouldn't recommend going lower than 270RPM). After you've

read in a few disks as a test, you can reassemble the case and for

get about it. You should still be able to read IBM and AppleII disks at

the reduced speed, but I wouldn't recommend writing to them.

Anyway, once you have adjusted the speed of your drive you

should be able to read 1541 disks to your heart's content. On my

A2000 (68040) it takes about 13 seconds to read a disk. (I could

actually reduce the time to under 10 seconds, but I didn't deem it

worth the effort). One more thing worth mentioning about "1541" is

how it deals with errors. When "1541" finds that it can't read a sec

tor, it places a special identifying mark on the sector

($00,$FF,"LAZARUS" to be precise). That way programs (like Get

Files, see below) that manipulate the disk image can identify sectors

that were unable to be read from the physical disk.

GetFiles

Command line: getfiles DiskImageName [DIR] [EXT]

DiskImageName: file name of disk image (wildcards accepted)

[DIR]: get a directory of the disk image (if not specified a directory

AND dump is performed)

[EXT]: add file type extensions (e.g. ".PRG") to the dumped files

Example: getfiles mydisk

GetFiles is used to extract the files (if any) from a disk image.

If you specify the DIR option a directory will be printed but the files

will not be dumped. Wildcards are accepted in the file name, so you

can easily directory and/or dump many disk images at once. Typi

cally you can use the universal wildcard (#?) because only files with

the exact size of a 1541 disk image will be considered.

GetFiles will rename files that contain illegal (for Amiga filena

mes) characters. For a "/", a "\" is substituted; for ":", ";" is substi

tuted; and for non-printable characters "-" is substituted. In addition,

files containing the special "LAZARUS" signature (see the "1541"

description above) will be reported as corrupt.

1541 files of type PRG, SEQ, USR, REL, and DEL in the nor

mal and locked forms are acceptable. Other types are ignored, but

an informative message is printed. Normally the files are stored "as

is" (except for illegal character substitution), but you can have the file

type added as an extension using the EXT option.

I should note that there is another freely-available program

that performs the same function as GetFiles: UNd64 by Jess Sos

noski. If you are dissatisfied with GetFiles for some reason, I sug

gest you try UNd64. The only major advantage to using GetFiles is

that it can detect the "LAZARUS" signature.

Speed

Command line: speed DriveNumber

DriveNumber: drive number (0-3)

The "speed" program measures the drive speed in RPM

(revolutions per minute). Normal drive speed is 300RPM. "Speed"

will continuously print the drive speed to the console (as usual, hit

Control-C to exit). You must have a disk inserted in the drive or

"speed" will not function.

In case you're curious, "speed" works by measuring the time

between index pulses. A simple index interrupt routine takes a

time-stamp of the "E clock" (CIA .7Mhz clock).