Lserial V21(3) for AMOS1.3 and above.

Lserial is (C) Niklas Sj�berg 1992

See IMPORTANT.TXT for more information on how to reach me or

register!

NOTE: Commands added since V1 is marked with *-, so just  load

you favorite textreader and search for "*-".

Commands added since V2 are marked ++

Lserial.doc is now  splitted  in  two  parts.  Part  I  covers

general    serial-commands  and  part  II  covers  all  necessary

information about the XPR-specifications.

DISTRIBUTION:

-------------

Lserial no longer is freely distributable, it is  now  shipped

under the term shareware.

Lserial is copyrighted by me, Niklas Sj�berg.

DISCLAIMER:

-----------

All the usual... You are using this program at your own  risk.

I can not be held responsible for any damage or whatsoever.

WHY?:

-----

AMOS's serial didn't work in 1.0/1.1, in  1.2/1.3  it  worked,

but  only sometimes  and only a little :-) The main 1.3-bug seems

to be when one tries to open the device after  closing  (this  is

NOT  a  devicebug since it is tested with serial.device from both

1.3.3 and 2.04. An old serial.device from WB1.2  causes  AMOS  to

hang  on  some occasions for some reason) it once and then trying

to open it again. You also get xpr and some other features.

HOW TO:

-------

In the days of V1.1/1.2 I used to  supply  a  program  for  my

extensions  which  could  change  any  (of  mine) extension to be

configured as any number. This is not possible  anymore,  due  to

the  layout  of  1.3  (well it IS possible but it would slow down

every access to the data-table). Because of this you  MUST  place

LSerial as extension number eleven (11). Your manual explains all

about  adding  extensions.  If  you  MUST  or  NEED  to  have  it

configured as  another number then send me a disk  full  of  nice

programs and some international replycoupuns to cover mailing.

PART I - General serialcommands

Before you start crashing your computer :

It is up to  you,  the  programmer,  to  make  sure  that  the

serial.device (or another device you might have selected) is open

and  ready  for  use before you try to access any read/write/xpr-

commands. If you for instance try to call Lser Send as the  first

instruction  in  your program it will hang OR crash the computer!

Always use Lser  Open! If the device  failed  to  open  you  will

receive  an  errormessage  and the program will be aborted, or if

you have a errorhandling-routine the error will be trapped  (that

is the best solution!). It is safe to call Lser Close even though

no  device  is open, so it might be a good idea to always do this

before your program exists or if you change device.

Lser Open - Open serial for use.

Lser Open BAUD,RWlen,STOP,BUF_SIZE,BRKtime,FLAGS,UNIT,"name.device"

where

BAUD     - Baudrate

RWlen    - number of bits when read/write (normally 8)

STOP     - Number of stop-bits (normally 1)

BUF_SIZE - Internal buffer for device. MUST be >512 bytes

BRKtime  - How long a BRK should last, in MICROseconds

(normally 250000)

FLAGS    - See Appendix B for list

UNIT     - Unit, normally 0 for external modems.

name     - Normally serial.device (located in devs:)

Lser Close - Close serial.device

Lser Close

++  Lser Params - Change the settings of the currently open device

++  Lser Params RWlen,STOP,BUF_SIZE,BRKtime,EXTFlags,FLAGS

++  where

++     All parameters mean the same thing like  in  Lser  Open.  Note

++  that  a  new  field  EXTFlags  is  available. This field may have

++  different values for different devices. It  is  used  to  control

++  mark  and  space parity instead of odd and even. Bit 24 "if mark-

++  space, use mark". Bit 25, use mark-space. These flags where added

++  mostly due to future compability and extensions.

++     According to C= Autodocs it is always best to decide if access

++  shall be shared or exclusive when opening the device. It is  also

++  wise  to  decide  if 7-wire or 3-wire protocol should be used. Do

++  not count on that these two options  can  be  changed  after  the

++  device has been opened. Use Lser Baud to change the baudrate.

++  NOTE! This command has been here all along,  I've just  missed it

++  in the documentation! Sorry..

++  Lser Baud - Change baudrate

++  Lser Baud NEWRATE

++  where

++     NEWRATE is the new baudrate.

Lser Send - Send data to the modem, non-multitasking

Lser Send A$

where

A$ contains any data. Note  that  this  instruction  does  not

return to AMOS until the whole string is sent.

Lser Query - Check if there is data to read

L=Lser Query

where

L - Contains the number of chars available to read.

Lser Read - Read all available characters.

A$=Lser Read

where

A$ will contain all data buffered by the  device.  WARNING  If

there  are  VERY  many  characters  to  read AMOS may crash. This

command should only be used if you often read  from  the  device,

like  in  a  comm-program which continually loops and read. Using

it when communicating with MIDI or via 0-modem can  be  dangerous

due to the very high speed!

Lser Mulsend - Send a string in a multitasking manner.

Lser Mulsend A$

where

A$ may contain any data. If you try  to  send  any  more  data

before Mulsend has completed only garbage will be output from the

modem.  Control  will  return  to  AMOS immediately. Changing the

contents of A$ before Mulsend has completed is a stupid thing  to

do..(I  have  not had time to test and see if serial.dev actually

copies the data)

Lser Mulcheck - Check if Mulsend has sent all data.

L=Lser Mulcheck

where

L will be true if Mulsend is ready.

Lser Get - Get a specified number of characters.

A$=Lser Get(N)

where

N Is the number of characters you  wish  to  read.  Note  that

control  will  not  return to AMOS until the number of characters

specified have been received.  This can cause  AMOS  to  hang  if

you  haven't any CARRIER and no data appears. A good advise is to

use Lser Query or Lcarrier before trying to read anything.

Lcarrier - Check for carrier

L=Lcarrier

where

L will be true if CARRIER exists.

++  Lser Status - Get statusflags from device

++  L=Lser Status

++  where

++  the 16 first bits of L have the following meaning:

++  (Active high means active if bit set, active low = active if

++   bit=0)

++   Bit#   Active Description

++   ----   ------ -----------

++     0           reserved

++     1           reserved

++     2    high   Ring indicator. Note that this flags also is

++                 "connected" to parallel's SEL. Do not relay to

++                 much on this bit!

++     3    low    Data Set Ready      (DSR)

++     4    low    Clear To Send       (CTS)

++     5    low    Carrier Detect      (CD)

++     6    low    Ready To Send       (RTS)

++     7    low    Data Terminal Ready (DTR)

++     8    high   hardware overrun

++     9    high   break sent (most recent output)

++    10    high   break received (as latest input)

++    11    high   transmit x-OFFed

++    12    high   receive x-OFFed

++ 13-15           reserved

++ 16-31           Always zero

The most interesting fields are DSR/DTR. You can use  Lcarrier

to check for carrier instead of using  this function (Lcarrier is

faster).

Lser Brk - Send a BRK to modem (time specified in Lser Open)

Lser Brk

*-  Linkey$ - Convert some AMOS-characters to ANSI-codes.

*-  A$=Linkey$

*-  where

*-     A$   will  contain one or more characters. Linkey$ works  just

*-  like  Inkey$  in  AMOS  except that it might return more than one

*-  character. It always reads one character from the keyboard but if

*-  for instance a cursor-key was pressed the ANSI-code will  require

*-  three  characters.  Currently  Linkey$  converts  all cursor-keys

*-  and all keys pressed while the CONTROL-key is held  down.  Escape

*-  backspace  a-z  etc. are not converted in any way. If no keys are

*-  pressed A$ will be empty. Always use this instead  of  Inkey$  if

*-  you are to be able to communicate with another (ANSI/VT100) term-

*-  inal.

PART II - The XPR specs

*-     XPR stands for eXternal PRotocol and  was  invented  by  W.G.J

*-  Langevald    in    1989.  The  idea  is  that  the  author  of  a

*-  communicationprogram  (or  of  any      program    involved    in

*-  filetransfers)    can    use    standard    Amiga-libraries   for

*-  filetransfers. The xpr*-libraries are supposed to  be  the  brain

*-  of  the protcol, checking the incoming data, if necessary request

*-  a re-send, save data to a file etc. etc. The (comm)program is the

*-  one which supplies all the basic routines for  serial-read/write,

*-  saving  files  etc.  These  routines  are  later  called  by  the

*-  xpr*.library. Since the XPR  is  quite  flexible,  allowing  many

*-  different  operations to be performed, you may count on that if a

*-  program supports ONE xpr.library it  will  probably  support  all

*-  future libraries. This means that the user may use protcols which

*-  even  wasn't  invented  when  the author of the program wrote his

*-  callbacks! It also means that the author of the program only need

*-  to write his callback routines once (which he already  needed  in

*-  his  originally program in the first place) in order to work with

*-  any protocols. Lately some XEM*.library (eXternal EMulation) have

*-  appeared. These can/could  actually  be  used  through  the  XPR-

*-  standard  using  XprHostMon  and XprUserMon. Due to the layout of

*-  AMOS (not using normal  intuition-screen)  these  cannot  be  run

*-  under AMOS in any way.

*-     When all the callbacks are written (which I have done for you)

*-  it  is  quite  easy to use any xpr.library. In general the 'flow'

*-  when using a xpr looks something like this  : (unnecessary  setup

*-  removed sine Lxpr handles this for you)

*-  1) Open the desired library

*-     (Did it open?)

*-  2) Call SetUp with a proper init-string.

*-     (See docs for the library in question)

*-  3) Check flags returned by SetUp

*-     (See below)

*-  4) Wait until user requests up-/download.

*-     (If  flags supported UserHostMon, download may be initiated

*-     automagically)

*-  5) Get appropriate filenames for up/download (if necessary, spe-

*-     cified in flags)

*-  6) Call Send or Receive.

*-  7) When the program is quit or xpr is changed, close it and/or

*-     jump to 1).

*-     APPENDIX C contains a complete example of how to use a xpr-

*-  library.

*-     Before we start dealing with the  Lxpr-syntax,  here  are  the

*-  callbacks   that  are  supported  by  Lxpr  (usually  the  needed

*-  callbacks for a library are stated in the library docs.)

*-  xpr_filename  - Support for both batch and singlefile transfer.

*-                 (actually this is a datafield, but always filled

*-                 in by Lxpr)

*-  xpr_fopen     - Opens file(s)

*-  xpr_fclose    - Close file(s)

*-  xpr_fread     - Read from file(s)

*-  xpr_fwrite    - Write to file(s)

*-  xpr_sread     - Read from serial.device (or spec. in Lser Open)

*-  xpr_swrite    - Write from serial.device(or spec. in Lser Open)

*-  xpr_sflush    - Clear all data in serial.device-buffer

*-  xpr_update    - Print info (see below)

*-  xpr_chkabort  - See if user aborted transfer

*-  xpr_chkmisc   - No need for this one, yet, not one xpr require is

*-                  so far.

*-  xpr_gets      - Get a string from the user

*-  xpr_setserial - Get and/or change serialparameters

*-  xpr_ffirst    - Get the first filename during batch transfer

*-  xpr_fnext     - Get the next filename during a batch transfer

*-  xpr_finfo     - Obtain fileinformation (filetype not supported)

*-  xpr_fseek     - Position in a file

*-  xpr_extension - All four (the four below).

*-  xpr_options   - Supported but currently not used (just exits)

*-  xpr_unlink    - Delete file(s)

*-  xpr_squery    - Return # of chars available from serial

*-  xpr_getptr    - Return customscreen-pointer (WB always returned)

*-  The xpr_update-function supports the following fields :

*-  xpru_protocol    - The name of the xpr currently used

*-  xpru_filename    - The file being transferd

*-  xpru_filesize    - Size of the file being transferd

*-  xpru_msg         - Last message from library

*-  xpru_errormsg    - Last errormessage

*-  xpru_blocks      - Number of blocks sent

*-  xpru_blocksize   - The current size of each block

*-  xpru_bytes       - Number of bytes sent

*-  xpru_errors      - Number of errors during transfer

*-  xpru_timeouts    - Number of timeouts during transfer

*-  xpru_blockcheck  - Type of error-check (usually CRC or checksum)

*-  xpru_expecttime  - Expected (total) transfertime

*-  xpru_elapsedtime - Time elapsed since start of transfer

*-  xpru_datarate    - cps for total transfer so far

*-  more fields may be supported in the future.

*-     The Lxpr-part only contains one command! This is  due  to  the

*-  inner workings of the AMOS compiler which treats all functions as

*-  local,  as  it  datas.  Lxpr  COULD have been split up on several

*-  commands, but would have meant that either all the  xpr-callbacks

*-  and  data  had  been stored in the global function (ie. extension

*-  init which always is linked if the extension is used) or a lot of

*-  copying and unecessary space waste  between  the  functions.  For

*-  those  of you who never use the xpr-functions less memory will be

*-  used.  The  only  thing  added  which  occupies  memory  are  the

*-  timer.device structure and its replyport.

*-  The syntax for Lxpr are as follows :

*-  A$=Lxpr("filename","setup-string","libraryname",FUNCTION)

*-  where

*-     FUNCTION is either, 0 for Send file(s), 1 for Receive file(s),

*-  2  for  Openlibrary,  3  for  Closelibrary,  4  for  Setup  (send

*-  parameters),  5  for  read  (works like Lser Read, but XprHostMon

*-  will be called if required, se below), 6 for  Write  (works  just

*-  Lser Send, but calls XprUserMon if required).

*-     I strongly suggest that you set up some global  variables  and

*-  assign appropriate values to them and then use them in all further

*-  xpr-calls :

*-  XPRSEND =0

*-  XPRREC  =1

*-  XPROPEN =2

*-  XPRCLOSE=3

*-  XPRSETUP=4

*-  XPRREAD =5

*-  XPRWRITE=6

++  XPRCUSTOMIZE

*-     XPRREAD and XPRWRITE are always checked for first,guaranteeing

*-  you as fast as possible reads and writes to/from the  device.  Of

*-  course you may overrule the library's request that XprHostMon and

*-  XprUserMon  should  be called, by calling Lser Read and Lser Send

*-  instead. These two routines are somewhat faster than the  XPRREAD

*-  and  WRITE and may safely be used if "flags" from SetUp indicates

*-  that the protocol doesn't require  XprHostMon  and/or  XprUserMon

*-  to be called.

Please note that it is up to your program to evaluate filepatterns requested

by the user. XPRSEND and XPRREC only accepts full filenames separated by

a space. If you use Ldos you can use the Lcat-commands in combination with

Lmatch (Release 2 only) in order to find matching filenames.

*-  XPRSEND - Send one or more files.

*-  A$=Lxpr(FILENAME$,"","",XPRSEND)

*-  where

*-     A$  currently  is  undefined  (empty  string).  FILENAME$  may

*-  contain  one or more filenames which are to be sent. If more than

*-  one filename is specified they must be separated  with  a  single

*-  space. Note that all libraries doesn't support batchtransfers.

*- WARNING! FILENAME$ MUST BE NULLTERMINATED! (FILENAME$=FILENAME$+Chr$(0))

*-  XPRREC - Receive one or more files.

*-  A$=Lxpr(FILENAME$,"","",XPRREC)

*-  where

*-     A$ currently is undefined (empty  string).  FILENAME$  can  be

*-  left  out  if  "flags"  specified  that  the protocol can receive

*-  filenames from sender (Ymodem-batch, Zmodem etc.)  FILENAME$  may

*-  contain  more  than  one  filename  but  protocols  which handles

*-  batchtransfers usually  can  receive  filenames  and  if  "flags"

*-  specify this you may set FILENAME$="".

*- WARNING! FILENAME$ MUST BE NULLTERMINATED! (FILENAME$=FILENAME$+Chr$(0))

*-  XPROPEN - Open a xpr-library for use.

*-  A$=Lxpr("","",LIBRARY$,XPROPEN)

*-  where

*-     A$ will be empty if the open failed, otherwise A$ will contain

*-  the string "OK" (in capitals). LIBRARY$ is the complete  name  of

*-  the library, for example xprzmodem.library.

*-  XPRCLOSE - Close a xpr-library.

*-  A$=Lxpr("","","",XPRCLOSE)

*-  where

*-     A$ currently is undefined. Always close your  current  library

*-  before opening a new library or exiting the program.

*-  XPRSETUP - Initialize the xpr and customize its performance.

*-  A$=Lxpr("",SETUP$,"",XPRSETUP)

*-  where

*-       SETUP$  is  a  valid  option-string,  as  specified  in  the

*-  protocol's documentation. A$ will contain a bit-pattern converted

*-  to  ASCII.  Please  note  that  the order of the bits are left to

*-  right  and  not  right  to  left  as  usual.  In    other   terms

*-  Mid$(A$,1,1)  means  bit  0 (NOT 1) and Mid$(A$,1,3) means bit 2.

*-  The bits have the following meanings :

*-  Bit 0 : If set, all was successful.

*-  Bit 1 :  Protocol  requires  no  filerequester / filenames   when

*-  receiving files. Note that the xpr may supply a filerequester

*-  itself, which probably will appear on the workbench-screen  since

*-  Lxpr  tells  the  xpr  to  use  this screen when/if xpr_getptr is

*-  called.

*-  Bit 2 : Protocol requires no filerequester/filnames when sending

*-  files.

*-  Bit 3 : The xpr would  like you  to use   XProtocolHostMon()  for

*-  all serial input. This means that you SHOULD call XPRREAD instead

*-  of  using  the  normal  Lser-functions. If you don't the protocol

*-  might not operate correctly and functions such as  autoactivation

*-  on download might be disabled. Note that it is perfectly legal to

*-  call  XPRREAD  even  if  this  bit  isn't  set.  Lxpr  only calls

*-  HostMon() if suitable.

*-  Bit 4 : The  xpr  would  like you to use  XProtocolUserMon()  for

*-  all  user  input (ie. it want  to check what you are sending). As

*-  above, you should call XPRWRITE if this bit is set. Note that  it

*-  is  perfectly  legal to call XPRWRITE even if this bit isn't set.

*-  Lxpr only calls UserMon() if suitable.

*-  Bit 5 : The  xpr  would  like you to use HostMon() even  when  no

*-  input  from  is  received. This feature is NOT supported by Lxpr!

*-  This can cause your program to be locked for an unknown time  and

*-  may drain all CPU-power you have got.

*-    As  an  example  xprzmodem  returns  :  "110100"  (if  AY  was

*-  requested).  The  first character ("1") says "All OK", the second

*-  character ("1") indicates that the xpr doesn't need filename when

*-  receiving files. The next "0"  indicates that the  protocol  need

*-  filenames  (ie. you supply requester) when  sending. The last "1"

*-  shows that the library want us to use XPRREAD since  it  want  to

*-  check  incoming  data  (auto download). The second last zero says

*-  that the library don't care what we send to the other  side  (you

*-  may  still  use XPRWRITE safely) and the last zero indicates that

*-  the protocol not will require a  call to HostMon() even  when  no

*-  data is present (which Lxpr doesn't support)

*- WARNING! SETUP$ MUST BE NULLTERMINATED! (SETUP$=SETUP$+Chr$(0))

*-  XPRREAD - Read any data that is currently available.

*-  A$=Lxpr("","","",XPRREAD)

*-  where

*-     A$ will contain the data from the serial.device. Note that  if

*-    the  xpr has requested HostMon() to be called the data can have

*-  been modified  by the  xpr.  For  instance,  as  soon  as  zmodem

*-  recognize  its  startsequence    for  download  it will start the

*-  download and return an empty string and A$ will thus be empty.

*-  XPRWRITE - Write a string to the serial.

*-  A$=Lxpr(STRING$,"","",XWRITE)

*-  where

*-     A$ always will be empty on return. Please note that if the xpr

*-  has requested XProtocolUserMon() to be called  you  can  never be

*-  sure that exactly the data you wanted to send really was sent. To

*-  my knowledge there are currently no xpr available which writes to

*-  your  data.  A  future  MNP-xpr however typically would need this

*-  function. STRING$ is a normal textstring.

++  XPRCUSTOMIZE - Change output from the xpr.

++  A$=Lxpr("x","text %code","",XCUSTOMIZE)

++  where

++     A$ always will be empty on return. "x"  is  a  special  "code"

++  which  tells  Lserial which text you wish to change. "text %code"

++  is the new text to be inserted. If you are to print a string, fx.

++  filename, you put  a  %s  where  you  want  the  filename  to  be

++  inserted.  If  you  are to print a number, like filesize, use %ld

++  instead. As said before, "x" is a character, ranging from "a"  to

++  "m"  for  the  various  strings. Default are: (just like in older

++  versions)

++  Code String

++  ---- ------

++  a    Protocol : %s

++  b    File name : %s

++  c    Size : %ld

++  d    Last message : %s

++  e    Last error : %s

++  f    # of Errors : %ld

++  g    # of Timeouts: %ld

++  h    # of Blocks : %ld, current size : %ld bytes

++  i    Transf. Bytes: %ld

++  j    Estimated : %s

++  k    Elapsed : %s

++  l    Error check : %s

++  m    CPS : %ld

++     Note that it is wise to include both position of text, plus  a

++  Chr$(26)  (Cline)  which  erases the current line before the text

++  is printed. If you want to remove any message (even  if  the  xpr

++  supports  it)  you  can  call XPRCUSTOMIZE with only a space " ".

++  Also note that Lserial no longer prints any  messages  until  the

++  xpr asks it to. That may cause for instance "Last error" to never

++  be printed, if no errors occur. Because of this you might want to

++  Print  (using  normal  Print)  these  fields to the screen before

++  calling Lxpr. An example:

++  A$=At(0,15)+"Last error : "

++  Print A$+"none so far."

++  then modify the xpr-string:

++  DUMMY$=Lxpr("e",Chr$(26)+A$+"%s","",XCUSTOMIZE)

++  To shut Last error up forever:

++  DUMMY$=Lxpr("e"," ","",XCUSTOMIZE)

++  To print more than one result in one line:

++  (NOTE! that messages are printed in the same order  as  they  are

++  listed above! You thus have to put the Chr$(26) to clear the line

++  in the string which is printed first!)

++  A1$=Chr$(26)+At(0,10)+"Estimated : %s"

++  A2$=At(30,10)+"(elapsed %s)"

++  DUMMY$=Lxpr("j",A1$,"",XCUSTOMIZE)

++  DUMMY$=Lxpr("k",A2$,"",XCUSTOMIZE)

++  while would produce something like:

++  "Estimated : 00:05:31 (elapsed 00:02:11)"

++     Please don't use to long strings, since some xprs return quite

++  long messages  sometimes.  Try  using  strings  shorter  than  30

++  characters if possible.

BUGS:

-----

None found during my tests  against  a  few BBS's.  One  thing

could  happen  though and is nothing special to LSerial : Garbage

collection (which never happens). When using a  lot  of  strings,

AMOS once in a while need to do a garbage collection, sorting all

strings  together.  It  seems  like AMOS has very hard to see for

itself when it is time for this and strange gurus may follow.  If

this  happens  call  Free  (DUMMY=Free) once in  a while. If this

doesn't help call Free VERY often or increase the buffer  a  bit.

If  something  strange  still happens you might have found a bug,

please contact me.

*-     Also you must be warned about the Lxpr-functions which doesn't

*-  bother with such trivial things like  checking  if  you  actually

*-  have  a library open! Lxpr return enough "codes" for your program

*-  to make sure that everything is OK before it tries  to  call  any

*-  other   function.  This  is  the  way  C,  Pascal  and  assembly-

*-  programmers have to work...

!PLEASE READ IMPORTANT.TXT!

APPENDIX A:

-----------

Errorcodes:

0 Serial already open

1 Invalid devicename

2 Unable to open device

3 Overflow in stringbuffer

4 Invalid read-size

++  5 Command need NULL-terminated string

++  6 You can't call with an empty argument

++  7 Invalid XPR-customstring (a-m)

++  8 Non valid XPR-command

APPENDIX B:

-----------

The FLAGS are specified when opening the device. Each functions

is assigned  to a value (or bit) and you may  add  the  different

values  together  to  select  one  or  more  options. For further

information see the docs for the serial shipped with AMOS.  NOTE:

all values are in HEX! (LISTING FROM GenAm3, edited somewhat)

Val   Name            Meaning

---   ----            -------

$1  = SERF_PARTY_ON  - Parity on (even if not $2)

$2  = SERF_PARTY_ODD - Odd Parity

$4  = SERF_7WIRE     - 7Wire

$8  = SERF_QUEUEDBRK - Break in background

$10 = SERF_RAD_BOOGIE- Highspeed (disables div. checks)

$20 = SERF_SHARED    - Other programs may open device

$40 = SERF_EOFMODE   - EOF recognition enabled

$80 = SERF_XDISABLED - xON/xOFF disabled

Typically you can set FLAGS to zero.

APPENDIX C:

-----------

++  This example is a bit old now. See the one included with Ldos

++  instead!

*-     This is sample-program which shows how to use  the  xpr's  and

*-  the  serial-commands  in  general. Actually it can be used as the

*-  worlds shortest communicationprogram! It supports the most common

*-  ANSI-sequences via Lansi (located in  Ldos) which converts  ANSI-

*-  sequences to AMOS controlcodes. The only thing we need to convert

*-  before  we  send it is the cursorkey-codes which are 3 bytes long

*-  in "ANSI-format"

(There are no *- in this code, so you may easily load it into AMOS)

Set Buffer 20      : Rem 20 Kb string-space

Global XSEND,XREC,XOPEN,XCLOSE,XSETUP,XREAD,XWRITE

XSEND=0            : Rem These are the globals which we use

XREC=1             : Rem when calling Lxpr.

XOPEN=2            : Rem This way you don't have to remember which

XCLOSE=3           : Rem command that belonged to which value.

XSETUP=4

XREAD=5

XWRITE=6

E$=Chr$($1B)+Chr$($5B) : Rem ANSI-startsequence.

CU$=E$+"A"             : Rem The codes for all

CD$=E$+"B"             : Rem cursormovements.

CR$=E$+"C"             : Rem Up,down,left and right

CL$=E$+"D"

Screen Open 1,640,256,8,Hires : Rem 8 colour ANSI is enough.

: Rem Lansi converts 16->8 automatically

Flash Off

Palette ,,,$FF0

Paper 0

Clw

Rem Open serial.device in "standard-mode" with a 8 Kb buffer

Lser Open 2400,8,1,8096,250000,$20,0,"serial.device"

Rem Just shows how to use Mul Send. Try this one in 300 bps!

Lser Mul Send "ATZ"+Chr$(13)

While Not Lser Mul Check : Rem wait until all data is sent

Print "Waiting for mulsend to complete.."

Wend

Centre "Press <!> to quit demo" : Print

Centre "Press <?> to SEND    files with Zmodem" : Print

Centre "Press <+> to RECEIVE files with Zmodem" : Print

Rem Try to open xprzmodem.library

STA$=Lxpr("","","xprzmodem.library",XOPEN) : Rem open

If STA$<>"OK" : Rem if STA$="OK" the call was successful.

Print "Failed to open xprzmodem.library!"

Lser Close

End

End If

Rem Initialize the library.

Rem Z1=Chop files, TN=Text mode off, KY=Keep partial files

Rem B4=Save buffer every 4Kb, OR=Owerwrite Resume, SN,RN=Don't send/

Rem receive full paths, AY=Autoactivate (call HostMon()), E10=allow

Rem 10 errors before abort.

STA$=Lxpr("","Z1,TN,KY,B4,OR,SN,RN,AY,E10"+Chr$(0),"",XSETUP)

STATUS=Val(Mid$(STA$,1,1)) : Rem All OK? 0 if failed.

FREQDL=Val(Mid$(STA$,2,1)) : Rem Need Freq for DL? (1 if not)

FREQUL=Val(Mid$(STA$,3,1)) : Rem Need Freq for UL? (1 if not)

Rem Don't care to check for User- and HostMon since we always call XREAD

Rem and XWRITE and let Lxpr handle this..

If STATUS=0

Print "Failed to setup parameters..."

A$=Lxpr("","","",XCLOSE)

Lser Close

End

End If

Do : Rem MainLoop

Multi Wait

A$=Inkey$

If A$="!" Then Exit 1

If A$="#" Then Clw : A$="" : Rem If the screen becomes garbled

If A$="?"

Clw

If FREQUL=0 : Rem Do we need a filerequester?

Proc _GET_FILE : Rem If so get file(s)

A$=Param$

Print "Requested file(s): ";A$

Else

A$=""

End If

STA$=Lxpr(A$,"","",XSEND) : Rem start download

A$=""

Home : Cline : Print "Press a key!" : Wait Key : Clw

End If

If(A$="+")

Clw

If FREQDL=0 : Rem Do we need to supply filename(s)

Proc _GET_FILE

A$=Param$

Print "Requested file(s): ";A$

Else

A$=""

End If

STA$=Lxpr(A$,"","",XREC)

Home

End If

If A$<>"" : Rem Check for

If A$=Chr$(30) : Rem CURSOR UP

A$=CU$

End If

If A$=Chr$(31) : Rem CURSOR DOWN

A$=CD$

End If

If A$=Chr$(28) : Rem CURSOR RIGHT

A$=CR$

End If

If A$=Chr$(29) : Rem CURSOR LEFT

A$=CL$      : Rem and replace with ANSI-sequence

End If

D$=Lxpr(A$,"","",XWRITE) : Rem Send to modem

End If

D=Free : Rem AMOS has always had problems with

Rem garbage collections. This prevents crashes

Rem when stringbuffer are use a lot.

A$=Lxpr("","","",XREAD) : Rem is there anything to read?

Print Lansi(A$); : Rem Convert ANSI->AMOS and print.

Loop

T$=Lxpr("","","",3) : Rem close library

Lser Close          : Rem and device.

Rem END OF PROGRAM

Procedure _GET_FILE

U:

Amos To Back

A$=Lfreq("Choose a file, press cancel when all are selected",$2+$4+$10+$40+$1000)

If A$="" Then Goto UT

If FILE$=""

FILE$=A$

Else

FILE$=FILE$+" "+A$

End If

Goto U

UT:

Amos To Front

End Proc[FILE$+Chr$(0)]