WHATPORT.COM

Version 3.7

June 8, 1990

For detecting installed serial ports,

type of mouse, and potential IRQ conflicts

Copyright 1989, 1990 Life Sciences Editorial Services

1236 River Bay Road

Annapolis, Maryland 21401

AUTHOR: Pete Petrakis (CIS 76555,1175)

IMPROVEMENTS IN THIS VERSION:

This version corrects a flaw in version 3.6 that prevented

detection of COM4 on a PS/2 system.  I regret the error, but writing

a bug-free program was complicated by my having no access to a PS/2

computer.  I thank Jim Luhman for testing this version on a PS/2

with four COM ports installed (not an easy setup to find, I tell

you) and verifying that it can detect them all.

DESCRIPTION:

This program detects what serial ports (COM1-COM4) are installed

in a standard (PC, XT, AT) or PS/2 system.  The type of mouse, if

present, is also shown (serial, bus, InPort, PS/2, HP) together with

the interrupt request (IRQ) line it's using (unless it's a PS/2).

The program warns of potential IRQ line conflicts between the

mouse and serial ports.  In the case of PS/2 computers, where

there should be no possibility of conflict between a PS/2 mouse

and a serial port, version 3.5 only reports the presence of the

PS/2 mouse and the serial ports that it finds installed (up to

COM4).

PRINCIPLE:

Because standard PCs have only two IRQ lines for serial ports,

COM3 must share COM1's IRQ line and COM4 must share COM2's

IRQ line.  Unfortunately, two devices using the same IRQ

line can't be active at the same time.  COM3 can't be used for

a printer or modem if a serial mouse is active on COM1, for

example.  Likewise, COM4 can't be used if a serial mouse is

active on COM2.  In other words, if you have four COM

ports and a mouse is attached to one of them, you do not

have three COM ports available for other devices, you only

have two.  The COM port that shares the IRQ line being

used by the mouse is not usable.  WHATPORT detects and

reports such conflicts.

The following table shows what ports are available for other

devices in a PC, XT, or AT when a serial port mouse is present

on a multiple-port system:

Serial port mouse on     Ports available for other devices

_____________________    _________________________________

COM1 or COM3 (IRQ4)          COM2 or COM4 (IRQ3)

COM2 or COM4 (IRQ3)          COM1 or COM3 (IRQ4)

Conflicts with serial ports are also possible in the case of a

bus mouse.  For example, if a bus mouse is configured to use

IRQ3, there will be a conflict with both COM2 and COM4.  If it

is configured to use IRQ4, there will be a conflict with both

COM1 and COM3.  WHATPORT also detects and reports these conditions

so a different IRQ can be selected for the bus mouse.

The PS/2 has a different arrangement.  COM1 in a PS/2 uses IRQ4,

and COM2, COM3, and COM4 share IRQ3.  There should not be any IRQ

conflicts with a PS/2 mouse in a PS/2 system since the PS/2 mouse

has its own port and does not use either IRQ3 or IRQ4.  However, it

seems theoretically possible that someone could attach an ordinary

(non-PS/2) serial or bus mouse to a PS/2 machine, in which case

there could be IRQ conflicts.  With that kind of setup, a bus mouse

using IRQ3 could make three serial ports (COM2, COM3, and COM4)

unusable on a PS/2.

HOW TO USE THE PROGRAM:

Just run it and read the results.  Adjustment for color or

monochrome display is automatic, but monochrome can be forced

by putting M on the command line.

Some special considerations apply in the case of PS/2 systems,

because the port addresses they use for COM3 and COM4 are different

from those in PCs, XTs, and ATs.  Version 3.7 of WHATPORT should

automatically detect the presence of an IBM PS/2 system and check

the right port addresses since it uses a standard interrupt for

testing the system type (INT 15h, function C0h).  It is not certain,

however, that this testing feature will work the same way with clones

of the IBM PS/2.  For this reason the program also allows you to

declare the presence of a PS/2 system by means of a command line

switch.  To declare a PS/2 simply type WHATPORT PS2 (or WHATPORT PS/2)

at the DOS prompt.  You probably won't need to do that, however.

The good news is that if a PS/2 mouse is installed you'll never need a

command line switch to tell the program you have that kind of system.

The program knows a PS/2 mouse when it sees one, and the detection

of that mouse is itself an adequate test for a PS/2 system (no other

type of system could have such a mouse).  Thus you'll need to be

concerned (mildly) about ability to detect a PS/2 system only if there

is no PS/2 mouse installed.

DISTRIBUTION:

WHATPORT.COM and this document are provided free by Life Sciences

Editorial Services, Annapolis, Maryland.  They may be copied and

distributed promiscuously provided no money is charged for them,

they are not modified, and they are not separated.  Although the

program has been extensively tested, Pete Petrakis makes no warranty

concerning its usefulness on all systems and he accepts no

responsibility or liability for any mishap resulting from its use.

Pete Petrakis

Annapolis, Maryland