TD 1.3  -  Time & Date Management Utility

=========================================

COPYRIGHT 1987 TransTech

Released for personal use only

May not be distributed for profit

Tired of having to use the Control Panel to set the time and date

every time you boot your system? Wish you didn't have to think about

it any more?

TD comes close to giving you a battery back-up clock just with software.

Should you buy and install the Dallas Semiconductor DS1216E SmartWatch,

TD will automatically take advantage of it.

CHANGES IN VERSION 1.3

======================

Certain changes were necessary for compatibility with the EZ-Kit. TD and

TDBOOT now also look for a SmartWatch in the cartridge port, for those

of you who have kludged something, or who choose to mount it under a

Cartridge ROM (no guarantees that this will work - call TransTech for info).

Finally two new functions were added to the keyboard interface (AM & PM).

INSTALLING TD

=============

To use TD, copy TD.PRG into the AUTO folder on your boot disk. If you

have a hard disk, copy TD.PRG to a convenient folder for when you may need

to reset the time/date from the desktop. Select TD.PRG with a single

mouse click, pull down OPTION on the desktop and click on Install

Application. Select TOS as the application type. Then pull OPTION down

again and click on Save DeskTop.

You may want to use TDBOOT.PRG program if you have a SmartWatch installed.

It is NOT recommended to have both TD and TDBOOT in the AUTO folder.

TD OPERATION WITHOUT A SMARTWATCH

=================================

When your system is first turned on TD will come up in time/date edit

mode. If TD has been run at least once with this boot disk, the time/date

entered at the last boot will appear. Edit the time and date as described

below. Type RETURN when the time is correct to continue booting your

system.

Whenever the system is reset, TD will automatically set the system time

and date using the ST's IKBD or intelligent keyboard. The IKBD maintains

the date and time, but GEMDOS is too stupid to use it automatically.

If you notice that the date and/or time need correcting, you may run TD

from the desktop by double-clicking as usual. You may edit the date/time

as described below, then press RETURN to set it.

TD OPERATION WITH SMARTWATCH

============================

Operation is greatly simplified with the addition of a SmartWatch. When

the system is turned on or reset TD reads the contents of the SmartWatch

and automatically sets the system time and date. Of course, the date and

time must initially be set after installation by running TD.PRG from the

desktop. From time to time you will notice that the SmartWatch will lose

or gain a few seconds, like any watch. Simply run TD.PRG from the

desktop when the time or date needs correction.

If you prefer not to have the date and time displayed at boot time, you

may use TDBOOT.PRG instead of TD.PRG. TDBOOT can only read the

SmartWatch and set TOS system time. It streamlines the booting process

by being smaller (only 2K) and not writing to the screen at all. It

is installed by placing it in the AUTO folder on your boot disks. Of

course, you shouldn't have both TD and TDBOOT installed in the AUTO

folder - that would just set the time/date twice every boot! TDBOOT can

be run from the desktop; it gives the copyright notice and sets the system

time and date as well.

You can force TD to ignore the SmartWatch's presence. Re-install TD

as TOS-Takes Parameters, then run TD and pass any command line

(except an empty one). TD will act as though no SmartWatch is present.

Although I used this for testing, you may find it useful if you

are 'fooling' the system by intentionally entering the wrong date,

or for demonstrating the program's non-SmartWatch mode.

EDITING THE TIME AND DATE

=========================

When TD enters the edit mode, the time and date are displayed with function

key labels underneath each field. To increment the value of a field

simply press the indicated function key. Shifting the appropriate function

key will lower the value. Values wrap around to their lowest or highest

values if the allowed range for that field is exceeded. Function Key F9

is a special case - it is the Next Day key and will automatically

adjust month and year to the next day as well as the date. The "A" and "P"

keys change the time to an AM or PM value, respectively. The day of the

week is automatically computed and cannot be changed. The number of days

in each month and leap years are controlled and compensated for. Once

the correct date and time have been entered, press RETURN at the exact

time entered. If for any reason you do not wish to set the time and date,

even though you may have edited it, just press UNDO and TD will exit

without setting either the SmartWatch or the system date and time.

INSTALLING A SMARTWATCH

=======================

The DS1216E SmartWatch is an intelligent IC socket manufactured by

Dallas Semiconductor. It can plug into any standard 28-pin 32K ROM socket.

The ROM is then plugged into the SmartWatch. Operation of the computer

should not be affected in any way. This is the same component provided

in many of the clock kits on the market today.

SmartWatch contains a special time-of-day and date clock IC, a lithium

battery and power-fail detect logic. It maintains the date and time with

no power applied to the system. In normal operation the SmartWatch should

last ten years.

Installation of the SmartWatch is not a casual thing. The installer should

be able use hand tools, perform fairly delicate manipulations, and

recognize such items as ROM chips, cables, plugs and sockets, etc. Patience

is another key requirement. This is not to say that the average person

cannot do it; with proper instructions anyone should be able to do the

job but it is safer with someone who has at least an introduction to the

inside of a computer. Word of warning:

NEVER WORK ON ANY ELECTRONIC OR ELECTRICAL DEVICE WITH POWER CONNECTED!

To install in the 520ST, remove the top plastic cover and the metal shield

covering the circuit board. Remove any of the ROM's from its socket, plug

the SmartSocket in its place, and plug the ROM into the SmartSocket. Be

sure to align pin one (indicated by notches or dots) for both the ROM

and the SmartWatch.

Normal installation in the 1040ST requires that the metal bracket holding

the power supply be modified to make room for the SmartWatch. Only the

raised square metal cover needs to be removed to get to the power supply

and the ROM chips. Be sure to remove all electrical components when

hacksawing or drilling that metal bracket. Note: this is not a task for

the faint-hearted! TransTech sells a kit which simplifies installation of

the SmartWatch in the 1040ST which requires no permanent modifications to

the computer.

GETTING A SMARTWATCH

====================

SmartWatch may be available from local or mail-order electronics dealers.

TransTech also has the SmartWatch available, as indicated when you

enter TD's edit mode.

Smartwatch (520ST or 1040ST)			$25

EZ-Kit for the 1040ST (SmartWatch not incl.)	$15

TransTech pays postage if you prepay the order.

C.O.D. Orders are accepted, YOU pay all shipping costs (about $3.50).

Quantity Discounts are available...call or write for information.

All prices subject to change without notice.

Transformation Technologies

10509 S.D. Mission Rd., Suite T

P.O. Box 880668

San Diego, CA 92108

(619) 283-9328

BUGS AND PROBLEMS

=================

The only known problem is that under MT C-Shell TD makes the cursor

disappear and gets the keyboard driver a little confused. The solution

is not to use TD under MT C-Shell, only from the desktop. Alternatively,

use the LOW.PRG or HIGH.PRG utility supplied with the developer's kit to

reset the console driver, or exit MT C-Shell and restart it.

TD should be accurate until the year 2080 AD. If accuracy or any other

problem is noticed, please drop me a line or give me a call at TransTech.

***  USE & ENJOY  ***

May God bless you all,

Jim Frimmel     May 12, 1987