PIN OUT CONFIGURATION FOR 50 PIN SCSI <-> 25DB

SIGNAL		SCSI <-------->	DB25

------------------------------------

DB0		2 <----------->	8

DB1		4 <----------->	21

DB2		6 <----------->	22

DB3		8 <----------->	10

DB4		10 <---------->	23

DB5		12 <---------->	11

DB6		14 <---------->	12

DB7		16 <---------->	13

DBP		18 <---------->	20

ATN		32 <---------->	17

BSY		36 <---------->	6

ACK		38 <---------->	5

RST		40 <---------->	4

MSG		42 <---------->	2

SEL		44 <---------->	19

C/D		46 <---------->	15

REQ		48 <---------->	1

I/O		50 <---------->	3

NOTE:	All odd SCSI pins except pin 25 are connected to ground.

IMPORTANT: PIN 25 ON DB25 CONNECTOR IS RESERVED.

DO NOT MAKE COMPLETED CONNECTIONS TO THIS PIN.