FORMAT11.DOC      VERSION 3.0

-----------------------------

Format11 Version 3.0 corrects a couple of bugs from Version

2.3 and incorporates a couple new added features:

Bugs Corrected

--------------

1. Restores the palette to whatever the colors were when the

program was invoked.

2. Corrects problem of multiple selections showing up in each

option group (e.g. Drive A & B both selected, Tracks 80 & 83

both selected... etc.). This problem only occurred in the

GEM Accessory Version.... F11GEM.ACC.

3. Now allows use of the same resource file for monochrome systems.

New Features

------------

1. Monochrome systems now supported.

2. EXTERNAL Drive "B" now supported.

3. Tracks selection now includes 81 & 82 Tracks/Disk.

4. Verification of formatted disks:

This is useful for drives

that are marginal when doing an 11 Sector/Track format. An error

will be reported upon receipt of any disk error during

the format procedure. This includes the most common errors

(Drive not ready, CRC errors, Sector not found, Track not

found....etc).

5. MSDOS Bootable:

This allows the creation of MSDOS Bootable 3 1/2"

disks ( 5 1/2" not supported) for portation to IBM or IBM clone

systems that have 3 1/2" disk drives.

MISC. NOTES

-----------

1. When copying files between different sector layout disks using

a single disk drive, make sure to open one disk as drive "A"

and the other as Drive "B". This way the OS will read the Bios

Parameter Block for each disk correctly and avoid any thrashing

of the destination disk.

2. Some floppy drives cannot handle more than 80 Tracks/Disk due

to a physical limitation ( plastic crash stops).

3. Also note that the faster your disk turns above the 300rpm

standard, the more likely you are to get errors when formatting

at 11 Sectors/Track. This is due to the fact that the 11 Sector

Layout cannot be successfully written because the disk has made

1 revolution before the last sector is completely written.

I would recommend using the VERIFY option on all 11 Sector/Track

formats.

4. The disks you format will have anywhere from 357,376 to 923,648

bytes available to the user when using the double sided 11 Sector

option. You get ~200k bytes Extra/Disk. 200k x 10 Disks/Box =

2 Mega Bytes Extra/Box. Thats almost 3 extra doubled sided disks!!

A lot of time and effort has went into this program to make it reliable

and user friendly. I hope you find it useful for some of yours needs.

Please send any comments and/or suggestions you might have to:

Matt Orsie                         Compuserve   73737,347

750 Eagles Ct. Apt. 2-D

Westminster, Md   21157