NETWORK V1.0

AC ELECTRONIC CIRCUIT ANALYSIS PROGRAM

Configured for the IBM by:         Bruce A. Trolli

Converted to ST by Mammoth

1

1.0) GENERAL

NETWORK  outputs  the ac frequency reponse for a user

described  electronic  circuit.  The circuit  description  is

prepared  as  an  input  text  file  containing  a  two  line

specification   for  each  element  in   the   circuit.   The

specification  of  each element requires the user  to  assign

node  numbers  to each node in the circuit.  This  method  is

similar  to  many  commercially  available  circuit  analysis

programs.

When  NETWORK is run,  it builds a matrix from  the

user's circuit description file and evaluates the output node

voltage at each frequency under the assumption that the input

node has a 1 volt signal with zero phase angle applied to it.

At run time the user may specify the  frequencies to be

calculated  as  either a linearly or  logarithmically  spaced

interval.  Output  is  sent to the screen by default  but  an

optional output datafile may be specified at runtime.

Models   are   provided  for   Resistors,   Capacitors,

Inductors,  Bipolar  transistors (current controlled  current

sources),  Fets  (voltage  controlled  current  sources,  and

Opamps (voltage controlled voltage sources).

2

2.0) ACKNOWLEDGEMENTS

Network  is  an adaptation of an  existing  circuit

analysis  program  to  run on  the  IBM  PC.   The  origional

article:  "Verify Network Frequency Response With This Simple

Basic Program",  Werner A. Schnider, EDN magazine, October 5,

1977,   was  written  for  a Hewlett  Packard  9830A  desktop

calculator.

The  program  was  modified to run on an Apple  II  and

reappeared  in  the  magazine  as:  "Basic  Program  Performs

Circuit   Analysis",   Richard  Steincross,   EDN   magazine,

September 1,  1982. This version expanded the capabilities of

the  program  by  adding inductors to  the  network  elements

supported.  It  also  allowed the user to specify  input  and

output nodes at run time.  After testing this program, I have

removed  this  feature because it did  not  produce  reliable

output for a number of simple circuits.

3

3.0) REQUIREMENTS

NETWORK  requires a single disk drive.

Because the program output has been limited to text, it

can  run  with either the monochrome or  color  adapter.  The

input  and output formats have purposely been kept simple  to

allow  most  systems to run the  program.  (Besides,  I  have

enough  problems making programs print the correct answer let

alone print it on a jazzed up screen.)

4

4.0) RUNNING THE PROGRAM

4.1) Number the nodes

To analyze your circuit, it will be necessary to number

the nodes on your schematic diagram.  These node numbers will

be  used to prepare a precise description of the  circuit  so

that NETWORK can construct the circuits network equations.

The  input  node must be node number 1 and  the  output

node must be the highest node in the circuit.

AC  ground is node 0.  The 1 volt input that is applied

to node number 1 is referenced to node 0.

4.2) Prepare Input File

Prepare   a  text  file  that  contains  a   two   line

description   for  each  circuit  element  according  to  the

definitions below. Note that comments may be added after the

element  type so that the origional reference designations of

your circuit's schematic may be retained as part of the input

file.  This should make it easy to go back and forth  between

your schematic and the input file.

Also  comments may be added anywhere in the input  file

by placing a ";" at the beginning of the line.

Resistors:

R

from node, to node, value (in ohms)

Example:  A 1000 ohm resistor between nodes 1 and 2.

R

1,2,1000

Capacitors:

C

from  node,  to node,value(in micro farads)

Example: A 10 uf capacitor between nodes 3 and 6.

C

3,6,10

5

Inductors:

L

from node, to node, value (in henries)

Fets:

F

gate node,  source node,  drain node,  transconductance

(amps/volt)

Example: A common source fet with 10 mmhos transconductance.

F

2,0,4,.01

Bipolar Transistors:

B

base node, emitter node, collector node, beta, b-e resistance

(ohms)

Example: Emitter follower with beta=100

B

3,4,0,100,.001

Bruce  Trolli

Cleveland RBBS

(216-3310510)

9

#-3310510)