ANALYSER - a simple ACSI/SCSI device scanner

--------------------------------------------

ANALYSER version 1.4 is Freeware by Roger Burrows, Anodyne Software.

It is hereby placed in the public domain, and may be freely copied and

distributed.  Please keep the program executable and this documentation

together - thanks.

Description

-----------

ANALYSER is a program to scan the peripheral bus(es) on STs, TTs, and

Falcons.  On the ST, it scans the ACSI bus; on the Falcon, it scans the

SCSI bus; on the TT, it scans both the ACSI and SCSI buses.  It issues

an INQUIRY command to each possible device id in order to determine if

there is a device at that id and, if so, what type of device it is.

If a device is present at a specific id, the following information is

listed:

1. The device type (as defined by the SCSI spec): one of:

direct-access (e.g. hard disk)

sequential-access (e.g. tape)

printer

processor

write-once read-multiple (e.g. optical disk)

read-only direct-access (e.g. cd-rom)

If the device isn't recognised as one of the above, it shows as

"unknown device".

2. The vendor and model identifier for the device, as obtained from

the data returned by the INQUIRY command.

Limitations

-----------

ANALYSER does not try to identify if there is more than one logical

unit at each address.  This situation is relatively rare when using

SCSI devices (either directly on a TT or via an ACSI-to-SCSI host

adapter).  It can occur if you are using older-style MFM drives

connected via an MFM-to-SCSI converter, or a changer-style CD-ROM

drive.  In such a case, only the data returned by logical unit zero

will be listed.

WARNING

-------

The Supercharger does not respond correctly to an INQUIRY command;

running ANALYSER can cause the Supercharger to grab the ACSI bus

and refuse to relinquish it, necessitating a re-boot.

Paranoia department

-------------------

As described above, this program only issues an INQUIRY command to

each device, which responds with basic device information.  I have

run it on my 1040ST, TT030 and Falcon030 with no problems.  However:

Roger Burrows and Anodyne Software shall not be liable for errors

contained in the software or the documentation or for incidental or

consequential damages in connection with the furnishing, performance

or use of the software or the documentation.

Aren't disclaimers wonderful?

Roger Burrows

Anodyne Software

9/Nov/95