StarTerm v2.0 --- 04-Mar-1986

by Jim Nangano & Steve Plegge

=============================

*Features*

----------

o  ASCII and XMODEM/XMODEM-CRC protocol file transfers

o  300 to 9600 baud rate

o  Telephone dialer

o  Function keys

o  Load file recognition and stripping

o  Text file conversion, both ASCII and XMODEM

o  Full/Half duplex

o  Smooth/coarse scrolling

o  Normal 'bordered' screen or full 80x25

o  User-set CR/LF options

o  Terminal configuration / function keys for each service

NOTE:  This is User Supported Software.  Select Project/About... for details

of suggested contribution and author's address.  Questions can be sent to

the following network addresses:

Delphi ------- JVNANGANO (Jim Nangano)

GEnie -------- JSP (Steve Plegge)

CompuServe --- [75225,1331] (Steve Plegge)

FidoNet ------ Steve Plegge on Net 10 / Node 100 (St. Louis)

This program and documentation may be freely distributed.

Telephone Dialer

----------------

Phone numbers are kept in the file "Star.Dir" in the current directory.

These numbers, and a dialing prefix ("ATDT" for Hayes, for instance), are

entered from the "Phone/Dial" menu selection.  Enter the service name in

the left column,  and the number on the right.  The dialing prefix goes in

the box to the right of the "SAVE" gadget.

Click any of the "Dial" gadgets to dial, "Cancel" to abort, or "SAVE" to

save to the phone file.

Function Keys

-------------

Function key definitions are reached via the "Terminal/Edit F-Keys" menu.

Enter the string you want each F-key to send.  If you want the F-key to send

a CR (return), use a "}".  You can enter control characters, and they will

saved and sent correctly, but they will be displayed as "box" characters.

Click "Cancel" to abort, or "SAVE" to save the function key definitions.

Newline (EOL) Control

---------------------

StarTerm can be set to send CR, LF, or CR/LF (Terminal/EOL Send); options

for receive are Strip CR, Leave CR, and Strip CR Insert LF (Terminal/EOL

Receive).  These settings are used on both ASCII and XMODEM protocol (text)

file transfer.

Saving by Service

-----------------

You can save the configuration and function keys by service defined in the

dialer requester box.  Click the "Dial" gadget for the desired service, then

save the function key definitions as above.  Configuration data will be

saved in "<service>.Defs"; function keys will be save in "<service>.Keys".

These files will then be read the next time you dial that service.  If you

haven't dialed a service, the files will be saved as "Defaults.Defs" and

"Defaults.Keys".

Load Files

----------

StarTerm recognizes load files on binary XMODEM downloads and does

apppropriate pad stripping of the trash.  No more CHOP!

Defaults

--------

1200 baud, full duplex, 8-bits, no parity, 1 stop (8-N-1).

7-E-1 is also available.

******* StarTerm is brought to you by Delphi/GEnie *StarShip* Amiga *******