Documentation for TI58C.PRG

---------------------------

Data Entry keys

___           ___

| 0 | through | 9 | - Enters digits 0 through 9.  The numeric keypad can

---           ---    also be used to enter numbers and functions.  The

ENTER key corresponds to the = key and the Clr Home

key corresponds to the CLR key.

___

| . | - Decimal point - Enters decimal point.

---

___   ___

|2nd| | 3 | - Enters the value for PI (3.141592) only to 6 digits until I

---   ---    can get true double precision variables.

___

|+/-| - Change sign - changes the sign of he displayed number.

---

Clearing Operations

___

| CE| - Clear Entry - Clears entries made with the digit, decimal point, and

---    change-sign keys only when pressed before a function key.

___

|CLR| - General Clear - Clears calculations in progress and the display.

---

___   ___

|2nd| |CMs| - Clear Memory Storage - Clears all 30 of the memory registers.

---   ---

___   ___

|2nd| | CP| - Clear Program - Clears the program currently in memory.

---   ---

Dual Function Keys

___

|2nd| - Enter function above any key.  Must be followed by the key whose

---    function lies just above the key.

___

|INV| - The inverse key reverses the purpose of the key that follows.  The

---    inverse key can be used with the following keys:

Function		Inverse Function

--------		----------------

lnx			e to the x power

log			10 to the x power

y to the x power	xth root of y

Int			fractional part

Prod			divide into memory

SUM			subtract from memory

___   ___   ___   ___   ___

Basic Functions - | + | | - | | X | | / | | = |   These functions are also

---   ---   ---   ---   ---    available by pressing the

corresponding key on the

numeric keypad where the

ENTER key is the = key.

___   ___

Parenthesis - | ( | | ) |

---   ---

___

|1/x| - Reciprocal - Calculate the reciprocal of the value x by dividing

---	x into 1.

___

|lnx| - Natural Logarithm - Calculates the natural logarithm (base e) of the

---	value in the display.

___   ___		       x

|INV| |lnx| - Natural Antilog e - Calculates the natural antilog, e to the

---   ---    power x, where x is the value in the display.

___

|log| - Common Logarithm - Calculates the common logarithm (base 10) of the

---	value in the display.

___   ___

|INV| |lnx| - Common Antilog - Calculates the common antilog, 10 to the

---   ---    power x, where x is the value in the display.

Angle Calculations

___   ___

|2nd| |Deg|  - angles will be measured in decimal degrees.

---   ---

___   ___

|2nd| |Rad|  - angles will be measured in radians.

---   ---

___   ____

|2nd| |Grad| - angles will be measured in grads.

---   ----

___   ___

|2nd| |sin|  - Calculates sine of an angle.

---   ---

___   ___

|2nd| |cos|  - Calculates cosine of an angle.

---   ---

___   ___

|2nd| |tan|  - Calculates tangent of an angle.

---   ---

___   ___   ___

|2nd| |sin| |1/x|  - Calculates cosecant of an angle.

---   ---   ---

___   ___   ___

|2nd| |cos| |1/x|  - Calculates secant of an angle.

---   ---   ---

___   ___   ---

|2nd| |tan| |1/x|  - Calculates cotangent of an angle.

---   ---   ---

Integer and Absolute Value

___   ___

|2nd| |Int| - Discards the fractional part of the number in the display.

---   ---

___   ___   ___

|INV| |2nd| |Int| - Discards the integer portion of the number in the display.

---   ---   ---

___   ___

|2nd| ||x|| - Absolute alue - Makes the value in the dislay positive.

---   ---

Fixed Point Display

___   ___

|2nd| |Fix |X - Sets fixed point to X decimal places where X ranges from 0-8.

---   ---

___   ___   ___

|INV| |2nd| |Fix| - Cancels fixed point display, returns display to normal.

---   ---   ---    INV 2nd 9 will do cause the same result.

Square and Square Root

___

|  2|

| x | - Calculates the square of the number in the display.

---

___

|  _|

|\/x| - Calculates the square root of the number in the display.

---

Roots and Powers

___

|  x|

| y | - Raises the display value y to the x power.  The entry sequence is

---    y key x followed by an operation or an = key.

___   ___

|   | |  x|

|INV| | y | - Takes the x root of the value y in the display.  The entry

---   ---    sequence is y INV key x followed by an operator of an = key.

Memory Functions

___

|STO|XX - Store - Stores the display value into memory register XX.

---

___

|RCL|XX - Recall - recalls and displays the value stored in memory registed XX

---      and retains the value in XX.

Direct Register Arithmetic

___

|SUM|XX - Memory Sum - adds the display value to the contents of the data

---      register XX and stores the result in XX.

___   ___

|INV| |SUM|XX - Memory Subtract - Subtracts the display value from the contents

---   ---      of data ragister XX and store the result in XX.

___

|Prd|XX - Memory Product - multiplies the display value to the contents of the

---      data register XX and stores the result in XX.

___   ___

|INV| |Prd|XX - Memory Divide - Divides the contents of data register XX by

---   ---      the display value and stores the result in XX.

___   ___

|2nd| |Exc|XX - Memory/Display exchange - exchanges the contents of data

---   ---      register XX with the display value.  The display value is

stored and the previous stored value is displayed.

Printing Functions

___   ___

|2nd| |Prt| - Print the displayed value on the line printer if it is on-line.

---   ---

___   ___

|2nd| |Adv| - Advance the paper in the printer one line.

---   ---

Mean, Variance, and Standard Deviation

___   ______

|2nd| |Sigma+| - Enter cumulative data.  Display will show number of entries

---   ------	 currently in table.

___   ___   ______

|INV| |2nd| |Sigma+| - Cancel cumulative entry.  Number of table entries will

---   ---   ------    decrease by 1.  a value must precede  these keys that

will be subrtacted from the table.  Display will show

number of entries currently in table.

___   ___

|2nd| | x |(above the 6 key) - Calcules the mean of the values entered

---   ---             in the above (Sigma+) fashion.

___   ___   ___

|INV| |2nd| | x |(above the 6 key) - Calculate the standard deviation of

---   ---   ---       values entered in the Sigma+ fashion.

___   ___

|2nd| | Op|11 - Calculates the variance of the values entered in the Sigma+

---   ---      fashion.

The total of the numbers entered in the table is in memory register 1, the

number ef enrties is in memory register 3, and the sum of the squares

of the entries is in memory register 2.

Op Functions

___   ___

|2nd| | Op|10 - Signum function - returns the following:

---   ---                display > 0 returns 1

display = 0 returns 0

display < 0 returns -1

___

|RST| - Reset - Resets the program counter to the first instruction of the

---       program.

Programming

___

|SST| - Single Step - Advance the display and program counter up by 1

---         instruction.

___

|BST| - Back Step - Move the diplay and program counter back one instruction.

---

___   ___

|2nd| |Ins| - Insert - Moves all the instructions starting at the current

---   ---         one up by one so another instruction may be put at the

current location.

___   ___

|2nd| |Del| - Delete - Deletes the current instruction and moves all of the

---   ---        following instructions down by one to fill the empty

memory location.

################################################################################################