Double Speed Format Utility

Copyright 1986,revised

by Brian Duggan

This utility requires the presence of TOS in Ram.  The Original 5-85

version is necessary as this program now modifies the actual coding of the

XBIOS format routine.

************ After formating DO NOT continue on WITHOUT rebooting ********

************     TOS into either RAM or in ROM, your choice.      ********

WHAT IT DOES:

Simply stated, all disk reads and writes (program/DOS permitting) are now

effectively doubled in speed.  That is all reads will be made at the rate of

23000 bytes/sec instead of 11,500 bytes/sec. (raw speed).

This is at the max. theoretical rate for a standard floppy,i.e. 5tracks/

per second or five times 4,608 bytes (nine 512 sectors) =23,040/sec.  Writes

with verify on ($444 address not equaling zero) are again at the max. speed

of 11,500 bytes/sec....as two turns of the disk are required for each write,

one to write on the disk and another to verify what was written.

If Verify is set to OFF by poking address $444 with zero, writes will also

now take place at the same speed as reads.

HOW IT DOES IT:

NO Changes are made to the format of the nine sectors. Additional format

information is placed in the dead space of the track after the nine sectors.

This is the space in which some utilities (this included) can write an extra

sector...giving 10 sector disks.

(Obviously, this speed-up is not possible with 10 sector disks as the space

required for the extra info has already been used up.   10 sector disks cannot

run with the speed-up mod....sorry.)

The additional format information allows the 68000 cpu to step directly to

the next track and continue reading/writing without having tomake any addi-

tional searches.

This is that 68000 power you have heard so much about(!)but seen so little

of with the disk.

CAVEATS:

Loading of large files (such as compilers, BASIC, graphics and STwriter

stuff) will show a net increase, including DOS overhead of 50% in speed.  That

is, BASIC for example will now load in 12 sec instead of 18 secs.

A load in of a large file will average  15,000 to 18,000 net bytes/sec.

This is comparable to some hard disks on the IBM....HAHA!(sorry Big Blue).

Despite what you have read in the various Byte magazine articles which all ran

disk copy benchmarks with Verify On (but only On with ATARI).....all WRITES can

occur at the 23,000 speed ...I challenge you to do the following:

(1) Format a disk with this modified utility...

(2) Load up BASIC and make the $444 poke.

(3) Load in STwriter, Then time how long it takes to load the STMAN

file from an ordinary disk (its almost exactly 90K)

(4) Time how long it takes to copy the STWMAN file to

the modified disk, and then how long it takes to load it

back in....

(5) Divide the time into the  90K to get the rates and  you will be

proud that you own an ATARI!..and have some figures to back it up.

NOW  the CAVEAT!!!!

Many programs are misbehaved... with miniscule input and output buffers that

clobber the ability of the DMA controller and 68000 cpu during disk

input/output.

Scream-holler-yell and write nasty things to these software houses...get them

on their collective toes...since this MOD is free, in the public domain...

But it is no excuse for lazy, third rate programming which butchers the

amazing ATARI.