SCMODE 1.0 by Mark Fontana

Released June 29, 1992

Do you have a Roland SCC-1 and hate wading through that buggy Roland

utility just to set your SCC-1 to MT-32 mode?

Do you have an external SC-55 or SC-155 and hate having to reach over

and do the on/off + hold button routine?

INTRODUCTION

SCMODE is a command line utility to set your Roland Sound Canvas series

tone module to either GS Standard mode or MT-32 emulation mode.  It is

suitable for use in batch files, ie. when starting games configured for the

MT-32, or resetting to GS mode before starting a sequencer.

SCMODE has been confirmed to work correctly with the SC-55, SCC-1, and

SC-155 tone modules.  I have not been able to test it on a JV-series

keyboard, but there's a fairly good chance it will work with those too.

At the moment, SCMODE supports only Roland MPU-compatible MIDI interfaces

(such as MusicQuest's), which covers all SCC-1 owners and a good chunk of

the remaining Sound Canvas user base.  I'll add Soundblaster MIDI support

if enough people ask about it.  As far as other interfaces, if you can

provide me with some (free) information on how to program for them, I'd

be happy to add support for these as well.

COMMAND LINE OPTIONS

SCMODE is pretty straightforward.  It accepts the following command line

options in any order:

mt32     ; set MT32 mode

gs       ; set GS Standard mode

-i n     ; use IRQ n (where n = 2, 3, 5, or 7)

-p n     ; use port n (where n is the base address of your MPU

interface, specified in hex, ie. "330" for 330h);

All options are case insensitive.  If no IRQ is specified, SCMODE will

usually be able to autodetect the correct IRQ level.  If no port is

specified, SCMODE assumes the default setting of 330h.

ERRORLEVELS

If SCMODE successfully changed modes, it returns errorlevel 0.  If any

errors were encountered, it returns errorlevel 1.

DISTRIBUTION

SCMODE is freeware.  You may use SCMODE indefinitely, and no payment or

registration is required.  You may freely distribute SCMODE as long as:

a) no fee is charged

b) no changes are made to the program or this documentation file

I would prefer that SCMODE _NOT_ be distributed by shareware houses

without first contacting me for approval.

DISCLAIMER

Use SCMODE at your own risk.  I assume no responsibility for damage

or loss resulting from the use of SCMODE.

ABOUT SCMODE

SCMODE was written using Borland C++ 3.0 and the C library of the

MusicQuest Programmer's Toolkit (which I highly recommend).

MIDI Interface ToolKit Copyright (c) 1987, 1989 by Music Quest, Inc.

I welcome all comments and bug reports concerning SCMODE.

Here's where you can reach me, in order of speed and reliability:

Internet  :  fontana@cis.ohio-state.edu

Phone     :  1 (614) 261-6117

FidoNet   :  Mark Fontana at Compu-Link BBS; Columbus, Ohio USA

1 (614) 846-1274; Fido address: 1:226/340

CompuServe:  75566,237 (I rarely log in)

UPCOMING PACKAGES

SCEDIT - A parameter editor/librarian and custom SYSEX message builder

for the Sound Canvas series.

SCANIM - An animation editor for the bitmapped display of external

Sound Canvas modules.  Overlays animations on Standard MIDI

files (like the Roland "Stargames" demo).