� Gravis Ultrasound Tech Specs �

��������������������������������

The Unofficial Dox

Written December, 1992

By Tran of Renaissance and Joshua Jensen

OK, folks, here it is.  This is the programming information that Gravis

didn't and won't give you any time soon.  These dox have been obtained

through long hours of debug sessions and experimentation.

Let's clear up some things that have been debated in comp.sys.ibm.pc.soundcard

for quit sometime now:

� The GUS can play 32 mono samples at once, fully pannable through 16

different positions.

� This is still a bit unclear.  The SDK says that samples have to be

32-byte aligned in memory.  I've been following this.  Maybe that is

not the case, though, and I haven't tested it.

Credits:  Tran came up with the initial information about programming the

card.  Josh documented it and included actual assembly language information

in this archive on how it works.  It's relatively simple, but some things are

unclear and are explained here in the dox and in the code.

������������������������������������������������������������������������������

Methods �

���������

@DELAY = (in byte 300h)*7

Description:  All this function does is give a short idle wait for the card

to do whatever processing it needs to.

������������������������������������������������������������������������������

Probe Ultra Sound �

�������������������

out BASE+103h, 4Ch

out BASE+105h, 0

@DELAY

@DELAY

out BASE+103h, 4Ch

out BASE+105h, 1

Poke Data Byte   Loc:   0h, Byte: AA    ; Don't think this HAS to be an AA.

Poke Data Byte   Loc: 100h, Byte: 55    ; Don't think this is needed.

Peek Data Byte   Loc:   0h

Store Byte

out BASE+103h, 4Ch

out BASE+105h, 0

Restore Byte

Is it AA?  If so, then we have found a GUS.

������������������������������������������������������������������������������

Test for amount of memory installed on UltraSound �

���������������������������������������������������

Poke Data Byte   Loc: 40000h, Byte: AA

Peek Data Byte   Loc: 40000h

Is it an AA?  If not, then there is 256k of DRAM.  Exit.

Poke Data Byte   Loc: 80000h, Byte: AA

Peek Data Byte   Loc: 80000h

Is it an AA?  If not, then there is 512k of DRAM.  Exit.

Poke Data Byte   Loc: C0000h, Byte: AA

Peek Data Byte   Loc: C0000h

Is it an AA?  If not, then there is 768k of DRAM.  Exit.

Poke Data Byte   Loc: 0FFFFFh, Byte: AA

Peek Data Byte   Loc: 0FFFFFh

Is it an AA?  If not, then there is 1024k of DRAM.  Exit.

Description:  Poke these bytes on the boundaries of 256k, 512k, 768k, and

1024k.  If the memory isn't there, then it won't return what

is poked.

������������������������������������������������������������������������������

Peek Data Byte �

����������������

out BASE+103h, 43h

out BASE+104h, low word address

out BASE+103h, 44h

out BASE+105h, high byte address

in  BASE+107h, INBYTE

Description:  The card's memory can be from 00000h to fffffh.  INBYTE is the

byte value returned from the card that was at that memory

location.

������������������������������������������������������������������������������

Poke Data Byte �

����������������

out BASE+103h, 43h

out BASE+104h, low word address

out BASE+103h, 44h

out BASE+105h, high byte address

out BASE+107h, OUTBYTE

������������������������������������������������������������������������������

Set Voice Frequency �

���������������������

out BASE+102h, voice number

out BASE+103h, 1

out BASE+104h, word frequency number (actual frequency / 19.0579083837)

Description:  Tran thought he'd give you the exact value :)  Dividing by 19

will give enough accuracy for the task, though.

������������������������������������������������������������������������������

Set Voice Balance �

�������������������

out BASE+102h, voice number

out BASE+103h, 0ch

out BASE+105h, byte balance (0-0fh)

Description: The Balance byte can range from 0 to 15.  0 is far left,

15 is far right, and 7 is right in the middle.

������������������������������������������������������������������������������

Set Voice Volume �

������������������

out BASE+102h, voice number

out BASE+103h, 9

out BASE+104h, word volume value (0-0ffffh, log ... not linear)

������������������������������������������������������������������������������

Set Loop Mode �

���������������

out BASE+102h, voice number

out BASE+103h, 80h

in  BASE+105h, TEMP

out BASE+103h, 0

out BASE+105h, (TEMP & 0e7h) or MODE

Description:  This is not really necessary.  Actually, the Set Loop Mode and

Stop Voice Immediately could be one function.

������������������������������������������������������������������������������

Stop Voice Immediately �

������������������������

out BASE+102h, voice number

out BASE+103h, 80h

in  BASE+105h, TEMP

out BASE+103h, 0

out BASE+105h, (TEMP & 0dfh) | 3

@DELAY

out BASE+103h, 0

out BASE+105h, (TEMP & 0dfh) | 3

Description:  Dunno why the extra delay and duplicated code is there.

������������������������������������������������������������������������������

Start Voice Playing �

���������������������

out BASE+102h, voice number

out BASE+103h, 0ah

out BASE+104h, word BEGIN >> 7

out BASE+103h, 0bh

out BASE+104h, word BEGIN << 9

out BASE+103h, 2

out BASE+104h, word START >> 7

out BASE+103h, 3

out BASE+104h, word START << 9

out BASE+103h, 4

out BASE+104h, word END >> 7

out BASE+103h, 5

out BASE+104h, word END << 9

out BASE+103h, 0

out BASE+105h, MODE & 0fch

������������������������������������������������������������������������������

Read Voice Location �

���������������������

out BASE+102h, voice number

out BASE+103h, 8ah

; in  BASE+104h, low byte TEMP0

; in  BASE+105h, high byte TEMP0

in  BASE+104h, word TEMP0

out BASE+103h, 8bh

; in  BASE+104h, low byte TEMP1

; in  BASE+105h, high byte TEMP1

in  BASE+104h, word TEMP1

LOC = ((TEMP0 << 7) | (TEMP1 >> 9)) & 0xfffff

Note: Word read at BASE+104h seems to work same as sequential byte reads

from BASE+104h then BASE+105h.

������������������������������������������������������������������������������

������������������������������������������������������������������������������

Ports � As you can see, there are a lot of blanks to be filled.  Hope Gravis

������� will take a hint soon.

BASE - write only, mix control

�������������������������������Ŀ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

���������������������������������

?   ?   ?   ?   ?   �   �   �

�   �   � 0 = Line in on.  1 = Line in off.

�   ����� 0 = Output on.   1 = Output off.

��������� 0 = Mic in off.  1 = Mic in on.   ???

BASE+102h - Active voice select (byte port)

BASE+103h - command port (byte port)

Bitmap so far:

�������������������������������Ŀ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �

���������������������������������

�   �   ?   ?   �   �   �   �

�   �           �   �   �   �

�   �           �   �   ���Ĵ

�   �           �   ��������� Command for voice

�   �           �������������

�   ������������������������� Command for card.

����������������������������� Read from voice.

0h - write voice mode (loop and data type)           - byte on 105h

�������������������������������Ŀ

� 7 � 6 � 5 � 4 � 3 � 2 � 1 � 0 �     Designated as MODE through dox

���������������������������������

?   ?   ?   �   �   �   �   �

�   �   �   �   � Voice On/Off (If both set to 0, voice on.)

�   �   �   ����� Voice On/Off (If both set to 1, voice off.)

�   �   ��������� 0 = 8 bit data.  1 = 16 bit data.

�   ������������� 0 = No loop.     1 = Loop Sample.

����������������� 0 = Go forward.  1 = Go backward.

1h - voice frequency                                 - word on 104h

2h - loop start location / 80h                       - word on 104h

3h - loop start location << 9                        - word on 104h

4h - loop end location / 80h                         - word on 104h

5h - loop end location << 9                          - word on 104h

9h - voice volume                                    - word on 104h

0ah - voice location / 80h                            - word on 104h

0bh - voice location << 9                             - word on 104h

0ch - voice balance                                   - byte on 105h

0eh - Unsure.  Has something to do with turning       - byte on 105h

stereo on and off.  See U_Reset for an

example of usage.

43h - low word of DRAM address                        - word on 104h

44h - high byte of DRAM address                       - byte on 105h

4Ch - Unsure exactly of what this does                - byte on 105h

However, a 1 must be output to BASE+105h for

the card to do memory writes.  At least, that's

the only way I could get it to work.

80h - read voice mode                                 - byte on 105h

8ah - read voice location                             - bytes on 104h, 105h

8bh - read voice location                             - bytes on 104h, 105h

Notes:  Any >= 80h plus a number might be the read mode of the value - 80h.

In other words, to read the voice volume (this is untested), you'd

output an 89h through BASE+103h and then read BASE+105h.

BASE+104h - word writes/reads

BASE+105h - byte writes/reads

������������������������������������������������������������������������������

Notes �

�������

To stop an instrument when the sample ends (not immediately), change its

loop type to 'no loop'.

If you are making a lot of changes to just ONE voice, the

OUT BASE+102h, Channel needs only to be done once.