Motorolla PINOUT DIAGRAMS

______________________________________________________________________________

CABLE FOR "Bag & Installed phones" (gray, silver, or black Box)

DB25 MALE to PRN PORT - - - - - - - - - connector "A"

DB25 FEMALE to TRANCEIVER - - - - - - - connector "B"

A.1 to B.18

A.2 to B.21

A.3 to A.14 to B.17

A.4 to B.1

A.12 to B.12

A.13 to B.11

A.17 to B.20

A.18 to B.14 to B.23

A.20 to B.23

B.3 to Ground (12 volt supply to Tranceiver)

B.4 & B.5 to +12 volts (use a cig lighter plug)

_______________________________________________________________________________

CABLE FOR "Classic, Ultra-Classic, also known as "brick" or 8000 series"

Numbers = PRN PORT DB25 MALE CONNECTOR

Letters = connections on back of phone

Back of phone - O o's are contacts

_______________________

1 to K                           | A  B  C |   | D  E  F |

2 to F                           |         |   |         |

3 to C                           | O  O  O |   | O  O  O |

4 to D                           |         |   |         |

12 to J                          | G  H  I |   | J  K  L |

13 to H                          | o  o  o |___| o  o  o |

18 to G                          |                       |

|                       |

|_______________________|

* NOTE ! *                      |                          |

this phone type uses

7.5 to 8.0 Volts !            |                          |

|  -    Battery conn.   +  |

|  O                    O  |

|__________________________|

_______________________________________________________________________________

CABLE FOR "Flip phone" 550 series (micro-tac portable)

A = PRN PORT DB25 CONNECTOR

B = "FLIP PHONE JACK"

A.1 to B.4

A.2 to diode (band towards A conn.) other side of diode to TEST LEAD

The TEST LEAD connects to the center pin of the Battery Connection Area

A.4 to diode (band towards A conn.) other side of diode to B.1

A.12 to B.5

A.13 to B.6

A.18 to B.8

also tie pins 18 thru 25 on conn. "A" together (for grounding)

Pin 8 on "B" connector to - (ground) of 8 Volt power supply

Pin 7 on "B" connector to + (positive) of the 8 Volt supply

* NOTE *

THIS PHONE TYPE REQUIRES 8.0 VOLTS TO PROGRAM

_______________________________________________________________________________