HOW TRAFFIC RADAR WORKS

The first thing to understand about traffic radar is that it has almost

nothing in common with the military radar used for civil defense, or the

aviation radar used by air-traffic controllers, or the weather radar used by

the meteorologist on the ten o'clock news.  Those sophisticated radars can all

measure speed of objects, distance of those objects from the radar station,

and general shape of those objects to aid in identification by a ground crew.

Those radars also cost hundreds of thousands of dollars.

Traffic radar, on the other hand, can't be any larger than what will fit on

the dashboard of a mid-size cruiser, and it can't cost any more than the low

bid of your municipality's procurement process.  Sometimes this is $400 or

less.  So traffic radar is necessarily a device with limited capabilities.

THE CONSTRICTED VIEW

The first limitation restricts the amount of territory traffic radar can

cover.  Military-aviation-weather radars use a rotating beam to sweep a full

360 degrees around the antenna tower.  All targets within range are displayed

on a radar screen.

Traffic radar uses only a stationary beam, much like a searchlight, and it

shines down the road, either forward or backward, but not both ways at once.

So the area under surveillance by traffic radar is quite limited.

The second limitation of traffic radar concerns the amount of information it

can provide.  Traffic radar doesn't have a radar screen.  Only a single

digital readout.  So, at any given time, the maximum amount of information

traffic radar can provide is one number.  This is a very significant

limitation and we'll talk more about it later.

LOOKING FOR TROUBLE

It's reasonable to think of a traffic radar beam as a searchlight because,

even though the beam is made up of microwaves, it behaves very much like a

light beam.  It travels in straight lines and is easily reflected.  Metallic

objects like cars, trucks, guard rails and over passes make the most effective

reflectors, sending glints of microwaves around in unpredictable directions,

just like glints of light.

Unlike light, however, you can't see this beam because microwaves are

invisible to the human eye.  But they are very easily seen by an electronic

eye, in this case a radio tuned to microwave frequency.  And, in fact, such a

radio connected to a compact antenna forms the basis of all traffic radars.

HERE'S LOOKING AT YOU, KID

Traffic radar works by shining its microwave searchlight down the road.  When

you come in range, the microwave beam bounces off your car, and the radar

antenna looks for the reflections.

How does radar get your speed from these microwave reflections? It used a

phenomenon of physics known as the Doppler principle.  We've all heard how the

Doppler principle works with sound waves.  The classic example is what you

hear when you stand along the railroad tracks.  As the train approaches, you

hear the sound at a fixed pitch.  The instant the train passes and begins to

move away, you hear a lower pitch.  In fact, the train itself is making the

same sound both coming and going, but to a stationary listener, the speed of

the train adds to the pitch of its sound as it approaches, and subtracts as it

departs.  This change from true pitch is called the Doppler Shift and the

magnitude of the change depends only upon the speed of the train.

SCIENCE ON THE GO

Traffic radar applies this Doppler principle to microwaves.  The microwave

reflection from an approaching car will be shifted upward in frequency.  A

departing car will cause a downward shift.  The radar compares the shifted

frequency of the reflection to the original frequency of the beam it sent out,

and from the difference it calculated speed, which it then displays on the

digital readout.  Thats really all there is to traffic radar.

A CHEAP BULB IS A DIM BULB

Traffic radar is purposely kept simple.  Going back to the searchlight

analogy, we all know there's a limit to the effectiveness of any flashlight or

spotlight.  The more powerful it is, the farther you can see.  The same

applies to radar.  And since power cost money, whether you're speaking of

searchlights or radar a traffic radar will have far less power than expensive

military radar.  And traffic radar's range will be limited accordingly.  It's

a fact of microwave life that the strength of the beam diminishes with

distance (radio waves in general act this way too (ed)).  The father it has to

travel, the less energy it'll have when it gets there.  For example, the radar

operator may be able to spray your car with microwaves while you're still a

mile away.  But the reflected signal has to travel that same mile back to the

radar before it's of any value, and if it's so weak when it gets back that the

radar's electronics can't read it, then it will be unable to compute speed.

You're out of range.

THE NEARSIGHTED ELECTRONIC EYE

In real life, radar range depends upon two things: power of the radar and the

reflectivity of the target.  We've already talked about power.  That's built

into the radar and there's nothing you can do about it.  Reflectivity of the

target is definitely something you should be aware of, however.  Perhaps

you've heard of an Air Force project called the "stealth bomber." This is an

attempt to build an airplane with poor radar reflectivity so it won't bounce

back the microwave beam.  Such a bomber operating at its normal altitude would

be invisible to enemy radar.  Radar reflectivity is mostly a matter of size

and shape, at least for purposes of describing highway vehicles.  The smaller

the vehicle, the smaller its reflection, and therefore the shorter its range.

Some cars are out of range on some radars until they drive within 500 feet of

the antenna.  On the other hand big, flat surfaces perpendicular to the beam

make excellent reflectors.  The same radar that may be blind to a small car

500 feet away can see a semi a mile and a half away under the right

conditions.  So the principle of radar is quite easily understood, but the

quirks of its operation behavior are hard to predict with accuracy.

THE BOTTOM LINE

The one thing you can be absolutely sure of, however, is that traffic radar

can only monitor one target at a time.  It has one readout and it displays one

number.  Where does it get the number? If there are several cars and trucks in

the beam, as there surely would be on a roadway with even moderate traffic,

how can it give only one number? This, in fact, is the most serious of all of

traffic radar's limitations.  Because it's made to a low-bid price, it must

necessarily be a relatively simple device.  At least the low power of traffic

radar is an asset here in that it pretty well limits surveillance to line of

sight.  And the simplest way of narrowing the field further - down to the

mandatory single number - is to program the electronics to consider only the

strongest reflection.  That has been done in all traffic radars.

And it's this one simplification, more than any other factor, that causes

errors.  This one simplification introduces the human element.  An operator

must look at the one number and decide what or who is responsible for it.

Humans have a tough time keeping track of invisible beams.

BELIEVING WHAT CAN'T BE SEEN

If there is only one vehicle in range, probably that vehicle is responsible

for the number, although it could be caused by an electrical interference or

blowing trash or some other less obvious distraction to the microwaves.  If

there is more than one vehicle in range, it's up to the operator to decide

which one is producing the strongest reflection.  Is it the closest one to the

antenna, or is the largest one in the pack? In truth, it could be either,

depending upon a host of subtleties.  A skilled operator intent on justice

wouldn't write a ticket unless he was absolutely sure.  A less skilled

operator might write the ticket thinking he had the right answer, and be

wrong.  A careless operator intent on filling his quota might see the number

and single out a likely perpetrator the red sports car - and be done with it.

When you deal with humans you take your chances.

THE PLOT THICKENS

So far, this discussion has been confined to stationary radar, the kind

waiting for you in a trap over the hill or hidden behind a bridge abutment.  A

more complex system exists - usually called "moving radar" - which allows an

operator to check the speed of oncoming traffic while the patrol car is in

motion.  The priniciples involved are the same as for stationary radar: a

microwave beam looks forward, monitoring the strongest reflection, which in

this case is the oncoming terrain.  Simultaneously the beam monitors the

second strongest reflection, which it takes to be oncoming traffic.  An

internal calculator then subtracts the terrain speed (same as patrol car

speed) from the closing speed of the patrol car and the oncoming traffic.  The

result - again, displayed as a single number - should be the road speed of the

target vehicle.

A FALSE WITNESS

Should be, but frequently isn't.  Unquestionably, military engineers could

design a moving radar that would deliver accuracy every time.  But on traffic

control budgets, no one has figured a way to eliminate all of the mistakes.

For example, moving radar frequently underestimates patrol car speed because

of "cosine error" with the result that the speed of the target car is

substantially overestimated.  So Well knows is this and other errors that in

some states, like Wisconsin, the courts do not take "judicial notice" of

moving radar, which is to say that radar evidence alone is not considered

persuasive.  The point to remember is that radar is a very sophisticated

electronic concept, a system of proven effectiveness, but traffic radar has

limitations that can't be ignored.  The best way to defend yourself is to have

a good working knowledge of these limitations.