AT&T Technologies                                    Document No. AP88-07DMOS

October, 1988

IMPLEMENTATION OF ADAPTIVE DIFFERENTIAL PULSE-CODE

MODULATION (ADPCM) TRANSCODER

WITH THE

WE(R) DSP16 DIGITAL SIGNAL PROCESSOR

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

enc32org.s   Orginal Program Listing 1. ADPCM Encoder Source Code, page 10

dec32org.s   Orginal Program Listing 2. ADPCM Decoder Source Code, page 26

adpcm32.s    Updated and modified program listing of both encoder and

decoder.  Code file received from author, Y.-S. Lai, on 9/4/90.

Code sections EXPAND (encoder) and COMPRESS (decoder) for

u-law/uniform PCM conversions are replaced with a simple

LINEAR code section.  Also, the synchronous code correction

is deleted.

enc32.s      Current Program Listing 1. ADPCM Encoder Source Code, page 10

dec32.s      Current Program Listing 2. ADPCM Decoder Source Code, page 26

The current code (9/17/90) was supplied by Peter J. Melsa,

who included the corrections of the 9/4/90 ADPCM32.S version.

Peter's notes are appended to the code header messages.

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1988 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

INDEX

~~~~~~

Introduction         (included below)

The ADPCM Algorithm  (included below)

Encoder

Decoder

Implementation

Hardware Interface

Software Implementation

References	     (included below)

INTRODUCTION

~~~~~~~~~~~~~

Direct access to the digital channel is now provided within some

private branch exchange (PBX) environments, in private digital networks,

by digital data services (DDS), and will be provided through integrated

services digital network (ISDN) connections. To make better use of these

digital channels, more sophisticated techniques can be applied to coding

analog signals at a rate lower than 64~kbits/sec so that more than one

analog signal, or concurrent analog and digital signals, can be

transmitted over the same 64~kbits/s channel.

Typically, telephone quality signals are digitally encoded using pulse-

code modulation (PCM) by sampling the analog signal at an 8~kHz rate, and

quantizing each sample using an 8-bit companding codec. This conversion

produces a 64~kbits/s digital signal from the original 4~kHz bandwidth

analog signal. The digital signal is then transmitted through the tele-

phone network using a 64~kbits/s channel before being converted back to

the analog domain at the receiving end. One solution for increasing the

capacity of the 64~kbits/s channels is to encode the voice channel

using ADPCM.

The ADPCM transcoder converts an A-law or u-law (or "mu"-law) 64~kbits/s

PCM data stream into a 32~kbits/s ADPCM data stream. Once converted, the

channel capacity of the current 64~kbits/s PCM is doubled, providing more

channels for transmitting and receiving data. A block diagram of a

full-duplex ADPCM transcoder is shown in Figure 1.

Two programs for implementing the ADPCM transcoder are described in

this note.  The programs implement a transcoder meeting the CCITT

recommendation G.721 [1].  A full-duplex implementation requires a

DSP16A running at 50~ns. This set of programs can be modified to allow

nonstandard operation on a DSP16 [2] running at 75~ns. Equivalent coding

quality can be achieved in this mode. This should be the program of choice

when interoperation with a CCITT G.721 transcoder is not required. The

modification is identified in the Source Code section.

THE ADPCM ALGORITHM

~~~~~~~~~~~~~~~~~~~~

The bit rate needed to transmit (or store) speech can be reduced by

removing any redundancy in the original PCM encoded signal. ADPCM

transcoders remove redundancy by using a predictor to estimate the

signal value at each sample.  By subtracting this estimate from the

original signal, a signal with less dynamic range is produced, which,

in turn can be quantized with fewer bits than the original PCM sample.

Adaptation of the predictor is based on a history of recent quantized

samples. Therefore, no additional information is needed by the decoder

to reconstruct the transmitted signal. The quantizer step-size adaptation

rate is also controlled to optimize the transcoders signal-to-noise ratio.

The quantizer stops adapting in the presence of narrow-band energy to

improve the performance for voiceband data signals. The decoder also

includes a synchronous coding adjuster which reduces the cumulative

distortion that occurs on synchronous tandem encodings.

The detailed functional blocks comprising the transcoder are

described in the Encoder and Decoder sections.

REFERENCES

~~~~~~~~~~~

[1] "Digital Processing of Voice-Band Signals - Algorithm for 24, 32,

40 bits/s ADPCM," American National Standard for Telecommunications

(T1Y1/87-040, T1Y1.2/887-090R1).

[2] WE(R) DSP16 Digital Signal Processor Information Manual,

June 1987

[3] "T7500 PCM Codec with Filters" AT&T Communication Devices Data Book,

March 1988

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=