FIX-FLOAT

===========

A LIBRARY FOR

RAPID HANDLING OF

DECIMAL NUMBERS

REPRESENTED

ON A

FIXED POINT

BINARY NOTATION

+

32 bit inexhaustible

???

random number

generator

-------------

| Version 1.1 |

-------------

REQUIREMENTS :

==============

The routines are written to be run in protected mode with 32 bit default

data size.  A C - compiler is required to set up data structures, this

could be done in any highlevel language that can export symbols visible

to assembly language.  C++ uses a strange naming convention of their

symbols making the task more difficult.  Support of floating point

processing is also required to setup data structures. No 486 or higher

instructions are used, a 386 is required however.  All timings given

in the function descriptions below are based on Intels 486.

INTRODUCTION :

==============

Fixfloat is a small library for handling decimal numbers represented on

a fixed point number format. A decimal number is represented with 32

bits, using the upper 16 bits as the integral part and the lower 16

bits as the decimal part :

31    24       16        8        0  ( bit nr )

________ ________ ________ ________

register :   |________|________|________|________|

integral part   ^  decimal part

A few decimal numbers and their hexadecimal representation as fixfloats:

Decimal number | Fixfloat (hex)

---------------------------------

1/16     |  0000 1000h

0.25     |  0000 4000h

0.5      |  0000 8000h

1        |  0001 0000h

7.1875   |  0007 1000h

In C language conversion from floats & doubles to fixfloats is

performed by multiplying by 65536 and taking the integral part of this

result.

ADVANTAGES & DISADVANTAGES :

============================

The advantage of this representation of decimal numbers are that

calculations are quicker,  usually by a factor 1.5 to 4, compared to

using floating point instructions,  the process of converting to an

integer is quick & simple ( 'shr eax,16' if eax contains a fixfloat in

assembler, (a>>16) in C if 'a' is a fixfloat ).  Converting from

floatingpoint register to integer takes 33 clockcycles on a 486,  it

takes 2 clock cycles to go from fixfloat to integer.  Another advantage

is that software using this format performs equally whether there is an

FPU available or not.  This remains an argument since many new x86

processors of various brands ships without floating point unit.  Adding

and subtracting operations use standard add & sub instructions and

computes 24 times faster than the fpu equivalents.  The primary

disadvantage of this format is of course that the range of the numbers

is limited to -32768 to 32767.  Other disadvantages are that precision

is limited and that for most operations a subroutine call is required

with added time for call & ret instructions.  This can be undone by

expanding the function into the calling body.  These routines are not

aimed towards applications where the result is numerically presented to

the user but rather where there is no need for better precision than a

4-5 decimal digits.  Another disadvantage is that multiplications are

relatively slow on the integer unit compared to the floating point

unit, this is true for both 486 & 586 processors.

ALGORITHMS :

============

Divisions and multiplications are performed by unsigned integer machine

instructions, using the 64 bit results of multiplication and the

possibility to use 64 bits in division denominators. No precision or

time is lost in this way.  The trancendental functions supported are

usually performed by a number of shifts to get the in value in an

appropriate range and then looking in a data table and some shifting of

the result. Precision is limited by how large data tables one wants.

Typically 12 bits precision of the incoming value is supported but this

is easily modified.

TRIGONOMETRY :

==============

The anglesum of a circle is 65536 instead of 2 PI,  thus a circle have

angles from 0 to 1. Here are some angles on different angle bases :

DEG  |  RAD   |  FIXFLOAT

===========================

45   |  PI/4  |  02000h

90   |  PI/2  |  04000h

180  |  PI    |  08000h

360  |  2PI   |  10000h

Sin & cos functions have 16384 steps on a circle, the asin, acos

functions goes from 0 to 1 in 4096 steps.  The tan function goes from

zero to 90 degrees in 1024 steps, if angles are larger than 84.375

degrees a separate 256 entry table is used and if angles are larger

than 88.59375 a last 256 entry table is used.  The atan functions uses

a table of 1024 entries if argument is less than 4 ( 0-4/1024 ). If

argument is less than 16 it uses a table of 256 entries ( 0-16/256 ) if

argument is less than 512 it uses a table of 256 entries ( 0-512/256 ),

if argument is less than 8192 it uses a table of 256 entries ( 0-8192/

256).

COMBINED ARITHMETIC FUNCTIONS :

===============================

By squaring a number larger than 256 the 16 bit integral part

overflows. By combining a few actions in a function one can sometimes

benefit from the 64 bit intermediate format. This is used by functions

ffhyp, fftrihyp.  The operations performed by ffhyp & fftrihyp are :

ffhyp = sqrt ( a*a+b*b )  fftrihyp = sqrt ( a*a+b*b+c*c )

It is also possible to eliminate intermediate shifts if both divison

and multiplication is performed.  This is used in following operations :

a * b                a * b

ffmuldiv  =  -------     ffmmd  = -------

c * d                  c

DATA TABLES :

================

The gen_fix_float_tables() function will setup the data tables needed

by the trancemdental functions. A table is also setup for a random

number generator. This function assumes that the compiler environment

has some sort of support for double numbers and elementary trancendental

functions. A random number generator must be present also. The rand

function is expected to return a random number covering at least bits

0-14 (15 bits). At current the data tables occupies approximately 64 kb.

The random table stands for 16 of those. Only the pure mul &  div

functions can be used without intializing data tables. It seems that

it takes something like a second to intialize all tables.

GENERAL INFO ON FUNCTIONS :

===========================

I have tried clocking a few of the functions using a hires timer but

the results are inconsistent and I don't want to put too much effort

into it.  Most instructions in the funtions execute in one clock,

except for shifts (2-4), multiplications (13-42), divisions (40), 16

bits loads (2).  This is true for a 486 processor,  for a 586 the

situation is different again. AGI stalls have been avoided to a large

extent. I have added time for loading data from math tables since this

data at most times won't be in the primary cache.  For a 16 bit load

( which is frequently use to lessen table size ) with a 'eax*2' offset

I have counted 7 clocks.

None of the functions use any local or global variables, a few uses some

stackspace (max 32 bytes).  The timings mentioned below are based on

instruction count and do not include the call and the ret statement.

If called from C the names below should be used, if called from within

assembly language a underbar must be added to end of functionname

( Watcom Std ). Parameters to the functions are used in the following

order :

eax - first argument

edx - second argument

ebx - third argument

ecx - fourth argument

Return values are in eax for all functions.

DESCRIPTION OF FIXFLOAT FUNCTIONS :

===================================

C FUNCTIONS :

=============

gen_fixfloat_tables :

---------------------

Will calculate all the tables needed by below functions, should be called

when using other first of all when using other functions than pure mul

and div functions.

Clocks  : c:a 20 000 000

fftoa(long ff, char *buf) :

---------------------------

Converts fixfloat ff to ascii decimal representation, puts result

where buf points. Zero truncated format, 3.25 represented by '3.25'.

fftoah(long ff, char *buf) :

---------------------------

Converts fixfloat ff to ascii hexadecimal representation, puts result

where buf points. Full format, 3.25 represented by '0003.4000'.

ffsmul :

--------

Multiplies the fixfloat in eax by the fixfloat in edx. Result in eax.

The result has sign according to the factors.

Affects : edx

Clocks  : 52

ffsdiv :

--------

Divides the fixfloat in eax by the fixfloat in edx. Result in eax.

Returns +/- infinity if denominator 0, does not guard against the

situation where the result doesn't fit into 32 bits. A div by zero

error is then generated. The result has sign according to the

nominator & denominator.

Affects : edx

Clocks  : 50

ffmul :

-------

Unsigned multiplication by two fixfloats, eax and edx. Result in eax.

This function consists of only 2 instructions and is therefore good

to expand into calling body :

mul     edx          ; these two instructions does the job

shrd    eax,edx,16

Affects : edx

Clocks  : 37

ffdiv :

-------

Unsigned division between two fixfloats, eax and edx, result in eax.

Checking as ffsdiv.

Affects : edx

Clocks  : 50

ffsin :

-------

eax contains angle to take sinus from. Value returned in eax.

Affects : none

Clocks  : 26

ffcos :

-------

eax contains angle to take cosinus from. Value returned in eax.

Affects : none

Clocks  : 27

ffasin :

-------

eax contains angle to take arcsinus from. Value returned in eax.

Affects : none

Clocks  : 21

ffacos :

-------

eax contains angle to take arccosinus from. Value returned in eax.

Affects : none

Clocks  : 23

fftan :

-------

eax contains angle to take tan from. Value returned in eax.

Affects : none

Clocks  : 23 - 33

ffatan :

-------

eax contains angle to make angle from. Value returned in eax.

Affects : none

Clocks  : 25 - 38

fflog2 :

--------

eax contains fixfloat to tak 2logarithm on. Value returned in eax.

Affects : none

Clocks  : 56

ffexp2 :

--------

eax contains the number to raise 2 to. Value returned in eax.

Affects : none

Clocks  : 20

ffpow :

-------

eax contains base and edx contains exponent. Value returned in eax.

This function is a combination of fflog2, ffsmul, ffexp2.

Affects : none

Clocks  : 152

ffsqrt:

-------

eax contains value to take square root of, returns squareroot in eax.

Affects : none

Clocks  : 61

ffhyp :

-------

Used to calculate hypothenuse of triangle. eax contains one side, edx

the other one. Result is returned in eax. An intermediate result

larger than 32 bits will not overflow, this is the advantage compared

to performing the operation step by step.

returns sqrt ( a*a + b*b)

Affects : none

Clocks  : 168

fftrihyp :

----------

As above but takes three arguments, the third is squared and added to

the two first ones.

returns sqrt ( a*a + b*b + c*c)

Affects : none

Clocks  : 212

ffmuldiv :

----------

Given four fixfloats a,b,c,d in register eax,ebx,ecx,edx it will

calculate (a * d) / (b*c) . This guards agains intermediate over-

flows and saves a bit of time. Returns +/- infinity if 0 in

denominator.

Affects : none

Clocks  : 150

ffmmd :

-------

Given three fixfloats in eax,edx,ebx it will calculate (eax*edx)/ebx

allowing for more than 32 bit intermediate result.  Returns +/- infinity

if 0 in denominator.

Affects : none

Clocks  : 94

ffortproj :

-----------

The function is short for 'ortogonal projection', it is used for finding

out how much a vector travels in another vectors direction. If it is

perpendicular to the other vector 0 is returned, if it travels the same

distance in the other vectors direction as the length of the other

vector 1 is returned. Negative meaning it goes in opposite direction

and a number above one that it goes further than the other vector in

its direction.

This function uses shifting (5 steps right ) to keep result within

format. Be aware !

Vector (b,d) is projected onto (a,c) :

eax - a      ( two vectors (a,c) , (b,d) )

edx - b

ebx - c

ecx - d

returns (a*b+c*d)/(a*a+c*c)

Affects : none

Clocks  : A lot :)

ffortproj1 :

-----------

Same as above function but it takes square root of denominator before

dividing, therefore it returns the length travelled in the other vectors

direction. This is what is usually meant by ortogonal projection.

This function uses shifting (5 steps right ) to keep result within

format. Be aware !

Vector (b,d) is projected onto (a,c):

eax - a      ( two vectors (a,c) , (b,d) )

edx - b

ebx - c

ecx - d

returns (a*b+c*d)/sqrt(a*a+c*c)

Affects : none

Clocks  : Uncountable ...

ffdot_through_hyps :

--------------------

Function does what the name says, having two vectors (a,c) & (b,d) it

returns the dot product of the vectors through the product of the

lengths of the vectors.

This function uses shifting (5 steps right ) to keep result within

format. Be aware !

eax - a

edx - b

ebx - c

ecx - d

returns (a*b+c*d)/(sqrt(a*a+c*c)*sqrt(b*b+d*d))

Affects : none

Clocks  : Many ...

DESCRIPTION OF INTEGER FUNCTIONS :

==================================

There is only one integer function ( taking integer as result, giving

integer as result supported. More may be added.

isqrt :

-------

Function returns a true square root of its arguement. It uses the same

table as ffsqrt, therefore needs gen_fixfloat_tables init to work

properly. A square root is always less than or equal to the actual square

root, isqrt(15) = 3, isqrt(16) = 4 etc. Observe that precision of this

one is not more than 3 digits.

Affects : none

Clocks  : 58

DESCRIPTION OF RANDOM NUMBER FUNCTIONS :

========================================

The function rand32 is responsible for producing pseudo random numbers.

The numbers are produced using a combination of a multiplication and a

data table containing pregenerated numbers.  8 bits of the data table

is used for each number,  it takes 16384 times to step through it.  The

multiplicand is constant during one such run, then it is changed. The

random table is also mutaded at this time.  The number generated is

always dependent on the previous one.  Even if the same multiplicand

reappears when restarting on a table it is highly unlikely that the

product in that moment would be the same as the one 256 million times

ago.  If this would happen the same random numbers would only appear

until a mutated entrance would occur,  which would be the first to come.

The period for the randomgenerator shold be :

16384 * 16384 * 2^32 * 2^(16384*32)  = 2 ^ 524438

Which I think is enough for many applications. ( A DEC Alpha 330 MHz

would produce 2^60 pseudo random numbers in 1000 years if dedicated to

it. )

There are 3 memory locations visible affecting the generator :

_rnd3 - contains the number by which to multiply the last random

number. Before multiplication a constant is added and then

the value is truncated to 12 bits to keep multiplication time

down.

_rnd2 - contains how many numbers generated so far.  Every time it

ticks over a 16 kb limit the _rnd3 is changed.

_rnd1 - contains the last number generated.

Any of these can be changed to produce 'randomize' affect. From a given

start it will always produce identical numbers.

rand32 :

--------

Returns a 32 bit pseudo random number.

Affects : none

Clocks  : 41

randpowint :

------------

eax contains a number to 'and' the 32 bit random number with, edx

contains the number above the maximum number desired.  This function

can be used when the interval of the randomnumber one want's is the

same from time to time, saves something like 25 clocks. If an interval

close to the 'and' value in eax is chosen it will have to loop back

fewer times.

Example :

// Want random number 0-99

long rn;

rn = randpowint(127,100)

....

// Want random number 0-2999

long rn;

rn = randpowint(4095,3000);

Affects : none

Clocks  : 47 - 94 ( Average c:a 80 )

randinter :

-----------

eax contains the number above the number desired. This functions

computes the 'and' value before calling rand32 and is a bit slower.

Example :

// Want random number 0-19

long rn;

rn = randinter(20);

Affects : none

Clocks  : 77 - 124 ( Average c:a 110 )