CHAPTER 11.  WRITING AN INSTALLABLE DEVICE DRIVER 26

Types of Device Drivers 39

Character Device Drivers 48

Block Device Drivers 55

How DOS 4.00 Installs Device Drivers 84

The Basic Parts of a Device Driver 134

The Device Driver Header 148

The Strategy Routine 274

The Interrupt Routines 283

How DOS 4.00 Passes a Request 300

Responding to Requests 392

Initialization Request 456

Media Check Request 511

Build BPB Request 630

Input and Output Requests 815

Nondestructive Input No Wait Request 879

Character Input and Output Status Requests 901

Character Input and Output Flush Requests 935

Open and Close Requests 951

Removable Media Request 983

Generic IOCTL Request 1008

Get Logical Device Request 1037

Set Logical Device Request 1055

CLOCK$ Device Driver Example 1073

CHAPTER 11.  WRITING AN INSTALLABLE DEVICE DRIVER

This chapter provides guide and system architecture information to support

successful creation of an installable device driver.

We recommend you use the information provided in this chapter in conjunction

with the information provided in the VDISK.ASM file on the utilities

diskette.  This file contains the fully documented source code for the VDISK

device driver, which emulates a fixed disk in RAM.

NOTE:  The example on the utilities diskette does not reflect the current

level of VDISK.SYS..

TYPES OF DEVICE DRIVERS

A device driver is a memory image file or an .EXE file that contains the code

needed to implement a device.  DOS 4.00 allows two types of device drivers to

be installed:

Character device drivers

Block device drivers.

CHARACTER DEVICE DRIVERS

Character device drivers perform character I/O in a serial manner and have

names such as CON, AUX, CLOCK$.  You can open handles or FCBs to perform

input and output to character devices.  Because character device drivers have

only one name, they support only one device.

BLOCK DEVICE DRIVERS

Block device drivers are the fixed disk or diskette drives on the system.

They perform random I/O in pieces called blocks, which are usually the

physical sector size of the disk.  Block devices are not named as character

devices are and you cannot open them.  Instead they are mapped using the

drive letters such as A, B, and C.

A single block device driver can be responsible for one or more disk or

diskette drives.  For example, the first block device driver in the device

chain may define four units such as A, B, C, and D.  The second device driver

may define three units:  E, F, and G.  The limit is 26 devices with the

letters A through Z assigned to drives.  The position of the driver in the

chain determines the way in which the drive units and drive letters

correspond.

Support for Media Greater than 32MB

A block device driver that runs in the DOS 4.00 environment can be written to

process 32-bit sector numbers.  This ability provides support for fixed disk

media greater than 32MB.  This support is described in these sections:

"The Device Driver Header" on page 11-3

"Build BPB Request" on page 11-14

"Input and Output Requests" on page 11-18.

DOS 4.00 also allows users to install their own device drivers that support

media greater than 32MB.

HOW DOS 4.00 INSTALLS DEVICE DRIVERS

DOS 4.00 installs device drivers dynamically at startup time by reading and

processing the DEVICE command in CONFIG.SYS.  For example, if you have

written a device driver called DRIVER1, include the following command in

CONFIG.SYS:

device=driver1

The DOS 4.00 device interface links device drivers together in a chain,

permitting you to add device drivers for optional devices.

Each device must be initialized.  The device driver's initialization

interrupt routine is called once when the device is installed.  The

initialization routine returns the location in memory of the ending address

of the device driver.  After setting the ending address field, a character

device driver sets the status word and returns.

DOS 4.00 processes installed character device drivers before handling default

devices.  To have DOS 4.00 install a new CON device (for example, in the

device driver header's Name/Unit field) name the device CON and set the

standard input device and standard output device bits in the attribute field.

Because DOS 4.00 installs drivers anywhere in memory, care must be taken in

any references to locations not in the segment.  Your driver will not always

be loaded at the same memory location each time.

Block devices are installed in the same manner as character devices, above.

Block devices return additional information such as the number of units.

This number identifies the devices' logical names.  For example, if the

current maximum logical device letter is F at the time of the install call,

and the block device driver initialization routine returns three logical

units, the logical names of the devices are G, H, and I.  The mapping is

determined by the position of the driver in the device list and the number of

units associated with the device.  The number of units returned by INIT

overrides the value in the name/unit field of the device header.

A block device also returns a pointer to a BIOS parameter block (BPB) array.

This is a pointer to an array of n word pointers, where n is the number of

units defined.  If all the units are the same, the array is able to point to

the same BIOS parameter block, thus saving space.  The array must be

protected below the ending address pointer set by the return.

The BPB contains information pertinent to the devices such as the sector size

and the number of sectors per allocation unit.  The sector size in the BPB

cannot be greater than the maximum allowed size set by DOS 4.00

initialization.

A block device returns the media descriptor byte passed to devices to report

which parameters DOS 4.00 is using for a particular drive unit.

THE BASIC PARTS OF A DEVICE DRIVER

A device driver is a memory image file or an .EXE file containing the code

needed to implement a device.  All DOS 4.00 installable device drivers have

these things in common:

A device driver header, which identifies the device to DOS 4.00 and

defines the strategy and interrupt entry points.  Since a device driver

is simply loaded and does not use a program segment prefix, the device

header must be located at physical location 0 of the device driver (ORG 0

or no ORG statement).

A strategy routine, which saves a pointer to the Request Header.

The interrupt routines, which perform the requested operation.

THE DEVICE DRIVER HEADER

A device driver requires a device header containing the following:

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Pointer to next device header                                  DWORD      |

+---------------------------------------------------------------------------+

| Attribute                                                      WORD       |

+---------------------------------------------------------------------------+

| Pointer to device strategy routine                             WORD       |

+---------------------------------------------------------------------------+

| Pointer to device interrupt routine                            WORD       |

+---------------------------------------------------------------------------+

| Name/unit field                                                8 BYTES    |

+---------------------------------------------------------------------------+

Pointer to Next Device Header

The device driver header is a pointer to the device header of the next device

driver.  It is a doubleword field set by DOS 4.00 at the time the device

driver is loaded.  The first word is an offset and the second word is the

segment.

If you are loading only one device driver, set the device header field to -1

before loading the device.  If you are loading more than one device driver,

set the first word of the device header field to the offset of the next

device driver's header.  Set the device header field of the last device

driver to -1.

Attribute Field

The attribute field identifies the device to DOS 4.00.

Bit 15

Bit 15 identifies whether the device is a block device or a character device.

If bit 15 is set to 0, this indicates a block device.  Setting bit 15 to 1

indicates a character device.  Note how the setting of bit 15 affects the

interpretation of the setting of the bits below.

Bit 14

Bit 14, for both character and block devices, tells DOS 4.00 whether the

device driver can handle control strings through IOCtl 44H, AL=2 through

AL=5.  Set bit 14 to 1 if control strings can be processed.  IOCtl

subfunctions permit the device driver to interpret the information passed to

it, such as setting a baud rate or changing form lengths, without performing

standard reads and writes.  Set bit 14 to 0 if control strings cannot be

processed.  DOS 4.00 will return an error if an IOCtl is issued to send or

receive control strings and bit 14 is set to 0.

Bit 13

Bit 13 is used for both block and character devices.  For block devices, set

bit 13 to 0 if the media is an IBM format.  Set bit 13 to 1 if the media is a

non-IBM format.  For character devices, set bit 13 to 0 if the driver

supports output-until-busy.  Set bit 13 to 1 if it does not.

With the support of output-until-busy, the device driver will send characters

to the device if the device is ready.  If the device is not ready, the device

driver will immediately return an error.

Bit 12

Bit 12 is reserved.

Bit 11

Set bit 11 if the device driver can handle removable media.  This bit is

called the open/close removable media bit.

Bits 10 through 7

Bits 10 through 7 are reserved.

Bit 6

Bit 6 is the generic IOCtl bit for both character and block device drivers.

If this bit is set to 1, the device driver supports generic IOCtl function

calls.  Setting this bit to 1 also indicates support of the Get/Set Logical

Drive function for a block device driver.

Bits 5 and 4

Bits 5 and 4 are reserved.

Bit 3

Set bit 3 to 1 if the character device is a clock device; set bit 3 to 0 if

it is not.

Bit 2

Set bit 2 to 1 if the character device is the NUL device; set bit 2 to 0 if

it is not.  Setting the bit tells DOS 4.00 whether the NUL device is being

used.  The NUL device cannot be reassigned.

Bit 1

If bit 15 is set to 0 for a character device, set bit 1 to 1 to indicate that

the character device is the current standard output device.  If bit 15 is set

to 1 for a block device, set bit 1 to 1 to indicate support for 32-bit sector

numbers; otherwise 16-bit sector number support is assumed.

Bit 0

Set bit 0 to 1 if the character device is the current standard input device;

set bit 0 to 0 if it is not the current standard input device.

Pointers to Strategy Routine and Interrupt Routines

When DOS 4.00 passes a request to a device driver, it calls the device driver

twice.  These two fields point to the first and second entry points:  the

strategy routine and the interrupt routine.  The fields are word values, so

they must be in the same segment as the device header.

Name/Unit Field

These 8-byte fields identify a character device by name or a block device by

unit.  A character device name is left-justified followed by spaces, if

necessary.  For block devices, although DOS 4.00 automatically fills in this

field with the value of number of units returned by INIT call, you may choose

to place the number of units in the first place.

THE STRATEGY ROUTINE

DOS 4.00 calls a device driver at the strategy routine at first, passing in a

request packet the information describing what DOS 4.00 wants the device

driver to do.

The strategy routine does not perform the request but queues the request or

saves a pointer to the request packet.

THE INTERRUPT ROUTINES

DOS 4.00 calls the device driver's interrupt routine with no parameters

immediately after the strategy routine returns.  An interrupt routine's

function is to perform the operation based on the queued request, process any

data in the request packet, and set up information being returned to DOS

4.00.

It is the responsibility of the device driver to preserve the system state.

For example, the device driver must save all registers on entry and restore

them on exit.  The stack maintained by DOS 4.00 is used to save all

registers.  If more stack space is needed, it is the device driver's

responsibility to allocate and maintain an additional stack.

All calls to device drivers are FAR calls.  FAR returns should be executed to

return to DOS 4.00.

HOW DOS 4.00 PASSES A REQUEST

DOS 4.00 passes a pointer in ES:BX to the request packet.  The packet

consists of a request header that contains information common to all

requests, followed by data pertinent to the request being made.

The structure of the request header is shown below.

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Length of the request header and subsequent data               BYTE       |

+---------------------------------------------------------------------------+

| Unit code for block devices only                               BYTE       |

+---------------------------------------------------------------------------+

| Command code                                                   BYTE       |

+---------------------------------------------------------------------------+

| Status                                                         WORD       |

+---------------------------------------------------------------------------+

| Reserved                                                       8-BYTE     |

+---------------------------------------------------------------------------+

| Data                                                           VARIABLE   |

+---------------------------------------------------------------------------+

Length Field

The length field identifies the length of the request header and subsequent

data in bytes.

Unit Code Field

The unit code field identifies the requesting unit in a block device driver.

If a block device driver has three units defined, for example, the possible

values for the unit code field are 0, 1, or 2.

Command Code Field

The command code identifies the request.  See "Responding to Requests" on

page 11-8 for a list of command code values and request descriptions.

Status Field

The status word field is zero on entry and is set by the driver interrupt

routine on return.

Bit 15

Bit 15 is the error bit.  If bit 15 is set to 1, the low order 8 bits of the

status word (7-0) indicate the error code.

Bits 14 - 10

Bits 14 through 10 are reserved.

Bit 9

Bit 9 is the busy bit.  As a response to status request call, character

device drivers can set the busy bit to indicate whether or not a device is

ready to perform input and output requests.  Block device drivers can set the

busy bit to indicate removable or nonremovable media.  See "Character Input

and Output Status Requests" on page 11-21 and "Removable Media Request" on

page 11-23 for more information about the calls.

Bit 8

Bit 8 is the done bit.  If set, the operation is complete.  The driver sets

the done bit to 1 when it exits.

Bits 7 - 0

Bits 7 through 0 are the low order 8 bits of the status word.  If bit 15 is

set, bits 7 through 0 contain the error codes.  The error codes and errors

are:

CODES     MEANING

00        Write protect violation

01        Unknown unit

02        Device not ready

03        Unknown command

04        CRC error

05        Bad drive request structure length

06        Seek error

07        Unknown media

08        Sector not found

09        Printer out of paper

0A        Write fault

0B        Read fault

0C        General failure

0D        Reserved

0E        Reserved

0F        Invalid disk change

RESPONDING TO REQUESTS

Each request packet that is passed to the device driver contains a command

code value in the request header to tell the driver which function to

perform.  The table below contains the DOS 4.00 device interface command code

values and the functions to be performed when these values are passed with

data.  Note that some of these functions are specific to either a block

device or a character device.

Following this table are detailed descriptions of request data structures and

what the interrupt routines are expected to do.  Some of these descriptions

pertain to more than one command code.

+---------------------------------------------------------------------------+

| COMMAND  REQUEST                                               DEVICE     |

| CODE     DESCRIPTION                                           TYPE       |

+---------------------------------------------------------------------------+

|  0       Initialization                                        Both       |

+---------------------------------------------------------------------------+

|  1       Media check                                           Block      |

+---------------------------------------------------------------------------+

|  2       Build BPB                                             Block      |

+---------------------------------------------------------------------------+

|  3       IOCtl input (called only if bit 14 of attribute is    Both       |

|          set to 1)                                                        |

+---------------------------------------------------------------------------+

|  4       Input (read)                                          Both       |

+---------------------------------------------------------------------------+

|  5       Nondestructive input no wait                          Character  |

+---------------------------------------------------------------------------+

|  6       Input status                                          Character  |

+---------------------------------------------------------------------------+

|  7       Input flush                                           Character  |

+---------------------------------------------------------------------------+

|  8       Output (write)                                        Both       |

+---------------------------------------------------------------------------+

|  9       Output (write with verify)                            Block      |

+---------------------------------------------------------------------------+

| 10       Output status                                         Character  |

+---------------------------------------------------------------------------+

| 11       Output flush                                          Character  |

+---------------------------------------------------------------------------+

| 12       IOCtl output (called only if bit 14 of attribute is   Both       |

|          set to 1                                                         |

+---------------------------------------------------------------------------+

| 13       Device open (called only if bit 11 of attribute is    Both       |

|          set to 1)                                                        |

+---------------------------------------------------------------------------+

| 14       Device close (called only if bit 11 of attribute is   Both       |

|          set to 1)                                                        |

+---------------------------------------------------------------------------+

| 15       Removable media (called only if bit 11 of attribute   Block      |

|          is set to 1)                                                     |

+---------------------------------------------------------------------------+

| 19       Generic IOCtl Request (called                         Both       |

|          only if bit 6 of attribute is set to 1)                          |

+---------------------------------------------------------------------------+

| 23       Get logical device (called only if bit 6 of           Block      |

|          attribute is set to 1)                                           |

+---------------------------------------------------------------------------+

| 24       Set logical device (called only if bit 6 of           Block      |

|          attribute is set to 1)                                           |

+---------------------------------------------------------------------------+

INITIALIZATION REQUEST

COMMAND CODE = 0

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

|  Number of units (not set by character devices)                BYTE       |

+---------------------------------------------------------------------------+

| Ending address of resident program code                        DWORD      |

+---------------------------------------------------------------------------+

| Pointer to BPB array (not set by character devices) pointer    DWORD      |

| to remainder of arguments                                                 |

+---------------------------------------------------------------------------+

| Drive number                                                   BYTE       |

+---------------------------------------------------------------------------+

| CONFIG.SYS Error Message control flag                          WORD       |

+---------------------------------------------------------------------------+

The driver must do the following:

Set the number of units (block devices only).

Set up the pointer to the BPB array (block devices only).

Perform any initialization code (to modems, printers, and others).

Set the ending address of the resident program code.

Set the status word in the request header.

To obtain information passed from CONFIG.SYS to a device driver at

initialization time, the BPB pointer field points to a buffer containing the

information passed in CONFIG.SYS following the =.  This string may end with

either a carriage return (0DH) or a linefeed (0AH).  This information is

read-only.  Only system calls 01H-0CH and 30H can be issued by the

initialization code of the driver.

The last byte parameter contains the drive letter for the first unit of a

block driver.  For example, 0=A, 1=B and so forth.

If an initialization routine determines that it cannot set up the device and

wants to terminate without using any memory, use the following procedure:

Set the number of units to 0.

Set the ending address offset to 0.

Set the ending address segment to the code segment (CS).

If there are multiple device drivers in a single memory image file, the

ending address returned by the last initialization called is the one DOS 4.00

uses.  IBM recommends that all device drivers in a single memory image file

return the same ending address.

If initialization of your device driver fails, and you want the system to

display the ERROR IN CONFIG.SYS LINE # error message, set the CONFIG.SYS

error message control flag to a non-zero value.

MEDIA CHECK REQUEST

COMMAND CODE = 1

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

| Media descriptor from DOS 4.00                                 BYTE       |

+---------------------------------------------------------------------------+

| Return                                                         BYTE       |

+---------------------------------------------------------------------------+

| A pointer to the previous volume ID (if bit 11 = 1 and disk    DWORD      |

| change is returned)                                                       |

+---------------------------------------------------------------------------+

When the command code field is 1, DOS 4.00 calls Media Check for a drive unit

and passes its current Media Descriptor byte.  See "Media Descriptor Byte."

Media Check returns one of the following:

Media not changed

Media changed

Not sure

Error code.

The driver must perform the following:

Set the status word in the request header.

Set the return byte to:

-1   for "media changed"

0  for "not sure"

1  for "media not changed."

The driver uses the following method to determine how to set the return byte:

If the media is nonremovable (a fixed disk), set the return byte to 1.

If less than 2 seconds since last successful access, set the return byte

to 1.

If changeline not available, set the return byte to 0.

If changeline is available but not active, set the return byte to 1.

If the media byte in the new BPB does not match the old media byte, set

the return byte to -1.

If the current volume ID matches the previous volume ID, or if the serial

number matches the previous serial number, set the return byte to 0.

Media Descriptor Byte

Currently the media descriptor byte has been defined for some media types.

This byte should be identical to the media byte if the device has the non-IBM

format bit off.  These predefined values are:

Media descriptor

byte -->        1  1  1  1  1  x  x  x

bits -->         7  6  5  4  3  2  1  0

BIT   MEANING

0   1=2-sided         0=not 2-sided

1   1=8 sector        0=not 8 sector

2   1=removable       0=not removable

3-7   must be set to 1

NOTE:  An exception to the above is that the media descriptor byte value of

F0, which is used to indicate any media types not defined, and F9,

which is used for 5.25-inch media with 2 sides and 15 sectors/tracks.

Examples of current DOS 4.00 media descriptor bytes:

+---------------------------------------------------------------------------+

| DISK                                         #      SECTORS/   MEDIA      |

| TYPE                                         SIDES  TRACK      DESCRIPTOR |

+---------------------------------------------------------------------------+

| Fixed disk                                   --     --         F8H        |

+---------------------------------------------------------------------------+

| 5.25 inch                                    2      15         F9H        |

+---------------------------------------------------------------------------+

| 5.25 inch                                    1       9         FCH        |

+---------------------------------------------------------------------------+

| 5.25 inch                                    2       9         FDH        |

+---------------------------------------------------------------------------+

| 5.25 inch                                    1       8         FEH        |

+---------------------------------------------------------------------------+

| 5.25 inch                                    2       8         FFH        |

+---------------------------------------------------------------------------+

| 8 inch                                       1      26         FEH        |

+---------------------------------------------------------------------------+

| 8 inch                                       2      26         FDH        |

+---------------------------------------------------------------------------+

| 8 inch                                       2       8         FEH        |

+---------------------------------------------------------------------------+

| 3.5 inch                                     2       9         F9H        |

+---------------------------------------------------------------------------+

| 3.5 inch                                     2      18         F0H        |

+---------------------------------------------------------------------------+

To determine whether you are using a single-sided or a double-sided diskette,

attempt to read the second side.  If an error occurs, you may assume the

diskette is single-sided.  Media descriptor F0H may be used for those media

types not described earlier.  Programs should not use the media descriptor

values to distinguish media.  DOS 4.00 internal routines use information in

the BIOS parameter block (BPB) to determine the media type of IBM-formatted

diskettes.  These media descriptor bytes do not necessarily indicate a unique

media type.

For 8-inch diskettes:

FEH (IBM 3740 Format) -- Single-sided, single-density, 128 bytes per

sector, soft-sectored, 4 sectors per allocation unit, 1 reserved sector,

2 FATs, 68 directory entries, 77*26 sectors.

FDH (IBM 3740 Format) -- Double-sided, single-density, 128 bytes per

sector, soft-sectored, 4 sectors per allocation unit, 4 reserved sectors,

2 FATs, 68 directory entries, 77*26*2 sectors.

FEH -- Double-sided, double-density, 1024 bytes per sector, soft

sectored, 1 sector per allocation unit, 1 reserved sector, 2 FATs, 192

directory entries, 77*8*2 sectors.

BUILD BPB REQUEST

COMMAND CODE = 2

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

| Media descriptor from DOS 4.00                                 BYTE       |

+---------------------------------------------------------------------------+

| Transfer address (buffer address)                              DWORD      |

+---------------------------------------------------------------------------+

| Pointer to BPB table                                           DWORD      |

+---------------------------------------------------------------------------+

DOS 4.00 calls Build BPB (BIOS Parameter Block) under the following two

conditions:

If "Media Changed" is returned

If "Not Sure" is returned, there are no used buffers.  Used buffers are

buffers with changed data not yet written to the disk.

The driver must do the following:

Set the pointer to the BPB

Set the status word in the request header.

The device driver must determine the media type that is in the unit to return

the pointer to the BPB table.  In previous versions of IBMBIO, the FAT ID

byte determined the structure and layout of the media.  The FAT ID byte has

only eight possible values (F8 through FF), so, as new media types are

invented, the available values will soon be exhausted.  With the varying

media layouts, DOS 4.00 needs to be aware of the location of the FATs  and

directories before it asks to read them.

The following paragraphs explain the method DOS 4.00 uses to determine the

media type.

The information relating to the BPB for a particular media is kept in the

boot sector for the media.  The format of the boot sector is as follows:

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| A 2-byte short JMP instruction (EBH), followed by a NOP        WORD       |

| instruction (90H).                                                        |

+---------------------------------------------------------------------------+

| Product name and version                                       8 BYTES    |

+---------------------------------------------------------------------------+

| Bytes per sector; must be power of 2                           WORD       |

+---------------------------------------------------------------------------+

| Sectors per allocation unit; must be power of 2                BYTE       |

+---------------------------------------------------------------------------+

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Reserved sectors starting at logical sector 0                  WORD       |

+---------------------------------------------------------------------------+

| Number of FATs                                                 BYTE       |

+---------------------------------------------------------------------------+

| Maximum number of root directory entries                       WORD       |

+---------------------------------------------------------------------------+

| Total number of sectors in media including the boot sector,    WORD       |

| FAT areas, and directories                                                |

+---------------------------------------------------------------------------+

| Media descriptor                                               BYTE       |

+---------------------------------------------------------------------------+

| Number of sectors occupied by a FAT                            WORD       |

+---------------------------------------------------------------------------+

| Sectors per track                                              WORD       |

+---------------------------------------------------------------------------+

| Number of heads                                                WORD       |

+---------------------------------------------------------------------------+

| Number of hidden sectors                                       WORD       |

+---------------------------------------------------------------------------+

The BPB information contained in the boot sector starts with the Bytes per

Sector entry.  The last three words are intended to help the device driver

identify the media.  The number of heads is useful for supporting different

multihead drives with the same storage capacity but a different number of

surfaces.  The number of hidden sectors is useful for supporting drive

partitioning schemes.

For drivers that support volume identification and disk change, this call

causes a new volume identification to be read from the disk.  This call

indicates that the disk has changed in a permissible manner.

To handle the partition that is bigger than 32MB, or one that starts beyond

or crosses the 32MB boundary, DOS 4.00 defines an extended BPB structure.

Depending on the size of the media, you can use either the existing BPB or

the extended one, which contains an additional DWORD field to indicate the

size of the partition in sectors.

Bit 1 of the attribute field in the block device driver header indicates

whether the device can process 32-bit sector numbers.  Set bit 1 to indicate

32-bit support.

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Bytes per sector                                               WORD       |

+---------------------------------------------------------------------------+

| Sectors per allocation unit                                    BYTE       |

+---------------------------------------------------------------------------+

| Reserved sectors starting at logical sector 0                  WORD       |

+---------------------------------------------------------------------------+

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Number of FATs -- 0 if not a FAT system                        BYTE       |

+---------------------------------------------------------------------------+

| Maximum number of root directory entries                       WORD       |

+---------------------------------------------------------------------------+

| Total number of sectors in the media.  This field is used to   WORD       |

| define a partition that is less than 32MB.  Setting this                  |

| field to 0 indicates to use the total (32-bit) number of                  |

| sectors in the media below.                                               |

+---------------------------------------------------------------------------+

| Media descriptor                                               BYTE       |

+---------------------------------------------------------------------------+

| Number of sectors occupied by a FAT                            WORD       |

+---------------------------------------------------------------------------+

| Sectors per track                                              WORD       |

+---------------------------------------------------------------------------+

| Number of heads                                                WORD       |

+---------------------------------------------------------------------------+

| Number of hidden sectors                                       DWORD      |

+---------------------------------------------------------------------------+

| Total (32-bit) number of sectors in the media. This field is   DWORD      |

| used to define a partition that is greater than 32MB, or one              |

| that crosses the 32MB boundary.                                           |

+---------------------------------------------------------------------------+

The extended BPB is a superset of the traditional BPB structure.  To achieve

the maximum compatibility between this structure and that of the traditional

BPB, DOS 4.00 uses the following rule:

If (the number of hidden sectors plus the total number of sectors in the

media) is greater than 64KB, use the 32-bit total number of sectors in

the media entry (DWORD).

Otherwise, use the TOTAL NUMBER OF SECTORS IN THE MEDIA entry (WORD).

A boot record exists at the beginning of each disk partition and each

extended DOS 4.00 partition volume.  DOS 4.00 automatically creates the

extended boot record.  The format of the extended boot record is:

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| A 2-byte short JMP instruction (EBH) followed by a NOP         WORD       |

| instruction (90H).                                                        |

+---------------------------------------------------------------------------+

| Product name and version                                       8 BYTES    |

+---------------------------------------------------------------------------+

| Extended BPB                                                   25 BYTES   |

+---------------------------------------------------------------------------+

| Physical drive number                                          BYTE       |

+---------------------------------------------------------------------------+

| Reserved                                                       BYTE       |

+---------------------------------------------------------------------------+

| Extended boot record signature                                 BYTE       |

+---------------------------------------------------------------------------+

| Volume serial number                                           DWORD      |

+---------------------------------------------------------------------------+

| Volume label                                                   11 BYTES   |

+---------------------------------------------------------------------------+

| Reserved                                                       8 BYTES    |

+---------------------------------------------------------------------------+

NOTE:  The value of Extended boot record signature is 29H.  The value of the

physical drive number is always 0H or 80H.

On all requests to extended drivers with a sector number in their request

headers, the sector number is a DWORD.  The standard DOS 4.00 block device

drivers set the attribute bit 1 for 32-bit support.

For each call to a device driver, DOS 4.00 checks to see if the starting

sector number passed in the request can be supported by the device driver.

If this value is greater than 64K for an old-style device driver, DOS 4.00

returns an UNKNOWN MEDIA (07H) device driver error.

INPUT AND OUTPUT REQUESTS

COMMAND CODES = 3, 4, 8, 9, 12

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

| Media descriptor byte                                          BYTE       |

+---------------------------------------------------------------------------+

| Transfer address (buffer address)                              DWORD      |

+---------------------------------------------------------------------------+

| Byte/sector count                                              WORD       |

+---------------------------------------------------------------------------+

| Starting sector number (If -1, use DWORD starting sector       WORD       |

| number.  This entry has no meaning for a character device.)               |

+---------------------------------------------------------------------------+

| For DOS 3.0 to DOS 4.00, pointer to the volume identification  DWORD      |

| if error code 0FH is returned                                             |

+---------------------------------------------------------------------------+

| Starting 32-bit sector number.  (Use this entry to the block   DWORD      |

| device driver with the attribute bit 1 set.)                              |

+---------------------------------------------------------------------------+

The DOS 4.00 INPUT/OUTPUT request structure can process 32-bit sector

numbers, providing support for media of more than 4 billion sectors.

The driver must do the following:

Set the status word in the request header

Perform the requested function

Set the actual number of sectors or bytes transferred.

No error checking is performed on an IOCtl call.  However, the driver must

set the return sector or byte count to the actual number of bytes

transferred.

Under certain circumstances the block device driver may be asked to do a

WRITE operation of 64KB that seems to be a wraparound of the transfer address

in the device driver request packet.  This occurs because an optimization is

added to the WRITE code in DOS 4.00.  It will only happen on WRITEs that are

within a sector size of 64KB on files that are being extended past the

current end of file.  The block device driver is allowed to ignore the

balance of the WRITE that wraps around.  For example, a WRITE of 10000H bytes

of sectors with a transfer address of XXXX:1 ignores the last two bytes.

Remember that a program using DOS 4.00 function calls cannot request an input

or output operation of more than FFFFH bytes because a wrap around in the

transfer buffer segment must occur.  You can ignore bytes that would have

wrapped around in the transfer segment.

If the driver returns an error code of 0FH (Invalid Disk Change), it must

provide a DWORD pointer to an ASCIIZ string identifying the correct volume ID

and the system prompts the user to reinsert the disk.

The reference count of open files on the disk maintained by OPEN and CLOSE

calls allows the driver to determine when to return error 0FH.  If there are

no open files (reference count=0) and the disk has been changed, the I/O is

valid, and error 0FH is not returned.  If there are open files (reference

count > 0) and the disk has been changed, an error 0FH situation may exist.

DOS 4.00 IBMDOS.COM will request an OPEN or CLOSE function only if SHARE is

loaded.

NONDESTRUCTIVE INPUT NO WAIT REQUEST

COMMAND CODE = 5

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

| Byte read from device                                          BYTE       |

+---------------------------------------------------------------------------+

The driver must do the following:

Return a byte from the device.

Set the status word in the request header.

If the character device returns busy bit = 0, meaning there are characters in

buffer, the next character that would be read is returned.  This character is

not removed from the input buffer, (that is, nondestructive input).  This

call allows DOS 4.00 to look ahead one input character.

CHARACTER INPUT AND OUTPUT STATUS REQUESTS

COMMAND CODES = 6, 10

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

The driver must do the following:

Perform the requested function.

Set the busy bit.

Set the status word in the request header.

The busy bit is set differently for output and input.

Output on Character Devices

If the busy bit is 1 on return, a write request would wait for completion of

a current request.  If the busy bit is 0, no request is waiting or running.

Therefore, a write request would start immediately.

Input on Character Devices with a Buffer

If the busy bit is 1 on return, a read request goes to the physical device.

If the busy bit is 0, characters are in the device buffer and a read returns

quickly.  This also indicates that the user has typed something.  DOS 4.00

assumes that all character devices have a type-ahead input buffer.  Devices

that do not have this buffer should always return busy = 0 so that DOS 4.00

does not loop endlessly, waiting for information to be put in a buffer that

does not exist.

CHARACTER INPUT AND OUTPUT FLUSH REQUESTS

COMMAND CODES = 7, 11

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

This call tells the driver to flush (terminate) all pending requests of which

it has knowledge.  Its primary use is to flush the input queue on character

devices.

The driver must set the status word in the Request Header upon return.

OPEN AND CLOSE REQUESTS

COMMAND CODES = 13, 14

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

These calls are designed to give the device information about current file

activity on the device if bit 11 of the attribute word is set.

On block devices, these calls can be used to manage local buffering.  The

device can keep a reference count.  Every OPEN increases the reference count.

Every CLOSE decreases the device reference count.  When the reference count

is 0, there are no open files on the device.  Therefore, the device should

flush buffers inside the device to which it has written because the user can

change the media on a removable media drive.  If the media has been changed,

reset the reference count to 0 without flushing the buffers.

These calls are more useful on character devices.  The OPEN call can send a

device an initialization string.  On a printer, the call can send a string to

set the font or the page size so the printer is always in a known state at

the start of an I/O stream.

Similarly, the CLOSE call can send a post string, such as a form feed, at the

end of an I/O stream.

Using IOCtl to set the preliminary and ending strings provides a flexible

mechanism for serial I/O device stream control.

REMOVABLE MEDIA REQUEST

COMMAND CODE = 15

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

To use this call, set bit 11 of the attribute field to 1.  Block devices can

use this call only by way of a subfunction of the IOCtl function call (44H).

This call is useful because it notifies a utility if it is dealing with a

removable or nonremovable media drive.  For example, the FORMAT utility needs

to know whether a drive is removable or nonremovable because it displays

different versions of some prompts.

The information is returned in the busy bit of the status word.  If the busy

bit is 1, the media is nonremovable.  If the busy bit is 0, the media is

removable.

No error bit checking is performed.  It is assumed that this call always

succeeds.

GENERIC IOCTL REQUEST

COMMAND CODE = 19

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

| Major function                                                 BYTE       |

+---------------------------------------------------------------------------+

| Minor function                                                 BYTE       |

+---------------------------------------------------------------------------+

| Contents of SI                                                 WORD       |

+---------------------------------------------------------------------------+

| Contents of DI                                                 WORD       |

+---------------------------------------------------------------------------+

| Pointer to Generic IOCTL request packet                        DWORD      |

+---------------------------------------------------------------------------+

The driver must:

Support the functions described under Generic IOCtl request

Maintain its own track table (TrackLayout).

See Appendix C, "I/O Control for Devices (IOCtl)" on page C-1 for a

description of the functions provided by generic IOCtl requests.

GET LOGICAL DEVICE REQUEST

COMMAND CODE = 23

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

| Input (unit code)                                              BYTE       |

+---------------------------------------------------------------------------+

| Command code                                                   BYTE       |

+---------------------------------------------------------------------------+

| Status                                                         WORD       |

+---------------------------------------------------------------------------+

| Reserved                                                       DWORD      |

+---------------------------------------------------------------------------+

SET LOGICAL DEVICE REQUEST

COMMAND CODE = 24

+---------------------------------------------------------------------------+

| FIELD                                                          LENGTH     |

+---------------------------------------------------------------------------+

| Request header                                                 13-BYTE    |

+---------------------------------------------------------------------------+

| Input (unit code)                                              BYTE       |

+---------------------------------------------------------------------------+

| Command code                                                   BYTE       |

+---------------------------------------------------------------------------+

| Status                                                         WORD       |

+---------------------------------------------------------------------------+

| Reserved                                                       DWORD      |

+---------------------------------------------------------------------------+

CLOCK$ DEVICE DRIVER EXAMPLE

This feature is a "Real-Time Clock" Board.  To allow this board to be

integrated into the system for TIME and DATE, a special device driver is

specified by an attribute word, the CLOCK$ device.  This device driver

defines and performs functions like any other character device driver.  Most

functions will be set done bit, reset error bit, return.  When a read or

write to this device occurs, 6 bytes are transferred.  The first 2 bytes are

a word, which is the count of days since 1-1-80.  The third byte is minutes;

the fourth is hours; the fifth is hundredths of seconds; and the sixth is

seconds.  Reading the CLOCK$ device gets the date and time; writing to it

sets the date and time.

PART 3.  APPENDIXES