APPENDIX A.  DOS 4.00 INTERRUPTS 13

20H Program Terminate 21

21H Function Request 41

22H Terminate Address 46

23H Ctrl-Break Exit Address 53

24H Critical Error Handler Vector 83

25H/26H Absolute Disk Read/Write 287

27H Terminate but Stay Resident 353

28H-2EH Reserved for DOS 4.00 392

2FH Multiplex Interrupt 396

30H-3FH Reserved for DOS 4.00 695

APPENDIX A.  DOS 4.00 INTERRUPTS

This chapter contains information to support use of the DOS 4.00 interrupts.

DOS 4.00 reserves interrupt types 20H to 3FH for its use.  Absolute memory

locations 80H to FFH are reserved by DOS 4.00.  All interrupt values are in

hexadecimal.

20H PROGRAM TERMINATE

Issue interrupt 20H to exit from a program.  This vector transfers to the

logic in DOS 4.00 to restore the terminate address, the Ctrl-Break address,

and the critical error exit address to the values they had on entry to the

program.  All file buffers are flushed and all handles are closed.  You

should close all files changed in length (see function call 10H and 3EH)

before issuing this interrupt.  If the changed file is not closed, its

length, date, and time are not recorded correctly in the directory.

For a program to pass a completion code or an error code when terminating, it

must use either function call 4CH (Terminate a Process) or 31H (Terminate

Process and Stay Resident).  These two methods are preferred over using

interrupt 20H, and the codes returned by them can be interrogated in batch

processing.  See function call 4CH for information on the ERRORLEVEL

subcommand of batch processing.

IMPORTANT: Before you issue interrupt 20H, your program must ensure that the

CS register contains the segment address of its program segment prefix.

21H FUNCTION REQUEST

Refer to each function call issued within 21H in Appendix B, "DOS 4.00

Function Calls" on page B-1.

22H TERMINATE ADDRESS

Control transfers to the address at this interrupt location when the program

terminates.  This address is copied into the program's Program Segment Prefix

at the time the segment is created.  You should not issue this interrupt; the

EXEC function call does this for you.

23H CTRL-BREAK EXIT ADDRESS

If the user presses the Ctrl and Break keys during standard input, standard

output, standard printer, or asynchronous communications adapter operations,

an interrupt 23H is executed.  If BREAK is on, the interrupt 23H is checked

on most function calls (except calls 06H and 07H).  If the user-written

Ctrl-Break routine saves all registers, it may end with a

return-from-interrupt instruction (IRET) to continue program execution.

If the user-written interrupt program returns with a long return, the carry

flag is used to determine whether the program will be ended.  If the carry

flag is set, the program is ended, otherwise execution continues (as with a

return by IRET).

If the user-written Ctrl-Break interrupt uses function calls 09H or 0AH, then

&hat.C, carriage-return and linefeed are output.  If execution is continued

with an IRET, I/O continues from the start of the line.

When the interrupt occurs, all registers are set to the value they had when

the original function call to DOS 4.00 was made.  There are no restrictions

on what the Ctrl-Break handler is allowed to do, including DOS 4.00 function

calls, as long as the registers are unchanged if IRET is used.

When the program creates a new segment and loads in a second program it

changes the Ctrl-Break address.  The termination of the second program and

return to the first causes the Ctrl-Break address to be restored to the value

it had before execution of the second program.  It is restored from the

second program's Program Segment Prefix.  You should not issue this

interrupt.

24H CRITICAL ERROR HANDLER VECTOR

A critical error is returned when a DOS function cannot be performed.  This

error is frequently caused by a hardware condition, such as the printer being

out of paper, a diskette drive door open, or a diskette out of space.  When a

critical error occurs within DOS 4.00, control is transferred with an

interrupt 24H.  On entry to the error handler, AH will have its bit 7=0

(high-order bit) if the error was a disk error (the most common occurrence),

bit 7=1 if it was not.

BP:SI contains the address of a Device Header Control Block from which

additional information can be retrieved.  See page A-5.

The registers are set up for a retry operation, and an error code is in the

lower half of the DI register with the upper half undefined.  The error codes

follow:

ERROR CODE  MEANING

0           Attempt to write on write-protected diskette

1           Unknown unit

2           Drive not ready

3           Unknown command

4           Data error (CRC)

5           Bad request structure length

6           Seek error

7           Unknown media type

8           Sector not found

9           Printer out of paper

A           Write fault

B           Read fault

C           General failure

The user stack is in effect and contains the following from top to bottom:

IP        DOS 4.00 registers from issuing INT 24H

CS

FLAGS

AX        User registers at time of original

BX        INT 21H request

CX

DX

SI

DI

BP

DS

ES

IP        From the original interrupt 21H

CS        from the user to DOS 4.00

FLAGS

The registers are set such that if an IRET is executed, DOS 4.00 responds

according to (AL) as follows:

(AL)  =0  ignore the error.

=1  retry the operation.

=2  terminate the program

through interrupt 22H.

=3  fail the system call

in progress.

NOTE:  Be careful when choosing ignore as a response because this causes DOS

4.00 to believe that an operation has completed successfully when it

may not have.

To return control from the critical error handler to a user error routine,

the following should occur:

Before an INT 24H occurs:

1.  The user application initialization code should save the INT 24H vector

and replace the vector with one pointing to the user error routine.

When the INT 24H occurs:

2.  When the user error routine receives control, it should push the flag

register onto the stack, and then execute a CALL FAR to the original INT

24H vector saved in step 1.

3.  DOS 4.00 gives the appropriate prompt, and waits for the user input

(Abort, Retry, Fail or Ignore).  After the user input, DOS 4.00 returns

control to the user error routine at the instruction following the CALL

FAR.

4.  The user error routine can now do any tasks necessary.  To return to the

original application at the point the error occurred, the error routine

needs to execute an IRET instruction.  Otherwise, the user error routine

should remove the IP, CS, and Flag registers from the stack.  Control can

then be passed to the desired point.

Disk Errors

If it is a hard error on disk (AH bit 7=0), register AL contains the failing

drive number (0 = drive A, and so on).  AH bits 0-2 indicate the affected

disk area and whether it was a read or write operation, as follows:

Bit 0=0 if read operation,

1 if write operation

Bits 2-1  (affected disk area)

0 0  DOS 4.00 area

0 1  file allocation table

1 0  directory

1 1  data area

AH bits 3-5 indicate which responses are valid.  They are:

Bit 3=0 if FAIL is not allowed

=1 if FAIL is allowed

Bit 4=0 if RETRY is not allowed

=1 if RETRY is allowed

Bit 5=0 if IGNORE is not allowed

=1 if IGNORE is allowed

Handling of Invalid Responses

If IGNORE is specified (AL=0) and IGNORE is not allowed (bit 5=0), make the

response FAIL (AL=3).

If RETRY is specified (AL=1) and RETRY is not allowed (bit 4=0), make the

response FAIL (AL=3).

If FAIL is specified (AL=3) and FAIL is not allowed (bit 3=0), make the

response END (AL=2).

Other Errors

If AH bit 7=1, the error occurred on a character device, or was the result of

a bad memory image of the FAT.  The device header passed in BP:SI can be

examined to determine which case exists.  If the attribute byte high-order

bit indicates a block device, then the error was a bad FAT.  Otherwise, the

error is on a character device.

If a character device is involved, the contents of AL are unpredictable and

the error code is in DI as above.

NOTES:

1.  Retry five times before giving this routine control for disk errors.

When the errors are in the FAT or a directory, retry three times.

2.  For disk errors, this exit is taken only for errors occurring during an

interrupt 21H function call.  It is not used for errors during an

interrupt 25H or 26H.

3.  This routine is entered in a disabled state.

4.  All registers must be preserved.

5.  This interrupt handler should refrain from using DOS 4.00 function calls.

If necessary, it may use calls 01H through 0CH, 30H, and 59H.  Use of any

other call destroys the DOS 4.00 stack and leaves DOS 4.00 in an

unpredictable state.

6.  The interrupt handler must not change the contents of the device header.

7.  If the interrupt handler handles errors itself rather than returning to

DOS 4.00, it should restore the application program's registers from the

stack, remove all but the last three words on the stack, then issue an

IRET.  This will return to the program immediately after the INT 21H that

experienced the error.  Note that if this is done, DOS 4.00 will be in an

unstable state until a function call higher than 0CH is issued;

therefore, it is not recommended.

The recommended way to end a critical error is to use FAIL and then test

the extended error code of the INT 21H.

8.  IGNORE requests (AL=0) are converted to FAIL for critical errors that

occur on FAT or DIR sectors.

9.  Refer to "Responding to Errors" on page B-5 and "Extended Error Codes" on

page B-5 for information on how to obtain additional error information.

10. For DOS 4.00, IGNORE requests (AL=0) are converted to FAIL requests for

certain critical errors (50-79).

The device header pointed to by BP:SI is formatted as follows:

+---------------------------------------------------------------------------+

| DWORD Pointer to next device (FFFFH if last device)                       |

+---------------------------------------------------------------------------+

| WORD Attributes:                                                          |

|    Bit 15  = 1 if character device                                        |

|             = 0 if block device                                           |

|    If bit 15 is 1:                                                        |

|        Bit 0 = 1 if current standard input                                |

|        Bit 1 = 1 if current standard output                               |

|        Bit 2 = 1 if current NULL device                                   |

|        Bit 3 = 1 if current CLOCK device                                  |

|    Bit 14 is the IOCtl bit                                                |

+---------------------------------------------------------------------------+

| WORD pointer to device driver strategy entry point.                       |

+---------------------------------------------------------------------------+

| WORD pointer to device driver interrupt entry point.                      |

+---------------------------------------------------------------------------+

| 8-BYTE character device named field for block devices.  The first byte is |

| the number of units.                                                      |

+---------------------------------------------------------------------------+

To tell if the error occurred on a block or character device, look at bit 15

in the attribute field (WORD at BP:SI+4).

If the name of the character device is desired, look at the eight bytes

starting at BP:SI+10.

25H/26H ABSOLUTE DISK READ/WRITE

Interrupt vectors 25H and 26H transfer control to the device driver.  They

have been extended to allow direct access to media greater than 32MB in size.

Their use of the CX register is what distinguishes them from the conventional

25H and 26H interrupts.  Note that if the conventional format parameters are

used in an attempt to access media greater than 32MB, an error code of 0207H

is returned in AX.

The request for extended 25H or 26H is:

MOV     AL,DRIVE       ; Drive number to process

; 0 = A

; 1 = B

; 2 = C . . .

LDS     BX,PACKET      ; Parameter list

MOV     CX,-1          ; Indicates extended format

INT     25H or 26H     ; Issue request to DOS 4.00

POP     AX             ; Discard stack word

JC      ERROR          ; Error code returned in AX

PACKET  LABEL   BYTE           ; Control packet

DD      RBA            ; RBA of first sector

; (0 origin)

DW      COUNT          ; Number of sectors to I/O

DD      BUFFER         ; Data buffer

On return, the original flags are still on the stack (put there by the INT

instruction).  This is necessary because return information is passed back in

the current flags.  Be sure to pop the stack to prevent uncontrolled growth.

WARNING:   IF DISK I/O HANDLED BY THIS INTERRUPT EXCEEDS THE LIMIT IMPOSED BY

THE 64KB DIRECT MEMORY ACCESS BOUNDARY, UNPREDICTABLE RESULTS CAN OCCUR.  WE

RECOMMEND YOU CAREFULLY CHECK THE SECTOR SIZE AND THE NUMBER OF SECTORS TO BE

READ OR WRITTEN TO BEFORE ISSUING THIS CALL.

The number of sectors specified is transferred between the given drive and

the transfer address.  Logical sector numbers (LSN) are obtained by numbering

each sector sequentially starting from track 0, head 0, sector 1 (logical

sector 0) and continuing along the same head, then to the next head until the

last sector on the last head of the track is counted.  Thus, logical sector 1

is track 0, head 0, sector 2; logical sector 2 is track 0, head 0, sector 3;

and so on.  Numbering continues with sector 1 on head 0 of the next track.

Note that although the sectors are sequentially numbered (for example,

sectors 2 and 3 on track 0 in the example above), they may not be physically

adjacent on the disk, because of interleaving.  Note that the mapping is

different from that used by DOS version 1.10 for dual-sided diskettes.

All registers except the segment registers are destroyed by this call.  If

the transfer was successful, the carry flag (CF) is 0.  If the transfer was

not successful CF=1 and (AX) indicate the error as follows.  (AL) is the DOS

4.00 error code that is the same as the error code returned in the low byte

of DI when an interrupt 24H is issued, and (AH) contains:

80H     Attachment failed to respond

40H     SEEK operation failed

08H     Bad CRC on diskette read

04H     Requested sector not found

03H     Write attempt on write-protected diskette

02H     Error other than types listed above

WARNING:   BEFORE ISSUING THIS INTERRUPT TO REMOVABLE MEDIA, THE MEDIA IN THE

DRIVE MUST BE ESTABLISHED CORRECTLY.  THIS CAN BE ACCOMPLISHED BY ISSUING

EITHER AN INT 21H GENERIC IOCTL (AH=44H), WITH A REQUEST TO RETURN THE BPB

THAT BUILD BPB RETURNS; OR AN INT 21H GET CURRENT DIRECTORY (AH=47H).

27H TERMINATE BUT STAY RESIDENT

This vector is used by programs that are to remain resident when COMMAND.COM

regains control.

DOS 4.00 function call 31H is the preferred method to cause a program to

remain resident, because this allows return information to be passed.  It

allows a program larger than 64KB to remain resident.  After initializing

itself, the program must set DX to its last address plus one, relative to the

program's initial DS or ES value (the offset at which other programs can be

loaded), and then execute an interrupt 27H.  DOS 4.00 then considers the

program as an extension of itself, so the program is not overlaid when other

programs are executed.  This concept is useful for loading programs such as

user-written interrupt handlers that must remain resident.

NOTES:

1.  This interrupt must not be used by .EXE programs that are loaded into the

high end of memory.

2.  This interrupt restores the interrupt 22H, 23H, and 24H vectors in the

same manner as interrupt 20H.  Therefore, it cannot be used to install

permanently resident Ctrl-Break or critical error handler routines.

3.  The maximum size of memory that can be made resident by this method is

64KB.

4.  Memory can be used more efficiently if the block containing a copy of the

environment is deallocated before terminating.  This can be done by

loading ES with the segment contained in 2C of the PSP, and issuing

function call 49H (Free Allocated Memory).

5.  DOS 4.00 function call 4CH allows the terminating program to pass a

completion (or error) code to DOS 4.00, which can be interpreted within

batch processing (see function call 31H).

6.  Terminate-but-stay-resident function calls do not automatically close

files.

28H-2EH RESERVED FOR DOS 4.00

These interrupts are reserved for DOS 4.00 use.

2FH MULTIPLEX INTERRUPT

Interrupt 2FH is the multiplex interrupt.  A general interface is defined

between two processes.  The specific application using interrupt 2FH defines

specific functions and parameters.

Every multiplex interrupt handler is assigned a specific multiplex number.

The multiplex number is specified in the AH register.  The specific function

that the handler is to perform is specified in the AL register.  Other

parameters are placed in the other registers, as needed.  The handlers are

chained into the interrupt 2FH interrupt vector.  There is no defined method

for assigning a multiplex number to a handler.  You must pick one.  To avoid

a conflict if two applications choose the same multiplex number, the

multiplex numbers used by an application should be patchable.

The multiplex numbers AH=00H through BFH are reserved for DOS 4.00.

Applications should use multiplex numbers C0H through FFH.

NOTE:  When in the chain for interrupt 2FH, if your code calls DOS 4.00 or if

you execute with interrupts enabled, your code must be reentrant and

recursive.

Function Codes

AH=1

AH=1 is the resident part of PRINT.

The following table contains the function codes that you can specify in AL to

request the resident portion of print to perform a specific function:

+---------------------------------------------------------------------------+

| FUNCTION                                                                  |

| CODES       DESCRIPTION                                                   |

+---------------------------------------------------------------------------+

| 0           Get installed state                                           |

+---------------------------------------------------------------------------+

| 1           Submit file                                                   |

+---------------------------------------------------------------------------+

| 2           Cancel file                                                   |

+---------------------------------------------------------------------------+

| 3           Cancel all files                                              |

+---------------------------------------------------------------------------+

| 4           Status                                                        |

+---------------------------------------------------------------------------+

| 5           End of status                                                 |

+---------------------------------------------------------------------------+

Print Error Codes

The following table contains the error codes that are returned from the

resident portion of print.

+---------------------------------------------------------------------------+

| ERROR                                                                     |

| CODES       DESCRIPTION                                                   |

+---------------------------------------------------------------------------+

|  1          Invalid function                                              |

+---------------------------------------------------------------------------+

|  2          File not found                                                |

+---------------------------------------------------------------------------+

|  3          Path not found                                                |

+---------------------------------------------------------------------------+

|  4          Too many open files                                           |

+---------------------------------------------------------------------------+

|  5          Access denied                                                 |

+---------------------------------------------------------------------------+

|  6          Queue full                                                    |

+---------------------------------------------------------------------------+

|  9          Busy                                                          |

+---------------------------------------------------------------------------+

| 12          Name too long                                                 |

+---------------------------------------------------------------------------+

| 15          Invalid drive                                                 |

+---------------------------------------------------------------------------+

AL=0  Get Installed State

This call must be defined by all interrupt 2FH handlers.  It is used by the

caller of the handler to determine if the handler is present.  On entry,

AL=0, AH=1.  On return, AL contains the installed state as follows:

AL=0   Not installed, permissible to install

AL=1   Not installed, not permissible to install

AL=FF  Installed

AL=1  Submit File

On entry, AL=1, AH=1, and DS:DX points to the submit packet.  A submit packet

contains the level (BYTE) and a pointer to the ASCIIZ string (DWORD in offset

segment form).  The level value for DOS 4.00 is 0.  The ASCIIZ string must

contain the drive, path, and filename of the file you want to print.  The

filename cannot contain global filename characters.  AL=2  Cancel File

On entry, AL=2, AH=1, and DS:DX points to the ASCIIZ string for the print

file you want to cancel.  Global filename characters are allowed in the

filename.  AL=3  Cancel all Files

On entry, AL=3 and AH=1.

AL=4  Status

This call holds the jobs in the print queue so that you can scan the queue.

Issuing any other code releases the jobs.  On entry, AL=4, AH=1.  On return,

DX contains the error count.  The error count is the number of consecutive

failures PRINT had while trying to output the last character.  If there are

no failures, the number is 0.  DS:SI points to the print queue.  The print

queue consists of a series of filename entries.  Each entry is 64 bytes long.

The first entry in the queue is the file currently being printed.  The end of

the queue is marked by a queue entry having a null as the first character.

AL=5  End of Status:  Issue this call to release the queue from call 4.  On

entry, AL=5 and AH=1.  On return, AX contains the error codes.  For

information on the error codes returned, refer to "Print Error Codes" on

page A-10.

AL=F8-FF  Reserved by DOS 4.00

AH=6

AH=6 is the resident part of ASSIGN.

The Get Installed State function

(AL=0) is supported.

AH=8H, 10H, 12H, 13H Reserved by DOS 4.00

AH=B7H

AH=B7H is the resident part of APPEND.

The Get Installed State function

(AL=0) is supported.

AL=2 Get APPEND version

This call is for distinguishing between

the PC LAN APPEND and the DOS 4.00 APPEND.

On return, if AX=FFFFH then the DOS 4.00

APPEND is loaded.

AL=4 Get APPEND Path Pointer

(DOS 4.00 APPEND only)

On return ES:DI points to the

currently active APPEND path.

AL=6 Get APPEND Function State

(DOS 4.00 APPEND only)

BX is returned with bits set indicating

if APPEND is currently enabled and what

functions are in effect.

+--------+------------------------------------------------------------------+

| BIT    | FUNCTION IN EFFECT IF BIT IS ON                                  |

+--------+------------------------------------------------------------------+

| 0      | APPEND enabled                                                   |

+--------+------------------------------------------------------------------+

| 13     | /PATH                                                            |

+--------+------------------------------------------------------------------+

| 14     | /E                                                               |

+--------+------------------------------------------------------------------+

| 15     | /X                                                               |

+--------+------------------------------------------------------------------+

NOTE:  The functions in effect do not change whether or not APPEND is

disabled.

AL=7 Set function state (DOS 4.00 APPEND

only)

On input BX is the new setting for all functions.

The suggested procedure is to get the current function

state, turn on or turn off the desired bits, then

use this call to set the function state.

AL=11H Set Return Found Name State (DOS 4.00 APPEND

only)

On request AL=17, a process system state flag is set.  If this flag is set,

then on the next ASCIIZ 3DH, 43H or 6CH function call within Interrupt 21H

that APPEND processes, APPEND returns the name it finds to the application

filename buffer.  This name may be different from the one the application

offered.  The application must provide enough space for the found name.

After APPEND has processed an

Interrupt 21H, it resets the Return Found Name state.  An example of this

process follows:

MOV     AH,0B7H         ; Indicate APPEND

MOV     AL,0            ; Get installed state

INT     2FH

CMP     AL,0            ; APPEND installed?

JE      NOT_INSTALLED

MOV     AH,0B7H         ; Indicate APPEND

MOV     AL,2            ; Get APPEND version

INT     2FH

CMP     AX,-1           ; DOS version?

PC_LAN_APPEND   ; AX<> -1 means PC LAN

JNE

APPEND

; The following functions are valid only if DOS 4.00 APPEND

MOV     AH,0B7H         ; Indicate APPEND

MOV     AL,4            ; Get APPEND path pointer

INT     2FH

; ES:DI = address of APPEND path

; (Buffer is 128 characters long)

MOV     AH,0B7H         ; Indicate APPEND

MOV     AL,6            ; Get function state

INT     2FH

; BX = function state

; 8000H = /X is on

; 4000H = /E is on

; 2000H = /PATH is on

; 0001H = APPEND enabled

; If off, similar to null

; APPEND path

; Set on by any non-status

; occurrence of APPEND

MOV     AH,0B7H         ; Indicate APPEND

MOV     AL,7            ; Set function state

MOV     BX,state        ; New state

INT     2FH

MOV     AH,0B7H         ; Indicate APPEND

MOV     AL,11H          ; Set Return Found

; Name state

INT     2FH

Example 2FH Handler

MYNUM           DB     X ; X = The specific AH

; multiplex number.

INT 2F NEXT     DD     ? ; Chain location

INT 2F:

ASSUME DS:NOTHING,ES:NOTHING,SS:NOTHING

CMP     AH,MYNUM

JE      MINE

JMP     INT 2F NEXT   ; Chain to next

; 2FH Handler

MINE:

CMP     AL,0F8H

JB      DO FUNC

IRET                  ; IRET on reserved

; functions

DO FUNC:

OR      AL,AL

JNE     NON INSTALL   ; Non Get Installed

; State request

MOV     AL,0FFH       ; Say I'm here

IRET                  ; All done

NON INSTALL:

.

.

.

Installing the Handler

The following example contains the functions necessary to install a handler:

MOV    AH,MYNUM

XOR    AL,AL

INT    2FH                      ; Ask if already

; installed

OR     AL,AL

JZ     OK INSTALL

BAD INSTALL:                       ; Handler already  installed .sp 3

OK INSTALL:                        ; Install my

; handler

MOV    AL,2FH

MOV    AH,GET INTERRUPT VECTOR

INT    21H                       ; Get multiplex

; vector

MOV    WORD PTR INT 2F NEXT+2,ES

MOV    WORD PTR INT 2F NEXT,BX

MOV    DX,OFFSET INT 2F

MOV    AL,2FH

MOV    AH,SET INTERRUPT VECTOR

INT    21H                       ; Set multiplex

; vector

.

.

.

30H-3FH RESERVED FOR DOS 4.00

These interrupts are reserved for DOS 4.00 use.