APPENDIX D.  EXPANDED MEMORY SUPPORT 3

APPENDIX D.  EXPANDED MEMORY SUPPORT

Expanded memory is memory addressable through a combination of an Expanded

Memory Specification (EMS) device driver and an EMS-capable hardware adapter.

The table below describes the Lotus/Intel/Microsoft (LIM) Expanded Memory

Manager Specification Version 4.0 functions and shows you which ones are

supported by DOS 4.00.  For detailed information and guidelines on the use of

these calls, refer to the LIM Specification.

LIM    INT                     DOS 4.00

SPEC.  67H         INTERFACE   SUPPORT     DESCRIPTION

1      AH = 40H    App         Yes         Get status

2      AH = 41H    App         Yes         Get page frame address

3      AH = 42H    App         Yes         Get unallocated page count

4      AH = 43H    App         Yes         Allocate pages

5      AH = 44H    App         Yes         Map/unmap handle page

6      AH = 45H    App         Yes         Deallocate pages

7      AH = 46H    App         Yes         Get EMM version

8      AH = 47H    App         Yes         Save page map

9      AH = 48H    App         Yes         Restore page map

10     Reserved

11     Reserved

12     AH = 4BH    App         Yes         Get EMM Handle count

13     AH = 4CH    App         Yes         Get EMM Handle pages

14     AH = 4DH    App         Yes         Get all EMM handle pages

15     AH = 4EH    App         Yes         Get/Set page map

16     AH = 4FH    App         Yes         Get/set partial page map

17     AH = 50H    App         Yes         Map/unmap multiple handle pages

18     AH = 51H    App         Yes         Reallocate pages

19     AH = 52H    App         No          Get/set handle attributes

20     AH = 53H    App         Yes         Get/set handle name

21     AH = 54H    App         Yes         Get handle directory

22     AH = 55H    App         Yes         Alter page map and jump

23     AH = 56H    App         Yes         Alter page map and call

24     AH = 57H    App         Yes         Move/exchange memory region

25     AH = 58H    App         Yes         Get mappable physical address

array

26     AH = 59H    OS          Yes         Get expanded memory hardware

information

27     AH = 5AH    App         Yes         Allocate new pages

28     AH = 5BH    OS          Yes         Alternate page map register set

29     AH = 5CH    App         Yes         Prepare expanded memory hardware

for warm boot

30     AH = 5DH    App         Yes         Enable/disable OS/E function set