-=----------------------------------=-

-=- Brown Box Plans..              -=-

-=-                                -=-

--------------------------------------

(>INTRODUCTION<)

THIS IS A FAIRLY SIMPLE MODIFICATION THAT CAN BE MADE TO ANY PHONE. ALL IT DOES

IS ALLOW YOU TO TAKE ANY 2 LINES IN YOUR HOUSE AND CREATE A PARTY LINE. SO FAR

I HAVE NOT HEARD OF ANY PROBLEMS WITH IT FROM MY FRIENDS THAT HAVE SET ONE UP

AND I HAVE NOT HAD ANY EITHER. THERE IS ONE THING THAT YOU WILL NOTICE WHEN YOU

ARE ONE OF THE TWO PEOPLE WHO IS CALLED BY A PERSON WITH THIS BOX. THE OTHER

PERSON WILL SOUND A LITTLE BIT FAINT.  I COULD OVERCOME THIS WITH SOME

AMPLIFIERS BUT THEN THERE WOULDN'T BE VERY MANY OF THESE BOXES MADE.

I THINK THAT THE CONVENIENCE OF HAVING TWO PEOPLE ON LINE AT ANY ONE TIME WILL

MAKE UP FOR THE MINOR VOLUME LOSS.

(>PHONE MODIFICATION INSTRUCTIONS<)

HERE IS THE DIAGRAM:

KEY:___________________________________

! PART                   ! SYMBOL !

-----------------------------------

!BLACK WIRE              !   *    !

!YELLOW WIRE             !   =    !

!RED WIRE                !   +    !

!GREEN WIRE              !   -    !

!SPDT SWITCH             !  _/_   !

!                           _/_   !

!VERTICAL WIRE           !   \    !

!HORIZONTAL WIRE         !   _    !

-----------------------------------

*    =    -    +

*    =    -    +

*    =    -    +

*    =    -    +

*    =    -    +

*    ==_/_-    +

*******_/_++++++

\

\

\

\_____PHONE____

---------------------------------------

IN SOME HOUSES THE BLACK AND YELLOW ARE ALREADY WIRED IN OTHERS YOU WILL HAVE

TO GO OUT TO YOUR BOX AND REWIRE IT. A GOOD WAY TO FIGURE OUT WHICH LINE IS

WHICH IS TO TAKE THE PHONE YOU ARE LOOKING FOR OFF THE HOOK.  THEN YOU

ONLY NEED TO TAKE THE RED AND GREEN WIRES ENTERING YOUR PHONE AND HOOK THEM

TO THE DIFFERENT PAIRS OF RED AND GREEN GOING INTO THE HOUSE.  YOU CAN'T HURT

ANYTHING IN THE PHONE OR TELEPHONE BY PROBEING.  WHEN YOU FIND THE PAIR THAT

YOU WANT TAKE THE BLACK FROM YOUR LINE AND ATTACH IT TO THE RED OF THE OTHER

LINE THEN TAKE THE YELLOW AND ATTACH IT TO THE GREEN LINE.  NOW YOU ARE ALL SET

TO GO.  FOR PEOPLE WITH ROTARY PHONES YOU CAN HAVE ONE PERSON CALL YOU THEN

PLACE THE SECOND CALL OUT TO THE OTHER PERSON. THOUGH NOT PHREAKERS TOOL, THE

BROWN BOX CAN BE PHUN.

Which G-file (Q=Quit) ?

Downloaded from Just Say Yes. 2 lines, More than 1500 files online!

Do you write? Give us a call!   415-922-2008 CASFA