How Ma Bell Works                               by the Jolly Roger

In  this  article,  I  will first describe  the  termination,

wiring,  and  terminal  hardware most commonly used in  the  Bell

system, and I will include section on methods of using them.

-------------

LOCAL NETWORK

-------------

The   local   telephone   network   between   the    central

office/exchange  and  the  telephone subscribers can  be  briefly

described as follows:

From  the  central office (or local exchange) of  a  certain

prefix(es), underground area trunks go to each area that has that

prefix  (Usually  more than one prefix per area.)  At  every  few

streets or tract areas, the underground cables surface. They then

go to the telephone pole (or back underground,  depending on  the

area)  and  then  to the subsribers house (or in the case  of  an

apartment building or mutliline business,  to a splitter or  dis-

tribution box/panel).

Now  that  we have the basics,  I'll try and go in-depth  on  the

subject.

------------------

UNDERGROUND CABLES

------------------

These  are sometimes inter-office trunks,  but usually in  a

residential  area they are trunk lines that go to bridging  heads

or  distribution  cases.  The cables are about 2-3  inches  thick

(varies),  and  are  either  in  a metal  or  pvc-type  pipe  (or

similiar).  Rarely (maybe not in some remote rural areas) are the

cables  just 'alone' in the ground.   Instead they are usually in

an  underground cement tunnel (resembles a small sewer or  storm-

drain.)   The manholes are >heavy< and will say 'Bell system'  on

them.  they can be opened with a 1/2 inch wide crowbar (Hookside)

inserted  in the top rectangular hole.  There are ladder rungs to

help you climb down.  You will see  the cable  pipes on the wall,

with the blue and white striped one being  the inter-office trunk

(at  least  in my  area).  The others  are local  lines,  and are

usually marked or color  coded.  There is almost  always a posted

color code  chart on the wall, not to mention  Telco manuals de-

scribing the cables and terminals, so I need not get into detail.

Also, there is usually  some kind  of  test equipment,  and often

Bell test sets are  left  in there.

--------------

BRIDGING HEADS

--------------

The  innocent-looking  grayish-green boxes.   These  can  be

either trunk bridges or bridging for residences.  The major trunk

bridging  heads  are usually larger,  and they have the  'Western

Electric' logo at the bottom,  whereas the normal bridging  heads

(which  may  be different in some areas-depending on the  company

you are served by.  GTE B.H.'s look slightly different.  Also, do

not  be  fooled by sprinkler boxes!)  They can be found  in  just

about every city.

To open a bridging head: if it is locked (and you're feeling

destructive),  put a hammer or crowbar (the same one you used  on

the  manhole) in the slot above the top hinge of the right  door.

Pull hard, and the door will rip off. Very effective! If it isn't

locked (as usual),  take a 7/8 inch hex socket and with it,  turn

the bolt  about 1/8 of a turn  to the  right  (you  should hear a

spring release inside). Holding the bolt, turn the handle all the

way to the left and pull out.

To Check for a test-set (which are often left by Bell employees),

go inside - First check  for a test-set  (which  are  often  left

by  Bell  employees).   There should be a panel of terminals  and

wires.   Push the panel back about an inch or so,  and rotate the

top  latch  (round with a flat section)  downward.   Release  the

panel  and it will fall all the way forward.  There is usually  a

large  amount  of wire and extra terminals.   The  test-sets  are

often hidden here,  so don't overlook it (Manuals,  as  well,  are

sometimes placed in the head).   On the right door is a metal box

of alligator clips.   Take a few (Compliments of Bell.).  On each

door is a useful little round metal device. (Says 'insert gently'

or'  clamp  gently - do not overtighten' etc..)  On the front  of

the disc, you should find two terminals.  These are for your test

set.   (If you dont have one, dont despair -I'll show you ways to

make basic test sets later in this article).

Hook the ring (-) wire to the 'r' terminal;  and the tip (+)

wire  to the other.  (By the way,  an easy way to  determine  the

correct polarity is with a 1.5v LED.   Tap it to the term.  pair,

if  it  doesnt light,  switch the poles until it does.   When  it

lights,find the longer of the two LED poles:  This one will be on

the  tip wire (+).   Behind the disc is a coiled  up  cord.  This

should have two alligator clips on it..  Its very useful, because

you dont have to keep connecting and disconnecting the fone (test

set) itself, and the clips work nicely.

On  the  terminal  board,  there should be  about  10  screw

terminals  per  side.   Follow the wires,  and you can see  which

cable pairs are active.  Hook the clips to the terminal pair, and

you're set!   Dial out if you want,  or just listen (If someone's

on theline).  Later,  I'll show you a way to set up a true  'tap'

that  will let the person dial out on his line and receive  calls

as normal, and you can listen in the whole time.  More about this

later...

On  major  prefix-area bridging heads,  you can  see  'local

loops' ,which are two cable pairs (cable pair = ring+tip,  a fone

line)  that are directly connected to each other on the  terminal

board.   These  'cheap  loops' as they are called,  do  not  work

nearLy  as  well  as the existing ones set up  in  the  switching

hardware  at the exchange office.   (Try scanning your  prefixes'

00xx  to 99xx #'s.)  The tone sides will announce themselves with

the 1008 hz loop tone,  and the hang side will give no  response.

The  first  person  should dial the 'hang' side,  and  the  other

person dial the tone side,  and the tone should stop if you  have

got the right loop.)

If  you want to find the number of the line that you're  on,

you  can either try to decipher the 'bridging log' (or whatever),

which is on the left door.  If that doesnt work,  you can use the

follwing:

---------------------------

ANI # (Automatic Number ID)

---------------------------

This  is a Telco test number that reports to you the  number

that youre calling from (It's the same, choppy 'Bell bitch' voice

that you get when you reach a disconnected #)

For the   213 NPA - Dial 1223

408 NPA - Dial 760

914 NPA - Dial 990

These  are  extremely useful when messing with any  kind  of line

terminals, house boxes, etc.

Now that we have bridging heads wired, we can go on... (don't

forget to close and latch the box after all... Wouldnt want GE

and Telco people mad, now, would we?)

-------------------------------------

"CANS" - Telephone Distribution Boxes

-------------------------------------

Basically, two types:

1> Large, rectangular silver box at the end of each street.

2> Black, round, or rectangular thing at every telephone pole.

Type 1 -   This is the case that takes the underground cable from

the  bridge and runs it to the telephone pole cable (The  lowest,

largest  one  on the telephone pole.)  The box is always  on  the

pole nearest the briging head, where the line comes up.  Look for

the 'Call before you Dig - Underground cable' stickers..

The  case box is hinged,  so if you want to climb the  pole,

you  can open it with no problems.   These usually have 2 rows of

terminal sets.

You  could try to impersonate a Telco technician and  report

the  number as 'new active' (giving a fake name and fake  report,

etc.)   I  dont recommend this,  and it  probably  won't  (almost

positively won't) work,  but this is basically what Telco linemen

do).

Type  2 - This is the splitter box for the group of houses around

the  pole  (Usually  4 or 5 houses).   Use it  like  I  mentioned

before.   The terminals (8 or so) will be in 2 horizontal rows of

sets.   The  extra  wires  that  are  just  'hanging  there'  are

provisions for extra lines to residences (1 extra line per house,

thats  why  the insane charge for line #3!)  If its the  box  for

your  house  also,  have fun and swap lines with  your  neighbor!

'Piggyback'  them  and  wreak havoc  on  the  neighborhood  (It's

eavesdropping  time...)  Again,  I don't recommend this,  and its

difficult to do it correctly.  Moving right along...

------------------------------

APARTMENT / BUSINESS MULTILINE

DISTRIBUTION BOXES

------------------------------

Found  outside the buliding (most often on the  right  side,

but not always...   Just follow the wire from the telephone pole)

or  in the basement.   It has a terminal for all the lines in the

building.   Use it just like any other termination box as before.

Usually says 'Bell system' or similar.  Has up to 20 terminals on

it (usually.) the middle ones are grounds (forget   these).   The

wires come from the cable to one row (usually the left one), with

the other row of terminals for the other row of terminals for the

building  fone wire pairs.   The ring (-) wire is usually the top

terminal if the set in the row (1 of 10 or more),  and the tip is

in the clamp/screw below it.  This can be reversed, but the cable

pair is always terminated one-on-top-of-each- other,  not on  the

one  next  to  it.  (I'm  not sure why the other  one  is  there,

probably as aprovision for extra lines) Don't use it  though,  it

is usually to close to the other terminals, and in my experiences

you get a noisy connection.

Final note:  Almost every apartment, business, hotel, or anywhere

there   is  more  than  2  lines  this  termination  lines   this

termination method is used.  If you can master this type, you can

be  in control of many things...   Look around in your area for a

building  that  uses this type,  and practice hooking up  to  the

line, etc.

As  an  added  help,here is the basic  'standard'  color-code  for

multiline terminals/wiring/etc...

Single line:  Red = Ring

Green = Tip

Yellow = Ground *

*  (Connected  to  the ringer coil  in  individual  and  bridged

ringer   phones (Bell only)  Usually  connected to the  green

(Tip)

Ring (-) = Red

White/Red Stripe

Brown

White/Orange Stripe

Black/Yellow Stripe

Tip (+) =  Green (Sometimes

yellow, see above.)

White/Green Stripe

White/Blue Stripe

Blue

Black/White Stripe

Ground =   Black

Yellow

----------------------

RESIDENCE TERMINAL BOX

----------------------

Small,  gray (can be either a rubber (Pacific Telephone) or  hard

plastic  (AT & T) housing deal that connects the cable pair  from

the  splitter box (See type 2,  above) on the pole to your  house

wiring.  Only 2 (or 4, the 2 top terminals are hooked in parallel

with the same line) terminals, and is very easy to use.  This can

be  used to add more lines to your house or add an external  line

outside the house.

---------

TEST SETS

---------

Well,  now  you  can  consider  yourself a minor  expert  on  the

terminals and wiring of the local telephone network.  Now you can

apply  it  to whatever you want to do..  Here's another  helpful

item:

How  to  make  a Basic Test-Set and how to use it  to  dial  out,

eavsdrop, or seriously tap and record line activity.

These are  the  (usually) orange hand set fones  used  by  Telco

technicians to test lines.   To make a very simple one,  take any

Bell (or other,  but I recommend a good Bell fone like a princess

or a trimline. gte flip fones work excllently, though..) fone and

follow the instructions below.

Note:    A 'black box' type fone mod will let you tap into  their

line,  and with the box o,  it's as if you werent there. they can

recieve  calls and dial out,  and you can be listening the  whole

time! very useful.  With the box off, you have a normal fone test

set.

Instructions:

A  basic black box works well with good results.   Take the cover

off  the fone to expose the network box (Bell type  fones  only).

The <RR> terminal should have a green wire going to it (orange or

different  if  touch tone - doesnt matter,  its the same  thing).

Disconnect the wire and connect it to one pole of an SPST switch.

Connect  a  piece  of wire to the other pole of  the  switch  and

connect it to the <RR> terminal.   Now take a 10k hm 1/2 watt 10%

resistor  and  put  it  between the <RR>  terminal  ad  the  <F>

terminal,  which  should have a blue and a white wire going to it

(different for touch tone).  It should look like this:

-----Blue wire----------<F>

!

----White wire-----!

!

10k Resistor

!

!

--Green wire--     !----<RR>

!    !

SPST

What  this  does in effect is keep the hookswitch  /  dial  pulse

switch (F to RR loop) open while holding the line high with  the

resistor.   This gives the same voltage effect as if the fone was

'on-hook',  while  the 10k ohms holds the voltage right above the

'off hook' threshold (around 22 volts or so, as compared to 15-17

or normal off hook 48 volts for normal 'on-hook'), giving

Test Set Version 2.

Another  design is similar to the 'type 1' test set (above),

but has some added features:

From >----------------Tip------<To Test

Alligator                        set

Clip >----------------Ring-----<phone

!                !

x                !

!                !

o                !

!    x---RRRRR---!

!   x            !

!---x            !

x----0------!

x    = Spst Switch

o    = Red LOD        0   = Green LED

RRRRR= 1.8k 1/2 watt  xxxx= Dpst switch

resistor

When the SPST switch in on, the LED will light, and the fone

will become active.   The green light should be on.  If it isn't,

switch  the dpst.   If it still isnt,  check the polarity of  the

line and the LEDs.   With both lights on, hang up the fone.  They

should all be off now.   Now flip the dpst and pick up the  fone.

The  red  LED shold be on,  but the green shouldnt.   If  it  is,

something is wrong with the circuit.  You wont get a dial tone if

all is correct.

When  you  hook  up  to the line with  the  alligator  clips

(Assuming you have put this circuit inside our fona and have  put

alligator clips on the ring and tip wires (As we did before)) you

should  have the spst #1 in the off posistion.  This will greatly

reduce the static noise involved in hooking up to a line. The red

LED can also be used to check if you have the correct polarity.

With  this  fone you will have the ability to listen  in  on

>all< audible line activity,  and the people (the 'eavesdropees')

can use their fone as normal.

Note that test sets #1 and #2 have true 'black boxes', and can be

used for free calls (see an article about black boxes).

Test Set Version 3

To do test set 3:

Using a trimline (or similar) phone, remove the base and cut

all  of  the wire leads off except for the red (ring -)  and  the

green  (tip +).   Solder alligator clips to the  lug.   The  wire

itself  is  'tinsel' wrapped in rayon,  and doesnt solder  well.

Inside  the one handset,  remove the light socket (if it has one)

and install a small slide or  toggle switch (Radio Shack's micro-

miniature  spst works well).   Locate the connection of the  ring

and the tip wires on the pc board near where the jack is  located

at the bottom of the handset.  (The wires are sometimes black or

brow instead of red and green,  respectively).   Cut the foil and

run 2 pieces of wire to your switch.  In parallel with the switch

add  a .25 uf 200 VDC capacitor (mylar,  silvered mica,  ceramic,

not  an electrolytic).  When the switch is  closed,  the  handset

functions normally.   With the switch in the other position,  you

can listen without being heard.

Note:  To  reduce the noise involved in connecting the clips to a

line,  add  a  switch selectable 1000 ohm 1/2  watt  resistor  in

series with the tip wire. Flip it in circuit when connecting, and

once on the line, flip it off again. (or just use the 'line disc-

onect'  type  switch as in the type 2 test  set  (above)).   Also

avoid   touching the alligator clips to any metal parts or  other

terminals,  for  i causes static on the line and raises  poeple's

suspicions.

---------

RECORDING

---------

If you would like to record any activity,  use test set 1 or

2  above  (for unattended recording of >all< line  activity),  or

just  any test set if you are going to be there to  monitor  when

they are dialing, talking, etc.

Place  a  telephone pickup coil (I recommend the Becoton  T-5  TP

coil  or equivalent) onto the test set,  and put the TP plug into

the mic.  jack of any standard tape recorder.  Hit play, rec, and

pause.   Alternate  pause when you want to record (I  dont  think

anyone should have any difficulty with this at all...)

Well,  if  you  still can't make a test set or you dont have  the

parts, there's still hope.  Alternate methods:

1>  Find  a  bell test set in a manhole or a  bridging  head  and

'Borrow it indefinately...

2>  Test sets can  be  purchased from:

Techni-Tool

5 Apollo Road

Box 368

Plymouth Meeting PA., 19462

Ask for catalog #28

They  are usually $300 - $600, and are supposed to have  MF

dialing  capability as well as TT dialing.  They are also of much

higher quality than the standard bell test sets.

If you would like to learn more about the subjects covered here,

I suggest:

1>  Follow Bell trucks and linemen or technicians and ask  subtle

questions. also try 611 (repair service) and ask questions..

2> Explore your area for any Bell hardware,  and experiment  with

it.  Don't try something if you are not sure what youre doing,

because you wouldnt want to cause problems, would you?

-----Mondain-----