-----------------chop-with-axe----------------chop-with-axe---------------------

Subject: Mag Stripe Card Info SUMMARY (long)

Summary of information on the SR&D MCR-175-1R-0803 mag stripe card

reader. Due to the large volume of summary requests I received, I am

posting this summary instead of mailing it. It's somewhat long, so hit

'n' now if you're not interested in this stuff.

[Editor's Note: The following is a concatenation of the replies I received

to a net.request about my mag stripe card reader. After the replies I have

included some software that I threw together to play with the card reader.

This file contains all the information I have on this subject.

Additions are most welcome. Note also that there was another summary

post in recent days with information on how the data on the cards is

encoded. See article <kms26oINNojs@network.ucsd.edu> for this info.

You'll notice I didn't get any farther than simply reading the raw signal

from the card; of the two card readers I ordered, one was completely DOA,

and the other had a faulty clock output (at least I assume that it was a

clock output; I was never able to read any sort of signal from that line).

Someone with a fully functional reader can easily extend what I wrote to get

it to decode the actual data content of the card. If you do decide to make

modifications and/or extensions, I'd appreciate a copy of whatever

changes you make (email to tmkk@uiuc.edu).  Enjoy!]

---

From: tmkk@uiuc.edu (K. Khan)

Subject: Re: Mag Card Swipe Reader: Need Help!

I am truly amazed that someone else is trying to use this device!  I got

mine about 2 years ago and spent some time trying to find the manufacturer.

I found a listing for SR&D in the Noth America technical directory at

the public library.  I found the listing for the American sales office

in Los Angeles.  I tried calling but the company had gone out of

business.  There was no listing in the local phone directory either.

I then tried calling the head office in Japan, but they also had

gone out of business.  I haven't seen the company listed in any recent

electronics directories, so I think they really are gone.

I have spent about an hour looking at the signals on the outputs

of the device.  One signal line is a /STATUS line which indicates

when a card is been moved through the unit.  The other 2 lines

pulse in response to a magnetic card.  I believe the IC performs

Manchester decoding and clock recovery for the read channel, so one

output line is DATA and the other is CLOCK.

That is as far as I got 2 years ago and I had forgot about it until

now.  If you receive any other info, please send a copy to me!

---

>Finally, there are 5 wires coming from the assembly and terminating in a

>small connector similar to power supply connectors for 3.5" floppy drives.

>The wires are red, yellow, green, blue, and black.

If its anything like the units I worked with, I think you will find

that the five wires are:

+5v

Gnd

Clock

Data

Card detected

But I don't know active levels, or which wire is what.

---

I picked few week ago a magnetic credit card reader from a another surplus

outfit. It cost about the sam es yours. My card reader was made by MAGTEK and

was diffrent from your reder in many ways. The reader I have has 4 ICs and

some of them are standard TTL chip, so I could easily quess the power

requiments (5V) and power connectors. My card reader had 6 pin connector. I

put the power to the reader and started to examine the signals with multimeter

and a little crystal earphone (my favourite electronics hacking tool). I found

that output signals were something like that: data out, data clock out, data

readable and and card ath the end of the reader. Then I connected the reader

to the joystick port of my 386SX and made a little Turbo Pascal program for

reading the card.

Spare printer port is the interface I use very often to connect diffrent

hardware circuit to my computer. This time I decided to use game port because

it can also provide the power to the user.

My program simply prints out the bits from the card. I have not found the way

to decode the bits to corresponding numbers. The program so prints all 237

bits form the card to screen. If you have any information about data coding, I

an interested in hearing about it.

Here is the meanings of the bytes in port $201:

D7: 0 -> card pushed to the end of the reader

D6: the read data from card

D5: 0 -> data stream readable

D4: the data clock

--- Pascal program ---

Program CardReader;

Uses Crt,Binary;

Const

gameport=$201;

Procedure Wait_start;

Begin

Repeat Until (Port[gameport] and 32)=0;

End;

Function data_readable:boolean;

Begin

data_readable:=((Port[gameport] and 32)=0);

End;

Procedure Wait_clock;

Begin

Repeat Until (Port[gameport] and 16)=0;

End;

Procedure Wait_clock_end;

Begin

Repeat Until (Port[gameport] and 16)=16;

End;

Function data_input:byte;

Begin

If (Port[gameport] and 64)=0 Then data_input:=0

Else data_input:=1;

End;

Function card_at_end:boolean;

Begin

card_at_end:=((Port[gameport] and 128)=0);

End;

Procedure test;

Begin

Wait_start;

Repeat

Writeln(ByteBin(Port[$201]));

Until keypressed;

End;

Begin

ClrScr;

Wait_start;

While data_readable Do Begin

Wait_clock;

Write(data_input);

Wait_clock_end;

End;

Repeat Until KeyPressed;

End.

---

Wiring color code for the SR&D MCR-175-1R-0803 mag stripe card reader:

Red:         +5V

Black:         Gnd

Yellow:         /Card Detect

Green:         Clock (?? - non-functional on the unit I have)

Blue:         /Data

The leading '/' indicates an active low TTL signal.

---

Quick 'n Dirty guide to the enclosed reader software

----------------------------------------------------

Hooking the SR&D MCR-175-1R-0803 card reader to your PC:

The included software is written specifically for the following configuration;

if your wiring is different, you'll need to make corresponding changes to the

software. Note also that the port address is hard-coded to look for LPT2's

status port (at address 0x279). If you're using a different port address, be

sure to change the port address value.

SR&D Wire       Printer Port Pin        Port Bit        Signal

---------       ----------------        --------        ------

Yellow          11                      7               /CARD DETECT

Blue            10                      6               /DATA

Black           18                      N/A             (Ground)

Power to the reader was provided by a separate power supply, basically one of

those black plastic DC power packs fed through a 7805 regulator chip.

Compiling the software:

Compile SWIPE.C (using SMALL memory model), assemble SWIPEISR.ASM, and

link the two together.

Using the software:

To use SWIPE.EXE, simply hook the reader up to your LPT2: port, power it

up, then run SWIPE. When you're ready, press the ENTER key, and swipe a

card through the reader. The program will read the data from the card and

store it in a buffer (but will not decode the data; that is left as an

excercise ;-). After the card has been read, press ENTER again and the

contents of the buffer will be dumped to stdout. To save the card data to

a file, simply redirect SWIPE's output on the command line, e.g.

SWIPE > citibank.out

Please let me know of any changes, bug fixes, or improvements you make to

this code. Send email to tmkk@uiuc.edu.

Thanks, and have fun!

--- CUT HERE ---

/*

* S W I P E . C

*

* Written:

*     1/11/92

*

* Description: Quick 'n Dirty reader program for SR&D mag stripe card reader.

*     Reads data from the input port as long as a card is detected in the

*     card slot. After sampling, the data is dumped to stdout, and may

*     be redirected to a file if desired.

*

* Note: Written for Borland C++ 3.0 - may require changes to compile under

*     MSC or others. Compile in SMALL model.

*

*/

#include <stdio.h>

#include <conio.h>

#include <process.h>

#include <dos.h>

#include <math.h>

#include <stdlib.h>

#include <assert.h>

#include <string.h>

/* timer chip programming register port addresses */

#define COMMAND_REG 0x43

#define CHANNEL0 0x40

/* size of sample buffer */

#define MAXSAMPLE 4096

typedef unsigned char byte;

/* global variables */

byte *databuf;  /* buffer for the sampled data */

/* interprocess communication data */

byte *bufp;         /* data buffer pointer */

unsigned nsamp;     /* number of samples to be made */

unsigned port;      /* input port address */

int enab=0;         /* flag to enable/disable sampling */

int start=0;        /* flag indicating that sampling has begun */

/* ISR prototype */

extern void interrupt shand(void);

void program_timer(int channel, unsigned count)

/*

* P R O G R A M _ T I M E R

*

* Description: Programs the given count value into the specified channel of

*     the IBM 825x timer chip. Channel 0 is the time-of-day-clock interrupt;

*     channel 2 is the speaker pulser.

*

* Parameter:

*     channel (in) - Channel to be programmed.

*     count (in) - Count value with which to program timer chip.

*

*/

{

outportb(COMMAND_REG, 0x36);  /* set up for reprogramming */

outportb(CHANNEL0 + channel, count & 0xff);  /* lo byte first */

outportb(CHANNEL0 + channel, count >> 8);  /* then hi byte */

}

void sample_data(int count)

/*

* S A M P L E _ D A T A

*

* Description: Sets up for data collection from the printer port using

*     the SHAND interrupt service routine (see SWIPEISR.ASM). This routine

*     reprograms the timer chip for the desired sampling rate, sets up

*     the interprocess communication area, and starts the sampling process.

*     The actual sampling is done in the SHAND procedure. This routine

*     waits until sampling has been completed before returning.

*

*/

{

void interrupt (*oldhand)(void);  /* pointer to old interrupt vector */

/* save old interrupt vector */

oldhand = getvect(0x1c);

/* clear enable flag */

enab = 0;

start = 0;

/* install new vector */

setvect(0x1c, shand);

/* set up interprocess communications area */

nsamp = 0;

bufp = databuf;

port = 0x279;  /* address of printer status register */

cprintf("Sampling at %fHz (= %fms)....",

1193180.0 / (float)count, (float)count / 1193.18);

/* reprogram timer chip */

program_timer(0, count);

/* enable sampling */

enab = 1;

/* wait until sampling is completed */

while (enab) ;

/* restore standard timing value */

program_timer(0, 0);

/* reinstall old handler vector */

setvect(0x1c, oldhand);

cprintf(" completed.\r\n");

}

void main()

{

unsigned i;

/* allocate memory */

databuf = calloc(MAXSAMPLE, sizeof(byte));

assert (databuf != NULL);

cprintf("Press <ENTER> when ready to swipe card:");

getchar();

sample_data(12);  /* This works out to about a 100kHz sampling rate  */

cprintf("Sampling completed, %u samples total.\r\n", nsamp);

cprintf("Press <ENTER> to dump data.\r\n\r\n");

getchar();

/* dump data to stdout */

for (i=0; i<nsamp; ++i)

if (kbhit())

break;

else

printf("%u\n", databuf[i]);

free (databuf);

}

--- CUT HERE ---

;

; S W I P E I S R . A S M

;

; Interrupt Service Routine to sample swipe reader.

;

_TEXT   segment byte public 'CODE'

DGROUP  group   _DATA,_BSS

assume  cs:_TEXT,ds:DGROUP,ss:DGROUP

_TEXT   ends

_DATA   segment word public 'DATA'

; Nuthin' in here!

_DATA   ends

_BSS    segment word public 'BSS'

; Nuthin' in here either!

_BSS    ends

_TEXT   segment byte public 'CODE'

;

; S H A N D

;

; Description: Sample interrupt handler for timer extension interrupt 0x1c.

;

; Note: Assumes that the /CARD DETECT line appears on bit 7 of the input

;     port. This bit corresponds to pin 11 on the standard PC printer port.

;     Also, the PC printer port card inverts the signal on this pin; thus,

;     bit 7 is *set* when a card is in the reader, even though /CARD DETECT

;     is an active low signal.

;

_shand  proc    far

push    ax

push    bx

push    dx

push    ds

push    si

push    di

push    bp

mov     bp,DGROUP

mov     ds,bp

cmp     word ptr DGROUP:_enab, 0                ; Are we turned on?

je      ciao                                    ; Nope - goodbye!

mov     dx, word ptr DGROUP:_port               ; Get port address

cmp     word ptr DGROUP:_start, 0               ; Has sampling begun?

jne     doit                                    ; Yup - hop to it!

;

; Sampling is enabled but has not actually begun because no card has yet been

;     detected in the slot of the reader. Check to see if a card has arrived

;     yet and, if it has, set the flag to indicate that sampling should begin.

;

in      al, dx                                  ; sample port

test    al, 80h                                 ; card detected?

jz      ciao                                    ; nope - later!

mov     word ptr DGROUP:_start, 1               ; card's there,

; start reading it

jmp     saveit

;

doit:

in      al, dx                                  ; Read the port

test    al, 80h                                 ; card detect bit set?

jz      disable                                 ; if not, quit

;

; Data is still valid; get buffer pointer and save the data

;

saveit:

mov     bx,word ptr DGROUP:_bufp                ; get addr. to write

mov     byte ptr [bx],al                        ; save sampled value

inc     word ptr DGROUP:_bufp                   ; and bump the ptr

inc     word ptr DGROUP:_nsamp                  ; update sample count

jmp     ciao                                    ; and skeedaddle

disable:

mov     word ptr DGROUP:_enab,0                 ; card no longer

; detected - disable

; sampling

ciao:

pop     bp                                      ; restore context

pop     di

pop     si

pop     ds

pop     dx

pop     bx

pop     ax

iret

_shand  endp

_TEXT   ends

_DATA   segment word public 'DATA'

;s@      label   byte

_DATA   ends

extrn   _nsamp:word

extrn   _port:word

extrn   _bufp:word

extrn   _enab:word

extrn   _start:word

_TEXT   segment byte public 'CODE'

extrn   _inportb:near

_TEXT   ends

public  _shand

end

=== END OF SUMMARY ===

-----------------chop-with-axe----------------chop-with-axe---------------------