PIC TIPS							Dec., 1992

Q.	I am using one of the PIC devices in a SOIC package, but am having a problem locating a supplier 	of converter sockets, so that I can program these devices with a third party programmer. Can you 	give me a list of suppliers for SOIC converter sockets

A.	The suppliers that we know of for converter sockets for the 0.3 mil SOIC packages are:

Emulation Technologies  @  (408) 982-0660

18LSO --> DIP,  Part # AS-18-18-01S-3

28LSO --> DIP,  Part # AS-28-28-03S-6-GANG

Another Co. as per Mike J.

Please verify that these converters will meet your requirements before purchasing.

NOTE:	A converter socket is not required for programming SOIC packages with the PRO

Master. Use either Microchip socket part # AC164002 (18 and 28 pin) or socket part #

AC164005 (18 PIN) with the PRO Master.

Q.	Can I replace a PIC16C5x HS device with the RC version?

A.	No.  Even if the frequency of the application would allow an RC version. The design of the RC 	oscillator is different from that of the XT device, due to the different characteristics of the respective 	inputs. Use of the wrong oscillator option device in an application, may cause unexpected 	operation and operational life reliability issues.

Q.	What is the maximum frequency for the PIC16C71 devices presently available?

A.	The Preproduction PIC16C71 devices can be ordered at either 16 MHz or 4 MHz maximum 	frequency rating. Note that the LP oscillator mode is rated to 200 KHz.