From: Detlef Mueller  <detlef@dmhh.hanse.de>

Subject: STOFIX Library v1.1

--> only intended for GX version L & M

This is the STOFIX library, version 1.1. It provides a

workaround for the STO and PINIT bug described below on

ROM L & M HP48GX's. It's 1187 bytes in size, its checksum

is #81E5h and its Library ID is 1222.

STOFIX is designed to run in any port on any revision

HP48 but is only useful in rev L & M

Installation

------------

* if you've a GX w/ two cards installed, remove the

card from port 1 as described in your manual if you

want to store the library into a port other than 0 or

1

* download STOFIX.LIB into your '48

* recall it onto the stack and purge the variable

containing the d/led copy

* enter the port number where the library should be

stored at, press STO

* warmstart your '48 by pressing ON-C or power cycle it

* reinstall the card in port 1 (if neccessary) as

described in your manual

Commands

--------

STOFIX provides 3 new commands, you'll see them after

entering the STOFIX menu (<right>-2 STOFI on a GX,

<left>-\|^ STOFI on a SX):

sto	 ( object1 object2	      -->	  )

This command is a replacement for the build-in STO

command, it does exactly the same, except that it

does stores directeced to a port > 1 in a correct

manner.

pinit	 (			      -->	  )

This command is a replacement for the PINIT command,

it does exactly the same, except it won't corrupt the

memory of card 1.

SFHLP	 (			      -->	  )

'StoFix HeLP' - shows three screens of help

information.

Usage

-----

Use the STOFIX commands on a L or M HP48GX in case you

have installed RAM cards in ports 1 & 2 for doing the

following tasks to avoid memory losts/ corruption:

* to store a library or backup into a port > 1, use

[sto]

* to store other objects into a port > 1, use [sto]

* to initialize a multi-port card, use [pinit]

Big description

---------------

There's a *really* bad BUG which appears in ROM rev L &

M HP48GX's.

Objects are stored into ports 1-33 from low to high

address, and each obj sequence is terminated by 5

0-nibbles following immediately the last obj of a port.

This marker is used as end of obj sequence and to

signal an intact port.

Two cards will share the address space starting at

address #C0000, port 1 will be active all the time, and

ports 2-33 are memory mapped into this address space

temporarily on access requests.

If you've plugged in two cards, and store a library

into, say, port 2 (with 2 STO - this may be a 32k,

128k, 256k, 512k or 1M card) the following actions

takes place:

a) port 2 is mapped to #C0000

b) the postion of the 00000 marker is determined

c) the library is added to the obj sequence in the

port

d) a new 00000 marker is written

e) port 1 is mapped back to #C0000

Here the bug appears: in ROM rev L & M '48s steps d)

and e) are swapped !

Consequences:

1) If you've a writeprotected card in port 1, only the

00000 marker wasn't written - because there's a

good chance that the remainder of the card isn't

filled with zeros, this will cause the 'Invallid

Card Data' message each time you turn on your '48.

On a rev M just run PINIT in this case. The rev L

don't have the PINIT command, so you should remove

the card in port 1 before storing something into

the card in port 2 (causing that port 2 is mapped

to #C0000 permanently)

2) If you've a writable, FREEd, card in port 1,

additional to the above effect the 00000 marker is

written into port 1 - depending on the fill level

of port 1 this will cause at least a modification

of one object. There may be a lot of different

effects caused by this - and there's a good chance

to loose memory if you access the modified data

(eg. by executing a modified library command or by

restoring a corrupt backup)

3) If you've a writable, MERGEd, card in port 1,

additional to the effects described in 1), the

00000 marker was written to the system RAM - again

this may cause various suspicious effects, the most

likely is to loose RAM later or earlier...

4) The PINIT command on a rev M '48 will trash your

card 1 too. It initializes ports 1-33 by storing a

small object into it and then immediately deleting

the object - the storing operation is performed by

the STO kernel... If you've a multiport card in

port 2, and execute PINIT, a bunch of 00000s may be

scattered all over card 1...

How to 'see' the bug:

Get two empty RAM cards, a memory scanner and a

library. Plug in the cards, and determine the size of

the library w/ DUP BYTES SWAP DROP 2 * R->B, assuming

the result is #1A51, use the memory scanner to examine

the address #C1A51 (this is port 1) and make shure that

there're no 0's. Now store the library into port 2 w/ 2

STO. Launch the scanner again and take a look at #C1A51

- if you see 00000, you got it. For beeing shure you

can additionaly remove the card of port 1, and take a

look at #C1A51 again (this time port 2) - assuming that

there wheren't any 0's before storing the lib, you

won't see the 00000 sequence...

Bye, 8-Detlef