The ORCHDVSA.COM is the VESA Bios extension program to conform with VESA

standards.  The following standard resolutions are supported:

------------------------------------------------------------------------

RESOLUTION:     VESA MODE:              VIDEO MEMORY REQUIREMENT

========================================================================

640x400x256     100h                    256k

640x480x256     101h                    512k

800x600x16      102h or 6Ah             256k

800x600x256     103h                    512k

1024x768x16     104h                    512k

1024x768x256    105h                    1 MB

------------------------------------------------------------------------

The cards supported are:

- Designer VGA  (supports 101h, 102h, 6ah)

- Designer 800 VGA  (supports 101h, 102h, 6ah)

- ProDesigner and ProDesigner Plus VGA  (supports 101h, 102h, 6ah, 103h, 104h)

- ProDesigner/e VGA  (supports 100h, 101h, 102h, 6ah,103h, 104h)

- ProDesigner II  (supports 100h, 101h, 102h, 6ah, 103h, 104h, 105h)

This program automatically detects the amount of memory installed on the

video card, and will only support the modes which fulfill the Video Memory

Requirements outlined on the table above.

Use the following instructions to install the Orchid VESA driver:

1. Copy the ORCHDVSA.COM file to your hard drive, or boot disk.

2. At the Dos prompt type:

ORCHDVSA [press Enter]

3. The VESA BIOS extension is now loaded and ready to work.