TVGA VESA SUPER VGA BIOS EXTENSIONS (Version 1.02)

==================================================

TVGAVESA is designed for TVGA 8800  &  8900, and provides all

functions  which  follow the Video Electronics Standards Association

(VESA) Super VGA BIOS Extension standard #VP891001.  These functions

include:

Function 00H - Return Super VGA information

01H - Return Super VGA mode information

02H - Set Super VGA video mode

03H - Return current video mode

04H - Save/Restore Super VGA video state

05H - CPU Video Memory Window Control

TVGAVESA will  detect  TVGA  board  type  automaticlly.  The

TVGAVESA BIOS supports different groups of Super VGA modes depending

on board type. For details please see the following list:

TVGA Board      Memory Size        Super VGA modes Supported

==================================================================

8800            256K             100H (640x400, 256 colors)

102H (800x600,  16 colors)

6AH (800x600,  16 colors)

8800            512K             100H (640x400, 256 colors)

101H (640x480, 256 colors)

102H (800x600,  16 colors)

104H (1024x768, 16 colors)

6AH (800x600,  16 colors)

8900            256K             102H (800x600,  16 colors)

6AH (800x600,  16 colors)

8900            512K             100H (640x400, 256 colors)

101H (640x480, 256 colors)

102H (800x600,  16 colors)

103H (800x600, 256 colors)

104H (1024x768, 16 colors)

6AH (800x600,  16 colors)

8900            1024K            100H (640x400, 256 colors)

101H (640x480, 256 colors)

102H (800x600,  16 colors)

103H (800x600, 256 colors)

104H (1024x768, 16 colors)

105H (1024x768,256 colors)

6AH (800x600,  16 colors)