SNOOK TUTORIAL: 0.5 keV Ar+-Cu(100) collision

1. Introduction

In this elementary project, named "copper", we shall simulate a single 0.5 keV Ar+ inelastic collision with a small (343 atom) Cu(100) cluster. (This is not a realistic simulation of a Cu crystal, because the target is too small to contain the collision.) The projectile will be directed towards the surface Cu atom at normal incidence, with an impact parameter of 0.1 Å (corresponding to an x-displacement of 0.1 Å, and a zero y-displacement). Our objective will be to generate data which can be used to plot atomic trajectories for any atoms that acquire kinetic energies above 20 eV before 100 fs have elapsed.

2. Creating input files (with Spider)

For this project, we need to create 6 input files. These are already supplied in the \tutorial directory as copper.trg, copper.prj, copper.run, copper.mdl, copper.imp, copper.inl. However, we will summarise the steps needed to create these files from the beginning in this section.

The first thing to do is create a new directory on your hard disk for storing the various files to be generated by this project. Go to this directory and invoke the command:

spider22

(or c:\sk22\bin\spider22 if you did not enter the c:\sk22\bin directory on your PATH statement in autoexec.bat). You can also achieve the same thing in various other ways under Win 95 (e.g. from Explorer).

2.1 Target file (copper.trg)

Select Target|New target|Face-centred cubic|[100] surface from the menus.

In the data dialog box that comes up, enter the following values:

Element: Cu   Lattice constant: 3.6147

Atomic number: 29   Atomic mass: 63.546

x-width: 3   x-origin: 0.0

y-width: 3   y-origin: 0.0

z-depth: 7   z-origin: 0.0

Flags: 0

Then click Ok and save the file as copper.trg.

These inputs will create a target file copper.trg with coordinates of your lattice. It will consist of 343 atoms: 7 atomic <110> rows in the x- and y-directions (surface plane), and 7 layers in the z-direction (surface normal).

(Optional) If you wish, you can examine this target file in an edit window (Edit|Open option), or as a graphical display (Target|Display/Orient).

2.2. Projectile file (copper.prj)

Select the Projectile|New projectile menu option from the menu.

Except for the projectile energy, use the default parameters in the dialog box that comes up, since these are correct for our purposes:

(Ar, 0.5 keV, Z = 18, mass = 39.948, Flags = 0)

Then click Ok and enter copper.prj in the file dialog box that comes up.

2.3. Run file (copper.run)

Select Run|New run from the menu. Move systematically through the dialog box, entering the following data:

Sampling frequency: check the 'periodic' box, and enter '10' for the period. Leave the 'At termination' box unchecked. Explanation: This will provide a 'snapshot' of the system after every 10 timesteps during the simulation.

Output data for: Check the box KE > 10 eV. Explanation: Output data will be recorded only for those particles having KE > 10.0 eV.

Ion incident angles: Use the default settings (polar = 90º, azimuthal = 0º). Explanation: these angles correspond to a normally incident projectile.

Periodic parameters: Set the following values:

Initial timestep = 2.0E-16 s;

Neighbour update =10 timesteps;

Termination time (min.) = 0.0 fs;

Termination time (max.) = 100 fs.

Explanation: (a) The simulation will start off with a time step of 0.2 fs: this value is mainly determined by the speed of the projectile, or the speed that could be attained by a displaced target atom (whichever is the larger). The idea is that no atom should move a great distance in any one timestep. (b) The neighbour update refers to the number of timesteps which elapse before the neighbour lists are updated. No error will result from setting this parameter too high or low. However, if too high, your simulation will waste memory; if too low, your simulation will waste time updating the neighbour lists unnecessarily; (c) Your simulation will terminate unconditionally after 100 fs have elapsed.

Cut-off parameters: We shall use a 3.7 Å cut-off for the potential (atom pairs located more distant than this will exert no influence on each other). The maximum no. of partners parameters will be set at 50: this parameter is used to allocate memory for the neighbour lists (here we assume that no atom will have more than 50 neighbours within its sphere of influence). The sphere of influence (computed by the program) is larger than the 3.7 Å specified for the interaction range because we have to take the range of the particle between updates into account (particles may move into interaction range between updates). The value of 50 partners is quite conservative, reflecting the heuristic nature of the parameter. When you run the simulation, you will find that snook reports that only about 30 of these partner slots were in fact needed.

Save the file (click Ok, enter copper.run in the file dialog box that comes up).

2.4. Model file (copper.mdl)

Select Model | New Model.

We shall use a ZBL screened Coulombic potential for the Ar-Cu interaction, and a Moliere-Lindhard screened Coulombic potential for the short range Cu-Cu interaction. The Cu-Cu interaction at long range (r > 2.1 Å) will be described by a Morse potential. The intermediate distance will be connected by a cubic spline, which is computed automatically once its range (1.5-2.1 Å) is specified. These conditions are SPIDER’s default parameters. To get information on individual parameters, select the dialog box item (input line, checkbox) with the mouse or Tab key, then hit the F1 key. Make sure the following values are set:

Projectile-Target potential: ZBL

Screening length correction: 1.0

Target-Target potential: Moliere-Lindhard

Screening length correction: 0.890869 (from the work of Harrison et al.)

Morse + Spline: Yes

Morse constants: D = 0.480 eV; a = 1.405 Å; r0 = 2.628 Å;

Spline range: Low = 1.5 Å; High = 2.1 Å;

We shall include target vibrational displacement effects in this exercise: Include = Yes; Mass = 63.546 (of Cu); Lattice temperature = 300 K. Debeye temperatures (from literature/online Help): Bulk = 343 K; Surface (perpendicular) = 192 K; Surface (parallel) = 292 K.

We shall ignore the surface energy  and bulk energy parameters. So enter these as 0.0.

We shall terminate the calculations when the energy of all atoms falls below 2.0 eV: Termination energy = 2.0 (Test projectile energy only: leave box unchecked).

Press Enter or click OK to save (as copper.mdl). A dialog box comes up telling you what are the calculated rms vibrational displacements corresponding to your input data (for your information only).

2.5. Impact file (copper.imp)

Select Impact|New

We are going to simulate a single trajectory, normally incident on Cu(100), aimed at a point between 2 atoms in a <110> row. The central Cu atom in the surface is located at (0,0,0) (examine the first line in the TRG file you created). We will examine a trajectory in which the Ar ion is aimed initially at the point (0.1,0.0) (in Å). In other words, bx  = 0.1 and by = 0.0. The impact parameter (b) is therefore ( (bx² + by²) = 0.1 Å).

First, specify the displacements: bx(min) = 0.1; bx(max) = 0.11; number of impacts = 1. The reason we use 0.11 for bx(max) is because the logic of the algorithm dictates that the max. value must be greater than or equal to the min. value. Sometimes if they are equal the logical comparison can fail because of conversion problems, so we increase the maximum value by an arbitrary amount: if the number of impacts is 1, then only the minimum value will be used.

Do the same for the by (min,max) values: 0.0 (min), 0.0 (max.) and 1 (number of impacts).

Keep the (x,y) angle at 90°, and projectile z0 at 3 Å (= vertical start position for projectile).

Click Ok or hit Enter to save the file.

You can use the Impact|Edit command to examine the coordinates in the file you created.

2.6. Inelastic file (copper.inl)

The Inelastic file is optional: simulations can be run using purely elastic models. Even if you do create an Inelastic file, it is still necessary to enable the inelastic loss options in Snook itself (via the simulation options menu command). In this example, we shall assume that the electronic loss can be calculated as an equipartition (i.e. equal scale factors of 0.5) between the (Lindhardt-Scharrf-Schiott) LSS and (Oen-Robinson) OR models.

Select Inelastic|New: a 4-tab dialog box comes up

Select the LSS tab

Click the Add Atom button, and fill out the grid as follows (the KLSS values were computed using the 3rd tab: 'Compute K(LSS), K(OR)'):

|  | Z1 | K(LSS) | Scale |
| --- | --- | --- | --- |
| atom 1 | 18 | 9.51 | 0.5 |
| atom 2 | 29 | 13.32 | 0.5 |

Enter 0.0 for velocity threshold.

Now click the OR tab

Click the add pair button twice

Fill out the grid as follows (for example, pair 1 represents Ar-Ar interactions: 18*18 = 324):

|  | Z1*Z2 | K(OR) | Scale |
| --- | --- | --- | --- |
| pair 1 | 324 | 0.00090 | 0.5 |
| pair 2 | 841 | 0.00157 | 0.5 |
| pair 3 | 522 | 0.00117 | 0.5 |

Enter 1.5 (Å) for the maximum apsidal distance, and 0.3 for the exponential parameter.

You can ignore the Shapiro-Tombrello tab, as we won't be using that model in this example.

Click the Ok button, and save as copper.inl.

The simulation project is now ready to run, so you can close Spider if you wish.

3. Running the simulation (with Snook)

Start Snook.

Select Run|Specify input files; navigate to the directory which contains your input files (e.g. c:\sk22\tutorial).

Click button marked ‘projectile’ and select copper.prj from the file dialog box, then click ok.

You will see that all the other files (copper.trg, copper.imp etc.) also appear in the main dialog box. Notice that the Inelastic file name appears dimmed - this is because you haven't yet specified which inelastic algorithms you want to use. Click ok.

The words ‘ready to run’ appear in the screen window. If you run the simulation at this point, no inelastic loss effects will be included. To include them, click on the Options|Simulation menu item. Look for the Inelastic Loss Models box, and select (check) the 2 boxes corresponding to OR and LSS losses (i.e., the models you set up in Spider).

There are a number of other options available in this dialog box. You can read about them in the online help. For now, leave them at their default values.

Select File|Run (or click the Run speed-button). The simulation begins by initialising variables and outputting information to the screen. To proceed, hit the Enter key, or click Ok with the mouse.

The simulation runs to completion (in a few seconds on a Pentium). Please note that every time you run this simulation again, you will get significantly different results. The reason is because the vibrational displacements are randomised on every new run. Since these displacements are of the same magnitude as the impact parameter, they influence the results considerably in this example project. If you disable this option (using Spider to modify the Model file), you will, however, get the same result from each run.

See the Annexe below for an explanation of the verbose output written by Snook to the screen

If you look on disk after running the simulation, you will find a file named DYNVARS.SNK, which contains the output data. The Win 95 version of Snook will store this file in the directory named in the “File|Specify output file” option (\SK22\BIN by default).

The images below are xz views of the collision as copied from the Graph window (click on the speed-button to open it during or before the simulation) at 26 fs and 101 fs elapsed time respectively (the projectile is indicated by an arrow in the upper figure):

4. Processing output (with Winnow)

We will now run Winnow from within SNOOK using the Winnow menu command. (You can run Winnow independently, if you prefer).

Select Winnow’s Process | Filter option. Enter DYNVARS.SNK as the source file in the file dialog box (use the Browse button to make sure you get the file path correct), then click Ok. Enter 20ev.snk as the destination file in the next dialog box. Click Ok.

Enter [ke/ep > 20.0] as the logical condition in the Filter input line. What this means is that only records satisfying this condition (particle kinetic energy above 20 eV) will be written to your output file (20ev.snk). Here, ke refers to kinetic energy in SI units, while ep is the proton charge. This is an example of Winnow’s query language. Click Ok: the file 20ev.snk is created.

Now select Process|Convert. Enter 20ev.snk as the file name, and click Ok. This will create a file named 20ev.dat which contains a listing of the dynamical variables recorded in 20ev.dat. In particular, the (x, y, z, px, py, pz) coordinates are listed in columns 1-6 respectively (in SI units). The remaining columns refer to the time, the particle mass, the particle 'row number', and the simulation 'run number' respectively. If you wish to examine the trajectories, you can do this with both the SNK file or the DAT file using the trajectory plotter (found on the Utilities menu). Alternatively, you can use your own plotting software to display the trajectories (the x, y, z coordinates respectively are found in the first 3 columns of the 20eV.dat file).

The trajectory plotter lets you step through the data on a frame-by-frame basis (among other options). The xz view on the left (obtained by running the trajectory plotter with the frame refresh option off) shows that only a few atoms in the system acquire energies of 20 eV or greater. The projectile path is arrowed.

The format of the 20eV.dat file is illustrated by the following output:

0.100095  -0.000142   2.018112   0.010648  -0.015284 -32.445780   2.00  6.633E-0026     0     1

0.103008  -0.003743   1.061738   0.405694  -0.483549 -29.177051   4.01  6.633E-0026     0     1

0.206295  -0.123288   0.571409   7.501106  -8.656532  -2.308187   6.02  6.633E-0026     0     1

That concludes the main tutorial.

As a further exercise you could try creating an energy spectrum based on the data written to dynvars.snk. (Select the Process|Spectrum option, and enter dynvars.snk as the input (source) file and “ke/ep” as the independent variable. Set a range of 1.0-100.0 (eV) and 15 channels. The resulting 'spectrum' is shown below (energy in eV on horizontal axis, counts on vertical axis).

This spectrum simply shows the time-averaged distribution of energies (for the various 'snapshots' that were periodically saved to disk). In this case, the modal energy recorded is about 15 eV.

5. Further development of the tutorial

In a real simulation you would not only use a larger target, but you would also study hundreds of trajectories directed into a symmetrically unique zone of the target lattice. The Impact file xcopper.imp which is found in your tutorial, defines a triangular zone containing just 48 such trajectories for illustration. The file was created in Spider using the following parameters: bx(min) = by(min) = 0.0; bx(max) = by(max) = 1.28Å; No. of impacts (both axes) = 10; xy angle = 45; proj. z0 = 3Å.

You can run a scaled-down version of a real simulation by substituting xcopper.imp for copper.imp in Snook's list of input files (File|Specify input files, then click the impact file Select button).

ANNEXE: Snook Screen Output

The following screen output was generated by the Copper project described in this tutorial with one minor change: the Include vibrational displacements option in copper.mdl was disabled (because the random vibrations would otherwise make individual trajectories non-reproducible). Most of the output can be ignored, but it is sometimes useful for diagnostic purposes.    Exceptional conditions (errors, warnings) are signalled by output in bright red. These are not always fatal conditions, but they always merit investigation. For example, you may have indicated that you would like to use the LSS inelastic model, but forgot to enter any parameters into the inelastic file. Snook will flag this with a warning about 'null parameters', but will allow you to continue.

C:\sk22\tutorial\copper.PRJ

C:\sk22\tutorial\copper.trg

C:\sk22\tutorial\copper.run

C:\sk22\tutorial\copper.mdl

C:\sk22\tutorial\copper.imp

--------------------------------------------------------------------------------------------------

SIMULATION PARAMETERS

Output: all particles with KE > 10 eV

Integration algorithm: Verlet

Sampling periodically (every 10 timesteps) but not at termination

Projectile: 0.500 keV Ar+ (Z=18, mass=39.94800, theta = 90.00, phi = 0.00)

Number of atoms in target: 343

General parameters:

- Calculations will start with 0.2000 fs timestep

- Time before termination tests begin: 0.00 fs

- Time before compulsory termination: 100.00 fs

- Testing all particles for termination energy

- Potential cuts off at 3.700 A

- Max. No. of collision partners = 50

- Including inelastic effects: LSS, OR, model(s)

- Ready to run calculations in earnest.

[The preceding information summarises the main parameters of the simulation, as deduced from the input files. A simulation consists of one or more runs. The same output format is repeated for every run. Each run is uniquely identified by a run number, which is counted from 1, and which gets written alongside any simulation output written to disk.]

Run count: 1

Force at cut-off distance: 0.232779 eV/A  (a)

Minimum force parameter: 0.023277 eV/A  (b)

[The preceding parameters are only of real interest if you are using the HGE algorithm. The Morse force between 2 particles separated by the cut-off distance is reported as (a). The parameter (b) is the "Minimum Force" parameter used only with one of the integration algorithms (HGE-B) - in practice, you will never need to use it. An explicit message will inform you if thermal displacements have been added to the atomic coordinates listed in the target file.]

Detected elemental target

[Snook detected that the target was an element. Snook needs to know whether the target is elemental or complex, since it affects the way forces are calculated. You can tell Snook to treat the target as elemental (even if it is not!) via the simulations options dialog box.]

- INITIALISED *

[The following lines give a summary of the system dynamical variables, in the form of net energies, momenta and angular momenta. The locations of particles #0 (projectile) and #1 (anchor target atom) respectively are also reported. Lastly, the (x, y, z) physical dimensions of the lattice region plus a bounding region equal in width to the cut-off distance are reported.  These numbers refer to the coordinates of 2 lattice corners. The lattice region extends from z = 6.7 (= 3.0 + 3.7) Å down to z = -14.7 Å.]

[NB. inelastic energy loss models do not conserve linear and angular momentum: see the file simprimr.doc, chapter 9. The potential energy reported here takes into account the potential at the cut-off distance, so it cannot be directly compared with the cohesive energy of the solid.]

Total System Energy: 56.18517 eV

Potential: -443.814 eV; Kinetic : 500.0000 eV

Px:-0.00000 Py:0.000000 Pz:-3.26007 (*1.0E+21 kg m/s)

Lx:0.000000 Ly:0.326007 Lz:0.000000 (*1.0E+31 Js)

Projectile (x,y,z) coordinates (A): 0.100000 0.000000 3.000000

Projectile (x,y,z) velocities (Mm/s): -0.00000 -0.00000 -0.04914

Particle 1 (x,y,z) coordinates (A): 0.000000 0.000000 0.000000

Particle 1 (x,y,z) velocities (Mm/s): 0.000000 0.000000 0.000000

Lattice zone lower coordinates (A): -11.3679 -11.3679 -14.5441

Lattice zone upper coordinates (A): 12.64593 12.64593 6.700000

[Discrete Oen-Robinson inelastic events are reported below as they occur. The particle indices are given, along with the associated energy loss. The distance of closest approach, before and after the energy subtraction, are also reported.]

OR event, particles: 0 and 1, dE = -4.54495 eV, R(old) = 0.742448 A, R(new) = 0.745853 A

OR event, particles: 1 and 123, dE = -2.03492 eV, R(old) = 1.021796 A, R(new) = 1.024722 A

OR event, particles: 123 and 173, dE = -0.50159 eV, R(old) = 1.431092 A, R(new) = 1.434991 A

OR event, particles: 123 and 172, dE = -0.50200 eV, R(old) = 1.429524 A, R(new) = 1.433401 A

OR event, particles: 1 and 130, dE = -0.44275 eV, R(old) = 1.347024 A, R(new) = 1.349481 A

OR event, particles: 123 and 214, dE = -0.70092 eV, R(old) = 1.266856 A, R(new) = 1.269671 A

OR event, particles: 130 and 137, dE = -0.33682 eV, R(old) = 1.377268 A, R(new) = 1.379375 A

OR event, particles: 207 and 214, dE = -0.34899 eV, R(old) = 1.392431 A, R(new) = 1.394751 A

OR event, particles: 137 and 144, dE = -0.31839 eV, R(old) = 1.396881 A, R(new) = 1.399034 A

* TERMINATED *

Maximum no. of collision partners required was: 31

[The above number shows that only 31 partners needed to be tracked in the interaction neighbour lists, rather than the 50 specified/anticipated in the Run file.]

Projectile energy (outside lattice zone) fell to 44.12408 eV

At last check, KE of fastest particle inside lattice zone was 12.58520 eV (for particle #175)

[Reports projectile energy, fastest lattice atom kinetic energy]

No. of timesteps executed = 198

Timestep at termination = 1.311838 fs

[The timestep is reported: it adapts during the simulation.]

Time elapsed = 100.55 fs.

Now getting final configuration...

No. of ejected atoms (z > 3 A): 0   Sputtered fraction: 0.00

[The dynamical variables are again reported, as at the start of the simulation. The energy loss dE reflects both errors in the integration process, and errors in the OR inelastic energy book-keeping. To isolate the former, you can temporarily disable the inclusion of OR inelastic effects (giving dE = 0.3 eV, i.e. book-keeping error of 1.0 eV). dE is also reported as a fraction of the total system energy (some authors use a different convention). The various inelastic energy losses for this run are also reported.].

Total System Energy: -7.86985 eV

Potential: -347.022 eV; Kinetic : 339.1524 eV

Px:-0.00679 Py:-0.00371 Pz:-2.84511 (*1.0E+21 kg m/s)

Lx:0.014892 Ly:0.508910 Lz:0.000567 (*1.0E+31 Js)

Projectile (x,y,z) coordinates (A): 8.500613 0.000000 11.34153

Projectile (x,y,z) velocities (Mm/s): 0.008669 0.000000 0.011746

Particle 1 (x,y,z) coordinates (A): -2.38670 0.000893 -5.07330

Particle 1 (x,y,z) velocities (Mm/s): 0.000049 0.000005 -0.00021

Lattice zone lower coordinates (A): -11.3679 -11.3679 -14.5441

Lattice zone upper coordinates (A): 12.64593 12.64593 6.700000

[dE is the energy error = integration error + inelastic correction error. This is an important parameter, since it essentially gauges the accuracy of the simulation.]

dE = 0.840689 eV; dE/(KE+PE) = 1.4963 %;  (Cumulative: 1.4963 %)

[The inelastic energy losses from the various channels are also summarised.]

LSS loss (eV): This run: -55.1643, cumulative: -55.1643

OR loss  (eV): This run: -9.73138, cumulative: -9.73138

ST loss  (eV): This run: 0.000000, cumulative: 0.000000