Pickboot, A Multi-boot Utility for x86 PC's

The accompanying utility, pickboot.com, is something I devised when

I was running Unix and still occasionally had to use some persnickety

DOS apps that needed to see real hardware instead of a virtual machine.

It worked well for me.

The source is included, in case somebody wants to improve it.  (or

has to see it to work up the courage to use it.  The purpose, use and

operation of the program is explained in the source code. (not here)

Because I have failed, so far, to make Windows NT run on my machine,

(and because I am unwilling to unevict OS/2 from my machine), I do not

have certain magic numbers for those operating systems.  The utility

must be modified or patched to become able to multi-boot those OS's.

The only change required is to place an "OS signature byte" (this is

my appellation) into a table within the program.

The best way to fix it is to install the correct, non-zero value

into the source at kos_init[2], (and kos_init[3] for 2 more OS's) and

regenerate the program.  (A makefile is suggested in the source.)

For those who do not have Microsoft's macro assembler (v6.0+), the

executable can be patched with debug or a binary file editor.  The

kos_init[] table begins at offset 0x2B2 in memory (when loaded at 100h

as .COM files are) or at offset 0x1B2 in the pickboot.com file.  The

kos_init[] table is ordered such that the Windows NT signature byte

must be at kos_init[2] and the OS/2 signature byte, (if also added),

must be at kos_init[3]. (offsets 0x2B4 and 0x2B5 using debug)

In either case, the signature byte value can be determined by

looking at offset 4 of the partition table entry corresponding to

the partition from which a particular OS may be booted.  For an OS

in the 1st partition (partition[0]), this would be at offset 0x1C2

of sector 0.  For an OS in the 2nd partition ... at offset 0x1D2.

(Note: This is raw sector 0, at head 0, sector 1, cylinder 0.  It

is not visible to debug.)

The signature byte can also be read by running "mboot -v"; the

hexadecimal signature byte value is shown at the end of each line.

I apologize for imposing this chore to make this utility work

for Windows NT.  If somebody wants to mail me the OS signature

values, I will incorporate them into the source and re-post the

(regenerated) pickboot.com executable.

Larry Brasfield

3441 79th Avenue SE

Mercer Island, WA  98040

(206)557-1207 (9-5)

(206)236-2121 (home)

Compuserve, user # 70413,3332