Chapter 1

This section is an alphabetic reference to the functions and data structures provided by Video for Windows for use by video capture applications. There are separate sections for functions, messages, and data structures. The messages and data structures are defined in MSVIDEO.H.

Video Capture Application Reference

Extensions are being added to the video capture functions to make it easier for applications to access video capture drivers. If your application needs video capture services, it should use the extensions rather than these functions. If you are developing video capture device drivers, you might use these functions for testing your drivers.

If you need information on the video capture extensions to develop your application, you can request the latest information from the following group:

Microsoft Corporation 
	Multimedia Systems Group
 	Product Marketing
 	One Microsoft Way
 	Redmond, WA 98052-6399

	FAX: (206) 93MSFAX

# Video Capture Function Reference

This section contains a listing of the functions used by video capture applications. The function definition is given, followed by a description of each parameter.

## Video Capture Function Summary

The following function operates on a single frame:

videoFrame

This function transfers a single frame from or to a video device channel.

The following functions are used to open, close, and communicate with a video capture device:

videoClose

This function closes the specified video device channel.

videoGetErrorText

This function retrieves a description of the error identified by the error number.

videoMessage

This function sends messages to a video device channel.

videoOpen

This function opens a channel on the specified video device.

The following functions control the configuration of a video capture device:

videoConfigure

This function sets or retrieves a configurable driver option.

videoConfigureStorage

This function saves or loads all configurable options for a channel.

videoDialog

This function displays a channel specific dialog box used to set configuration parameters.

videoGetChannelCaps

This function retrieves a description of the capabilities of a channel.

videoGetNumDevs

This function returns the number of MSVIDEO devices installed.

videoUpdate

This function directs a channel to repaint the display.

The following functions control video capture streaming:

videoStreamAddBuffer

This function sends a buffer to a video device channel.

videoStreamFini

This function terminates streaming from the specified device channel.

videoStreamGetError

This function returns the most recent error encountered.

videoStreamGetPosition

This function retrieves the current position of the specified video device channel.

videoStreamInit

This function initializes a video device channel for streaming.

videoStreamPrepareHeader

This function prepares a buffer for video streaming.

videoStreamReset

This function stops streaming on the specified video device channel, returns all video buffers from the driver, and resets the current position to zero.

videoStreamStart

This function starts streaming on the specified video device channel.

videoStreamStop

This function stops streaming on a video channel.

videoStreamUnprepareHeader

This function cleans up the preparation performed by videoStreamPrepareHeader.

## Video Capture Function Alphabetic Reference

videoClose

DWORD videoClose(hVideo)

Syntax

This function closes the specified video device channel.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel. If the function is successful, the handle will be invalid after this call.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_INVALHANDLE

Specified device handle is invalid.

DV_ERR_NONSPECIFIC

The driver failed to close the channel.

If buffers have been sent with videoStreamAddBuffer and they haven’t been returned to the application, the close operation fails. You can use videoStreamReset to mark all pending buffers as done.

Comments

videoOpen, videoStreamInit, videoStreamFini, videoStreamReset

See Also

videoConfigure

DWORD videoConfigure(hVideo, msg, dwFlags, lpdwReturn, 
                                        lpData1, dwSize1, lpData2,

Syntax

This function sets or retrieves a configurable driver option.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

UINT msg

Specifies the option to set or retrieve.

DVM_PALETTE

Indicates a palette is being sent to the driver or retrieved from the driver.

DVM_PALETTERGB555

Indicates an RGB555 palette is being sent to the driver.

DVM_FORMAT

Indicates format information is being sent to the driver or retrieved from the driver.

DWORD dwFlags

Specifies flags for configuring or interrogating the device driver. The following flags are defined:

VIDEO_CONFIGURE_SET

Indicates values are being sent to the driver.

VIDEO_CONFIGURE_GET

Indicates values are being obtainded from the driver.

VIDEO_CONFIGURE_QUERY

This flag is used to determine if the driver supports the option specified by msg. This flag should be combined with either the VIDEO_CONFIGURE_SET or VIDEO_CONFIGURE_GET flag. If this flag is set, the lpData1, dwSize1, lpData2, and dwSize2 parameters are ignored.

VIDEO_CONFIGURE_QUERYSIZE

Returns the size, in bytes, of the configuration option in lpdwReturn. This flag is only valid if the VIDEO_CONFIGURE_GET flag is also set.

VIDEO_CONFIGURE_CURRENT

Requests the current value. This flag is only valid if the VIDEO_CONFIGURE_GET flag is also set.

VIDEO_CONFIGURE_NOMINAL

Requests the nominal value. This flag is only valid if the VIDEO_CONFIGURE_GET flag is also set.

VIDEO_CONFIGURE_MIN

Requests the minimum value. This flag is only valid if the VIDEO_CONFIGURE_GET flag is also set.

VIDEO_CONFIGURE_MAX

Get the maximum value. This flag is only valid if the VIDEO_CONFIGURE_GET flag is also set.

LPDWORD lpdwReturn

Points to a DWORD used for returning information from the driver. If the VIDEO_CONFIGURE_QUERYSIZE flag is set, lpdwReturn is filled with the size of the configuration option.

LPVOID lpData1

Specifies a pointer to message specific data.

DWORD dwSize1

Specifies the size of the lpData1 buffer in bytes.

LPVOID lpData2

Specifies a pointer to message specific data.

DWORD dwSize2

Size of the lpData2 buffer in bytes.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_INVALHANDLE

Specified device handle is invalid.

DV_ERR_NOTSUPPORTED

Function is not supported.

videoOpen, videoMessage

See Also

videoConfigureStorage

DWORD videoConfigureStorage(hVideo, lpstrIdent, dwFlags)

Syntax

This function saves or loads all configurable options for a channel. Options can be saved and recalled for each application or each application instance.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

LPSTR lpstrIdent

Identifies the application or instance. Use an arbitrary string which uniquely identifies your application or instance.

DWORD dwFlags

Specifies flags for the storage. The following flags are defined:

VIDEO_CONFIGURE_GET

Requests that the values be loaded.

VIDEO_CONFIGURE_SET

Requests that the values be saved.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_INVALHANDLE

Specified device handle is invalid.

DV_ERR_NOTSUPPORTED

Function is not supported.

The method used by a driver to save configuration options is device dependent.

Comments

videoOpen

See Also

videoDialog

DWORD videoDialog(hVideo, hWndParent, dwFlags)

Syntax

This function displays a channel-specific dialog box used to set configuration parameters.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

HWND hWndParent

Specifies the parent window handle.

DWORD dwFlags

Specifies flags for the dialog box. The following flag is defined:

VIDEO_DLG_QUERY

If this flag is set, the driver immediately returns zero if it supplies a dialog box for the channel, or DV_ERR_NOTSUPPORTED if it does not.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_INVALHANDLE

Specified device handle is invalid.

DV_ERR_NOTSUPPORTED

Function is not supported.

Typically, each dialog box displayed by this function lets the user select options appropriate for the channel. For example, a VIDEO_IN channel dialog box lets the user select the image dimensions and bit depth.

Comments

videoOpen, videoConfigureStorage

See Also

videoFrame

DWORD videoFrame(hVideo, lpVHdr)

Syntax

This function transfers a single frame from or to a video device channel.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel. The channel must be of type VIDEO_IN or VIDEO_OUT.

LPVIDEOHDR lpVHdr

Specifies a far pointer to an VIDEOHDR structure.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_INVALHANDLE

Specified device handle is invalid.

Use this function with a VIDEO_IN channel to transfer a single image from the frame buffer. Use this function with a VIDEO_OUT channel to transfer a single image to the frame buffer.

Comments

To determine the size of the buffer needed for capturing data, use videoConfigure. Set the size of the buffer in the dwBufferLength field of the VIDEOHDR structure. If the buffer size is not large enough, the function can fail.

videoOpen

See Also

videoGetChannelCaps

DWORD videoGetChannelCaps(hVideo, lpChannelCaps, dwSize)

Syntax

This function retrieves a description of the capabilities of a channel.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

LPCHANNEL_CAPS lpChannelCaps

Specifies a far pointer to a CHANNEL_CAPS structure.

DWORD dwSize

Specifies the size of the CHANNEL_CAPS structure.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_UNSUPPORTED

Function is not supported.

The channel capabilities structure returns the capability information. For example, capability information might include whether or not the channel can crop and scale images.

Comments

videoGetErrorText

DWORD videoGetErrorText(hVideo, wError, lpText, wSize)

Syntax

This function retrieves a description of the error identified by the error number.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel. This might be NULL if the error is not device specific.

UINT wError

Specifies the error number.

LPSTR lpText

Specifies a far pointer to a buffer which is filled with a null-terminated string corresponding to the error number.

UINT wSize

Specifies the length of the buffer pointed to by lpText.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_BADERRNUM

Specified error number is out of range.

If the error description is longer than the buffer, the description is truncated. The returned error string is always null-terminated. If wSize is zero, nothing is copied and the function returns zero.

Comments

videoGetNumDevs

DWORD videoGetNumDevs()

Syntax

This function returns the number of MSVIDEO devices installed.

None

Parameters

Returns the number of MSVIDEO devices listed in the [drivers] section of the SYSTEM.INI file.

Return Value

videoOpen

See Also

videoMessage

DWORD videoMessage(hVideo, wMsg, dwP1, dwP2)

Syntax

This function sends a message to a video device channel.

HVIDEO hVideo

Parameters

Specifies the handle to the device.

UINT wMsg

Specifies the message to send.

DWORD dwP1

Specifies the first parameter for the message.

DWORD dwP2

Specifies the second parameter for the message.

Returns the message specific value returned from the driver.

Return Value

This function is used for configuration messages such as DVM_SRC_RECT and DVM_DST_RECT and device specific messages.

Comments

videoConfigure

See Also

videoOpen

DWORD videoOpen(lphvideo, dwDeviceID, dwFlags)

Syntax

This function opens a channel on the specified video device.

LPHVIDEO lphvideo

Parameters

Specifies a far pointer to a HVIDEO handle. The video capture driver uses this location to return a handle that uniquely identifies the opened video device channel. Use this handle to identify the device channel when calling other video functions.

DWORD dwDeviceID

Identifies the video device to open. The value of dwDeviceID varies from zero to one less than the number of video capture devices installed in the system.

DWORD dwFlags

Specifies flags for opening the device. The following flags are defined:

VIDEO_EXTERNALIN

Specifies the channel is opened for external input. Typically, external input channels capture images into a frame buffer.

VIDEO_EXTERNALOUT

Specifies the channel is opened for external output. Typically, external output channels display images stored in a frame buffer on an auxiliary monitor or overlay.

VIDEO_IN

Specifies the channel is opened for video input. Video input channels transfer images from a frame buffer to system memory buffers.

VIDEO_OUT

Specifies the channel is opened for video output. Video output channels transfer images from system memory buffers to a frame buffer.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_BADDEVICEID

Specified device ID is out of range.

DV_ERR_ALLOCATED

Specified resource is already allocated.

DV_ERR_NOMEM

Unable to allocate or lock memory.

At a minimum, all capture drivers support a VIDEO_EXTERNALIN and a VIDEO_IN channel. Use videoGetNumDevs to determine the number of video devices present in the system.

Comments

videoClose, videoGetNumDevs

See Also

videoStreamAddBuffer

DWORD videoStreamAddBuffer(hVideo, lpvideoHdr, dwSize)

Syntax

This function sends a buffer to a video device channel. After the buffer is filled by the device, the device sends it back to the application.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

LPVIDEOHDR lpvideoHdr

Specifies a far pointer to a VIDEOHDR structure that identifies the buffer.

DWORD dwSize

Specifies the size of the VIDEOHDR structure.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_INVALHANDLE

The device handle specified is invalid.

DV_ERR_UNPREPARED

The lpvideoHdr structure hasn’t been prepared.

The data buffer must be prepared with videoStreamPrepareHeader before it is passed to videoStreamAddBuffer. The VIDEOHDR data structure and the data buffer pointed to by its lpData field must be allocated with GlobalAlloc using the GMEM_MOVEABLE and GMEM_SHARE flags, and locked with GlobalLock.

Comments

To determine the size of the buffer needed for capturing data, use videoConfigure. Set the size of the buffer in the dwBufferLength field of the VIDEOHDR structure. If the buffer size is not large enough, the driver might return DV_ERR_NONSPECIFIC.

videoStreamPrepareHeader

See Also

videoStreamFini

DWORD videoStreamFini(hVideo)

Syntax

This function terminates streaming from the specified device channel.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_INVALHANDLE

The device handle specified is invalid.

DV_ERR_STILLPLAYING

There are still buffers in the queue.

If there are buffers that have been sent with videoStreamAddBuffer that haven’t been returned to the application, this operation will fail. Use videoStreamReset to mark all pending buffers as done.

Comments

Each call to videoStreamInit must be matched with a call to videoStreamFini.

For VIDEO_EXTERNALIN channels, this function is used to halt capturing of data to the frame buffer.

For VIDEO_EXTERNALOUT channels that support overlay, this function is used to disable the overlay.

videoStreamInit

See Also

videoStreamGetError

DWORD videoStreamGetError(hVideo, lpdwErrorID, lpdwErrorValue)

Syntax

This function returns the error most recently encountered.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

LPDWORD lpdwErrorID

Specifies a far pointer to the DWORD to be filled with the error ID.

LPDWORD lpdwErrorValue

Specifies a far pointer to the DWORD to be filled with the number of frames skipped.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_INVALHANDLE

The device handle specified is invalid.

While streaming video data, a capture driver can fill buffers faster than the client application can save the buffers to disk. In this case, the DV_ERR_NO_BUFFERS error is returned in lpdwErrorID and lpdwErrorValue contains a count of the number of frames missed. After receiving this message and returning the error status, the driver should reset its internal error flag to DV_ERR_OK and the count of missed frames to zero.

Comments

Applications should send this message frequently during capture since some drivers which do not have access to interrupts use this message to trigger buffer processing.

videoOpen

See Also

videoStreamGetPosition

DWORD videoStreamGetPosition(hVideo, lpInfo, dwSize)

Syntax

This function retrieves the current position of the specified video device channel.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

LPMMTIME lpInfo

Specifies a far pointer to an MMTIME structure.

DWORD dwSize

Specifies the size of the MMTIME structure.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_INVALHANDLE

Specified device handle is invalid.

Before using videoStreamGetPosition, set the wType field of the MMTIME structure to indicate the time format you desire. After videoStreamGetPosition returns, check the wType field to  determine if the your time format is supported. If not, wType will specify an alternate format. Video capture drivers typically provide the milliseconds time format.

Comments

The position is set to zero when streaming is started with videoStreamStart.

videoStreamInit

DWORD videoStreamInit(hVideo, dwMicroSecPerFrame, dwCallback, dwCallbackInstance, dwFlags)

Syntax

This function initializes a video device channel for streaming.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

DWORD dwMicroSecPerFrame

Specifies the number of microseconds between frames.

DWORD dwCallback

Specifies the address of a callback function or a handle to a window called during video streaming. The callback function or window processes messages related to the progress of streaming.

DWORD dwCallbackInstance

Specifies user instance data passed to the callback function. This parameter is not used with window callbacks.

DWORD dwFlags

Specifies flags for opening the device channel. The following flags are defined:

CALLBACK_WINDOW

If this flag is specified, dwCallback is a window handle.

CALLBACK_FUNCTION

If this flag is specified, dwCallback is a callback procedure address.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_BADDEVICEID

The device ID specified in hVideo is not valid.

DV_ERR_ALLOCATED

The resource specified is already allocated.

DV_ERR_NOMEM

Unable to allocate or lock memory.

If a window is chosen to receive callback information, the following messages are sent to the window procedure function to indicate the progress of video input: MM_DRVM_OPEN at the time of videoStreamInit, MM_DRVM_CLOSE at the time of videoStreamFini, MM_DRVM_DATA when a buffer of image data is available, MM_DRVM_ERROR when an error occurs.

Comments

If a function is chosen to receive callback information, the following messages are sent to the function to indicate the progress of video input: MM_DRVM_OPEN, MM_DRVM_CLOSE, MM_DRVM_DATA, MM_DRVM_ERROR. The callback function must reside in a DLL. You do not have to use MakeProcInstance to get a procedure-instance address for the callback function.

void CALLBACK videoFunc(hVideo, wMsg, dwInstance, dwParam1, dwParam2)

Callback

videoFunc is a placeholder for the application-supplied function name. The actual name must be exported by including it in an EXPORTS statement in the DLL’s module-definition file.

HVIDEO hVideo

Callback Parameters

Specifies a handle to the video device channel associated with the callback.

DWORD wMsg

Specifies the MM_DRVM_ message.

DWORD dwInstance

Specifies the user instance data specified with videoOpen.

DWORD dwParam1

Specifies a parameter for the message.

DWORD dwParam2

Specifies a parameter for the message.

Because the callback is accessed at interrupt time, it must reside in a DLL and its code segment must be specified as FIXED in the module-definition file for the DLL. Any data that the callback accesses must be in a FIXED data segment as well. The callback may not make any system calls except for PostMessage, timeGetSystemTime, timeGetTime, timeSetEvent, timeKillEvent, midiOutShortMsg, midiOutLongMsg, and OutputDebugStr.

Callback Comments

For VIDEO_EXTERNALIN channels, this function is used to initiate capturing of data to the frame buffer.

For VIDEO_EXTERNALOUT channels which support overlay, this function is used to enable the overlay.

videoOpen, videoStreamFini, videoClose

See Also

videoStreamPrepareHeader

DWORD videoStreamPrepareHeader(hVideo, lpvideoHdr, dwSize)

Syntax

This function prepares a buffer for video streaming.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

LPVIDEOHDR lpvideoHdr

Specifies a pointer to a VIDEOHDR structure that identifies the buffer to be prepared.

DWORD dwSize

Specifies the size of the VIDEOHDR structure.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_INVALHANDLE

Specified device handle is invalid.

DV_ERR_NOMEM

Unable to allocate or lock memory.

Use this function after videoStreamInit or after videoStreamReset to prepare the data buffers for streaming data.

Comments

The VIDEOHDR data structure and the data block pointed to by its lpData field must be allocated with GlobalAlloc using the GMEM_MOVEABLE and GMEM_SHARE flags, and locked with GlobalLock. Preparing a header that has already been prepared will have no effect and the function will return zero. Typically, this function is used to insure that the buffer will be available for use at interrupt time.

videoStreamUnprepareHeader

See Also

videoStreamReset

DWORD videoStreamReset(hVideo)

Syntax

This function stops streaming on the specified video device channel and resets the current position to zero. All pending buffers are marked as done and are returned to the application.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_INVALHANDLE

The device handle specified is invalid.

videoStreamReset, videoStreamStop, videoStreamAddBuffer, videoStreamClose

See Also

videoStreamStart

DWORD videoStreamStart(hVideo)

Syntax

This function starts streaming on the specified video device channel.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_INVALHANDLE

The device handle specified is invalid.

videoStreamReset, videoStreamStop, videoStreamAddBuffer, videoStreamClose

See Also

videoStreamStop

DWORD videoStreamStop(hVideo)

Syntax

This function stops streaming on a video channel.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_INVALHANDLE

Specified device handle is invalid.

If there are any buffers in the queue, the current buffer will be marked as done (the dwBytesRecorded field in the VIDEOHDR header will contain the actual length of data), but any empty buffers in the queue will remain there. Calling this function when the channel is not started has no effect, and the function returns zero.

Comments

videoStreamStart, videoStreamReset

See Also

videoStreamUnprepareHeader

DWORD videoStreamUnprepareHeader(hVideo, lpvideoHdr, dwSize)

Syntax

This function cleans up the preparation performed by videoStreamPrepareHeader.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

LPVIDEOHDR lpvideoHdr

Specifies a pointer to a VIDEOHDR structure identifying the data buffer to be cleaned up.

DWORD dwSize

Specifies the size of the VIDEOHDR structure.

Returns zero if the function was successful. Otherwise, it returns an error number. The following errors are defined:

Return Value

DV_ERR_INVALHANDLE

The device handle specified is invalid.

DV_ERR_STILLPLAYING

The structure identified by lpvideoHdr is still in the queue.

This function is the complementary function to videoStreamPrepareHeader. You must call this function before freeing the data buffer with GlobalFree. After passing a buffer to the device driver with videoStreamAddBuffer, you must wait until the driver is finished with the buffer before calling videoStreamUnprepareHeader. Unpreparing a buffer that has not been prepared has no effect, and the function returns zero.

Comments

videoStreamPrepareHeader

See Also

videoUpdate

DWORD videoUpdate(hVideo, hWnd, hDC)

Syntax

This function directs a channel to repaint the display. It applies only to VIDEO_EXTERNALOUT channels.

HVIDEO hVideo

Parameters

Specifies a handle to the video device channel.

HWND hWnd

Specifies the handle of the window to be used by the channel for image display.

HDC hDC

Specifies a handle to a device context.

Returns zero if the function was successful. Otherwise, it returns an error number. The following error is defined:

Return Value

DV_ERR_UNSUPPORTED

Specified message is unsupported.

This message is normally sent whenever the client window receives a WM_MOVE, WM_SIZE, or WM_PAINT message.

Comments

# Video Capture Data Structure Reference

This section lists data structures used by video capture applications. The data structures are presented in alphabetical order. The structure definition is given, followed by a description of each field.

## Video Capture Data Structure Alphabetic Reference

CHANNEL_CAPS

The CHANNEL_CAPS structure is used with videoGetChannelCaps to return the capabilities of a channel to an application.

typedef struct channel_caps_tag {

DWORD  dwFlags;

DWORD  dwSrcRectXMod;

DWORD  dwSrcRectYMod;

DWORD  dwSrcRectWidthMod;

DWORD  dwSrcRectHeightMod;

DWORD  dwDstRectXMod;

DWORD  dwDstRectYMod;

DWORD  dwDstRectWidthMod;

DWORD  dwDstRectHeightMod;

} CHANNEL_CAPS;

The CHANNEL_CAPS structure has the following fields:

Fields

dwFlags

Returns flags giving information about the channel. The following flags are defined:

VCAPS_OVERLAY

Indicates the channel is capable of overlay. This flag is used only for VIDEO_EXTERNALOUT channels.

VCAPS_SRC_CAN_CLIP

Indicates that the source rectangle can be set smaller than the maximum dimensions.

VCAPS_DST_CAN_CLIP

Indicates that the destination rectangle can be set smaller than the maximum dimensions.

VCAPS_CAN_SCALE

Indicates that the source rectangle can be a different size than the destination rectangle.

dwSrcRectXMod

Returns the granularity allowed when positioning the source rectangle in the horizontal direction.

dwSrcRectYMod

Returns the granularity allowed when positioning the source rectangle in the vertical direction.

dwSrcRectWidthMod

Returns the granularity allowed when setting the width of the source rectangle.

dwSrcRectHeightMod

Returns the granularity allowed when setting the height of the source rectangle.

dwDstRectXMod

Returns the granularity allowed when positioning the destination rectangle in the horizontal direction.

dwDstRectYMod

Returns the granularity allowed when positioning the destination rectangle in the vertical direction.

dwDstRectWidthMod

Returns the granularity allowed when setting the width of the destination rectangle.

dwDstRectHeightMod

Returns the granularity allowed when setting the height of the source rectangle.

Some channels can only use source and destination rectangles which fall on 2, 4, or 8 pixel boundaries. Similarly, some channels only accept capture rectangles widths and heights that are multiples of a fixed value. Rectangle dimensions indicated by modulus operators are considered advisory. When requesting a particular rectangle, the application must always check the return value to insure the request was accepted by the driver. For example, if dwDstRectWidthMod is set to 64, the application might try to set destination rectangles with widths of 64, 128, 192, 256, ..., and 640 pixels. The driver might actually support a subset of these sizes and indicates the supported sizes with the return value of the DVM_DST_RECT message. If a channel supports arbitrarily positioned rectangles, with arbitrary sizes, the values above should all be set to 1.

Comments

VIDEOHDR

The VIDEOHDR structure defines the header used to identify a video data buffer.

typedef struct videohdr_tag {

LPSTR  lpData;

DWORD  dwBufferLength;

DWORD  dwBytesUsed;

DWORD  dwTimeCaptured;

DWORD  dwUser;

DWORD  dwFlags;

DWORD  dwReserved[4];

} VIDEOHDR;

The VIDEOHDR structure has the following fields:

lpData

Specifies a far pointer to the video data buffer.

dwBufferLength

Specifies the length of the data buffer.

dwBytesUsed

Specifies the number of bytes used in the data buffer.

dwTimeCaptured

Specifies the time (in milliseconds) when the frame was captured relative to the first frame in the stream.

dwUser

Specifies 32 bits of user data.

dwFlags

Specifies flags giving information about the data buffer. The following flags are defined for this field:

VHDR_DONE

Set by the device driver to indicate it is finished with the data buffer and it is returning the buffer to the application.

VHDR_PREPARED

Set by Windows to indicate the data buffer has been prepared with videoStreamPrepareHeader.

VHDR_INQUEUE

Set by Windows to indicate the data buffer is queued for playback.

VHDR_KEYFRAME

Set by the device driver to indicate a key frame.

dwReserved[4]

Reserved for use by the device driver. Typically, these maintain a linked list of buffers in the queue.