Appendix A

tc "Common Return Codes"§Common Return Codes

This appendix describes the primary and, if applicable, secondary return codes that are common to several APPC verbs.  The return codes are listed in alpha-betical order by environment.

Verb-specific return codes are described for the individual verbs in Chapter 3, "APPC Verbs."

The following primary (primary_rc) and secondary (secondary_rc) return codes are common to all environments.

tc "Return codes"§Return codes

AP_ALLOCATION_ERROR

primary_rc; APPC has failed to allocate a conversation.  The conversation state is set to RESET.xe "Allocation errors"§

This code may be returned through a verb issued after [MC_]ALLOCATE.

The associated secondary return codes are as follows:

AP_ALLOCATION_FAILURE_NO_RETRY

secondary_rc; The conversation cannot be allocated because of a permanent condition, such as a configuration error or session protocol error.  To determine the error, the system administrator should examine the error log file.  Do not attempt to retry the allocation until the error has been corrected.

AP_ALLOCATION_FAILURE_RETRY

secondary_rc; The conversation could not be allocated because of a temporary condition, such as a link failure.  The reason for the failure is logged in the system error log.  Retry the allocation.

AP_CONVERSATION_TYPE_MISMATCH

secondary_rc; The partner LU or TP does not support the conversation type (basic or mapped) specified in the allocation request.

AP_PIP_NOT_ALLOWED

secondary_rc; The allocation request specified PIP data, but either the partner TP does not require this data, or the partner LU does not support it.

AP_PIP_NOT_SPECIFIED_CORRECTLY

secondary_rc; The partner TP requires PIP data, but the allocation request specified either no PIP data or an incorrect number of parameters.

AP_SECURITY_NOT_VALID

secondary_rc; The user ID or password specified in the allocation request was not ac-cepted by the partner LU.

AP_SYNC_LEVEL_NOT_SUPPORTED

secondary_rc; The partner TP does not support the sync_level (AP_NONE or AP_CONFIRM_SYNC_LEVEL) specified in the allocation request, or 
the sync_level was not recognized.

AP_TP_NAME_NOT_RECOGNIZED

secondary_rc; The partner LU does not recognize the TP name specified in the allocation request.

AP_TRANS_PGM_NOT_AVAIL_NO_RETRY

secondary_rc; The remote LU rejected the allocation request because it was unable to start the requested partner TP.  The condition is permanent.  The reason for the error may be logged on the remote node.  Do not retry the allo-cation until the cause of the error has been corrected.

AP_TRANS_PGM_NOT_AVAIL_RETRY  secondary_rc;

The remote LU rejected the allocation request because it was unable to start the requested partner TP.  The condition may be temporary, such as a time-out.  The reason for the error may be logged on the remote node.  Retry the allocation.

AP_COMM_SUBSYSTEM_ABENDED

primary_rc; This primary return code indicates one of the following conditions:

u	The node used by this conversation has abended.

u	The connection between the TP and the node type 2.1 has been broken (which is a LAN error).

u	The Network Access Process at the TP's machine has abended. n

The system administrator should examine the error log to determine the reason for the abend.

AP_COMM_SUBSYSTEM_NOT_LOADED

primary_rc; primary_rc; A required component such as a local node or link service has not been loaded.  Thus, communication cannot take place.  Make sure all components required for APPC support are set for your environment.  Contact the system administrator.

When this return code is used with the [MC_]ALLOCATE verb, it may indi-cate that no communications subsystem could be found to support the local LU.  (For example, the local LU alias specified with the TP_STARTED verb is incorrect or has not been configured.)  Note that if lu_alias or mode name is less than 8 characters, you must ensure that these fields are filled with spaces to the right.  This error is returned if these parameters are not filled with spaces, since there isn't a node available that can satisfy the [MC_]ALLOCATE request.  n

CS/CW

When the [MC_]ALLOCATE verb produces this return code for a Comm Server system configured with multiple nodes, there are two secondary return codes as follows:

CS

X'F0000001'

secondary_rc; No nodes have been started.

X'F0000002

secondary_rc; At least one node has been started, but the local LU (when the TP_STARTED verb is issued) is not configured on any active nodes.  The problem could be either of the following:

u	The node with the local LU is not started

u	The local LU is not configured.  n

primary_rc; When the [MC_]ALLOCATE verb produces this return code for Comm Workstation, there are two secondary return codes as follows:

CW

X'F0000001'

secondary_rc; The local node has not been started.

X'F0000002'

secondary_rc; The local node has been started., but the local LU (when the TP_STARTED verb is issued) is not configured. n

AP_CONV_FAILURE_NO_RETRY

primary_rc; The conversation was terminated because of a permanent condition, such as a session protocol error.  The system administrator should examine the system error log to determine the cause of the error.  Do not retry the conversation until the error has been corrected.

AP_CONV_FAILURE_RETRY

primary_rc; The conversation was terminated because of a temporary error.  Restart the TP to see if the problem occurs again.  If it does, the system administrator should examine the error log to determine the cause of the error.

AP_CONVERSATION_TYPE_MIXED

primary_rc; The TP has issued both basic and mapped verbs.  Only one type can be issued in a single conversation.

AP_DEALLOC_NORMAL

primary_rc; This return code does not indicate an error.

The partner TP issued the [MC_]DEALLOCATE verb with dealloc_type set to one of the following:

u	AP_FLUSH

u	AP_SYNC_LEVEL with the synchronization level of the conversation specified as AP_NONE

AP_PROG_ERROR_NO_TRUNC

primary_rc; The partner TP has issued one of the following verbs while the conversation was in SEND state:

u	SEND_ERROR with err_type set to AP_PROG

u	MC_SEND_ERROR

Data was not truncated.

AP_PROG_ERROR_PURGING

primary_rc; While in RECEIVE, PENDING_POST (OS/2 only), CONFIRM, CONFIRM_SEND, or CONFIRM_DEALLOCATE state, the partner TP issued one of the following verbs:

u	SEND_ERROR with err_type set to AP_PROG

u	MC_SEND_ERROR

Data sent but not yet received is purged.

AP_STACK_TOO_SMALL

primary_rc; APPC was unable to process the verb because it detected that the stack was too small.  The minimum stack size is 2.5 KB.

AP_UNEXPECTED_DOS_ERROR

primary_rc; The operating system has returned an error to APPC while processing an APPC call from the local TP.  The operating system return code is returned through the secondary_rc.  It appears in Intel byte-swapped order.  If the problem persists, consult the system administrator.

The following primary return code (primary_rc) is valid for an OS/2 operating environment.

OS/2

AP_TP_BUSY

primary_rc; The local TP has issued a call to APPC while APPC was processing another call for the same TP.  This may occur if the local TP has multiple threads, and more than one thread is issuing APPC calls using the same tp_id. n

The following primary return code (primary_rc) is valid for both OS/2 and the Windows operating environment.

OS/2—Win

AP_INVALID_VERB_SEGMENT

primary_rc; The VCB extends beyond the end of the data segment that is to contain it.  n

The following primary return code (primary_rc) is returned for mapped conversations.

Mapped

AP_DEALLOC_ABEND

primary_rc; The conversation has been deallocated for one of the following reasons:

u	The partner TP has issued the MC_DEALLOCATE verb with dealloc type set to AP_ABEND.

u	The partner TP has abended, causing the partner LU to send an MC_DEALLOCATE request.  n

The following primary return codes (primary_rc) are returned for basic conversations.

Basic

AP_DEALLOC_ABEND_PROG

primary_rc; The conversation has been deallocated for one of the following reasons:

u	The partner TP has issued the DEALLOCATE verb with 
dealloc_type set to AP_ABEND_PROG.

u	The partner TP has abended, causing the partner LU to send a DEALLOCATE request.

AP_DEALLOC_ABEND_SVC

primary_rc; The conversation has been deallocated because the partner TP issued the DEALLOCATE verb with dealloc_type set to AP_ABEND_SVC.

AP_DEALLOC_ABEND_TIMER

primary_rc; The conversation has been deallocated because the partner TP has issued the DEALLOCATE verb with dealloc_type set to AP_ABEND_TIMER.

AP_PROG_ERROR_TRUNC

primary_rc; In SEND state, after sending an incomplete logical record, the partner TP issued a SEND_ERROR verb with err_type set to AP_PROG.  The local TP may have received the first part of the logical record through a receive verb.

AP_SVC_ERROR_NO_TRUNC

primary_rc; While in SEND state, the partner TP (or partner LU) issued a SEND_ERROR verb with err_type set to AP_SVC.  Data was not truncated.

AP_SVC_ERROR_PURGING

primary_rc; The partner TP (or partner LU) issued a SEND_ERROR verb with err_type set to AP_SVC while in RECEIVE, PENDING_POST (OS/2 only), CONFIRM, CONFIRM_SEND, or CONFIRM_DEALLOCATE state.  Data sent to the partner TP may have been purged.

AP_SVC_ERROR_TRUNC

primary_rc; In SEND state, after sending an incomplete logical record, the partner TP (or partner LU) issued a SEND_ERROR verb.  The local TP may have received the first part of the logical record.