Chapter 1

The Installable Compression Manager (ICM) provides services for applications that want to compress or decompress video image data stored in AVI files. This chapter explains the programming techniques used to access these services.  It covers the following topics:

Using the Installable Compression Manager

·	General information about the ICM and the Video for Windows architecture

·	Information on how to compress and decompress video image data from your application

·	An alphabetic reference to the ICM functions and data structures

Before reading this chapter, you should be familiar with the video services available with Windows. For information about these Windows topics, see the Microsoft Windows Programmer’s Reference.

# Video Compression and Decompression Header Files

The function prototypes, constants, flags, and data structures applications use to access the ICM services are defined in COMPMAN.H and COMPDDK.H.

# ICM Architecture

The ICM is used by the Video for Windows editing tool (VidEdit) and the playback engine (MCIAVI) to handle compression and decompression of image data.   ICM is the intermediary between the application and the actual video compression and decompression drivers.  It is the video compression/decompression drivers that do the real work of compressing and decompressing individual frames of data.

This chapter covers the ICM and the functions a video editing or playback application uses to communicate with it.   For information on the video compression and decompression drivers, see Chapter 10, Video Compression and Decompression Drivers.

As the application makes calls to the ICM to compress or decompress data, the  ICM translates this to a message to be sent to the appropriate compressor or decompressor which does the work of compressing or decompressing the data.   The ICM gets the return from the driver and then returns back to the application.

The ICMAPP sample application illustrates routines that compress data, decompress data, and display a dialog box. You might find the helper functions defined in ICM.C useful in developing your application.

# Using ICM Services

In general, an application performs the following tasks to use ICM services:

·	Locate, open, or install the appropriate compressor or decompressor

·	Configures or obtains configuration information about the compressor or decompressor

·	Uses a series of functions to compress, decompress, and (for decompressors with drawing capabilities) draw the data

These tasks are covered in the following sections. The sample application, ICMApp, shows how to use the ICM services to do all of the above functions to compress and decompress images.

## Error Returned from the ICM Functions

For most ICM functions, return values of less than zero indicate an error. Your application should check these return values to see if the ICM function encounters an error. To keep the example fragments in this chapter simple, many of them do not check for errors. For more complete examples, see the ICMAPP and ICM examples included with the development kit.

## Locating and Opening Compressors and Decompressors

To use ICM, an application must open a compressor or decompressor. If your application does not know about the compressors or decompressors installed on a system, it must find a suitable compressor to open. Once your application finishes with a compressor or decompressor, it closes it to free any resources used for compression or decompression. Your application can use the following functions for finding compressors and decompressors, and opening and closing them:

ICInfo

This function obtains information about compressor or decompressor.

ICOpen

This function opens a compressor or decompressor.

ICClose

This function closes a compressor or decompressor.

ICLocate

This function locates a specific type of compressor or decompressor.

If your application knows the compressor or decompressor it needs, it can open the compressor with the ICOpen function. Your application uses the handle returned by this function to identify the opened compressor or decompressor when it uses other ICM functions. The ICOpen function has the following syntax:

BOOL  ICOpen(fccType, fccHandler,  wMode)

The fccType and fccHandler parameters are four character codes used to describe the type and handler type for the compressor. Compressor and decompressors are identified by two four-character codes. Applications open a specific compressor or decompressor  by using the four-character codes for the type and handler. The first four-character code describes the type of the compressor or decompressor. For video compressors and decompressors, this is always 'vidc'. The second four-character code identifies the specific compression handler type.  For example, this value is 'msvc' for the Video 1 compressor. Your application can use NULL if it does not know this four-character code.

In an AVI file, the stream header contains information about the stream type and the specific handler for that stream. For video streams, the stream type is 'vidc' and the handler type is the appropriate handler four-character code. As in the previous example, Video 1 compressed streams use 'msvc'.

Note:

The wMode parameter specifies flags passed to the compressor or decompressor. For ICOpen, these flags let the compressor or decompressor know why it is opened and they can prepare for subsequent operation. The following flags are defined:

ICMODE_COMPRESS

Advises a compressor it is opened for compression.

ICMODE_DECOMPRESS

Advises a decompressor it is opened for decompression.

ICMODE_DRAW

Advises a decompressor it is opened to decompress an image and draw it directly to hardware.

ICMODE_QUERY

Advises a compressor or decompressor it is opened to obtain information.

If your application does not know which compressors and deompressors are installed on a system, it can use ICInfo to enumerate them. This function has the following syntax:

BOOL  ICInfo(fccType, fccHandler, lpicinfo)

The fccType parameter specifies a four-character code indicating the type of compressor or decompressor. To enumerate the compressors or decompressors, your application specifies an integer for fccHandler. Your application receives return information for integers between  0 and the number of installed compressors or decompressors of the type specified for fccType. The compressor or decompressor returns information about itself in a ICINFO data structure pointed to by lpicinfo. The ICInfo function returns TRUE if it can locate the specified compressor or decompressor.

The following example enumerates the compressors or decompressors in the system to find one that can handle the format of its images. (The example uses ICCompressQuery and ICDecompressQuery to determine if a compressor or decompressor supports the image format. The use of these functions is described in “Compressing Image Data” and “Decompressing Image Data.”)

for (i=0; ICInfo(p->fccType, i, &p->icinfo); i++)

{

hic = ICOpen(p->icinfo.fccType, p->icinfo.fccHandler, ICMODE_QUERY);

if (hic)

{

// skip this compressor if it can't handle the specifed format

if (p->fccType == ICTYPE_VIDEO &&

p->pvIn != NULL &&

ICCompressQuery(hic, p->pvIn, NULL) != ICERR_OK &&

ICDecompressQuery(hic, p->pvIn, NULL) != ICERR_OK)

{

ICClose(hic);

continue;

}

// find out the compressor name.

ICGetInfo(hic, &p->icinfo, sizeof(p->icinfo));

// stuff it into the combo box

n = ComboBox_AddString(hwndC,p->icinfo.szDescription);

ComboBox_SetItemData(hwndC, n, hic);

}

}

Applications can use ICLocate to find a compressor or decompressor of a specific type, and to obtain a handle to it for use in other ICM functions. The ICLocate function has the following syntax:

HIC ICLocate (fccType, fccHandler, lpbiIn, lpbiOut, wFlags)

The fccType and fccHandler parameters are four-character codes used to describe the type and handler type for the compressor. Your application can specify NULL for fccHandler if it does not know the handler type or if it can use any handler type.

The lpbiIn parameter contains a pointer to a BITMAPINFOHEADER structure describing the input format the compressor or decompressor will handle. The lpbiOut parameter contains a pointer to a BITMAPINFOHEADER structure describing the output format desired by the application. If your application does not care what output format is returned by a compressor or decompressor, you can set lpbiOut to NULL. The wFlags parameter indicates the type of operation you want the driver to do: compress, decompress or directly decompress and draw.

For example, the following fragment tries to find a compressor that can compress an 8-bit per pixel bitmap:

BITMAPINFOHEADER    bih;

HIC                 hIC

// inialize the Bitmap structure

bih.biSize = sizeof(BITMAPINFOHEADER);

bih.biWidth = bih.biHeight = 0;

bih.biPlanes = 1;

bih.biCompression = BI_RGB;      // standard RGB bitmap

bih.biBitcount = 8;              // 8bpp format

bih.biSizeImage = 0;

bih.biXPelsPerMeter = bih.biYPelsPerMeter = 0;

bih.biClrUsed = bih.biClrImportant = 256;

hIC = ICLocate (ICTYPE_VIDEO, 0L,

(LPBITMAPINFOHEADER)&bih, NULL, ICMODE_COMPRESS);

The following fragment tries to locate a specific compressor to compress the 8-bit RGB format to an 8-bit RLE format:

BITMAPINFOHEADER    bihIn, bihOut;

HIC                 hIC

// initialize the Bitmap structure

bihIn.biSize = bihOut.biSize = sizeof(BITMAPINFOHEADER);

bihIn.biWidth = bihIn.biHeight = bihOut.biWidth = bihOut.biHeight = 0;

bihIn.biPlanes = bihOut.biPlanes= 1;

bihIn.biCompression = BI_RGB;              //standard RGB bitmap for input

bihOut.biCompression = BI_RLE8;            // 8-bit RLE for output format

bihIn.biBitcount = bihOut.biBitCount = 8;  // 8bpp format

bihIn.biSizeImage = bihOut.biSizeImage = 0;

bihIn.biXPelsPerMeter = bih.biYPelsPerMeter = 
    bihOut.biXPelsPerMeter = bihOut.biYPelsPerMeter = 0;

bihIn.biClrUsed = bih.biClrImportant = 
    bihOut.biClrUsed = bihOut.biClrImportant = 256;

hIC = ICLocate (ICTYPE_VIDEO, 0L,

(LPBITMAPINFOHEADER)&bihIn,

(LPBITMAPINFOHEADER)&bihOut, ICMODE_COMPRESS);

## Installing and Removing Compressors and Decompressors

There are three methods of installing compressors and decompressors. They might be installed during the set-up of Video for Windows or other software relating to Video for Windows. Users can also install compressors and decompressors with the Drivers option of the Control Panel. Applications can also install custom compressors and decompressors functions required for their operation.  Most applications will not need to install or remove compressors or decompressors. Your application can use the following functions for installing and removing compressors and decompressors:

ICInstall

This function installs compressor or decompressor.

ICRemove

This function removes an installed compressor or decompressor.

Compressors and decompressor are usually installed by a setup program or by the user with the Drivers option of the Control Panel. An application might, however, install a compressor directly or install a function as a compressor. In these cases, the application uses ICInstall and ICRemove.

The ICInstall function creates a new entry in the SYSTEM.INI for a compressor or decompressor. After installation, the compressor or decompressor must still be opened. The ICInstall function has the following syntax:

BOOL ICInstall (fccType, fccHandler, lParam, szDesc, wFlags)

The fccType and fccHandler parameters specify four-character codes describing the compressor type and handler type. Flags set in wFlags specify the meaning of lParam. The following flags are defined:

ICINSTALL_DRIVER

Indicates lParam points a null-terminated string containing the name of a compressor or decompressor.

ICINSTALL_FUNCTION

Indicates lParam points to a compressor function.

If you are installing a driver, lParam specifies the name of the driver. If you are installing a function as a compressor or decompressor, use lParam to specify a far pointer to your function. This function should be structured like the DriverProc entry point function used by installable drivers. For more information on the DriverProc entry point function, see Chapter 10, “Video Compression and Decompression Drivers.”

Use the szDesc parameter for a descriptive name for the compressor or decompressor. This information is not used and your application does not have to supply a name.

For example, the following fragment installs the ICSample driver:

result = ICInstall ( ICTYPE_VIDEO, mmioFOURCC('s','a','m','p'),
		(LPARAM)(LPSTR)"icsample.drv", "Sample Codec Driver", ICINSTALL_DRIVER)

The following fragment shows how an application would install a function as a compressor or decompressor.

// This function looks like a DriverProc entry point

LRESULT MyCodecFunction(DWORD dwID, HDRVR hDriver, UINT uiMessage,

LPARAM lParam1, LPARAM lParam2);

// This function installs the MyCodecFunction as a compressor

result = ICInstall ( ICTYPE_VIDEO, mmioFOURCC('s','a','m','p'),

(LPARAM)(FARPROC)&MyCodecFunction, NULL, ICINSTALL_FUNCTION);

Usually the Drivers option of the Control Panel is used to remove a compressor or decompressor. Applications installing a function as a compressor or decompressor must remove the function before the application terminates so other applications do not try to use the function. Applications can use ICRemove to remove the function installed. The ICRemove function has the following syntax:

BOOL ICRemove (fccType, fccHandler, wFlags)

The fccType and fccHandler parameters specify four-character codes describing the compressor type and handler type. The wFlags parameter is not used.

## Configuring Compressors and Decompressors

Applications can configure the compressor or have the compressor display a dialog box to let the user to do the configuration. The following functions are available for these operations:

ICQueryConfigure

Determines if the compressor or decompressor supports a configuration dialog box.

ICConfigure

Displays the configuration dialog box of the compressor or decompressor.

ICGetStateSize

Determines the size of the state data for the compressor or decompressor.

ICGetState

Obtains the state data from the compressor or decompressor.

ICSetState

Sends the state data to the compressor or decompressor.

If practical, your application should let the user configure the compressor with the compressor’s configuration dialog box. Typically, this makes your application independent of the compressor and you do not need to consider all the options available to a compressor.

Your application uses ICQueryConfigure to determine if a compressor can display a configuration dialog box. If the compressor can display a configuration dialog box, your application uses ICConfigure to display it. Both of these functions use the handle your application obtained when it located the compressor. The ICConfigure function also requires a handle to the parent window. Your application can use the following fragment to test for support of the configuration dialog box and display it:

if (ICQueryConfigure(hIC)){

// If compressor handles a configuration dialog box, display it

// using our app window as the parent window.

ICConfigure(hIC, hwndApp);

}

Your application might also directly get and set the state information for a compressor. If your application creates or modifies the state data, it must know the actual layout of the compressor data before restoring a compressor state. Alternatively, if your application obtains state data from a compressor and uses it to restore the state in a subsequent session, it must make sure that it only restores state information obtained from the compressor it is currently using. The following fragments show how to obtain the state data:

if (size > 0) {

dwStateSize = ICGetStateSize(hIC);        // get size of buffer required

h = GlobalAlloc(GHND, dwStateSize);       // allocate data buffer

lpData = GlobalLock(h);                   // lock data buffer

ICGetState(hIC, (LPVOID)lpData, dwStateSize); // get the state data

// Store the state data as required

}

The following fragments show how to restore state data:

ICSetState(hIC, (LPVOID)lpData, dwStateSize); // set the new state data

GlobalUnlock(h);

## Getting Information about Compressors and Decompressors

The following functions can be used to get information about a compressor or decompressor:

ICGetInfo

Obtains general information about the compressor or decompressor.

ICGetDefaultKeyFrameRate

Determines the default key frame rate of a compressor or decompressor.

ICGetDisplayFormat

Determines the 'best' format a compressor or decompressor has for displaying on the screen.

ICGetDefaultQuality

Determines the default quality value of a compressor or decompressor.

To obtain information about a compressor or decompressor, your application can use ICGetInfo. This function fills an ICINFO structure with information about the compressor or decompressor. Your application must allocate the memory for the ICINFO structure and pass a pointer to it in ICGetInfo. The ICINFO structure has the following definition:

typedef struct {

DWORD    dwSize;

DWORD    fccType;

DWORD    fccHandler;

DWORD    dwFlags;

DWORD    dwVersion;

DWORD    dwVersionICM;

char     szName[16];

char     szDescription[128];

char     szDriver[128];

} ICINFO;

The dwSize field contains the size of the ICINFO structure.

The fccType field contains the four-character code 'vidc' for image compression.

The fccHandler field identifies the compressor or decompressor with its four-character code.

The dwVersion field contains the version number of the compression driver.

The dwVersionICM field will contain the ICM version number supported by the compressor or decompressor. This is 1.0 (0x00010000) if the compressor or decompressor is written for Video for Windows 1.0.

The szName field contains the short name of the compressor or decompressor. The name is used in list-boxes for choosing compression methods.

The szDescription field contains the long description for the compressor or decompressor.

The szDriver field contains the actual module name that contains the compressor or decompressor.

The dwFlags field contains flags indicating capabilities of the compressor or decompressor. The following flags are defined:

VIDCF_QUALITY

Indicates the compressor or decompressor supports quality levels.

VIDCF_CRUNCH

Indicates a compressor supports compressing to an arbitrary frame size.

VIDCF_TEMPORAL

Indicates the compressor or decompressor supports inter-frame compression.

VIDCF_DRAW

Indicates decompressor can draw to hardware. (These decompressors support the ICDraw functions.)

VIDCF_FASTTEMPORAL

Indicates a compressor can do temporal compression but it doesn't need previous frame.

Unless your application is looking for a particular compressor or decompressor, the flags give your application the most useful information. Your application can check the flags to determine the capabilities of the compressor or decompressor. For example, if quality is supported, your application might enable a quality selection control in a compression dialog box.

The following fragment shows how to obtain the ICINFO information from a compressor or decompressor:

ICINFO		ICInfo;

ICGetInfo(hIC, &ICInfo, sizeof(ICInfo));

The ICGetDefaultKeyFrameRate and ICGetDefaultQuality functions let your application determine the default key frame rate and default quality value. Both of these functions require only the handle to the compressor or decompressor. The following fragment uses both functions to obtain the default values:

DWORD    dwKeyFrameRate, dwQuality;

dwKeyFrameRate = ICGetDefaultKeyFrameRate(hIC);

dwQuality = ICGetDefaultQuality(hIC);

Your application can use the following functions to display the about dialog box of a compressor or decompressor:

ICQueryAbout

Determines if a compressor or decompressor supports an about dialog box.

ICAbout

Displays the about dialog box of a compressor or decompressor.

The ICQueryAbout function lets your application determine if a compressor or decompressor can display the about dialog box. The ICAbout function actually displays the dialog box. The following examples uses these two functions:

if ( ICQueryAbout(hIC)){

// If the compressor has an about dialog box, show it
    ICAbout(hIC, hwndApp);

}

Compressing Image Data

Your application uses a series of functions to coordinate compressing video data. The coordination involves the following activities:

·	Specifying the input format and determining the compression format

·	Preparing the compressor for compression

·	Compressing the video

·	Ending compression

Your application uses the following functions for these activities:

ICCompress

Compress data.

ICCompressBegin

Prepare compressor driver for compressing data.

ICCompressEnd

Tell the compressor driver to end compression.

ICCompressGetFormat

Determine the output format of a compressor.

ICCompressGetFormatSize

Get the size of the output format data.

ICCompressGetSize

Get the size of the compressed data.

ICCompressQuery

Deterimine if a compressor can compress a specific format.

### Specifying the Input Format and Determining the Compression Format

When your application wants to compress data and the output format is not important, it must first locate a compressor that can handle the input format. When the output format is not important to your application, it can use ICCompressGetFormat to have the compressor suggest a format. If the compressor can produce multiple formats, it returns the format that preserves the greatest amount of information rather than one that compresses to the most compact size. This will preserve image quality if the video data is later edited and recompressed. The ICCompressGetFormat function has the following syntax:

LRESULT  ICCompressGetFormat(hic, lpbiInput, lpbiOutput)

The hic parameter specifies the compressor handle. The lpbiInput parameter specifies a far pointer to a BITMAPINFO structure indicating the format of the input data. The lpbiOutput parameter specifies a far pointer to a buffer used to return the output format suggested by the compressor. Your application can determine the size of the buffer needed for the buffer with ICCompressGetFormatSize.

Your application can use the output format data as the 'strf' chunk in the AVI RIFF file. This data starts out like a BITMAPINFOHEADER data structure. The compressor can include any additional information required to decompress the file after this information. A color table (if used) follows this information. If the compressor has format data following the BITMAPINFOHEADER structure, it updates the biSize field to specify the number of bytes used by the structure and additional data.

The following example fragment shows how an application can determine the output format that a compressor wants to use.

LPBITMAPINFOHEADER	lpbiIn, lpbiOut;

// *lpbiIn must be initialized to the input format

dwFormatSize = ICCompressGetFormatSize(hIC, lpbiIn); // get output buffer size

h = GlobalAlloc(GHND, dwFormatSize);             // allocate format buffer

lpbiOut = (LPBITMAPINFOHEADER)GlobalLock(h);     // lock format buffer

ICCompressGetFormat(hIC, lpbiIn, lpbiOut);       // fill the format information

If your application requires a specific output format, it should use ICCompressQuery to interrogate a compressor to determine if it supports the output format your application suggests. This function has the following syntax:

LRESULT  ICCompressQuery(hic, lpbiInput, lpbiOutput)

The hic parameter specifies the compressor handle. Your application typically obtains this with ICLocate or ICOpen. The lpbiInput and lpbiOutput parameters specify far pointers to the data structures defining the input and output formats your application prefers. If the compressor can handle both formats it returns ICERR_OK. If it cannot handle the formats, it returns ICERR_BADFORMAT. If the compressor returns ICERR_BADFORMAT and the output format is critical to your application, your application will have to find an alternate compressor. If an alternate output format is satisfactory, your application might choose to use ICCompressQuery with the alternate formats to determine if the compressor can handle them. Or your application can use ICCompressGetFormat to have the compressor suggest the output format.

If your application specifies NULL for the lpbiOutput parameter of ICCompressQuery, the compressor will select the output format. Typically, your application specifies NULL when it only wants to know if the compressor can handle the input format. The output format information is not returned to your application.

The following fragment uses ICCompressQuery to determine if a compressor can handle both the input and output format:

LPBITMAPINFOHEADER	lpbiIn, lpbiOut;

// Both *lpbiIn & *lpbiOut must be initialized to the respective formats

if (ICCompressQuery(hIC, lpbiIn, lpbiOut) == ICERR_OK){

// format is supported - use the compressor

}

Your application will also need the size of the data returned from the compressor after compression is complete. Use ICCompressGetSize to obtain the worst case (largest) buffer required by the compressor. The number of bytes returned should be used to allocate a buffer used for subsequent compression of images. The following example determines the buffer size and allocates a buffer of that size:

// find the worst-case buffer size

dwCompressBufferSize = ICCompressGetSize(hIC, lpbiIn, lpbiOut);

// allocate a buffer and get lpOutput to point to it

h = GlobalAlloc(GHND, dwCompressBufferSize);

lpOutput = (LPVOID)GlobalLock(h);

### Initialization for the Compression Sequence

Once your application selects a compressor that handles the input and output formats it needs, it can prepare the compressor to start compressing data. The ICCompressBegin function initializes the compressor. This function requires the compressor handle and the input and output format. It returns ICERR_OK if it initializes properly for the specified formats. If the compressor cannot handle the formats, or if they are incorrect, it returns the error ICERR_BADFORMAT.

### Compressing the Video

The ICCompress function does the actual compression. Your application must use this function repeatedly until all the frames are compressed. This function has the following syntax:

LRESULT  ICCompress(hic, dwFlags, lpbiOutput, lpData, lpbiInput, lpBits, 
                                     lpckid, lpdwFlags, lFrameNum, dwFrameSize, 
                                    dwQuality, lpbiPrev, lpPrev)

The hic parameter specifies the handle to the compressor.

The dwFlags parameter specifies any applicable flags for the compression. Your application can use ICM_COMPRESS_KEYFRAME to have the compressor make the frame a key frame. (A key frame is one that does not require data from a previous frame for decompression.) When this flag is set, compressors use this image as the initial one in a sequence.

The lpbiInput and lpBits parameters specify far pointers to the data structure defining the input format and the location of the input buffer. Similarly, the lpbiOutput and lpData parameters specify far pointers to the data structure defining the output format and the location of the buffer for the output data. Your application must allocate the memory for these buffers. When control returns to your application, it typically stores the compressed data in  lpbiOutput and lpData in a subsequent operation. If your application needs to move the compressed data, it can find the size used for the data in the biSizeImage field in the BITMAPINFO structure specified for lpbiOutput.

The lpckid and lpdwFlags are used for AVI file data returned by the compressor. The lpckid specifies a far pointer to a DWORD used to hold a chunk ID for data in the AVI file. The lpdwFlags specifies a far pointer to a DWORD holding the return flags used in the AVI index. The compressor will set this flag to AVIIF_KEYFRAME to correspond to the ICM_COMPRESS_KEYFRAME flag. The AVIIF_KEYFRAME flag marks the key-frames in the AVI file. If your application creates AVI files, it should save the information returned for these parameters in the file.

The lFrameNum parameter specifies the frame number. Your application provides and, if necessary, increments this information. Compressors use this value to check if frames are being sent out of order when they are doing fast temporal compression. If your application has a frame recompressed, it should use the same frame number used when the frame was first compressed. If your application compresses a still frame image, it can specify zero for lFrameNum.

The dwFrameSize parameter specifies the requested frame size in bytes. If set to zero, the compressor chooses the frame size. If set to a non-zero value, the compressor tries to compress the frame to within the specified size. To obtain the size goal, the compressor might have sacrificed image quality (or made some other trade-off). Compressors recognize the frame size value only if they return the VIDCF_CRUNCH flag for ICGetInfo.

The dwQuality parameter specifies the requested quality value for the frame. Compressors support this only if they set the VIDCF_QUALITY flag for ICGetInfo.

The lpbiPrev and lpPrev parameters specify far pointers to the data structure defining the format and the location of the previous uncompressed image. Compressors use this data if they perform temporal compression (that is, they need the previous frame to compress the current frame). Compressors need this information only if they return the VIDCF_TEMPORAL flag. Compressors returning the VIDCF_FASTTEMPORAL flag can perform temporal compression without the previous frame.

The ICCompress function returns ICERR_OK if successful. Otherwise, it returns an error code.

After your application has compressed its data, it uses ICCompressEnd to notify the compressor that it has finished. To restart compression after using this function, your application must re-initialize the compressor with ICCompressBegin.

The following fragment compresses image data for use in an AVI file. It assumes the compressor does not support VIDCF_CRUNCH or VIDCF_TEMPORAL flags but it does support VIDCF_QUALITY.

DWORD    dwCkID;

DWORD    dwCompFlags;

DWORD    dwQuality;

LONG     lNumFrames, lFrameNum;

// assume dwNumFrames is initialized to the total number of frames

// assume dwQuality holds the proper quality value (0-10000)

// assume lpbiOut, lpOut, lpbiIn and lpIn are all initialized properly.

if (ICCompressBegin(hIC, lpbiIn, lpbiOut) == ICERR_OK){

// If o.k. to start, compress each frame

for ( lFrameNum = 0; lFrameNum < lNumFrames; lFrameNum++){

if (ICCompress(hIC, 0, lpbiOut, lpOut, lpbiIn, lpIn,

&dwCkID, &dwCompFlags, lFrameNum,

0, dwQuality, NULL, NULL) == ICERR_OK){

// Write compressed data the AVI file.

.

.

.

// set lpIn to be the next frame in the sequence

} else {

// handle compressor error

}

}

ICCompressEnd(hIC);    // terminate compression

} else {

// handle error

}

## Decompressing Image Data

Similar to compressing data, your application uses a series of functions to control the decompressor used to decompress the video data. Decompressing data involves the following activities:

·	Specifying the input format and determining the decompression format

·	Preparing to decompress video

·	Decompressing the video

·	Ending decompression

Your application uses the following functions for these activities:

ICDecompress

Decompress data.

ICDecompressBegin

Prepare decompressor for decompressing data.

ICDecompressEnd

Tell decompressor to end decompression.

ICDecompressGetFormat

Determine the output format of a decompressor.

ICDecompressGetFormatSize

Get the size of the output data format.

ICDecompressGetPalette

Get the palette for the output format of a decompressor.

ICDecompressQuery

Determine if a decompressor can decompress a specific format.

Decompression is handled very much like compression except that the input format is a compressed format and the output is a displayable format. The input format for decompression is usually obtained from the video stream header in the AVI file. After determining the input format, your application can use ICLocate or ICOpen to find a decompressor that can handle it.

### Specifying the Input Format and Determining the Decompression Format

Because your application allocates the memory required for decompression, it needs to determine the maximum memory the decompressor can require for the output format. The ICDecompressGetFormatSize function obtains the number of bytes the decompressor uses. This function has the following syntax:

DWORD  ICDecompressGetFormatSize(hic, lpbi)

The hic parameter specifies a handle to a decompressor. The lpbi specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

If your application wants the decompressor to suggest a format, it can use ICDecompressGetFormat to obtain the format. This function has the following syntax:

DWORD  ICDecompressGetFormat(hic, lpbiInput, lpbiOutput)

Like ICDecompressGetFormatSize, the hic and lpbiInput parameters specify a handle to the decompressor and a far pointer to the structure indicating the format of the input data. The decompressor returns its suggested format in the BITMAPINFO structure pointed to by lpbiInput. Your application should check that the decompressor returns ICERR_OK for the return value before accessing the lpbiOutput information. If the decompressor cannot handle the input format, it returns ICERR_BADFORMAT. The following fragment shows how an application can use ICDecompressGetFormat:

LPBITMAPINFOHEADER	lpbiIn, lpbiOut;

// assume *lpbiIn points to the input (compressed) format

dwFormatSize = ICDecompressGetFormatSize(hIC, lpbiIn); // get output 
                                                       // buffer size

h = GlobalAlloc(GHND, dwFormatSize);         // allocate format buffer

lpbiOut = (LPBITMAPINFOHEADER)GlobalLock(h); // lock format buffer

ICDecompressGetFormat(hIC, lpbiIn, lpbiOut); // fill the format information

If your application needs a specific output format, in can use ICDecompressQuery to determine if the decompressor can handle both the input and output format. This function uses the same parameters as ICDecompressGetFormat except that your application sets lpbiOutput to point at the structure defining the desired output format. If your application is just determining if the decompressor can handle the input format, it can specify NULL for lpbiOutput. The following fragment shows how an application can use this function:

LPBITMAPINFOHEADER	lpbiIn, lpbiOut;

// assume both *lpbiIn & *lpbiOut are initialized to the respective formats

if (ICDecompressQuery( hIC, lpbiIn, lpbiOut) == ICERR_OK){

// format is supported - use the decompressor

}

If your application creates its own format, it must also obtain a palette for the bitmap. (Most applications use standard formats and do not need to obtain a palette.) Your application can obtain the palette with ICDecompressGetPalette. This function has the following syntax:

DWORD  ICDecompressGetPalette(hic, lpbiInput, lpbiOutput)

Like the other functions, hic and lpbiInput specify a handle to a decompressor and point to a BITMAPINFO structure indicating the format of the input data. The lpbiOutput parameter points to a BITMAPINFO structure used to return the color table. The space reserved for the color table must have an entire 256 color palette table reserved at the end of the structure. The following fragment shows how to get the palette information:

ICDecompressGetPalette(hIC, lpbiIn, lpbiOut);

// move up to the palette

lpPalette = (LPBYTE)lpbiOut + lpbi->biSize;

### Initialization for the Decompression Sequence

Once your application selects a decompressor that handles the input and output formats it needs, it can prepare the decompressor to start decompressing data. The ICDecompressBegin function initializes the compressor. This function requires the compressor handle and the input and output format. It returns ICERR_OK if it initializes properly for the specified formats. If the compressor cannot handle the formats, or if they are incorrect, it returns the error ICERR_BADFORMAT.

### Decompressing the Video

The ICDecompress function does the actual decompression. Your application must use this function repeatedly until all the frames are decompressed. This function has the following syntax:

DWORD  ICDecompress(hic, dwFlags, lpbiFormat, lpData, lpbi, lpBits)

The hic parameter specifies the handle to the decompressor.

The dwFlags parameter specifies any applicable flags for decompression. If your video presentation is starting to lag other components (such as audio), your application can use ICM_DECOMPRESS_HURRYUP to have the decompressor decompress at a faster rate. To speed up decompression, a decompressor might extract only the information it needs to decompress the next frame and not fully decompress the current frame. Thus, when your application uses this flag, it should not try to draw the decompressed data.

The lpbiFormat and lpData parameters specify far pointers to the data structure defining the input format and the location of the input buffer. Similarly, the lpbi and lpBits parameters specify far pointers to the data structure defining the output format and the location of the buffer for the output data. Your application must allocate the memory for these buffers. When control returns to your application, it will use the information in  lpbi and lpBits for subsequent processing of the decompressed data.

The ICDecompress function returns ICERR_OK if successful. Otherwise, it returns an error code.

After your application has decompressed its data, it uses ICDecompressEnd to notify the decompressor that it has finished. To restart decompression after using this function, your application must re-initialize the decompressor with ICDecompressBegin. The following fragment shows how an application can initialize a decompressor, decompress a frame sequence, and terminate decompression:

LPBITMAPINFOHEADER  lbpiIn, lpbiOut;

LPVOID              lpIn, lpOut;

LONG                lNumFrames, lFrameNum;

// assume lpbiIn and lpbiOut are initialized to the input and output format

// and lpIn and lpOut are pointing to the data buffers.

if (ICDecompressBegin(hIC, lpbiIn, lpbiOut) == ICERR_OK){

for (lFrameNum = 0; lFrameNum < lNumFrames, lFrameNum++){

if (ICDecompress(hIC, 0, lpbiIn, lpIn, lpbiOut, lpOut) == ICERR_OK){

// frame decompressed OK so we can process it as required

} else {

// handle decompression error

}

}

ICDecompressEnd(hIC);

} else {

// handle error for decompression initialization

}

## Using Hardware Drawing Capabilities

Some decompressors have the ability to draw directly to video hardware as they decompress video frames. These decompressors return the VIDCF_DRAW flag in response to ICGetInfo. When using this type of decompressor, your application does not have to handle the decompressed data. It lets the decompressor retain the decompressed data for drawing. The following functions are used to for decompressing and drawing with decompressors that have drawing capabilities:

ICDrawBegin

This function prepares a decompressor for drawing.

ICDrawEnd

This function stops a decompressor’s drawing operations.

ICDrawFlush

This function flushes the buffers in the decompressor.

ICDrawQuery

This function determines if the decompressor can render data in a specific format.

ICDrawStart

This function starts the internal clock a decompressor uses for drawing.

ICDrawStop

This function stops the internal clock a decompressor uses for drawing.

ICGetBuffersWanted

This function determines the pre-buffering requirements of a compressor.

If your application uses a decompressor with drawing capabilities, it must handle the following activities:

·	find a decompressor that can decompress and draw a bitmap with the input format specified

·	prepare for decompression

·	decompress data

·	terminate the decompression process

### Specifying the Input Format

Since your application no longer needs to draw the final data, it does not need to be concerned with the output format. However, it must make sure the decompressor can draw the input format. Your application can use ICDrawQuery to determine if a decompressor can handle the input format. While this function can determine if a deompressor can handle the format, it does not determine if the a decompressor has all the capablities needed to draw a bitmap. If your application is uncertain if the decompressor can render the bitmap as required, use this function with ICDrawBegin The following section describes ICDrawBegin. The following fragment shows how to check the input format with ICDrawQuery:

// lpbiIn points to BITMAPINFOHEADER structure indicating the input format

if (ICDrawQuery(hIC, lpbiIn) == ICERR_OK){

// decompressor recognizes the input format

} else {

// we need a different decompressor

}

### Preparing to Decompress Video

The ICDrawBegin function initializes a decompressor and it informs the decompressor about the destination of drawing. The ICDrawBegin function has the following syntax:

DWORD ICDrawBegin(hIC, dwFlags, hPal, hwnd, hdc, xDst, yDst, dxDst, dyDst,
								lpbi, xSrc, yScr, dxSrc, dySrc, dwRate, dwScale)

The hIC parameter contains the handle to the decompressor. The dwFlags parameter specifies any applicable flags. The following flags are defined for this function:

ICDRAW_QUERY

Use to determine if the decompressor can handle the decompression. The decompressor does not draw when this flag is used.

ICDRAW_FULLSCREEN

Indicates that the decompressor will draw to the full screen rather than to a window.

ICDRAW_HDC

Indicates that the decompressor will use a window and display context for drawing.

The hPal parameter specifies a handle to the palette used for drawing. Decompressor ignore this information and your application can set it to null.

The hwnd and hdc parameters define the window and display context used for drawing. Your application must set these values if it uses the ICDRAW_HDC flag.

The xDst, yDst, dxDst and dyDst parameters define the destination rectangle used for drawing. Specify the destination rectangle values relative to the current window or display context. Your application should set these parameters to the desired destination rectangle if it uses ICDRAW_HDC. It can set them to zero if it uses the ICDRAW_FULLSCREEN flag.

The xSrc, ySrc, dxSrc, and dySrc parameters specify the source rectangle used for clipping the frames of the image. The decompressor will stretch the rectangle specified as the source into the rectangle specified by the destination when drawing.

The lpbi parameter should contain a pointer to the BITMAPINFO structure for the input format. Your application uses the dwRate and dwScale parameters to specify the decompression rate. The integer value specified for dwRate divided by the integer value specified for dwScale defines the play rate in frames per second. This is used by the decompressor when it is responsible for timing of frames on playback.

The following fragment shows the initialization sequence to have the decompressor draw full screen:

// assume lpbiIn has the input format, dwRate has the data rate

if (ICDrawBegin(hIC, ICDRAW_QUERY|ICDRAW_FULLSCREEN, NULL, NULL,

NULL, 0, 0, 0, 0, lpbiIn, 0, 0, 0, 0, dwRate, dwScale) == ICERR_OK){

// decompressor supports this drawing so set up to draw.

ICDrawBegin(hIC, ICDRAW_FULLSCREEN, hPal, NULL, NULL, 0, 0, 0, 0, lpbiIn,

0, 0, lbpi->biWidth, lpbi->biHeight, dwRate, dwScale);

// we're ready to start decompressing and drawing frames now

// drawing done so terminate

ICDrawEnd(hIC);

} else {

// do drawing myself

}

Some decompressors buffer the compressed data for more efficient operation. Your application can use ICGetBuffersWanted to determine how many data frames it should send to the decompressor before it has the decompressor draw them.

### Decompressing the Video

The ICDraw function has the decompressor do the actual decompression. This function has the following syntax:

LRESULT ICDraw(hIC, dwFlags, lpFormat, lpData, cbData, lTime)

The hIC is the handle to the decompressor. The dwFlags are flags set by the application and used by the decompressor. These flags can be:

ICDRAW_HURRYUP

Tell the decompressor to decompress at a faster rate.

ICDRAW_UPDATE

Tell the decompressor to update the screen based on the last data received. In this case the lpData parameter should be NULL.

The lpFormat parameter specifies a pointer to the format of the input data. The lpData parameter contains the actual data to be decompressed and later drawn.

The cbData parameter specifies the number of bytes in lpData.

The lTime parameter specifies the time to draw this frame. The decompressor divides this integer by the time scale specified with ICDrawBegin obtain the actual time. Time for the ICDraw functions is relative to ICDrawStart. (That is, ICDrawStart sets the clock to zero.) For example, if your applications specifies 1000 for the time scale and 75 for lTime, the decompressor draws the frame 75 milliseconds into the sequence.

The decompressor starts decompressing data in response to ICDraw, however, it does not start drawing data until your application calls ICDrawStart. (Your application should not use ICDrawStart until it has sent the number of frames the decompressor returned for ICGetBuffersWanted.) When your application uses ICDrawStart, the decompressor begins to draw the frames at the rate specified by dwRate specified with the ICDrawBegin. Drawing continues until your application stops the decompressor drawing clock with ICDrawStop. The following fragment uses the ICDraw functions:

DWORD		dwNumBuffers;

// find out how many buffers need filling before drawing starts
ICGetBuffersWanted(hIC, &dwNumBuffers);

for (dw = 0; dw < dwNumBuffers; dw++){

ICDraw(hIC, 0, lpFormat, lpData, cbData, dw); // fill the pipeline

    // Point lpFormat and lpData to next format and data buffer

}

ICDrawStart(hIC); // start the clock

while (fPlaying){

ICDraw(hIC, 0, lpFormat, lpData, chData, dw); // fill more buffers

// Point lpFormat and lpData to next format and data buffer, update dw

}

ICDrawStop(hIC);  // when done stop drawing and decompressing

ICDrawFlush(hIC); // flush any existing buffers

ICDrawEnd(hIC);   // end decompression

### Controlling Drawing Parameters

The following functions provide more control over decompressors that can draw the decompressed data:

ICDrawGetTime

This function obtains the current time from the decompressor.

ICDrawRealize

This function has the decompressor realize the palette used for drawing.

ICDrawSetTime

This function sets the value of the internal clock for the decompressor.

ICDrawWindow

This function has the decompressor redraw the window.

If your application wants to monitor or change the clock of the decompressor, it can use ICDrawGetTime and ICDrawSetTime. If your application wants to change the playback position while the decompressor is drawing, it can use ICDrawWindow for repositioning the decompressor. If the playback window gets a palette realize message, your application must call ICDrawRealize to have the decompressor realize the palette again for playback.

Video Compression and Decompression Application Reference

This section is an alphabetic reference to the functions and data structures provided by ICM for applications using video compression and decompression services. There are separate sections for functions and data structures. The COMPMAN.H and COMPDDK.H files define the functions and data structures.

## Video Compression and Decompression Function Reference

Applications use the following functions for compressing video data:

ICCompress

This function compresses a single video image.

ICCompressBegin

This functions prepares a compressor for compressing data.

ICCompressEnd

This function tells a compressor to end compression.

ICCompressGetFormat

This function determines the output format of a compressor.

ICCompressGetFormatSize

This function obtains the size of the output format data.

ICCompressGetSize

This function obtains the size of the compressed data.

ICCompressQuery

This function determines if a compressor can compress a specific format.

Applications use the following functions for decompressing video data:

ICDecompress

The function decompresses a single video frame.

ICDecompressBegin

This functions prepares a decompressor for decompressing data.

ICDecompressEnd

This function tells a decompressor to end decompression.

ICDecompressGetFormat

This function determines the output format of a decompressor.

ICDecompressGetFormatSize

This function obtains the size (in bytes) of the output format data.

ICDecompressGetPalette

This function obtains the palette for the output format of a decompression.

ICDecompressQuery

This function determines if a decompressor can decompress data with a specific format.

ICDecompressQuery

This function determines if a decompressor can render a specific format.

Applications use the following functions to control video decompressors that draw directly to the display:

ICDraw

This function decompresses an image for drawing.

ICDrawBegin

This function is used to start decompressing data directly to the screen.

ICDrawEnd

This function tells a decompressor to end drawing.

ICDrawFlush

This function flushes the image buffers used for drawing.

ICDrawGetTime

This function obtains the current value of the internal clock if the decompressor is handling the timing of drawing frames.

ICDrawRealize

This function tells  decompressor to realize its palette used while drawing.

ICDrawSetTime

This function sets the value of the internal clock if the decompressor is handling the timing of drawing frames.

ICDrawStart

This function tells a decompressor to start its internal clock for the timing of drawing frames.

ICDrawStop

This function tells a decompressor to stop its internal clock used for the timing of drawing frames.

ICDrawWindow

This function tells a decompressor to redraw the window when it has moved.

ICGetBuffersWanted

This function obtains information about the pre-buffering needed by a compressor.

Applications use the following functions to obtain information about a compressor or decompressor and display its dialog boxes:

ICQueryAbout

This function determines if a compressor supports an about dialog box.

ICAbout

This function instructs a compressor to display its about dialog box.

ICQueryConfigure

This functions determines if a compressor supports a configuration dialog box.

ICConfigure

This function displays the configuration dialog box of the specified compressor.

ICGetInfo

This function asks a compressor for information about itself.

ICInfo

This function returns information about specific installed compressors, or it enumerates the compressors installed.

ICGetDefaultKeyFrameRate

This function obtains the default key frame rate value.

ICGetDisplayFormat

Given an input format and optionally an open compressor handle, finds the "best" format it can for displaying on the screen.

Applications use the following functions to set and retrieve the state information of a compressor or decompressor:

ICGetState

This function gets the state of a compressor.

ICGetStateSize

This function gets the size of the state data used by a compressor.

ICSetState

This function sets the state of a compressor.

Applications use the following functions to locate, open, and close a compressor or decompressor:

ICOpen

This function opens a compressor or decompressor.

ICClose

This function closes a compressor or decompressor.

ICLocate

This function finds a compressor with specific attributes.

Applications use the following functions to install and remove a compressor or decompressor and send messages directly to it:

ICInstall

This function installs a new compressor.

ICRemove

This function removes a compressor function installed ICInstalled.

ICSendMessage

This  function sends a message to a compressor.

# Video Compression and Decompression Functions

This section contains an alphabetical list of the functions applications can use for compressing and decompressing video data. The functions are identified with the prefix IC.

ICAbout

LRESULT  ICAbout(hic, hwnd)

Syntax

This function instructs a compressor or decompressor to display its about dialog box.

HIC  hic

Parameters

Specifies the handle to the installable compressor.

HWND  hwnd

Specifies a handle to the parent window.

Returns ICERR_OK after the compressor or decompressor displays the about dialog box. It returns ICERR_UNSUPPORTED if it does not support an about dialog box.

Return Value

ICQueryAbout

See Also

ICClose

LRESULT  ICClose(hic)

Syntax

This function closes a compressor or decompressor.

HIC  hic

Parameters

Specifies a handle to a compressor or decompressor.

Returns ICERR_OK if successful, otherwise it returns an error number.

Return Value

ICLocate ICOpen

See Also

ICCompress

LRESULT  ICCompress(hic, dwFlags, lpbiOutput, lpData, lpbiInput, lpBits, lpckid, lpdwFlags, lFrameNum, dwFrameSize, dwQuality, lpbiPrev, lpPrev)

Syntax

This function compresses a single video image.

HIC  hic

Parameters

Specifies the handle of the compressor to use.

DWORD  dwFlags

Specifies applicable flags for the compression. The following flag is defined:

ICM_COMPRESS_KEYFRAME

Indicates that the compressor should make this frame a key frame.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure holding the output format.

LPVOID  lpData

Specifies a far pointer to output data buffer.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure containing the input format.

LPVOID  lpBits

Specifies a far pointer to the input data buffer.

LPDWORD  lpckid

Specifies a far pointer to a DWORD used to hold a chunk ID for data in the AVI file.

LPDWORD  lpdwFlags

Specifies a far pointer to a DWORD holding the return flags used in the AVI index. The following flag is defined:

AVIIF_KEYFRAME

Indicates this frame is a key-frame.

LONG  lFrameNum

Specifies the frame number.

DWORD  dwFrameSize

Specifies the requested frame size in bytes. If set to zero, the compressor chooses the frame size.

DWORD  dwQuality

Specifies the requested quality value for the frame.

LPBITMAPINFOHEADER  lpbiPrev

Specifies a far pointer to a BITMAPINFO structure holding the previous frame's format.

LPVOID  lpPrev

Specifies a far pointer to the previous frame's data buffer.

This function returns ICERR_OK if successful. Otherwise, it returns an error code.

Return Value

The lpData buffer should be large enough to hold a compressed frame. You can obtain the size of this buffer by calling ICCompressGetSize.

Comments

Set the dwFrameSize parameter to a requested frame size only if the compressor returns the VIDCF_CRUNCH flag in response to ICGetInfo. Without this flag, set this parameter to zero.

Set the dwQuality parameter to a quality value only if the compressor returns the VIDCF_QUALITY flag in response to ICGetInfo. Without this flag, set this parameter to zero.

ICCompressBegin, ICCompressEnd, ICCompressGetSize, ICGetInfo

See Also

ICCompressBegin

LRESULT  ICCompressBegin(hic, lpbiInput, lpbiOutput)

Syntax

This function prepares a compressor for compressing data.

HIC  hic

Parameters

Specifies a handle to a compressor.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure holding the input format.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure holding the output format.

Returns ICERR_OK if the specified compression is supported, otherwise it returns ICERR_BADFORMAT if either the input or output format is not supported.

Return Value

ICCompress, ICCompressEnd, ICDecompressBegin, ICDrawBegin

See Also

ICCompressEnd

LRESULT  ICCompressEnd(hic)

Syntax

This function ends compression by a compressor.

HIC  hic

Parameters

Specifies a handle to the compressor.

Returns ICERR_OK if successful, otherwise it returns an error number.

Return Value

ICCompressBegin, ICCompress, ICDecompressEnd, ICDrawEnd

See Also

ICCompressGetFormat

LRESULT  ICCompressGetFormat(hic, lpbiInput, lpbiOutput)

Syntax

This function determines the output format of a compressor.

HIC  hic

Parameters

The compressor handle.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure used to return the output format.

Returns the size of the output format.

Return Value

ICCompressGetFormatSize

See Also

ICCompressGetFormatSize

LRESULT  ICCompressGetFormatSize(hic, lpbi)

Syntax

This function obtains the size of the output format data.

HIC  hic

Parameters

Specifies a handle to a compressor.

LPBITMAPINFOHEADER  lpbi

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

Returns the size of the output data format structure.

Return Value

Use this function to determine the size of the output format buffer you need to allocate when using ICCompressGetFormat.

Comments

ICCompressGetFormat

See Also

ICCompressGetSize

LRESULT  ICCompressGetSize(hic, lpbiInput, lpbiOutput)

Syntax

This function obtains the size of the compressed data.

HIC  hic

Parameters

Specifies a handle to a compressor.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the output format.

Returns the maximum number of bytes a single compressed frame can occupy.

Return Value

ICCompressQuery, ICCompressGetFormat

See Also

ICCompressQuery

LRESULT  ICCompressQuery(hic, lpbiInput, lpbiOutput)

Syntax

This function determines if a compressor can compress a specific format.

HIC  hic

Parameters

Specifies the compressor handle.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure indicating the input data.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the data output. If NULL, then any output format is acceptable.

Returns ICERR_OK if the compression is supported, otherwise it returns ICERR_BADFORMAT.

Return Value

ICCompressGetFormat

See Also

ICConfigure

LRESULT  ICConfigure(hic, hwnd)

Syntax

This function displays the configuration dialog box of a compressor.

HIC  hic

Parameters

Specifies a handle to the compressor.

HWND  hwnd

Specifies a handle to the parent window.

Returns ICERR_OK after the configuration dialog box is displayed. It returns ICERR_UNSUPPORTED if the compressor does not support a a configuration dialog box.

Return Value

ICQueryConfigure

See Also

ICDecompress

LRESULT  ICDecompress(hic, dwFlags, lpbiFormat, lpData, lpbi, lpBits)

Syntax

The function decompresses a single video frame.

HIC  hic

Parameters

Specifies a handle to the decompressor to use.

DWORD  dwFlags

Specifies any applicable flags for decompression. The following flag is defined:

ICDECOMPRESS_HURRYUP

Indicates the decompressor should try to decompress at a faster rate. When an application uses this flag, it should not draw the decompressed data.

LPBITMAPINFOHEADER  lpbiFormat

Specifies a far pointer to a BITMAPINFO structure containing the format of the compressed data.

LPVOID  lpData

Specifies a far pointer to the input data.

LPBITMAPINFOHEADER  lpbi

Specifies a far pointer to a BITMAPINFO structure containing the output format.

LPVOID  lpBits

Specifies a far pointer to a data buffer for the decompressed data.

Returns ICERR_OK on success, otherwise it returns an error code.

Return Value

The lpBits parameter should point to a buffer large enough to hold the decompressed data. Applications can obtain the size of this buffer with ICDecompressGetSize.

Comments

ICDecompressBegin, ICDecompressEnd, ICDecompressGetSize

See Also

ICDecompressBegin

LRESULT  ICDecompressBegin(hic, lpbiInput, lpbiOutput)

Syntax

This function prepares a decompressor for decompressing data.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the output data.

Returns ICERR_OK if the specified decompression is supported, otherwise it returns ICERR_BADFORMAT if either the input or output format is not supported.

Return Value

ICDecompress, ICDecompressEnd, ICCompressBegin, ICDrawBegin

See Also

ICDecompressEnd

LRESULT  ICDecompressEnd(hic)

Syntax

This function tells a decompressor to end decompression.

HIC  hic

Parameters

Specifies a handle to a decompressor.

Returns ICERR_OK if successful, otherwise it returns an error number.

Return Value

ICDecompressBegin, ICDecompress, ICCompressEnd, ICDrawEnd

See Also

ICDecompressGetFormat

LRESULT  ICDecompressGetFormat(hic, lpbiInput, lpbiOutput)

Syntax

This function determines the output format of a decompressor.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure used to return the format of the output data.

Returns the size (in bytes) of the output format.

Return Value

ICDecompressGetFormatSize

See Also

ICDecompressGetFormatSize

LRESULT  ICDecompressGetFormatSize(hic, lpbi)

Syntax

This function obtains the size (in bytes) of the output format data.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPBITMAPINFOHEADER  lpbi

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

Returns the size of the output data format structure.

Return Value

Use this function before ICDecompressGetFormat to find the size needed to allocate the output format buffer.

Comments

ICDecompressGetFormat

See Also

ICDecompressGetPalette

LRESULT  ICDecompressGetPalette(hic, lpbiInput, lpbiOutput)

Syntax

This function obtains the palette for the output format of a decompression.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure used to return the color table. The space reserved for the color table must be at least 256 bytes.

Returns the size of the output format or an error code.

Return Value

ICDecompressGetFormat

See Also

ICDecompressQuery

LRESULT  ICDecompressQuery(hic, lpbiInput, lpbiOutput)

Syntax

This function determines if a decompressor can decompress data with a specific format.

HIC  hic

Parameters

Specifies a handle to to a decompressor.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

LPBITMAPINFOHEADER  lpbiOutput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the output data. If NULL, any output format is acceptable.

Returns ICERR_OK if the decompression is supported, otherwise it returns ICERR_BADFORMAT.

Return Value

ICDecompressGetFormat

See Also

ICDecompressQuery

LRESULT  ICDecompressQuery(hic, lpbiInput)

Syntax

This function determines if a decompressor can render a specific format.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPBITMAPINFOHEADER  lpbiInput

Specifies a far pointer to a BITMAPINFO structure indicating the format of the input data.

Returns ICERR_OK if the decompression is supported, otherwise it returns ICERR_BADFORMAT.

Return Value

ICDecompressQuery

See Also

ICDraw

LRESULT  ICDraw(hic, dwFlags, lpFormat, lpData, cbData, lTime)

Syntax

This function decompress an image for drawing.

HIC  hic

Parameters

Specifies a handle to a decompressor.

DWORD  dwFlags

Specifies any flags for the decompression. The following flags are defined:

ICDRAW_HURRYUP

Indicates the decompressor should try to increase its decompression rate.

ICDRAW_UPDATE

Tells the decompressor to update the screen based on data previously received. Set lpData to NULL when this flag is used.

LPVOID  lpFormat

Specifies a far pointer to a BITMAPINFOHEADER structure containing the input format of the data.

LPVOID  lpData

Specifies a far pointer to the actual input data.

DWORD  cbData

Specifies the size of the input data (in bytes).

LONG  lTime

Specifies the time to draw this frame based on the time scale sent with ICDrawBegin.

Returns ICERR_OK on success, otherwise an appropriate error number.

Return Value

This function is used to decompress the image data for drawing by the decompressor. Actual drawing of frames does not occur until ICDrawStart is called. The application should be sure to pre-buffer the required number of frames before drawing is started (you can obtain this value with ICGetBuffersRequired).

Comments

ICDrawBegin, ICDrawEnd, ICDrawStart, ICDrawStop, ICGetBuffersRequired

See Also

ICDrawBegin

LRESULT  ICDrawBegin(hic, dwFlags, hpal, hwnd, hdc, 
xDst, yDst, dxDst, dyDst, lpbi, xSrc, ySrc, dxSrc, dySrc, 
dwRate, dwScale)

Syntax

This function starts decompressing data directly to the screen.

HIC  hic

Parameters

Specifies a handle to the decompressor to use.

DWORD  dwFlags

Specifies flags for the decompression. The following flags are defined:

ICDRAW_QUERY

Determines if the decompressor can handle the decompression. The decompressor does not actually decompress the data.

ICDRAW_FULLSCREEN

Tells the decompressor to draw the decompressed data on the full screen.

ICDRAW_HDC

Indicates the decompressor should use the window handle specified by hwnd and the display context handle specified by hdc for drawing the decompressed data.

HPALETTE  hpal

Specifies a handle to the palette used for drawing.

HWND  hwnd

Specifies a handle for the window used for drawing.

HDC  hdc

Specifies the display context used for drawing.

int  xDst

Specifies the x-position of the upper-right corner of the destination rectangle.

int  yDst

Specifies the y-position of the upper-right corner of the  destination rectangle.

int  dxDst

Specifies the width of the destination rectangle.

int  dyDst

Specifies the height of the destination rectangle.

LPBITMAPINFOHEADER  lpbi

Specifies a far pointer to a BITMAPINFO structure containing the format of the input data to be decompressed.

int  xSrc

Specifies the x-position of the upper-right corner of the source rectangle.

int  ySrc

Specifies the y-position of the upper-right corner of the source rectangle.

int  dxSrc

Specifies the width of the source rectangle.

int  dySrc

Specifies the height of the source rectangle.

DWORD  dwRate

Specifies the data rate. The data rate in frames per second equals dwRate divided by dwScale.

DWORD  dwScale

Specifies the data rate.

Returns ICERR_OK if it can handle the decompression, otherwise it returns ICERR_UNSUPPORTED.

Return Value

Decompressors use the hwnd and hdc parameters only if an application sets ICDRAW_HDC flag in dwFlags. It will ignore these parameters if an application sets the ICDRAW_FULLSCREEN flag. When an application uses the ICDRAW_FULLSCREEN flag, it should set hwnd and hdc to NULL.

Comments

The destination rectangle is specified only if ICDRAW_HDC is used. If an application sets the ICDRAW_FULLSCREEN flag, the destination rectangle is ignored and its parameters can be set to zero.

The source rectangle is relative to the full video frame. The portion of the video frame specified by the source rectangle will be stretched to fit in the destination rectangle.

ICDraw, ICDrawEnd

See Also

ICDrawEnd

LRESULT  ICDrawEnd(hic)

Syntax

This function tells a decompressor to end drawing.

HIC  hic

Parameters

Specifies a handle to a compressor.

Returns ICERR_OK if successful, otherwise it returns an error number.

Return Value

ICDrawBegin, ICDraw, ICCompressEnd, ICDecompressEnd

See Also

ICDrawFlush

LRESULT  ICDrawFlush(hic)

Syntax

Flush the image buffers used for drawing.

HIC  hic

Parameters

The compressor handle.

Returns ICERR_OK on success.

Return Value

ICDraw, ICDrawBegin, ICDrawEnd, ICDrawStart, ICDrawStop

See Also

ICDrawGetTime

LRESULT  ICDrawGetTime(hic, lplTime)

Syntax

This function obtains the current value of the internal clock if the decompressor is handling the timing of drawing frames.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPLONG  lplTime

Specifies a far pointer to a LONG buffer used to return the current time value. The value will be in samples (frames for video).

Returns ICERR_OK if successful.

Return Value

ICDrawStart, ICDrawStop, ICDrawSetTime

See Also

ICDrawRealize

LRESULT  ICDrawRealize(hic, hdc, fBackground)

Syntax

This function tells a decompressor to realize its palette used while drawing.

HIC  hic

Parameters

Specifies a handle to a decompressor.

HDC  hdc

Specifies the display context used to realize the palette.

BOOL  fBackground

Specifies TRUE if the palette is to be realized in the background. It specifies FALSE if it is to be realized in in the foreground.

Returns ICERR_OK if palette is realized. The compressor returns ICERR_UNSUPPORTED if it doesn't support this function.

Return Value

ICDrawBegin

See Also

ICDrawSetTime

LRESULT  ICDrawSetTime(hic, lpTime)

Syntax

This function sets the value of the internal clock if the installable decompressor is handling the timing of drawing frames.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPLONG  lpTime

Specifies the current time that the compressor should be rendering. This value should be in samples. For video, this corresponds to frames.

Returns ICERR_OK if successful.

Return Value

ICDrawStart, ICDrawStop, ICDrawGetTime

See Also

ICDrawStart

void  ICDrawStart(hic)

Syntax

This function tells a decompressor to start its internal clock for the timing of drawing frames.

HIC  hic

Parameters

Specifies a handle to a compressor.

This function should only be used with hardware decompressors that do their own asynchronous decompression, timing and drawing.

Comments

ICDraw, ICDrawStop, ICDrawBegin, ICDrawEnd

See Also

ICDrawStop

void  ICDrawStop(hic)

Syntax

This function tells a decompressor to stop its internal clock used for the timing of drawing frames.

HIC  hic

Parameters

Specifies a handle to a decompressor.

This function should only be used with hardware decompressors that do their own asynchronous decompression, timing and drawing.

Comments

ICDraw, ICDrawStart, ICDrawBegin, ICDrawEnd

See Also

ICDrawWindow

LRESULT  ICDrawWindow(hic, prc)

Syntax

This function has a decompressor redraw the window when is has moved.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPRECT  prc

Specifies a pointer to the destination rectangle. The destination rectangle is specified in screen coordinates.

Returns ICERR_OK if successful.

Return Value

This function is only supported by hardware which does its own asynchronous decompression, timing and drawing. The rectangle is set to empty if the window is totally hidden by other windows.

Comments

ICGetBuffersWanted

LRESULT  ICGetBuffersWanted(hic, lpdwBuffers)

Syntax

This function obtains information about the pre-buffering needed by a decompressor.

HIC  hic

Parameters

Specifies a handle to a decompressor.

LPDWORD  lpdwBuffers

Specifies a far pointer to a DWORD used to return the number of samples the decompressor needs to get in advance of when they will be rendered.

Returns ICERR_OK if successful, otherwise it returns ICERR_UNSUPPORTED.

Return Value

This function is used only with a decompressor that uses hardware to render data and wants  to ensure that hardware pipelines remain full.

Comments

ICGetDefaultKeyFrameRate

LRESULT  ICGetDefaultKeyFrameRate(hic)

Syntax

This function obtains the default key frame rate value.

HIC  hic

Parameters

Specifies a handle to a compressor.

Returns the default key frame rate.

Return Value

ICGetDisplayFormat

HIC  ICGetDisplayFormat(hic, lpbiIn, lpbiOut, BitDepth, dx, dy)

Syntax

This function returns the "best" format available for display a compressed image. The function will also open a compressor if a handle to an open compressor is not specified.

HIC  hic

Parameters

Specifies the decompressor that should be used. If this is NULL, an appropriate compressor will be opened and returned.

LPBITMAPINFOHEADER  lpbiIn

Specifies a pointer to BITMAPINFOHEADER structure containing the compressed format.

LPBITMAPINFOHEADER  lpbiOut

Specifies a pointer to a buffer used to return the decompressed format. The buffer should be large enough for a BITMAPINFOHEADER and 256 color entries.

int  BitDepth

If non-zero, specifies the preferred bit depth.

int  dx

If non-zero, specifies the width to which the image is to be stretched.

int  dy

If non-zero, specifies the height to which the image is to be stretched.

Returns a handle to a decompressor if successful, otherwise, it returns zero.

Return Value

ICGetInfo

LRESULT  ICGetInfo(hic, lpicinfo, cb)

Syntax

This function obtains information about a compressor or decompressor.

HIC  hic

Parameters

Specifies a handle to a compressor or decompressor.

ICINFO FAR *  lpicinfo

Specifies a far pointer to ICINFO structure used to return information about the compressor or decompressor.

DWORD  cb

Specifies the size of the structure pointed to by lpicinfo.

Returns zero if successful.

Return Value

ICGetState

void  ICGetState(hic, pv, cb)

Syntax

This function gets the state of a compressor or decompressor.

HIC  hic

Parameters

Specifies a handle to the compressor or decompressor.

LPVOID  pv

Specifies a pointer to a buffer used to return the state data.

DWORD  cb

Specifies the byte count for state buffer.

Use ICGetStateSize before calling ICGetState to determine the size of buffer to allocate for the call.

Comments

ICGetStateSize, ICSetState

See Also

ICGetStateSize

LRESULT  ICGetStateSize(hic)

Syntax

This function gets the size of the state data used by a compressor or decompressor.

HIC  hic

Parameters

Specifies a handle to the compressor or decompressor.

Returns the number of byte used by the state data.

Return Value

Use this function to get the size of the state data for the ICGetState and ICSetState buffers.

Comments

ICGetState, ICSetState

See Also

ICInfo

BOOL  ICInfo(fccType, fccHandler, lpicinfo)

Syntax

This function returns information about specific installed compressors and decompressors, or it enumerates the compressors installed.

DWORD  fccType

Parameters

Specifies a four-character code indicating the type of compressor or decompressor.

DWORD  fccHandler

Specifies a four-character code identifying a specific compressor or decompressor, or a number between 0 and the number of installed compressors of the type specified by fccType.

ICINFO FAR *  lpicinfo

Specifies a far pointer to a ICINFO structure used to return information about the compressor.

Returns a compressor or decompressor handle if successful, otherwise, it returns zero.

Return Value

ICInstall

BOOL  ICInstall(fccType, fccHandler, lParam, szDesc, wFlags)

Syntax

This function installs a new compressor.

DWORD  fccType

Parameters

Specifies a four-character code indicating the type of data used by the compressor. Use 'vidc' for video compressors.

DWORD  fccHandler

Specifies a four-character code identifying a specific compressor.

LPARAM  lParam

Identifies what to install. The meaning of this parameter is defined by the flags set for wFlags.

LPSTR  szDesc

Specifies a pointer to a null-terminated string describing the installed compressor.

UINT  wFlags

Specifies flags defining the contents of lParam. The following flags are defined:

ICINSTALL_DRIVER

Indicates lParam is a pointer to a null-terminated string containing the name of the compressor to install.

ICINSTALL_FUNCTION

Indicates lParam is a far pointer to an installable compressor function. This function should be structured like the DriverProc entry point function used by compressors and decompressors.

Returns a handle to a compressor or decompressor.

Return Value

ICRemove

See Also

ICLocate

HIC  ICLocate(fccType, fccHandler, lpbiIn, lpbiOut, wFlags)

Syntax

This function finds a compressor or decompressor that can handle images with the formats specified, or it finds a decompressor that can decompress an image with a specified format directly to hardware.

DWORD  fccType

Parameters

Specifies the type of compressor or decompressor the application wants to open. For video, this is ICTYPE_VIDEO.

DWORD  fccHandler

Specifies a single preferred handler of the given type that should be tried first. Typically, this comes from the stream header in an AVI file.

LPBITMAPINFOHEADER  lpbiIn

Specifies a pointer to BITMAPINFOHEADER structure defining the input format. A compressor handle will not be returned unless it can handle this format.

LPBITMAPINFOHEADER  lpbiOut

Specifies zero or a pointer to BITMAPINFOHEADER structure defining an optional decompressed format. If lpbiOut is nonzero, a compressor handle will not be returned unless it can create this output format.

WORD  wFlags

Specifies any flags defining the use of the compressor or decompressor. This parameter must contain one of the following values:

ICMODE_COMPRESS

Indicates the compressor should be able to compress an image with a format defined by lpbiIn to the format defined by lpbiOut.

ICMODE_DECOMPRESS

Indicates the decompressor should be able to decompress an image with a format defined by lpbiIn to the format defined by lpbiOut.

ICMODE_DRAW

Indicates the decompressor should be able to decompress an image with a format defined by lpbiIn and draw it directly to hardware.

Returns a handle to a compressor or decompressor if successful, otherwise it returns zero.

Return Value

ICOpen

HIC  ICOpen(fccType, fccHandler,  wMode)

Syntax

This function opens a compressor or decompressor.

DWORD  fccType

Parameters

Specifies the type of compressor or decompressor the application wants to open. For video, this is ICTYPE_VIDEO.

DWORD  fccHandler

Specifies a single preferred handler of the given type that should be tried first. Typically, this comes from the stream header in an AVI file.

UINT  wMode

Specifies any flags defining the use of the compressor or decompressor. This parameter can contain the following values:

ICMODE_COMPRESS

Advises a compressor it is opened for compression.

ICMODE_DECOMPRESS

Advises a decompressor it is opened for decompression.

ICMODE_DRAW

Advises a decompressor it is opened to decompress an image and draw it directly to hardware.

ICMODE_QUERY

Advises a compressor or decompressor it is opened to obtain information.

Returns a handle to a compressor or decompressor if successful, otherwise it returns zero.

Return Value

ICClose ICLocate

See Also

ICQueryAbout

BOOL  ICQueryAbout(hic)

Syntax

This function determines if a compressor or decompressor supports an about dialog box.

HIC  hic

Parameters

Specifies the handle to the installable compressor.

Returns TRUE if the installable compressor supports an about dialog box, otherwise it returns FALSE.

Return Value

ICAbout

See Also

ICQueryConfigure

BOOL  ICQueryConfigure(hic)

Syntax

This function determines if a compressor or decompressor supports a configuration dialog box.

HIC  hic

Parameters

Specifies a handle to the compressor or decompressor.

Returns TRUE if compressor supports a configuration dialog box, otherwise it returns FALSE.

Return Value

ICConfigure

See Also

ICRemove

BOOL  ICRemove(fccType, fccHandler, wFlags)

Syntax

This function removes a compressor function installed with ICInstall.

DWORD  fccType

Parameters

Specifies a four-character code indicating the type of data used by the compressor. Use 'vidc' for video compressors.

DWORD  fccHandler

Specifies a four-character code identifying a specific compressor.

UINT  wFlags

Not used.

Returns TRUE if successful.

Return Value

ICInstall

See Also

ICSendMessage

LRESULT  ICSendMessage(hic, wMsg, dw1, dw2)

Syntax

This function sends a message to a compressor or decompressor.

HIC  hic

Parameters

Specifies the handle of the compressor or decompressor to receive the message.

UINT  wMsg

Specifies the message to send.

DWORD  dw1

Specifies additional message-specific information.

DWORD  dw2

Specifies additional message-specific information.

Returns a message-specific result.

Return Value

ICSetState

void  ICSetState(hic, pv, cb)

Syntax

This function sets the state of a compressor or decompressor.

HIC  hic

Parameters

Specifies a handle to the compressor or decompressor.

LPVOID  pv

Specifies a pointer to the state data to set.

DWORD  cb

Specifies the size (in bytes) of the buffer containing the state data.

ICGetState, ICGetStateSize

See Also

# Video Compressor and Decompressor Data Structure Reference

This section lists data structures used by video compressors and decompressors. The data structures are presented in alphabetical order. The structure definition is given, followed by a description of each field.

ICINFO

The ICINFO structure is filled by a video compressor when it receives the ICM_GETINFO message.

typedef struct {

DWORD  dwSize;

DWORD  fccType;

DWORD  fccHandler;

DWORD  dwFlags;

DWORD  dwVersion;

DWORD  dwVersionICM;

char  szName[16];

char  szDescription[128];

char  szDriver[128];

} ICINFO;

The ICINFO structure has the following fields:

Fields

dwSize

Should be set to the size of an ICINFO structure.

fccType

Specifies a four-character code representing the type of stream being compressed or decompressed. Set this to 'vidc' for video streams.

fccHandler

Specifies a four-character code identifying a specific compressor.

dwFlags

Specifies any flags. The following flags are defined for video compressors (ICINFO.fccHandler == 'vidc'):

VIDCF_QUALITY

The compressor supports quality.

VIDCF_CRUNCH

The compressor supports crunching to a frame size.

VIDCF_TEMPORAL

The compressor supports inter-frame compression.

VIDCF_DRAW

The compressor supports drawing.

VIDCF_FASTTEMPORAL

The compressor can do temporal compression and doesn’t need the previous frame.

dwVersion

Specifies the version number of the compressor.

dwVersionICM

Specifies the version of the ICM supported by this compressor; it should be set to 1.0 (0x00010000)

szName[16]

Specifies the short name for the compressor. The null-terminated name should be suitable for use in list boxes.

szDescription[128]

Specifies a null-terminated string containing the long name for the compressor.

szDriver[128]

Specifies a null-terminated string for the module that contains the compressor.