Serial Fix 1992

by Medical Designs Software

Version 1.00

Written by: Bill Penner

Icons created by: Bruce D. Noonan, M.D.

This  program  may  be  distributed and used, free of charge,

provided  that  the  program  is  distributed in complete form and

UNmodified.   This  software  is  not  to be sold, however, may be

included  on  disks  which  are  sold  for  a small handling fee.

Medical  Design  Software  retains  all copyright exclusive rights

except  exclusive  distribution.   This  software  consists of the

following  files:  SERIALFX.PRG,  SERIALFX.CPX,  and  SERIALFX.DOC

(this file).

PURPOSE

Serial  Fix  was  written  as  an upgrade to the old TurboCTS

program,  written  by  Bill Penner, long ago, as an attempt to fix

the  old  RTS/CTS  problem.  The problem the ST/STe/TT has, occurs

when  a high speed modem is used in conjunction with ZMODEM or any

near-continuous  flow  transfer program is used to send data.  The

ST/TT  sometimes  has problems properly recognizing when the modem

deasserts CTS (telling the ST/TT that the buffer is full and don't

send  any  more data), resulting in the ST overflowing the modem's

buffer.   This results in bad data being sent which results in CRC

errors, packet size problems, and other data transmission errors.

TurboCTS was only a partial fix of the problem.  TurboCTS did

manage  to  speed  things  up  to the point where CTS response was

generally  ok,  however,  that program did not work on the TT, did

not  support  XBRA  programming  requirements, and only worked for

RTS/CTS handshaking.

Serial  FIX  is a replacement for TurboCTS and will work with

all  versions of TOS (1.0 through 3.06).  Serial FIX supports FOUR

forms of handshaking:  (1) XON/XOFF, (2) RTS/CTS, (3) BOTH RTS/CTS

AND  XON/XOFF,  and (4) NONE.  The desired handshaking mode is set

from  the RS232 control panel or other program as you currently do

now.   The new fix is a REAL fix to the RTS/CTS problem.  Previous

fixes  (like  TurboCTS)  only  fixed the symptoms of the problem.

NOTE:   Although  documented  as legal in official Atari bindings,

TOS  normally  will  NOT allow both XON/XOFF and RTS/CTS to be set

simultaneously.  Serial FIX DOES support this option, however.

INSTALLATION:

Serial  FIX  is composed of two parts, a program for the AUTO

folder  and  a  CPX  file  to  be used with the extensible CONTROL

(XCONTROL.ACC) panel.

SERIALFX.PRG  is  the  heart  of  the  Serial  FIX solution.

SERIALFX.PRG  is  intended  to  be  installed in your AUTO folder,

although  it can be ran from the desktop, and replaces the current

serial  routines  for  the  ST compatible port.  The ST compatible

port  is  the only port on the ST and STe machines, or the Modem 1

port (ONLY!) on the MegaSTe and TT machines.

SERIALFX.CPX   is  a  control  panel  to  be  used  with  the

Extensible  Control Panel (XCONTROL.ACC).  The CPX is used to lock

the  serial  routines  into  using  at  least RTS/CTS handshaking,

regardless  of the desires of a program to the contrary.  The file

SERIALFX.CPX is to be placed in in your folder of CPX files (often

called 'CPX').

OPERATION:

SERIALFX.PRG:   Once  the  program  has  been started (either

booted  in  the 'AUTO' folder or run from the desktop), no special

operations  are  needed.  The program replaces the system routines

and  the  port speed and handshaking are set just as they are now,

either  by  terminal  software,  CONTROL  panel, or the Extensible

Control Panel.

SERIALFX.CPX:   This  program  will  allow  you  to  lock the

handshaking  in  RTS/CTS  mode.   This  is  often needed because a

number  of programs have been written to ignore RTS/CTS mode (like

FLASH).   If  the CPX is not used to lock the mode on, and RTS/CTS

mode  is  SET  from  the  control panel, AND a terminal program is

started,  very  likely,  the program would turn off RTS/CTS.  With

the  mode  locked, the program will be unable to disable RTS/CTS.

The CPX will give you the option to save the current lock setting.

Saving  the  setting  will  cause  the selected mode to be entered

every  time  the  CPX  boots  up.   NOTE:   to be able to lock the

RTS/CTS  mode  on,  you  must  use  BOTH  THE SERIALFX.PRG AND THE

SERIALFX.CPX.

PROGRAMMING INFORMATION:

This  version  of  the  software  should  now  be  fully XBRA

compliant.   When the software starts and installs the replacement

routines,  the TOS version is checked to determine the best method

of installation (read for maximum speed).  Under TOS 1.0, traps 1,

13, and 14 are intercepted and processed.  Under TOS 1.2, 1.4, and

1.6 (non-Bconmap() compatible machines after 1.2), the xcon vector

table is used, and only trap 14 is used to receive Rsconf() calls.

Under  Bconmap()  compatiable  systems,  bconmap (port 6) and xcon

vectors are modified so that no trap interception is necessary.

Other  software  can  be  used  to  lock or unlock the serial

routines.   To perform this function, look in the cookie jar for a

'MDSa'  cookie.   The long data byte is a pointer to the following

data structure:

struct {

int   lockfg;         /*  RTS/CTS locked flag (bit 1 =

locked) */

int   (*setlock)()    /*  Routine  to  set lock status

*/

} serfx;

The  lock  status  is  set by pushing an integer with the value of

either  0  (to disable lock) or 1 (to enable) and then jsr'ing the

setlock  routine  (i.e. (*serfx->setlock)(1) to enable the lock).

The  call can be made in user or super mode.  That is all there is

to it.

WRAP UP

As  normal,  PLEASE let me know if you have any problems with

the  program.   I have attempted to make the program as compatible

with other software as possible.

Bill Penner, IAAD Member

3235 Wright Avenue

Bremerton, WA  98310-4753

Phone: (206) 373-4840 (before 9pm Pacific)

GEnie: BPENNER